IEEE P802.11
Wireless LANs

IEEE 802.11 TGn Comparison Criteria

Date:	December 03, 2003

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines comparison criteria that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document.)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P Stephens | Intel | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Pieter-Paul Giesberts | Agere |  |  |  |  |
| Bruno Jechoux | Mitsubishi | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | jechoux@tcl.ite.mee.com |
| Jeff Gilbert | Atheros |  |  |  |  |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Empty skeleton submitted to satisfy the automatic document system | 20 October 2003 |
| R1 | APS initial contribution | 31 October 2003 |
| R2 | Updated following FRCC telecon on 4 Nov 2003 to add proposed comparison criteria evaluating degree of support for HT usage models | 10 November 2003 |
| R3 | Updates made during 13 November 2003 TGn session while editing the FR document. Added revision history. Updated with changes made at the FRCC telecon on 18 November 2003. | 19 November 2003 |
| R4 | Merged in comments from: - Pieter-Paul Giesberts (Agere) - Bruno Jechoux (Mitsubishi) Reorganised existing CC into sections according to Pieter-Paul’s contribution. + APS comments on the above | 27 November 2003 28 November 2003 |
| R5 | Updated at 2 December 2003 FRCC telecon. Includes changes agreed at the telecon. Includes proposed definition of maturity by Jeff Gilbert. | 2 December 2003 |

# Introduction

## Purpose of document

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in [2] and to disclose results according to the comparison criteria defined in this document.

## Form of Disclosure

A proposal shall disclose its results using the template or form defined below in section 5..

## Relationship to Functional Requirements

The main purpose of the comparison criteria is to define metrics to enable comparison of TGn roposals.

In addition, the functional requirements [2] may define that specific criteria meet specific values.   This document defines how those measurements are to be made and reported so that compliance to the functional requirements can be evaluated.

As such, the functional requirements [2] are dependent on this document, but not the other way around.

## Relationship to Simulation Scenarios

The IEEE 802.11 TGn FRCC (Functional Requirements and Comparison Criteria Special Committee) has defined usage models from which simulation scenarios have been created [3].

These simulation scenarios are intended to define the input to a simulation in sufficient detail so that the simulation results from different proposals can be meaningfully compared.

This document may define certain criteria given the conditions defined in a certain simulation scenario.  As such certain parts of this  document are dependent on the simulation scenarios contained in [3], but not the other way around.

## Requirements of the Comparison Criteria Document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The criteria defined here:

- Shall be defined unambiguously

- Can be obtained from a reasonable simulation environment, or obtained by examination of the proposed submission

- Are compliant to the 802.11 HT PAR [5] and 5C [6]

Ideally, most criteria should be single values.

In some cases (e.g. transport delay), a metric might need to be presented as a graph or table.  The definition of the metric shall include the exact form in which it is to be presented.

## Summary Status of this Document

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet discussed by FRCC | 5 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC |  |
| Number of Comparison Criteria approved by TGn |  |
| Total number of Comparison Criteria | 44 |

# Definitions

| Term | Definition |
| --- | --- |
| Goodput | Goodput is defined by totaling the number of bits in MSDUs successfully received and dividing by the simulation duration (s). |

# Additional Disclosures

(This section contains requirements for additional information that shall be disclosed with a proposal)

| Number | Name | Definition |  | Status of this AD |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reference submissions | A list of related IEEE submissions, both documents and presentations. | PPG (Agere) proposal Agreed 6/0. |  |  |  |

# Comparison Criteria

## General

| Number | Name |  |  | Definition | Simulation Scenario |  |  | Status of this CC | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Regulatory compliance | Proposal shall state the regulatory domains it is intended to be compliant with. The proposal shall state its intended regulatory compliance with at least the following domains: USA, Japan, EU, China. |  |  |  | Suggestion made at TGn session. Vote 14/4. |  |  | Definition of EU is needed. |
|  |  | Regulatory non-compliance | Proposal shall identify any known problems with existing regulations The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, EU, China. |  |  |  | Suggestion made at TGn session, split out by Joe Levy from previous. Vote 7/0 |  |  |  |

## Marketability

| ###### Number | ###### Name |  |  | ###### Definition | Simulation Scenario |  |  | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | PPG (Agere) proposal Removed unanimously, as covered by other CCs. |  |
|  | Maturity of solution and technology |  |  | Give an indication of the maturity of the solution and its technology. For example, reference to similar technology in successful products. Reference articles in the literature. Alternative proposal by Jeff Gilbert: Give an indication of the maturity of the solution and its technology by listing relevant publications, presentations and implementations within and outside of the IEEE. The burden will be on the proposer to list sufficient relevant publications and implementations to convince the body of the maturity of the key technology of the proposal. |  |  |  | PPG (Agere) proposal No consensus 9/7 PPG to produce better language. Augmented by Jeff Gilbert (Atheros) | Maturity needs precise definition |
|  | Power consumption estimate |  |  | Give an estimate of the power consumption in TX, RX (decoding packet), IDLE (listening but no packet). Specify model and assumptions. To specify for a mode of operation that achieves 100Mbps goodput for the point-point use case. Tx consumption measured at total tx power = 20dBm. |  |  |  | PPG (Agere) proposal 7/0 |  |
|  |  | Power consumption | Total power consumed (mW) by a realistic implementation (define implementation technologies). TBD - Note, For consistency, this document will need to define which parts of an implementation are included in the power consumption measurements. Show results separately for: - Signal Acquisition - Receiving - Transmitting (at Tx power = 20dBm) Show results for highest achievable PHY rate, 54Mbps (or closest rate) and 6 Mbps (or closest rate). |  |  | If relevant, choose one channel model from [4]. | APS proposal |  |  |

## MAC Related

| Number | Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | Required changes to 802.11 MAC | A list of required changes to the 802.11 MAC. |  | PPG (Agere) proposal | "Change" needs precise definition. How is this not satisfied by their technical specification? |

## PHY Related

| Number | Name | Definition |  | Simulation Scenario |  | Status of this CC |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required changes to 802.11 PHY | A list of required changes to the 802.11 PHY. |  |  |  | APS proposal based on list of required changes to 802.11 MAC, for completeness. |  | "Change" needs precise definition. How is this not satisfied by their technical specification? |
|  | Data rates | A list of PHY layer data rates, with per data rate the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. |  |  | PPG (Agere) proposal Accepted unanimously. |  |  |  |

## Interoperability and Coexistence

|  | Name |  |  | Definition | Simulation Scenario |  |  | Status of this CC | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Backward compatibility and interoperability with 802.11a |  |  | Identify the means of achieving backward compatibility and interoperability with 802.11a. Also indicate for which of the modes and data rates that this is applicable. |  |  |  | PPG (Agere) proposal | Should reference technical specification and perhaps summarise? |  |
|  | Backward compatibility and interoperability with 802.11g |  |  | Identify the means of achieving backward compatibility and interoperability with 802.11g. Also indicate for which of the modes and data rates that this is applicable. |  |  |  | PPG (Agere) proposal | Should reference technical specification and perhaps summarise? |  |
|  | Impact on options in previous 802.11 standards |  |  | List the impact on the options of other 802.11 standards, such as .11a/g/b or .11e. |  |  |  | PPG (Agere) proposal | Impact needs definition. Such as needs definition. |  |
|  |  | Legacy Share | Two measures are made. Firstly the throughput (T1) of a fully backlogged legacy STA transmitting to its AP. Secondly the throughput (T2) of the legacy STA when a fully-backlogged co-channel co-incident HT STA and its AP are introduced. The legacy share is defined as (T2 / T1). |  |  | TBD geometry. TBD channel model. 1500B MSDU size. | APS proposal |  |  |  |

## Channelization

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | Spectral characteristics | a. Define the transmission spectral mask b. Specify the channelization |  | PPG (Agere) proposal |  |
|  | Co-channel interference rejection | Give the number for the system capacity reduction factor e according to Appendix 1 in 802.11-03/802r7. |  | PPG (Agere) proposal |  |
|  | Adjacent channel interference rejection | Specify the required adjacent channel rejection and the impact of adjacent channel interference |  | PPG (Agere) proposal | "Impact" needs definition, and under what circumstances. |

## RF Characteristics

| ###### Number | ###### Name |  |  | Definition | Simulation Scenario |  |  | Status of this CC | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required carrier frequency accuracy |  |  | Indicate the required carrier frequency accuracy (in PPM). |  |  |  | PPG (Agere) proposal |  |  |
|  | RF PA backoff |  |  | Identify the RF PA backoff from 1dB compression point according to the RAPP model. |  |  |  | PPG (Agere) proposal | Simulation should be run at oversample rate of 4x. Use RAPP power amplifier model as specified in document 00/294. Use P-parameters of 2 and 3. Specify backoff from full saturation used in the simulation calculated as PABackoff = –10 log10(Average TX Power/Power at saturation) |  |
|  |  | HT Spectral Efficiency | The number of bps/Hz when demonstrating a throughput value of 100Mbps |  |  | Report results for all the channel models described in [4]. | APS proposal |  |  |  |

## Complexity

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | RF/IF complexity | Give an indication of the complexity of the RF/IF part of the solution, relative to current 802.11a PHYs. |  | PPG (Agere) proposal | Analogue vs RF/IF (i.e. what if IF is digital)? |
|  | Baseband processing complexity | Give an indication of the baseband processing complexity, relative to current 802.11a PHYs (gate counts, MIPS, etc.). |  | PPG (Agere) proposal |  |

## Coverage of Usage Models

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | HT Usage Models Supported (non QoS) | This metric relates to the ability of the proposal to support the non-QoS application service level of each usage model, as defined by its simulation scenario. TBD: consider counting the number of TCP flows that meet 50 % of their offered load divided by the total number of such flows. Note: Would prefer a distribution of this ratio Aggregate TCP traffic goodput at each STA. | All mandatory simulation scenarios | APS TBD proposal resulting from FRCC telecon on 4 Nov 2003. |  |
|  | HT Usage Models Supported (QoS) | This metric relates to the ability of the proposal to support the QoS application service level of each usage model, as defined by its simulation scenario. For each flow (i.e. application at a station) with a delay and PLR specification, the proposal shall report the packet loss rate (defined below) and compare to the maximum specified packet loss rate. The proposal shall also report the number of these flows that are less than or equal to the PLR specified in [2] divided by the total number of such flows. For the purpose of this criterion, packet loss rate (PLR) is defined by the number of MSDUs that are lost in transit, or whose MAC SAP to MAC SAP transit time exceeds the maximum delay for the application specified in [2] divided by the total number of MSDUs transmitted during the simulation for that application at that STA. | All mandatory simulation scenarios | APS proposal resulting from FRCC telecon on 4 Nov 2003. |  |

## Performance Measurements at the MAC SAP

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | BSS Aggregate Goodput at the MAC data SAP | This metric is defined as the total number of bits in all MSDUs transmitted successfully summed across all STA in a BSS throughout a simulation run divided by the simulated duration (s). Note, MSDUs not transmitted successfully (e.g. due to retry limits or lifetime limits being exceeded) do not count. Note, bits are counted at the MAC data SAP and are not sensitive to the number of transmission attempts. | All mandatory simulation scenarios | APS proposal Started discussion 18 Nov 2003 |  |
|  | MAC Efficiency | MAC efficiency is defined as the the aggregate goodput at the MAC-SAP divided by the average physical layer data rate. | All mandatory simulation scenarios | BJC, HBE proposal |  |
|  | MSDU Delay | This metric gives the MSDU delay distribution for each flow type (i.e. application aggregated on all the STA) Curves shall be represented as 1-cdf(Delay) | All mandatory simulation scenarios | BJC, HBE proposal |  |
|  | Throughput / Range | Presented as a curve of Throughput (bps) vs range (m). Throughput is defined by totaling the number of bits in MSDUs successfully transmitted and dividing by the simulation duration (s). | Present results for two isolated STA (one may be an AP), one is fully backlogged with 1500B MSDUs addressed to the other. Vary the distance between the two STA. Report results for all the channel models described in [4]. TBD scenario from [3]. | APS proposal | Need a link test simulation scenario to go in [3]. |
|  | Throughput / Range in 20MHz | TBD |  |  |  |
|  | HT Range | The range value where the Throughput / Range curve passes through 100Mbps. | Report results for all the channel models described in [4]. | APS proposal |  |
|  | 10Mbps range | The range value where the Throughput / Range curve passes through 10Mbps. | Report results for all the channel models described in [4]. | WFA proposal (moved from FR) |  |
|  | Uniform BSS operation with HT rate supported | Continuous coverage of HT range and 10Mbps range requirements should be at least 95% (i.e., maximum 1 out of 20 stations within range specified may have to accept lower throughput, also providing for mobile stations and movement in the environment) | TBD | WFA proposal (moved from FR) | Needs implementable definition. |

### Throughput and Overhead

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | Preambles | What are the possible preambles? |  | PPG (Agere) proposal | Why is this relevant? |
|  | Maximum data throughput | Maximum data throughput at all combinations of: a. Packet sizes of 100B, 1000, and 10000B b. All proposed preamble lengths. |  | PPG (Agere) proposal | Incomplete, other conditions need to be specified. |

## PHY Performance

### Additive White Gaussian Noise Interference

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | AWGN PER performance | Identify performance in an AWGN channel, and for packet lengths of 100B, 1000B, and 10000B. |  | PPG (Agere) proposal | Why is AWGN relevant? |

### Non-Ideal Power Amplifier Effects

| ###### Number | ###### Name | Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | PER performance versus AWGN with packet lengths of 1000B with non-ideal power amplifier. | Simulation results of PER performance for 1000B packets, with non-ideal power amplifier. |  | PPG (Agere) proposal | Simulation should be run at oversample rate of 4x. Use RAPP power amplifier model as specified in document 00/294. Use P-parameters of 2 and 3. Specify backoff from full saturation used in the simulation calculated as PABackoff = –10 log10(Average TX Power/Power at saturation) APS Comment: why call out this one impairment specially? |
|  | Spectral characteristics with non-ideal power amplifier | Using the RAPP power amplifier model in doc. 00/294, show change in spectral characteristics due to non-ideal power amplifier as input power is swept over a reasonable range. |  | PPG (Agere) proposal |  |
|  | Pulse shaping filter | Describe the pulse shaping filter used at the input to the power amplifier in items above. Show the resulting power spectrum at the input to the PA. |  | PPG (Agere) proposal |  |

### Non-AWGN Distortions

| ###### Number | ###### Name | ###### Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | PER performance in non AWGN channels | PER versus Eb/No and Es/No (where Es is measured at the output of the transmitter) with 1000B packets simulated down to a PER of 0.01 or further. |  | PPG (Agere) proposal | Create waterfall curves for all channel models defined in 03/940r1. APS comment: type of averaging needs to be defined carefully. |

### Non-ideal Receiver Effects

| ###### Number | ###### Name | ###### Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | Carrier frequency offset | Identify the carrier frequency offset and degradation at worst case carrier frequency offset. |  | PPG (Agere) proposal |  |
|  | Baseband timing offset accuracy | Identify the baseband timing offset accuracy and degradation at worst case baseband timing offset. |  | PPG (Agere) proposal |  |
|  | Phase noise sensitivity | Simulate sensitivity to phase noise using model in document 11-??-296r1. Use 3dB bandwidth of 20kHz. Sweep the RMS phase noise in degrees over a reasonable range. Show influence of carrier degradation in AWGN. Provide all assumptions, e.g. whether or not tracking loop is enabled or not. (Also reference doc. 98-156r3.) |  | PPG (Agere) proposal | APS comment: is AWGN realistic? |
|  | DC offset sensitivity | Identify the DC offset sensitivity. |  | PPG (Agere) proposal |  |

## Diversity

| ###### Number | ###### Name | ###### Definition | Simulation Scenario | Status of this CC | Notes |
| --- | --- | --- | --- | --- | --- |
|  | Receiver training | Specify how the preamble allows for training of the receiver. |  | PPG (Agere) proposal | APS comment: Isn't this part of the technical specification? |
|  | Receiver and antenna diversity | Specify how the design supports receiver and antenna diversity. |  | PPG (Agere) proposal | APS comment: Isn't this part of the technical specification? |

# Template for Comparison Criteria Submissions

The results for a complete submission shall include a the values of comparison criteria defined in section 2 using the format defined in this section.

This is TBD.

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/813, TGn Functional Requirements

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)