IEEE P802.11
Wireless LANs

IEEE 802.11 TGn Comparison Criteria

Date:	14 January 2004

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines comparison criteria that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document. please let Adrian know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P Stephens | Intel | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Pieter-Paul Giesberts | Agere |  |  |  |  |
| Bruno Jechoux | Mitsubishi | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | jechoux@tcl.ite.mee.com |
| Jeff Gilbert | Atheros | Atheros Communications 529 Almanor Ave | + 1 408-773-5261 |  | gilbertj@atheros.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2°, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Herve Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | bonneville@tcftcl.ite.mee.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Frans Hermodsson | TeliaSonera |  | +46 40 10 51 97 | +46 40 30 70 29 | Frans.Hermodsson@teliasonera.com |
| David Bagby | Calypso Consulting |  |  |  |  |
| Sean Coffey | Texas Instruments |  |  |  |  |
| Günter Kleindl | SIEMENS AG Austria |  | +43 51707 35738 |  | guenter.kleindl@siemens.com |
| Jacob Sharony | Symbol |  |  |  | Jacob Sharony [sharony@symbol.com] |
| John Ketchum | Qualcomm |  | +1 781 276 0915 |  | jwk@qualcomm.com |
| Sanjiv Nanda | Qualcomm | 5775 Morehouse Dr, San Diego CA 92121 USA | +1 858 845 2375 |  | snanda@qualcomm.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Empty skeleton submitted to satisfy the automatic document system | 20 October 2003 |
| R1 | APS initial contribution | 31 October 2003 |
| R2 | Updated following FRCC telecon on 4 Nov 2003 to add proposed comparison criteria evaluating degree of support for HT usage models | 10 November 2003 |
| R3 | Updates made during 13 November 2003 TGn session while editing the FR document. Added revision history. Updated with changes made at the FRCC telecon on 18 November 2003. | 19 November 2003 |
| R4 | Merged in comments from: - Pieter-Paul Giesberts (Agere) - Bruno Jechoux (Mitsubishi) Reorganised existing CC into sections according to Pieter-Paul’s contribution. + APS comments on the above | 27 November 2003 28 November 2003 |
| R5 | Updated at 2 December 2003 FRCC telecon. Includes changes agreed at the telecon. Includes proposed definition of maturity by Jeff Gilbert. | 2 December 2003 |
| R6 | Merged in multiple comments and changes - Bjorn Bjerke - Herve Bonneville - Rahul Malik - Frans Hermodsson - David Bagby - Sean Coffey - Jeff Gilbert - Guenter Kleindl Added in "priority" column and give initial priority assignment. | 15 December 2003 |
| R7 | Updated during the Dec 16, 2003 telecon of the FRCC. | 16 December 2003 |
| R8 | Contribution from Jacob Sharony | 12 January 2004 |
| R9 | Change marks removed prior to split-out sessions to enable tracking of changes made in those sessions. | 13 January 2004 |
| R10 | Changes from Interop group session merged in Changes from Mktg group session merged in | 13 January 2004 |
| R11 | Changes from MAC group merged in Changes from PHY group in doc 11-04-0053r5 merged in | 14 January 2004 |
| R12 | Updated during TGn FRCC session | 14 January 2004 |

# Introduction

## Purpose of document

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in [2] and to disclose results according to the comparison criteria defined in this document.

## Form of Disclosure

A proposal shall disclose its results using the template or form defined below in section 6..

## Relationship to Functional Requirements

The main purpose of the comparison criteria is to define metrics to enable comparison of TGn roposals.

In addition, the functional requirements [2] may define that specific criteria meet specific values.   This document defines how those measurements are to be made and reported so that compliance to the functional requirements can be evaluated.

As such, the functional requirements [2] are dependent on this document, but not the other way around.

## Relationship to Simulation Scenarios

The IEEE 802.11 TGn FRCC (Functional Requirements and Comparison Criteria Special Committee) has defined usage models from which simulation scenarios have been created [3].

These simulation scenarios are intended to define the input to a simulation in sufficient detail so that the simulation results from different proposals can be meaningfully compared.

This document may define certain criteria given the conditions defined in a certain simulation scenario.  As such certain parts of this  document are dependent on the simulation scenarios contained in [3], but not the other way around.

## Requirements of the Comparison Criteria Document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The criteria defined here:

- Shall be defined unambiguously

- Can be obtained from a reasonable simulation environment, or obtained by examination of the proposed submission

- Are compliant to the 802.11 HT PAR [5] and 5C [6]

Ideally, most criteria should be single values.

In some cases (e.g. transport delay), a metric might need to be presented as a graph or table.  The definition of the metric shall include the exact form in which it is to be presented.

## Summary Status of  11-03-813r5

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet discussed by FRCC | 39 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC |  |
| Number of Comparison Criteria approved by TGn |  |
| Total number of Comparison Criteria | 44 |

## Summary Status of  11-03-813r6

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet discussed by FRCC | 75 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC | 5 |
| Number of Comparison Criteria approved by TGn |  |
| Total number of Comparison Criteria | 80 |

## Summary Status of  11-03-813r7

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet approved by FRCC | 74 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC | 6 |
| Number of Comparison Criteria approved by TGn | 0 |
| Total number of Comparison Criteria | 80 |

# Definitions

| Term | Definition |
| --- | --- |
| Goodput | Goodput is defined by totaling the number of bits in MSDUs indicated at the MAC DATA SAP, and dividing by the simulation duration (s). |
| Backward Compatible | Supports all the mandatory modes of the subject standard in the band or bands covered by the proposal. |
| Interoperable | Supports some or all mandatory or optional modes of the subject standard. |

# Additional Disclosures

(This section contains requirements for additional information that shall be disclosed with a proposal)

| Number | Name | Definition | Status of this AD |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. |  | PPG (Agere) proposal Agreed 6/0. |  |  |

# Comparison Criteria

| Number |  |  | Name | Definition | Simulation Scenario | Status of this CC | Notes |  | Pri |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## General |  |  |  |  |  |  |  |  |  |
| CC2 |  |  | Regulatory compliance | The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, Europe, China. |  | Mktg group consensus | Compliance according to regulatory body within each domain, FCC, CEPT, MPHTP, etc. |  | 6 (low) |
| ## Marketability |  |  |  |  |  |  |  |  |  |
| CC3 |  |  | List of mandatory usage models covered at HT rate | List the mandatory usage models for which 100 Mbps goodput can be achieved and the modes used to achieve this goodput. |  | Based on WFA proposal. unanimously Mktg group consensus |  |  | 2 |
| CC4 |  |  | Cost of Overall Solution that addresses all market segments for both AP and Station | Estimated cost of a device capable of meeting the mandatory features of proposal, relative to an 802.11a or g device. |  | Based on WFA proposal unanimous Mktg group consensus |  |  | 5 |
| CC6 |  |  | PHY complexity | Give an indication of the PHY complexity, relative to current 802.11a PHYs (e.g. gate counts, MIPS, filtering, clock rate, etc.). |  | Mktg group consensus | Intentially allowing flexibility in the responses. |  | 1 (high) |
| CC7 |  |  | MAC processing complexity | Give an indication of the MAC processing complexity, relative to current 802.11-1999 and 802.11e implementations (e.g. gate counts, MIPS, memory, clock rate, etc.) |  | Mktg group consensus | Intentially allowing flexibility in the responses. |  | 3 |
| CC9 |  |  | Power consumption estimate | Using the output of the goodput vs. range CC(27) for the minimum mode that meets a throughput of 100 Mbps: 1) Specify EIRP for the transmitter 2) Estimate the total active receive power consumption in the 5GHz band relative to 54Mbps 802.11a mode. |  | Mktg group consensus |  |  | 4 |
| ## Interoperability and Coexistence with Legacy Devices (consideration of coexistence with non-802.11 devices is deferred until after initial stages of the selection process to reduce simulation effort) |  |  |  |  |  |  |  |  |  |
| CC11 |  |  | Backward compatibility and interoperability with 802.11-1999 (Rev 2003) | Provide a summary description of the means by which backward compatibility with 802.11-1999 (Rev 2003) is achieved in the band(s) covered by the proposal, including references to the sections in the technical proposal document where the complete details are given. |  | Ad Hoc proposal covering CC 11 and CC 12 Interop Group consensus | ? |  | 100% |
| CC15 |  |  | Sharing of medium with legacy devices | Goodput of legacy device in HT network and impact of legacy device on the goodput of the HT devices. The following measurements are made: --T1: the goodput reported in simulation scenario 17. --T2: the goodput reported in simulation scenario 18. --T3: the goodput reported for STA1 in simulation scenario 19. --T4: the goodput reported for STA2 in simulation scenario 19. The legacy share is defined as (T3 / T1). The legacy impact is defined as (T2-T4)/T2 | Simulation scenarios 17-19. | Based on WFA proposal Interop Group consensus |  |  | 75% |
| ## MAC Related |  |  |  |  |  |  |  |  |  |
| ### Performance Measurements at the MAC SAP |  |  |  |  |  |  |  |  |  |
| CC18 |  |  | HT Usage Models Supported (non QoS) | This metric relates to the ability of the proposal to support the non-QoS application service level of each usage model, as defined by its simulation scenario. For each mandatory simulation scenario: Report Goodput per non-QoS flow. Report aggregate goodput for non-QoS flows. Report the ratio of aggregate Goodput for non-QoS flows to the total offered load for non-QoS flows. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | All mandatory simulation scenarios Note, this is measured with QoS flows turned on. | MAC group consensus |  |  | H 13 0 |
| CC19 |  |  | HT Usage Models Supported (QoS) | This metric relates to the ability of the proposal to support the QoS application service level of each usage model, as defined by its simulation scenario. For each QoS flow, the proposal shall report the packet loss rate (defined below) and compare it to the specified QoS objective. The proposal shall also report the number of these flows that satisfy their QoS objective . Also report the fraction of QoS flows that satisfy their QoS objective. For the purpose of this criterion, packet loss rate (PLR) for a QoS flow is defined as the number of MSDUs that are not delivered at the Rx MAC SAP within the specified delay bound, divided by the total number of MSDUs offered at the Tx MAC SAP for that flow during the simulation . | All mandatory simulation scenarios Note, this is measured with non- QoS flows turned on. | MAC group consensus |  |  | H 15 0 |
| CC20 |  |  | BSS Aggregate Goodput at the MAC data SAP | Three aggregate goodput metrics are to be reported. Metric 1 is defined as the sum of goodput across all flows in the simulation scenario. Metric 2 is defined as the aggregate number of bits in MSDUs that are delivered at the Rx MAC SAP within the specified delay bound of the flow’s defined QoS, divided by the simulation duration. Metric 3 is defined as the sum of goodput across all flows that meet their QoS objective in the simulation scenario. Notes: Metric 1 includes flows that fail to meet their QoS objectives. Metric 2 excludes MSDUs that exceed the delay bound. Metric 3 excludes all MSDUs from flows that fail to meet their QoS objectives. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | All mandatory simulation scenarios | MAC group consensus |  |  | H 17 0 |
| CC24 |  |  | MAC Efficiency | MAC efficiency is defined as the the aggregate Metric 2 goodput in CC20 divided by the average physical layer data rate. Note: The average data rate is computed as the time-weighted average of PHY data rates during successful transmissions of MPDUs that carry data frames. | All mandatory simulation scenarios | MAC group consensus |  |  | L 13 1 |
| CC25 |  |  | Scalability | Provide analysis of MAC efficiency with PHY data rates increased by a) 50% and b) 100% relative to the maximum PHY rate of the proposal. Give efficiency numbers as percentages of the PHY data rate. |  | MAC group consensus |  |  | L 9 5 |
| CC27 |  |  | Throughput / Range | Report curves of Goodput (bps) vs range (m) at each PHY rate specified in the proposal. Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [3]: Model B Residential Model D Typical Office | Simulation Scenario 16 | MAC group consensus |  |  | M 13 0 |
| CC28 |  |  | Throughput / Range in 20MHz | Report curves of Goodput (bps) vs range (m) at each PHY rate specified in the proposal, when operating in a 20 MHz bandwidth. Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [3]: Model B Residential Model D Typical Office | Simulation scenario 16 | MAC group consensus |  |  | M 13 0 |
| ### MAC Changes |  |  |  |  |  |  |  |  |  |
| CC46 |  | MAC Compatibility and parameters. |  | Provide a list of mandatory and optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used. Include references to the sections in the technical proposal document where the complete details are given. | None required | Interop Group Consensus |  |  | 100% |
| CC47 |  | MAC extensions |  | Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given. |  | Interop Group Consensus |  |  | 100% |
| CC50 |  | Encryption impacts |  | Provide a summary description of the impact of secured vs unsecured traffic on the throughput of the proposed HT operation. Include references to the sections in the technical proposal document where the complete details are given. |  | Interop Group Consensus | Revised to eliminate required simulation to address this CC. |  | 25% |
| ## PHY Related |  |  |  |  |  |  |  |  |  |
| In this section, the performance of the physical layer will be shown given realistic implementation and radio impairments under a subset of channel conditions. The results will be given in terms of PER for a given data rate and thus should be independent of MAC aspects of the proposals. Both phy-only and complete proposals should be required to complete this section. ### PHY Rates |  |  |  |  |  |  |  |  |  |
| CC51 | Data rates |  |  | A list of PHY layer data rates, and for each data rate, specify the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. |  | Accepted by FRCC Phy subgroup. |  |  | H |
| CC42 | Preambles |  |  | What are the proposed preambles? Describe and evaluate the cross-correlation and auto-correlation properties of all preambles. If the system uses multiple concurrent transmit streams, describe and evaluate the cross-correlation properties of the preambles in the different streams. Describe the auto-correlation properties within each stream. State the PAPR of all preambles. These analyses should be conducted on the transmit waveforms independent of any channel mode. |  | FRCC PHY proposal voted on by FRCC full session. |  |  | CC42 |
| ### Channelization |  |  |  |  |  |  |  |  |  |
| CC51.5 | Channelization |  |  | Specify the channelization – i.e. the adjacent channel spacing. |  | Accepted by FRCC Phy subgroup. |  |  |  |
| CC52 | Spectral Mask |  |  | Show the transmit spectral mask that all transmissions in the proposal meet. List for each channelization. This must be under the same PA backoff per rate used for performance simulations. |  | Accepted by FRCC Phy subgroup. |  |  | H |
| ### Efficiency |  |  |  |  |  |  |  |  |  |
| CC58 | HT Spectral Efficiency |  |  | The number of bps/Hz during the PSDU carrying a Data MPDU when demonstrating a goodput value of at least 100Mbps. Specify the phy data rate used during this test. | Using simulation scenario 16 defined in [3]. | Accepted by FRCC Phy subgroup. |  |  | H |
| ### PHY Performance |  |  |  |  |  |  |  |  |  |
| CC59 | AWGN PER performance |  |  | Identify performance in idealized channel for a packet length of 1000B. The rows or columns of the channel should be orthogonal to each other as follows: Take the first Nr x Nt elements of the fourier matrix with dimension max(Nr, Nt). Show the PER versus SNR curves for 5 supported data rates representative of your rate set including your maximum and minimum rates. The SNR should be calculated in –10dB signal bandwidth. Nt=Nr=1,2,3,4 if proposal has any mode generating 2 or more independent streams otherwise Nt=Nr=1. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. |  | Accepted by FRCC Phy subgroup |  |  | L |
| CC67 | PER performance in non AWGN channels |  |  | Show the PER curves for 5 supported data rates representative of your rate set including your maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Plot PER versus SNR averaged over time per receive antennas in –10dB signal bandwidth for 1000B packets. Total received signal power is summed over all transmit antennas. Averaging should occur over a minimum of 100 packet errors down to 1% PER. Each packet should use an independent channel realization. There shall not be any a priori knowledge of the channel at the receiver. This should be simulated for channel models B, D, and F. The simulations should all include the Doppler effect as specified in the text of the channel model document. All models should be run without the florescent effect but additionally model D should be run with the florescent effect on the highest data rate. The shadowing variance should be 0. |  | Accepted by FRCC Phy subgroup |  |  | H |
| ### PHY Changes |  |  |  |  |  |  |  |  |  |
| CC80 | Required changes to 802.11 PHY |  |  | Give a summary description of changes to a legacy 802.11 PHY. Give references to sections in your specification the give the complete details. |  | Accepted by FRCC Phy subgroup |  | L |  |

# Physical layer impairments

These impairments are to be used in section Error: Reference source not found.

| Number | Name | Definition | Status of this IM |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| IM1 | PA non-linearity | Simulation should be run at oversample rate of at least 4x. Use RAPP power amplifier model as specified in document 00/294. Use P-parameter of 3. Specify backoff from full saturation used in the simulation calculated as PABackoff = –10 log10(Average TX Power/Power at saturation) for each rate. For this test, Psat is specified to be 28dBm. |  | Accepted by FRCC Phy subgroup. |  |  |
| IM2 | Carrier frequency offset | Simulations should be run using a random total tx and rx carrier frequency offset ranging from –40ppm to +40ppm. The symbol clock shall have the same relative frequency offset as the RF carrier frequency offset. |  | Accepted by FRCC Phy subgroup. |  |  |
| IM3 | Baseband timing offset accuracy | Simulations shall include timing acquisition on a per-packet basis. |  | Accepted by FRCC Phy subgroup. |  |  |
| IM4 | Phase noise | The phasenoise will be specified with a pole-zero model. PSD at 0Hz: -100dBc/Hz PSD floor -130dBc/Hz Break freq: 250kHz |  |  |  |  |
| IM5 | Noise figure | Input referred total noise figure from antenna to output of the A/D will be 10dB. |  | Accepted by FRCC Phy subgroup. |  |  |

# Comparison Criteria Submissions

The results for a complete submission shall include a the values of comparison criteria defined in section 2 using the format defined in this section.

| Number | Name | Definition |
| --- | --- | --- |
| CC2 | Regulatory compliance | Textual Statement |
| CC3 | List of mandatory usage models covered at HT rate | List of mandatory usage models matching the criterion defined in CC3 |
| CC4 | Cost of Overall Solution that addresses all market segments for both AP and Station | Cost estimate in any suitable form |
| CC6 | PHY complexity | Numeric value in any suitable form |
| CC7 | MAC processing complexity | Numeric value in any suitable form |
| CC9 | Power consumption estimate | EIRP value (dBm) Receive power consumption figure (mW). |
| CC11 | Backward compatibility and interoperability with 802.11-1999 (Rev 2003) | Bullet-point summary and refence to section in technical specification. |
| CC15 | Sharing of medium with legacy devices | Legacy impact and legacy share values as specified in CC15 |
| CC17 | SAP compatibility | Bullet-point summary description and reference to section in the technical specification. |
| CC18 | HT Usage Models Supported (non QoS) | For each mandatory simulation scenario: Goodput per non-QoS flow. Aggregate goodput for non-QoS flows. Ratio of aggregate Goodput for non-QoS flows to the total offered load for non-QoS flows. |
| CC19 | HT Usage Models Supported (QoS) | For each QoS flow, report the packet loss rate and compare it to the specified QoS objective. Report the number of these flows that satisfy their QoS objective . Also report the fraction of QoS flows that satisfy their QoS objective. |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | Report values of metrics 1-3 defined in CC20 |
| CC24 | MAC Efficiency | Report MAC efficiency metric defined in CC24 |
| CC25 | Scalability | Report MAC efficiency values for the two PHY rate increases specified in CC25. |
| CC27 | Throughput / Range | For the following channel models from [3]: Model B Residential Model D Typical Office Present curves of Goodput (bps) vs range (m) at each PHY rate specified in the proposal. Disclose all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. |
| CC28 | Throughput / Range in 20MHz | For the following channel models from [3]: Model B Residential Model D Typical Office Present curves of Goodput (bps) vs range (m) at each PHY rate specified in the proposal, when operating in a 20 MHz bandwidth. Disclose all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. |
| CC51 | Data rates | Text list of rates supported, each marked mandatory or optional |
| CC42 | Preambles | Reference to section in technical specificatio defining preambles. For each preamble type supported: Mean and std of peak to sidelobe ratio of the autocorrelation function PAPR values. Description and evaluation of cross-correlation properties. |
| CC51.5 | Channelization | Minimum channel spacing for all operational modes |
| CC52 | Spectral Mask | For each channelization: graph of spectral output (dBm) vs frequency offset from center frequency (MHz). |
| CC58 | HT Spectral Efficiency | The number of bps/Hz and specified data rate. |
| CC59 | AWGN PER performance | For proposals which have any mode generating 2 or more independent streams, show 4 graphs for the values Nr=Nt from 1 to 4. For proposals that do not have any mode generating 2 or more independent streams, show a single graph for Nr=Nt=1. For each graph: Plot a curve of log PER vs SNR (dB) for each of 5 supported data rates on the same axes. |
| CC67 | PER performance in non AWGN channels | For each of channel models B, D and F: Plot a curve of log PER vs SNR (dB) for each of 5 supported data rates. Additionally, for channel model D: Plot a curve of log PER vs SNR (dB) for the highest supported data rate incoporating the fluorescent effect on the same graph as the other curve for channel model D. |
| CC80 | Required changes to 802.11 PHY | A bullet-point list of main features with reference to the section in the technical specification. |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/813, TGn Functional Requirements

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)

[7] ITU-R 1390 Method