IEEE P802.11
Wireless LANs

IEEE 802.11 TGn Comparison Criteria

Date:	March 18, 2004

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines comparison criteria that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document. please let Adrian know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P Stephens | Intel | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Pieter-Paul Giesberts | Agere |  |  |  |  |
| Bruno Jechoux | Mitsubishi | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | jechoux@tcl.ite.mee.com |
| Jeff Gilbert | Atheros | Atheros Communications 529 Almanor Ave | + 1 408-773-5261 |  | gilbertj@atheros.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2°, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Herve Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | bonneville@tcl.ite.mee.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Frans Hermodsson | TeliaSonera |  | +46 40 10 51 97 | +46 40 30 70 29 | Frans.Hermodsson@teliasonera.com |
| David Bagby | Calypso Consulting | 2028 Arbor Ave. Belmont CA. 94002 | +1 (650) 637-7741 |  | david.bagby@ieee.org |
| Sean Coffey | Texas Instruments |  |  |  |  |
| Günter Kleindl | SIEMENS AG Austria |  | +43 51707 35738 |  | guenter.kleindl@siemens.com |
| Jacob Sharony | Symbol |  |  |  | Jacob Sharony [sharony@symbol.com] |
| John Ketchum | Qualcomm |  | +1 781 276 0915 |  | jwk@qualcomm.com |
| Sanjiv Nanda | Qualcomm | 5775 Morehouse Dr, San Diego CA 92121 USA | +1 858 845 2375 |  | snanda@qualcomm.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Empty skeleton submitted to satisfy the automatic document system | 20 October 2003 |
| R1 | APS initial contribution | 31 October 2003 |
| R2 | Updated following FRCC telecon on 4 Nov 2003 to add proposed comparison criteria evaluating degree of support for HT usage models | 10 November 2003 |
| R3 | Updates made during 13 November 2003 TGn session while editing the FR document. Added revision history. Updated with changes made at the FRCC telecon on 18 November 2003. | 19 November 2003 |
| R4 | Merged in comments from: - Pieter-Paul Giesberts (Agere) - Bruno Jechoux (Mitsubishi) Reorganised existing CC into sections according to Pieter-Paul’s contribution. + APS comments on the above | 27 November 2003 28 November 2003 |
| R5 | Updated at 2 December 2003 FRCC telecon. Includes changes agreed at the telecon. Includes proposed definition of maturity by Jeff Gilbert. | 2 December 2003 |
| R6 | Merged in multiple comments and changes - Bjorn Bjerke - Herve Bonneville - Rahul Malik - Frans Hermodsson - David Bagby - Sean Coffey - Jeff Gilbert - Guenter Kleindl Added in "priority" column and give initial priority assignment. | 15 December 2003 |
| R7 | Updated during the Dec 16, 2003 telecon of the FRCC. | 16 December 2003 |
| R8 | Contribution from Jacob Sharony | 12 January 2004 |
| R9 | Change marks removed prior to split-out sessions to enable tracking of changes made in those sessions. | 13 January 2004 |
| R10 | Changes from Interop group session merged in Changes from Mktg group session merged in | 13 January 2004 |
| R11 | Changes from MAC group merged in Changes from PHY group in doc 11-04-0053r5 merged in | 14 January 2004 |
| R12 | Updated during TGn FRCC session | 14 January 2004 |
| R13 | Updated during TGn FRCC session during consideration of comments | 15 January 2004 |
| R14 | Updated during TGn FRCC telecon + editorial updates | 27 January 2004 |
| R15 | Updated during TGn FRCC telecon | 10 February 2004 |
| R16 | Editorial corrections + disclosures added for MAC changes Changes agreed at FRCC telecon | 13 February 2004 25 February 2004 |
| R17 | Changes agreed at FRCC telecon Disclosure format moved from section 5 in to CC. Section 5 removed. | 9 March 2004 |
| R18 | Changes made at TGn session | 16 March 2004 |
| R19 | Editorial Comments from 11-04-343r1 implemented Updated during March 17 FRCC session. | 17 March 2004 |
| R20 | Updated during March 18 FRCC session | 18 March 2004 |
| R21 | Ditto (continued) (completed consideration of impairments column and mandatory/optional column) | 18 March 2004 |
| R22 | Updated during March 18 FRCC session (Editorial changes and TBD resolutions proposed). | 18 March 2004 |

# Introduction

## Purpose of document

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in [2] and to disclose results according to the comparison criteria defined in this document.

## Form of Disclosure

A proposal shall disclose its results as required by section 3 and as required by the Disclosure column of the comparison critieria table in section 4..

## Relationship to Functional Requirements

The main purpose of the comparison criteria is to define metrics to enable comparison of TGn roposals.

In addition, the functional requirements [2] may define that specific criteria meet specific values.   This document defines how those measurements are to be made and reported so that compliance to the functional requirements can be evaluated.

As such, the functional requirements [2] are dependent on this document, but not the other way around.

## Relationship to Simulation Scenarios

The IEEE 802.11 TGn FRCC (Functional Requirements and Comparison Criteria Special Committee) has defined usage models from which simulation scenarios have been created [3].

These simulation scenarios are intended to define the input to a simulation in sufficient detail so that the simulation results from different proposals can be meaningfully compared.

This document may define certain criteria given the conditions defined in a certain simulation scenario.  As such certain parts of this document are dependent on the simulation scenarios contained in [3], but not the other way around.

# Definitions

| Term | Definition |
| --- | --- |
| Goodput | Goodput is defined by totaling the number of bits in MSDUs indicated at the MAC DATA SAP, and dividing by the simulation duration (s). |
| Backward Compatible | Supports all the mandatory modes of the subject standard in the band or bands covered by the proposal. |
| SNR | The signal to noise ratio is defined as the ratio of the signal power in the aggregate of the -10dB signal bandwidths divided by the noise power in the aggregate of the -10dB signal power bandwidths. |

# Additional Disclosures

(This section contains requirements for additional information that shall be disclosed with a proposal)

| Number | Name | Definition | Status of this AD |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. |  | PPG (Agere) proposal Agreed 6/0. |  |  |

# Comparison Criteria

| Number | Name | Definition | Simulation Scenario / Impairments | Status of this CC | Notes | Mandatory / optional | Disclosure |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ## General |  |  |  |  |  |  |  |
| CC2 | Regulatory compliance | The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, Europe, China. | None required |  | Compliance according to regulatory body within each domain, FCC, CEPT, MPHTP, etc. | Mandatory | Textual Statement |
| ## Marketability |  |  |  |  |  |  |  |
| CC3 | List which of usage models 1 and 4 are covered at HT rate | List the mandatory usage models for which 100 Mbps goodput can be achieved and the modes used to achieve this goodput. | None required |  |  | Mandatory | List of mandatory usage models matching the criterion |
| CC6 | PHY complexity | Give an indication of the PHY complexity, relative to current 802.11a PHYs (e.g. gate counts, MIPS, filtering, clock rate, etc.). | None required |  | Intentially allowing flexibility in the responses. | Optional | Numeric value in any suitable form |
| CC7 | MAC processing complexity | Give an indication of the MAC processing complexity, relative to implementations of 802.11-1999 (Rev 2003), and published amendments (e.g. gate counts, MIPS, memory, clock rate, etc.) | None required |  | Intentially allowing flexibility in the responses. | Optional | Numeric value in any suitable form |
| ## Backward Compatibility and Coexistence with Legacy Devices |  |  |  |  |  |  |  |
| (consideration of coexistence with non-802.11 devices is deferred until after initial stages of the selection process to reduce simulation effort) |  |  |  |  |  |  |  |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Provide a summary description of the means, if any, by which backward compatibility with 802.11-1999 (Rev 2003) and 802.11g is achieved in the band(s) covered by the proposal, including references to the sections in the technical proposal document where the complete details are given. | None required |  | ? | Mandatory | Bullet-point summary and refence to section in technical specification. |
| CC15 | Sharing of medium with legacy devices | Goodput of legacy device in HT network and impact of legacy device on the goodput of the HT devices. Report the following measurements : --T1: the goodput reported in simulation scenario 17. --T2: the goodput reported in simulation scenario 18. --T3: the goodput reported for STA1 in simulation scenario 19. --T4: the goodput reported for STA2 in simulation scenario 19. | Simulation scenarios 17-19. No impairments |  |  | Mandatory | Legacy impact and legacy share values as specified here |
| ## MAC Related |  |  |  |  |  |  |  |
| ### Performance Measurements at the MAC SAP Note, for all performance measurements, statistics recorded shall be during steady state conditions. |  |  |  |  |  |  |  |
| CC18 | HT Usage Models Supported (non QoS) | This metric relates to the ability of the proposal to support the non-QoS application service level of each usage model, as defined by its simulation scenario. For each simulated scenario: Report Goodput per non-QoS flow. Report aggregate goodput for non-QoS flows. Report the ratio of aggregate Goodput for non-QoS flows to the total offered load for non-QoS flows. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. Note, backward TCP Ack flows shall be counted as non-QoS flows. | Simulation Scenarios 1, 4, and 6. Note, this is measured with QoS flows turned on. IM1,5,6 |  |  | Mandatory | For each simulated scenario report results as defined in this CC. |
| CC19 | HT Usage Models Supported (QoS) | This metric relates to the ability of the proposal to support the QoS application service level of each usage model, as defined by its simulation scenario. For each QoS flow, the proposal shall report the packet loss rate (defined below) and compare it to the specified QoS objective. The proposal shall also report the number of these flows that satisfy their QoS objective . Also report the fraction of QoS flows that satisfy their QoS objective. For the purpose of this criterion, packet loss rate (PLR) for a QoS flow is defined as the number of MSDUs that are not delivered at the Rx MAC SAP within the specified delay bound, divided by the total number of MSDUs offered at the Tx MAC SAP for that flow during the simulation . | Simulation Scenarios 1 and4 . Note, this is measured with non- QoS flows turned on. IM1,5,6 |  |  | Optional | For each simulated scenario report results as defined in this CC. |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | Three aggregate goodput metrics are to be reported. Metric 1 is defined as the sum of goodput across all flows in the simulation scenario. Metric 2 is defined as the aggregate number of bits in MSDUs that are delivered at the Rx MAC SAP within the specified delay bound of the flow’s defined QoS, divided by the simulation duration. Metric 3 is defined as the sum of goodput across all flows that meet their QoS objective in the simulation scenario. Notes: Metric 1 includes flows that fail to meet their QoS objectives. Metric 2 excludes MSDUs that exceed the delay bound. Metric 3 excludes all MSDUs from flows that fail to meet their QoS objectives. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | Simulation Scenarios 1 and4 . IM1,5,6 |  |  | Mandatory | For each simulated scenario, report values of metrics 1-3 defined in here |
| CC24 | MAC Efficiency | MAC efficiency is defined as the the aggregate Metric 2 goodput in CC20 divided by the average physical layer data rate. Let r(n) denote the rate at which the nth successful Data MPDU is transmitted. Let t(n) denote the PPDU transmission time of the nth successfully transmitted Data MPDU (i.e. including preamble, PLCP header). Define the average PHY data rate r as r = ∑ r(n)t(n)/ ∑t(n) | Simulation Scenarios 1, 4, and 6. IM1,5,6 |  |  | Mandatory | For each simulated scenario, Report MAC efficiency metric defined in here |
| CC27 | Throughput / Range | Report curves of Goodput (bps) vs range (m). Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [4]: Model B Residential Model D Typical Office | Simulation Scenario 16 IM1,5,6 |  |  | Mandatory | For the simulated channel models, Present the required curves. Also provide, textual description of parameter and setting values. |
| CC28 | Throughput / Range in 20MHz | Report curves of Goodput (bps) vs range (m) , when operating in a 20 MHz bandwidth. Also provide all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. For the following channel models from [4]: Model B Residential Model D Typical Office | Simulation scenario 16 IM1,5,6 |  |  | Mandatory | For the specified channel models from [3]: Present curves of Goodput (bps) vs range (ml, when operating in a 20 MHz bandwidth. Disclose all MAC parameters and settings (e.g., interframe spacings, fragmentation thresholds etc.) and all PHY parameters and settings used to obtain the curves. |
| ### MAC Changes |  |  |  |  |  |  |  |
| CC46 | MAC Compatibility and parameters. | Provide a list of optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used. Include references to the sections in the technical proposal document where the complete details are given. | None required |  |  | Mandatory | Text list of features plus references to sections in proposed technical specification document. |
| CC47 | MAC extensions | Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given. | None required |  |  | Mandatory | Text list of features plus references to sections in proposed technical specification document. |
| ## PHY Related |  |  |  |  |  |  |  |
| In this section, the performance of the physical layer will be shown given realistic implementation and radio impairments under a subset of channel conditions. The results will be given in terms of PER for a given data rate and thus should be independent of MAC aspects of the proposals. Both phy-only and complete proposals should be required to complete this section. PHY Rates |  |  |  |  |  |  |  |
| CC51 | Data rates | A list of PHY layer data rates, and for each data rate, specify the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. For adaptive rate proposals, specify the range of achievable rates or, if possible, state the achievable rates in a closed form expression. | None required |  |  | Mandatory | Text list of rates supported, each marked mandatory or optional |
| CC42 | Preambles | Specify the proposed preambles. Summarize the important properties of each part the proposed preambles. Include references to the sections in the technical proposal document where the complete details are given. Specify how the use of any new preamble affects reception by legacy STA. | None required |  |  | Mandatory | Reference to section in technical specificatio defining preambles. For each preamble type supported: Mean and std of peak to sidelobe ratio of the autocorrelation function PAPR values. Description and evaluation of cross-correlation properties. |
| ### Channelization |  |  |  |  |  |  |  |
| CC51.5 | Channelization | Specify the channelization – i.e. the adjacent channel spacing. | None required |  |  | Mandatory | Minimum channel spacing for all operational modes |
| CC52 | Spectral Mask | Define the transmit spectral mask requirements for each channelization of the proposal. This must be under the same PA backoff used for performance simulations. | None required IM1 |  |  | Mandatory | For each channelization: graph of spectral output (dBm) vs frequency offset from center frequency (MHz). |
| ## Efficiency |  |  |  |  |  |  |  |
| CC58 | HT Spectral Efficiency | The number of bps/Hz during the PSDU carrying a Data MPDU when demonstrating a goodput value of at least 100Mbps. Specify the phy data rate used during this test. | Using simulation scenario 16 defined in [3]. IM1,5,6 |  |  | Mandatory | The number of bps/Hz and specified data rate. |
| ### PHY Performance |  |  |  |  |  |  |  |
| CC59 | AWGN PER performance | Identify the performance in an idealized channel for a PSDU length of 1000B. The rows or columns of the channel shall be orthogonal to each other as follows: take the first Nr x Nt elements of the Fourier matrix with dimension max(Nr,Nt). Show PER versus SNR curves for 5 supported data rates representative of the proposal's rate set, including the maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Note that when computing the SNR, the signal and noise bandwidth shall be identical. Averaging should occur over a minimum of 100 packet errors down to 1% PER Refer to [7] for a definition of the fourier matrix. | None required None |  |  | Mandatory | For proposals which have any mode generating 2 or more independent streams, show 4 graphs for the values Nr=Nt from 1 to 4. For proposals that do not have any mode generating 2 or more independent streams, show a single graph for Nr=Nt=1. For each graph: Plot a curve of log PER vs SNR (dB) for each of 5 supported data rates on the same axes. |
| CC67 | PER performance in non AWGN channels | Show either or both of the following two sets of performance curves: 1.) Show the PER curves for 5 supported data rates representative of your rate set including your maximum and minimum rates. If the proposal supports fewer than 5 data rates, all supported data rates should be shown. Plot PER versus SNR averaged over time per receive antennas for PSDUs of length 1000B. Averaging should occur over a minimum of 100 MPDU errors down to 1% PER. 2.) Show curves for both achieved average physical layer data throughput and PER, as a function of total SNR for 1000B PSDUs. These results should be generated with a rate selection algorithm active. Data throughput is defined as the total number of bits successfully received in the data portion of the PPDU, divided by total transmission time, not including overheads such as preamble and backoff. The throughput shall be averaged over at least 100 independent realizations of the channel. PER is computed by averaging over a minimum of 100 MPDU errors. Total received signal power is summed over all transmit antennas. Each packet should use an independent channel realization. There shall not be any a priori knowledge of the channel at the receiver. This should be simulated for channel models B, D, and F. The simulations should all include the Doppler effect as specified in the text of the channel model document. All models should be run without the fluorescent effect but additionally model D should be run with the fluorescent effect on the highest data rate. The shadowing variance should be 0. These simulations are performed using the NLOS version of the specified channel models. | None required IM1-6 |  |  | Mandatory | For each of channel models B, D and F: Plot a curve of log PER vs SNR (dB) for each of 5 supported data rates. Additionally, for channel model D: Plot a curve of log PER vs SNR (dB) for the highest supported data rate incorporating the Fluorescent effect on the same graph as the other curve for channel model D. |
| CC67.2 | Offset Compensation | Provide the impact on PER of carrier frequency offset and symbol clock offset by comparing to the PER achieved at the lowest average SNR that achieves a 10% PER for PSDUs of length 1000 bytes in channel E (NLOS) with no carrier and symbol clock offset. The symbol clock shall have the same relative frequency offset as the carrier frequency offset. Also, provide that same impact on PER using an SNR of 50dB in channel E (LOS). The carrier offset difference at the receiver relative to the transmitter shall be -40ppm and +40ppm. The results shall be presented in such a manner that it is clear whether there are specific values of offset for which the proposed system has better or worse performance relative to no offset. Note Simulations shall be performed under the same conditions as those in CC 59 & 67. | None required IM1-5 |  |  | Mandatory | Numeric results |
| ### PHY Changes |  |  |  |  |  |  |  |
| CC80 | Required changes to 802.11 PHY | Give a summary description of changes to a legacy 802.11 PHY. Give references to sections in your specification that give the complete details. | None required |  |  | Mandatory | A bullet-point list of main features with reference to the section in the technical specification. |

# Physical layer impairments

| Number | Name | Definition | Status of this IM |
| --- | --- | --- | --- |
| IM1 | PA non-linearity | Simulation should be run at oversample rate of at least 4x. Use RAPP power amplifier model as specified in document 00/294 with p = 3. Calculate backoff as the output power backoff from full saturation: PA Backoff = 10 log10(Average TX Power/Psat). Total TX power shall be limited to no more than 17 dBm. Disclose: (a) EIRP and how it was calculated, (b) PA Backoff, and (c) Psat per PA. Note: the intent of this IM is to allow different proposals to choose different output power operating points. Note: the value Psat = 25dBm is recommended. |  |
| IM2 | Carrier frequency offset | Simulations for all comparisons except Offset Compensation shall be run using a fixed carrier frequency offset of –13.675 ppm at the receiver, relative to the transmitter. The symbol clock shall have the same relative offset as the carrier frequency offset. Simulations shall include timing acquisition on a per-packet basis. |  |
| IM4 | Phase noise | The phasenoise will be specified with a pole-zero model. PSD at 0Hz: -100dBc/Hz PSD floor -130dBc/Hz Break freq: 250kHz Note, this impairment is modelled at both transmitter and receiver. |  |
| IM5 | Noise figure | Input referred total noise figure from antenna to output of the A/D will be 10dB. | . |
| IM6 | Anntena Configuration | The antenna configuration at both ends of the radio link shall be a uniform linear array of isotropic antennas with separation of one-half wavelength, with an antenna coupling coefficient of zero. All antennas shall have the same vertical polarization. |  |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/813, TGn Functional Requirements

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/940r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)

[7] IEEE 802.11-04-176r0, TGn Comparison Criteria – Proposal for CC59