IEEE P802.11
Wireless LANs

Proposed K-factors for TGn Channel Models

Date:                                                     November 5, 2003

Author:                                                     Vinko Erceg
                                                                 Zyray Wireless
                                 11455 El Camino Real, Suite 350, San Diego, CA 92139
                                                          Phone: +1-858-704-0430
                                                e-Mail: verceg@zyraywireless.com

Abstract

This document provides proposed K-factors and their mappings to environments for the TGn channel models.

| Model | Condition | K (dB) LOS/NLOS | RMS delay spread (ns) | # of clusters |
| --- | --- | --- | --- | --- |
| A (optional) | LOS/NLOS | 6 / - | 0 | 1 tap |
| B | LOS/NLOS | 6 / - | 15 | 2 |
| C | LOS/NLOS | 6 / - | 30 | 2 |
| D | LOS/NLOS | 6 / - | 50 | 3 |
| E | LOS/NLOS | 6 / - | 100 | 4 |
| F | LOS/NLOS | 6 / - | 150 | 6 |
| G | NLOS | - | 600 | 6 |

Table IIa: Summary of model parameters for LOS/NLOS conditions

| Environment | Condition | Model |
| --- | --- | --- |
| Residential | LOS | B – LOS |
|  | NLOS | B - NLOS |
| Residential/ Small Office | LOS | B - LOS |
|  | NLOS | C - NLOS |
| Typical Office | LOS | C - LOS |
|  | NLOS | D - NLOS |
| Large Office | LOS | D - LOS |
|  | NLOS | E - NLOS |
| Large Space/ (Indoors and Outdoors) | LOS | E - LOS |
|  | NLOS | F - NLOS |
| Stadium/ Microcell | LOS | F - LOS |
|  | NLOS | G - NLOS |

Table IIb: Environment – Model Mapping.