IEEE P802.11
Wireless LANs

IEEE802.11k Draft PICS Proforma

Date:	November 5th, 2003

Author:	Simon Black, Hasse Sinivaara
	Nokia

emails: simon@motix.demon.co.uk, hasse.sinivaara@nokia.com

Abstract

This paper proposes a Protocol Implementation Conformance Statement (PICS) proforma for draft IEEE802.11k. The proposal is based on the functionality and section numbering present in draft D0.8

The PICS status symbols below are reproduced from Annex A in ISO/IEC 8802-11 1999 as a guide to interpreting this submission.

M	Mandatory

O	Optional

O.<n>	Optional, but support of at least one group of options labelled by the same number <n>

Pred:	conditional symbol, including predicate identification

A predicate is either:

- An item-reference for an item in the PICS proforma: the value of the predicate is true if the item is marked as supported, and is false otherwise.

- A boolean expression constructed by combining item-references using the boolean operator OR: the value of the predicate is true if one or more of the items is marked as supported, and is false otherwise.

Each item referenced in a predicate, or in a preliminary question for group conditional items, is indicated by an asterisk in the Item column.

More information concerning the syntax and use of the PICS proforma can be found in the introductory text of the annex referred to above.

# Annex A – PICS

### A.4.3 IUT configuration

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF10 | Is Radio Resource Measurement supported? | 7.3.1.4 | O | Yes, No |

### A.4.11 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM1 | Are the following Radio Resource Measurement capabilities supported? Radio Measurement Capability | 7.3.1.4 | M | Yes, No |
| RRM2 RRM2.1 RRM2.2 | Action frame protocol for measurements Measurement Request frame Measurement Report frame | 7.4 7.4.1.1, 7.3.2.19 7.4.1.2 | M M M | Yes, No Yes, No Yes, No |
| RRM3 RRM3.1 RRM3.2 RRM3.3 RRM3.4 RRM3.5 | General protocol for requesting and reporting of measurements Parallel Measurements Enable Measurement Requests Enable Autonomous Report Incapable Indication Refused Indication | 7.3.2.19, 7.3.2.20, 11.7, 11.7.6 7.3.2.19, 7.3.2.20 7.3.2.19, 11.7.6 7.3.2.19, 11.7.6 7.3.2.20 7.3.2.20 | M O M M M M | Yes, No Yes, No Yes, No Yes, No Yes, No Yes, No |
| RRM4 RRM4.1 RRM4.2 RRM4.3 RRM4.4 * RRM4.5 RRM4.6 RRM4.7 | Beacon Measurement Type Beacon Request Active Scan mode Passive Scan mode Beacon table mode Periodic Measurement Reporting Conditions Beacon Report | 11.7, 11.7.7.1 7.3.2.19.1 7.3.2.19.1 7.3.2.19.1 7.3.2.19.1 7.3.2.19.1 7.3.2.19.1 7.3.2.20.1 | M M M M M O RRM4.5:M M | Yes, No Yes, No Yes, No Yes, No Yes, No Yes, No Yes, No, N/A Yes, No |
| * RRM5 RRM5.1 RRM5.2 | Frame Measurement Type Frame Request Frame Report | 11.7, 11.7.7.2 7.3.2.19.2 7.3.2.20.2 | O RRM5:M RRM5:M | Yes, No Yes, No, N/A Yes, No, N/A |
| RRM6 RRM6.1 RRM6.2 | Channel Load Measurement Type Channel Load Request Channel Load Report | 11.7, 11.7.7.3 7.3.2.19.3 7.3.2.20.3 | M M M | Yes, No Yes, No Yes, No |
| * RRM7 RRM7.1 RRM7.2 | Noise Histogram Measurement Type Noise Histogram Request Noise Histogram Report | 11.7, 11.7.7.4 7.3.2.19.4 7.3.2.20.4 | O RRM7:M RRM7:M | Yes, No Yes, No, N/A Yes, No, N/A |
| * RRM8 RRM8.1 RRM8.2 | Hidden Node Measurement Type Hidden Node Request Hidden Node Report | 11.7, 11.7.7.5 7.3.2.19.5 7.3.2.20.5 | O RRM8:M RRM8:M | Yes, No Yes, No, N/A Yes, No, N/A |
| * RRM9 RRM9.1 RRM9.2 | Medium Sensing Time Histogram Measurement Type Medium Sensing Time Histogram Request Medium Sensing Time Histogram Report | 11.7, 11.7.7.6 7.3.2.19.6 7.3.2.20.6 | O RRM9:M RRM9:M | Yes, No Yes, No, N/A Yes, No, N/A |
| RRM10 RRM10.1 RRM10.2 | STA Statistics Measurement Type STA Statistics Request STA Statistics Report | 11.7, 11.7.7.7 7.3.2.19.7 7.3.2.20.7 | M M M | Yes, No Yes, No Yes, No |
| RRM11 | AP Channel Report | 7.3.2.9, 7.3.2.21 | CF1:M | Yes, No, N/A |
| RRM12 | Action Frame Protocol for Site Reports Site Report Request Site Report Response | 7.4 7.4.1.3 7.4.1.4, 7.3.2.22 | M M M | Yes, No Yes, No Yes, No |
| RRM13 | Site Report Procedure | 11.8.1, 11.8.2 | M | Yes, No |
| RRM14 | Disassociation Imminent | 7.4.1.5 | CF1:O | Yes, No, N/A |
| RRM15 | RCPI Measurement RCPI Measurement for DSSS PHY at 2.4GHz RCPI Measurement for OFDM PHY at 5GHz RCPI Measurement for HR DSSS PHY at 2.4GHz | 15.4.8.5 17.3.10.6 18.4.8.5 | CF4: M CF6: M CF7: M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM16 RRM16.1 RRM16.2 | Management Information Base Dot11peerStatsTable AP Service Load | Annex D Annex D | CF1:M CF1:M | Yes, No Yes, No |