IEEE P802.11
Wireless LANs

Normative Text Changes for Call Admission Control in EDCA

Date:	November 11, 2003

Author:	Mathilde Benveniste
	Avaya Labs - Research
	233 Mt. Airy Road, Basking Ridge, NJ USA
	Phone: 973-761-6105
	e-Mail: benveniste@ieee.org

Abstract

This document contains the material proposed to GTE for inclusion in the draft in the form of insertions into and replacements for material in IEEE Std 802.11-1999, as updated by IEEE Std 802.11a-1999, IEEE Std 802.11b-1999 (already present in TGE Draft 5.0).

Editorial notes appear in bold italic Times New Roman font, informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Added text appears in blue with a dotted underline.  Deleted text appears in red with a strikethrough line.

Add the following in section 7.3.2.14

All ACs with priority higher than that of an AC with ACM flag equal to 1 must have admission control mandatory

.

Insert the following at the end of section 9.9.1.7

If an ADDTS request has not been accepted for downlink traffic

in an AC with ACM equal to 1, the QAP must transmit frames in such AC using a lower priority AC that does not require admission control, if such frames are transmitted.  The QAP shall transmit frames destined to a legacy STA in the BE AC.