IEEE P802.11
Wireless LANs

802.11 TGn Functional Requiremenets

Date:	October 21, 2003

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

Abstract

This document defines functional requirements that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing email contributions / review comments of this document)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Skeleton document created | 20 October 2003 |
| R1 | New content partially presented to discussed at the FRCC telecon. Includes updates agreed at the telecon. | 21 October 2003 |

# Introduction

## Purpose of document

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in this document.

## Form of Disclosure

A proposal shall declare its compliance with these functional requirements using the template defined below.

## Relationship to Comparison Criteria and Usage Models

Functional requirements may be described in terms of metrics defined in the 802.11 TGn comparison criteria document [2], or the simulation scenarios defined in the 802.11 TGn Usage Models document [3].

## Requirements for this document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The functional requirements document shall include a set of requirement statements.  Each of these:

- Shall be defined unambiguously

- Can be verified from a reasonable simulation environment, or verified from an examination of the proposed submission

- Are compliant to the 802.11 HT PAR [1] and 5C [2]

- Should be relevant to one or more mandatory use cases in UM doc (TBD)

Note, at the September TGn meeting, it was decided that this document should relate only to mandatory features of the proposal.  Optional features, qualities, performances are considered in [2].

# Functional Requirements – Proposed amendments taken from 	Wi-Fi Alliance MRD (prepared by J. Kruyz/P. Feinberg/R. DeVegt)

| Number | Name | Requirement | Status of Requirement | Notes |
| --- | --- | --- | --- | --- |
| 1 | HT Usage Models supported | Mandatory HT Usage Models supported. Note: supported => PLR of QoS streams is satisfied. Note: packet loss is a packet that doesn’t arrive or arrives late according to the specified delay bound. | Consensus of FRCC, 21 Oct 2003. |  |
|  | Single PHY | Same implemenation across all HT usage models |  |  |
|  | Point-point HT rate supported | Supports a point-point throughput of 100Mbps at the top of the MAC at a range of 15m (TBD) for channel models (B, C, D - TBD) defined in [4]. | APS initial proposal |  |
|  | Point-point HT rate supported at range | Supports a point-point throughput of 10Mbps at the top of the MAC at a range of 100m (TBD) for channel models (B, C, D – TBD) defined in [4] |  |  |
|  | Uniform BSS operation with HT rate supported | Continuous coverage of previous two requirements should be at least 95% (i.e., maximum 1 out of 20 stations within range specified may have to except lower throughput, also providing for mobile stations and movement in the environment) |  |  |
|  | Robust operation with HT rate supported | Supports BSS (1AP,1STA) operation with throughput of 100Mbps at the top of the MAC at a range of 15m (TBD) for channel models (B, C, D - TBD) defined in [4] and in the presence of two other APs operating on the same channel, each within 15m of the AP under test (small enterprise) |  |  |
|  | SAP Compatibility | Backwards compatible with existing 802.11 devices and software, such that the current MAC/SAP functionality is retained |  |  |
|  | Current Spectral flexibility | Protocol supports 2.4GHz ISM and 5GHz UNII bands | APS initial proposal |  |
|  | Japanese Bands Spectral flexibility | Protocol supports .11j 10MHz channels | APS initial proposal |  |
|  | Cost / Complexity flexibility | Protocol provides options that allow low-cost lower performance variants and high-cost higher performance variants. | APS initial proposal |  |
|  | .11b BSS compatibility | A HT STA can communicate with a legacy .11b STA operating within a BSS managed by a legacy .11b AP | APS initial proposal |  |
|  | .11g BSS compatibility | A HT STA can communicate with a legacy .11g STA operating within a BSS managed by a legacy .11g AP without degradation to legacy device operation | [5] PAR, section 18 |  |
|  | .11a BSS compatibility | A HT STA can communicate with a legacy .11a STA operating within a BSS managed by a legacy .11a AP without degradation to legacy device operation | [5] PAR, section 18 |  |
|  | .11b STA compatibility | A HT STA can communicate with a legacy .11b STA operating within a BSS managed by a HT AP without degradation to legacy device operation | APS initial proposal |  |
|  | .11g STA compatibility | A HT STA can communicate with a legacy .11g STA operating within a BSS managed by a HT AP | [5] PAR, section 18 |  |
|  | .11a STA compatibility | A HT STA can communicate with a legacy .11a STA operating within a BSS managed by a HT AP without degradation to legacy device performance | [5] PAR, section 18 |  |
|  | .11b IBSS Backward compatibility | HT and legacy .11b STA can communicate directly within a single IBSS consisting of a mixture of legacy .11b and HT devices | APS initial proposal |  |
|  | .11g IBSS Backward compatibility | HT and legacy .11g STA can communicate directly within a single IBSS consisting of a mixture of legacy .11g and HT devices | APS initial proposal |  |
|  | .11a IBSS Backward compatibility | HT and legacy .11a STA can communicate directly within a single IBSS consisting of a mixture of legacy .11a and HT devices | APS initial proposal |  |
|  | CSMA/CA Medium Access Procedures | Activation of legacy CSMA/CA MAC procedures are user controlled. |  |  |
|  | All changes pertain to higher throughput | All the proposed mechanisms have a positive effect on throughput | APS initial proposal |  |
|  | Spectral Efficiency | The value of metric (spectral efficiency) has a value of 3bps/Hz for at least one mode of operation of the system | APS initial proposal |  |
|  | Fair sharing | A HT BSS shares the medium fairly with a co-channel co-located legacy BSS. | APS initial proposal |  |
|  | Fair Medium Sharing | Ensure fair sharing of medium sharing with legacy 802.11 systems; i.e., if some stations in a BSS are operating in HT mode the STAs operating in legacy modes should not see their performance reduced |  |  |
|  | Increased Range for current rates | The protocol supports increased ranges for rates matching the legacy rates. | APS initial proposal |  |
|  | Power management | The protocol permits operation of the 802.11 (+e) power-saving modes | APS initial proposal |  |
|  | Regulatory | The protocol provides signalling of any constraints on modes of operation specific to regulatory constraints due to geopolitical or regional regulations | APS initial proposal |  |
|  | Regulatory Compliance | All HT operating modes shall be compliant with current and emerging regulations affecting 802.11 products |  |  |
|  | Baseline | The protocol builds on the baseline specification defined by 802.11 and ammendments a,b,d,e (including the WME transitory subset),g,h,i,j,k | [5] PAR, section 18 |  |

# Template for Complete Submissions

The results for a complete submission shall include a statement of coverage of the functional requirements and results for simulation scenarios following the outline presented here.

## Coverage of Functional Requirements

Each requirement shall be declared to be covered or not covered.  For each requirement relating to measurements, a reference to the document/page showing the appropriate performance measurements should be added.

| Number | Name | Requirement | Coverage | Results Reference |
| --- | --- | --- | --- | --- |
| R1 | TBD | TBD |  |  |
| Rest of table will be filled in when the functional requirements are agreed… |  |  |  |  |

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/814, TGn Comparison Criteria

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)