IEEE P802.11
Wireless LANs

Border Flag in IEEE802.11k Site Reporting

Date:	November 5th, 2003

Author:	Hasse Sinivaara, Simon Black
	Nokia

emails: hasse.sinivaara@nokia.com, simon@motix.demon.co.uk

Abstract

Earlier submissions [1], [2] have introduced the idea of a border, or boundary flag as a part of the IEEE802.11k site report protocol. The purpose of such a flag is to assist in the transition between WLAN and cellular access in STAs that are capable of operation in both types of network. This paper deals only with this topic, rather than present an entire site reporting proposal and builds on protocol elements already included in IEEE802.11k draft 0.8.

# Introduction

Earlier submissions have introduced the concept of a border, or boundary flag as part of the TGk site reporting, or BSS information distribution protocol. The purpose of this flag is to provide a hint to a dual-mode WLAN /cellular terminal for use in determining when it would be sensible to initiate a hand-off from one network to another. This principle is well described in [3], a paper that also considers other elements of the handoff process.

Consider the building coverage scenario in figure 1 below. The ovals represent coverage of a number of access points within a building. There are two exits to the building – one at the front door, the other at the side of the building to the parking lot. There is assumed to be cellular coverage overlaying the whole building (as is often the case). At location A the terminal has good WLAN service. As the terminal moves to location B, it moves into an area of weaker signal strength. WLAN only terminals in such circumstances frequently roam between access points as signal quality varies. A dual-mode terminal in these circumstances may attempt to hand-off to the cellular network, only to return to the WLAN moments later. The signaling traffic and potential for service disruption is likely to be far greater in the WLAN-cellular transition, than in an roam between two BSSs within the same ESS. This type of transition is therefore undesirable [3].

As the terminal reaches location C, there is good signal strength still on the WLAN, but the terminal is near a building exit. At this point it would be sensible to start to prepare for a WLAN-cellular hand-off given that the user may exit the building and the ESS coverage area.

The principle of the border bit is thus to identify the BSSs close to the exit points of ESS coverage such that:

- a strong hint is provided for dual-mode WLAN-cellular terminals that a hand-off to a network beyond the WLAN may be required shortly; and

- unnecessary inter-network transitions are avoided when it would offer a better service to stay within the WLAN

Typically the border flag would be set by a system administrator and the normative text proposal that follows assumes that control of this is made available via the TGk MIB.

A

B

C

Border Flag Set

Border Flag Clear

Front Door

Access to Parking

Figure 1 – Border Flag Example

The remainder of this submission concerns normative text modifications to support the border flag proposal.

# Proposed Normative Text

The proposed normative text is presented in two parts. The first adds a flag for the border function within the site report, renaming a field for clarity. Rather than propose the related MIB in relation to Draft 0.8 (which contains no MIB for site reporting) [4], this proposal builds on the MIB proposed in document 11-03-837 [5].

Amend clause 7.3.2.22 as follows:

# 7.3.2.22 Site Report element

Site Report element contains pertinent information on a collection of AP’s that are candidates to which STAs can roam to. The format of the Site Report element is shown in Figure [reference removed].

|  | Element ID | Length |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |  |  |
|  | BSSID | BSSID Match Status BSSID Information | Current Channel | PHY Type | The element contains zero or more quadruplets |  |  |
| Octets: | 6 | 2 | 1 | 1 |  |  |  |

Figure [reference removed] –Site Report element format

The Element ID field shall be equal to the Site Report value in [reference removed].

The Length field is variable and shall be set to the number of the quadruplets that follow multiplied by the size of the quadruplet (i.e., 10). Each quadruplet describes an AP and consists of BSSID, BSSID Status, Current Channel and PHY type. The minimum value of the Length field is 0 (e.g., with no neighbour AP in the Site Report element).

The BSSID is the address of the STA contained in the AP.  The subsequent fields; BSSID Match Status, Current Channel and PHY type are presumed to be for the corresponding BSSID.

The BSSID Match StatusInformation is a two octet field containing the following information.  The format of the BSSID Match StatusInformation is shown as follows:

|  | ESS Border | ESS Match | Capability Match | Supported Rate Match | Reserved |
| --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 3 | 4 | 7-15 |

Figure [reference removed] – BSSID Match StatusInformation field format

- The ESS Border bit shall be set to 1 if dot11ESSBorder is true, otherwise it shall be set to 0.

- The ESS Match bit, if set, indicates that the AP represented by this BSSID advertises the same SSID as the current AP. If the bit is not set it indicates that either the SSID does not match, or the AP does not have that information at the current time.

- The Capability Match bit, if set, indicates that the AP represented by this BSSID’s advertised Capabilities match the current AP’s capabilities.  If the bit is not set it indicates that either the capabilities do not match, or the information is not available to the AP at this time.

- Supported Rates Match bit, if set indicates that the AP represented by this BSSID’s advertised Supported Rates matches the current AP’s Supported Rates.  If the bit is not set it indicates that either the Supported Rates do not match, or the information is not available to the AP at this time.

Current Channel field specifies the AP’s current operation channel.

PHY Type is a field that represents the PHY type of the AP represented by this BSSID.  If the BSSID has more than one PHY type there will be a duplicate BSSID entry in the Site Report. The PHY type will be encoded as defined by dot11PHYType.

Amend the Site Report MIB proposed in 11-03-837 as follows, renumbering attributes following the change:

Dot11RRMRequestEntry ::=

SEQUENCE {

dot11RRMSiteReportIFIndex		INTEGER,

dot11RRMSiteReportBSSID		 	MacAddress,

dot11RRMSiteReportMatchStatus		Unsigned32,

dot11RRMSiteReportESSBorder		TruthValue

dot11RRMSiteReportESSMatch		TruthValue

dot11RRMSiteReportCapMatch		TruthValue

dot11RRMSiteReportSuppRatesMatch	TruthValue

dot11RRMSiteReportCurentChannel	INTEGER,

dot11RRMSiteReportPhyType		INTEGER,

dot11RRMSiteReportRowStatus		RowStatus

}

dot11RRMSiteReportMatchStatus OBJECT-TYPE

SYNTAX INTEGER (0, 2, 8, 10, 16, 18, 24, 26)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Is a match to the STA sending the site report.

Valid values are

00 = No Match
02 = Match on ESS

08 = Match on capabilities

10 = Match on ESS and capabilities

16 = Match on supported rates

18 = Match on ESS and supported rates

24 = Match on capabilities and supported rates

26 = Match on ESS, capabilities, and supported rates."

DEFVAL { 26 }

::= { dot11RRMSiteReportEntry 3 }

dot11RRMSiteReportESSBorder OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“BSS reported is at the boundary of the ESS”

DEFVAL { False }

::= { dot11RRMSiteReportEntry 3 }

dot11RRMSiteReportESSMatch OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“BSS reported is a match on ESS”

DEFVAL { True }

::= { dot11RRMSiteReportEntry 4 }

dot11RRMSiteReportCapMatch OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“BSS reported is a match on Capabilities”

DEFVAL { True }

::= { dot11RRMSiteReportEntry 5 }

dot11RRMSiteReportSuppRatesMatch OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“BSS reported is a match on Supported Rates”

DEFVAL { True }

::= { dot11RRMSiteReportEntry 6 }

# References

[1] 11-03-338r0, STA Statistics & RRM Information Distribution, Black/Sinivaara (Nokia)

[2] 11-03-0544r0, Revised BSS Neighbourhood Distribution, Black/Sinivaara (Nokia)

[3] Seamless Handoff of Mobile Terminal from WLAN to cdma2000 Network, Hemish Parikh (Univ of Missouri – Columbia) & Hemant Chaskar, Dirk Trossen, Govind Krishnamurthi (Nokia Research Centre), World Wireless Congress 2003

[4] IEEE Std 802.11k/D0.8 Specification for Radio Resource Measurement, TGk Editor

[5] 11-03-837r3, Radio Resource Measurements MIB