IEEE P802.11
Wireless LANs

Rectification for Block Ack Setup Procedures

Date:	November 11, 2003

Author:	Jennifer Bray
	Cambridge Silicon Radio
	Address: Science Park, Milton Road, Cambridge, CB4 OWH, UK
	Phone: +44 1223 692388
	Fax: +44 1223 692001
	e-Mail: Jennifer.Bray@csr.com

Isaac Lim Wei Lih

Panasonic
	e-Mail: wllim@psl.com.sg

Abstract

Recently there was a discussion on the 802.11 Technical email reflectors of Tge comment 498: “A rule should be specified to determine either originator or recipient initiate the BlockAck setup.” This contribution proposes normative changes to the Tge draft 5.1 to resolve the comment by requiring that the originator of the traffic to be sent under Block Ack is always the originator of the ADDBA request.

This implies that the Block Ack Initiator parameter is no longer required, as the initiator of the traffic is known to be the originator of the ADDBA request.

It also implies that if an Access Point grants a TSPEC which sets the downlink acknowledgement policy to Block Ack there is a requirement for the access point to initiate a Block Ack Negotiation by sending an ADDBA request, and if an Access Point grants a TSPEC which sets bidirectional policy to block ack there is a requirement for both QSTAs to send an ADDBA request.

Editorial notes already in Tge draft appear in bold italic Times New Roman font. Informative notes appear in normal Arial font, and normative text appears in normal Times New Roman font.  Open issues are highlighted using red text in normal Arial font, and begin with "OPEN ISSUE:".  Added text from Tgg and Tgh drafts appears red with a solid underline. Other added text appears in blue with a solid underline.  Deleted text due to Tgg and Tgh appears red with a strikethrough line. Other Deleted text appears in blue with a strikethrough line.

#### 7.3.1.16 Block Ack Parameter Set field

The Block Ack Parameter Set field is used in ADDBA frames to signal the parameters for setting up a Block Ack. The length of the Block Ack Parameter Set field is 2 octets. The Block Ack Parameter Set field is illustrated in Figure 33.6.

|  | B0 | B1 | B2 | B3 | B4 | B7 | B8 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reserved |  |  | Block Ack Policy | TID |  | Buffer Size |  |
| Octets: | 2 |  |  |  |  |  |  |  |

Figure 33.6 – Block Ack Parameter Set fixed field

The Block Ack Policy subfield is set to 1 for immediate Block Ack and 0 for delayed Block Ack. The Block Ack Policy subfield value assigned by the originator of the QoS data type frames is advisory.

TID contains the value of the TC or TS for which the Block Ack is being requested.

The Buffer Size indicates the number of buffers of size 2304 octets available for this particular TID.

In an ADDBA request frame, the Buffer Size subfield is intended to provide guidance for the frame receiver to decide its re-ordering buffer size, and is advisory only. If the Buffer Size subfield is set to 0, it implies that the transmitter of the request frame has no information to specify its value.

In an ADDBA response framewith the ADDBA Result Code is “Success”, the Buffer size is set to a value of at least 1.

7.4.3.1 ADDBA request

An ADDBA request is sent by  a QSTA to request permission to transmit data frames under Block Ack to another QSTA. . The frame body of an ADDBA request frame contains the information shown in Table 20.16.

Table 20.16 – ADDBA request frame body

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | Action |
| 3 | Dialog Token |
| 4 | Reserved (1 octet) |
| 5 | Block Ack Parameter Set |
| 6 | Block Ack Timeout Value |

The Category field is set to 3 (representing Block Ack).

The Action field is set to 0 (representing ADDBA request).

The Dialog Token field is set to a non-zero value chosen by the STA.

The Block Ack Parameter Set field is defined in 7.3.1.16.

The Block Ack Timeout value is defined in 7.3.1.17.

## 9.10 Block acknowledgment

### 9.10.1 Introduction (Informative)

The Block Acknowledgement (Block Ack) mechanism allows a block of QoS Data MPDUs to be transmitted, each separated by a SIFS period, between two QSTAs. The mechanism is for improving the channel efficiency by aggregating several acknowledgements into one frame. There are two types of Block Ack mechanisms: immediate and delayed. Immediate Block Ack is suitable for high-bandwidth, low latency traffic while the delayed Block Ack is suitable for applications that tolerate moderate latency.  In this clause, the QSTA with data to send using the Block ACK mechanism is referred to as the originator and the receiver of that data as the recipient.

The Block Ack mechanism is initialized by an exchange of ADDBA request/response frames. The ADDBA request frame is transmitted by the originator only. The ADDBA response frame is transmitted by the recipient only. After initialization, blocks of QoS data type frames can be transmitted. A block may be started within a polled TXOP or by winning EDCA contention. The MPDUs within this block usually fit within a single TXOP and are all separated by a SIFS. The number of frames in the block is limited, and the amount of state that must be kept by the recipient is bounded. The MPDUs within the block of frames are acknowledged by a BlockAck control frame, which is requested by a BlockAckReq control frame.

The Block Ack mechanism does not require setting up of a TS; however QSTAs using the TS facility may choose to signal their intention to use Block Ack mechanism for the scheduler’s consideration in assigning TXOPs.  Acknowledgements of frames belonging to the same TID, but transmitted during multiple TXOPs, may also be combined into a single Block Ack frame.  This mechanism allows the originator flexibility regarding the transmission of Data MPDUs. The originator may split the block of frames across TXOPs, separate the data transfer and the block acknowledgement exchange and interleave blocks of MSDUs for different TIDs or RAs.

Figure 62.8 illustrates the message sequence chart for the set up, data and block acknowledgement transfer and the tear down of Block Ack mechanism and are discussed in detail in the following subclauses.

ADDBA Request

ACK

ADDBA Response

ACK

QoS Data MPDU

BlockAckReq

BlockAck

DELBA Request

ACK

Originator

Recipient

(a) Setup

(b) Data & Block Ack

(c) Tear Down

multiple

times

Figure 62.8 – Message Sequence Chart for Block Ack Mechanism: (a) Set up, (b) Data and acknowledgement transfer and (c) Tear down

### 9.10.2 Set up and modification of the Block Ack parameters

A QoS data type frame originating QSTA that intends to use the Block Ack mechanism should first check if the intended peer QSTA is capable of participating in Block Ack mechanism by discovering and examining its Block Ack capability bit. If the intended peer QSTA is capable of participating, the originator sends an ADDBA request frame indicating the TID for which the Block Ack is being set up. The Block Ack policy and the Buffer Size fields in ADDBA request frame are advisory and may be changed by the recipient. The receiving QSTA, which is the intended peer QSTA, shall respond by an ADDBA response frame. The receiving QSTA has the option of accepting or rejecting the request. When the QSTA accepts, it indicates the type of Block Ack and the number of buffers that it shall allocate for the support of this block. If the QSTA rejects the request, then the originator shall not use the Block Ack mechanism.

If the Block Ack mechanism is being set up for a TS, bandwidth negotiation (using ADDTS request and response QoS Action management frames) should precede the set up of the Block Ack mechanism.

Once the Block Ack exchange has been set up, Data and acknowledgements are transferred using the procedure described in clause 9.10.3.

### 9.10.3 Data and acknowledgement transfer

After setting up for the Block exchange following the procedure in 9.10.2, the originator may transmit a Block of QoS data type frames separated by SIFS period, with the total number of frames not exceeding the Buffer Size subfield in the associated ADDBA response frame. Each of the frames shall have the Ack policy subfield in the QoS Control set to “Block Acknowledgement”. The RA field of the frames shall be the recipient's unicast address. The originator requests acknowledgement of outstanding QoS data type frames by sending a BlockAckReq frame. The recipient shall maintain a Block Ack record for the block.

### 10.3.14 Block Ack

This mechanism supports the initiation (or modification) and termination of Block Ack.

The primitives used for this mechanism are called Block Ack primitives, which include MLME-ADDBA.xxx and MLME-DELBA.xxx primitives, where xxx denotes request, confirm, indication, or response. Each primitive contains parameters that correspond to a Block Ack Action frame body.  Requests and responses may cause these frames to be sent.  Confirms and indications are emitted when an appropriate Block Ack action frame is received.

#### 10.3.14.1 MLME-ADDBA.request

##### 10.3.14.1.1 Function

This primitive requests the initiation (or modification) of Block Ack with a peer MAC entity.

##### 10.3.14.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDBA.request	(

PeerQSTAAddress,

DialogToken,

TID,

BlockAckPolicy

BufferSize,

BlockAckTimeout,

ADDBAFailureTimeout

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerQSTAAddress | MAC Address | N/A | Specifies the address of the peer MAC entity with which to perform the Block Ack initiation (or modification). |
| DialogToken | Integer | 0-255 | Specifies a number unique to the Block Ack action primitives and frames used in defining or deleting a Block Ack. |
| TID | Integer | 0-15 | Specifies the TID of the data. |
| BlockAckPolicy | Enumeration | Immediate, Delayed | Specifies the Block Ack Policy |
| BufferSize | Integer | 0-127 | Specifies the number of MPDUs that can be held in its buffer (in the case of originator SME issuing this primitive) or that can be in the block for the specified TID (in the case of recipient SME issuing this primitive). |
| BlockAckTimeout | Integer | 0-65535 | Specifies the number of TUs without a frame exchange between peers after which the Block Ack is considered to be torn down. |
| ADDBAFailureTimeout | Integer | Greater than or equal to 1 | Specifies a time limit (in TU) after which the BA set up procedure is terminated. |

##### 10.3.14.1.3 When generated

This primitive is generated by the SME at a QSTA to request initiation (or modification) of Block Ack with the specified peer MAC entity.

##### 10.3.14.1.4 Effect of receipt

The QSTA shall send the ADDBA request frame to the specified peer MAC entity.

#### 10.3.14.2 MLME-ADDBA.confirm

##### 10.3.14.2.1 Function

The primitive reports the results of initiation (or modification) of the Block Ack attempt with the specified peer MAC entity

##### 10.3.14.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDBA.confirm	(

PeerQSTAAddress,

DialogToken,

TID,

ResultCode,

BlockAckPolicy,

BufferSize,

BlockAckTimeout

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerQSTAAddress | MAC Address | N/A | Specifies the address of the peer MAC entity with which the Block Ack initiation (or modification) was attempted. This value must match the PeerQSTAAddress parameter specified in MLME-ADDBA.request. |
| DialogToken | Integer | 0-255 | Specifies a number unique to the Block Ack action primitives and frames used in defining or deleting a Block Ack. This value must match the Dialog Token parameter specified in MLME-ADDBA.request. |
| TID | Integer | 0-15 | Specifies the TID of the data. This value must match the TID specified in MLME-ADDBA.request |
| ResultCode | Enumeration | SUCCESS, REFUSED, TIMEOUT | Indicates the result of the corresponding MLME-ADDBA.request. |
| BlockAckPolicy | Enumeration | Immediate, Delayed | Specifies the Block Ack Policy |
| BufferSize | Integer | 0-127 | Specifies the maximum number of MPDUs in the block for the specified TID (in the case of originator SME issuing this primitive) or that can be held in its buffer (in the case of recipient SME issuing this primitive). |
| BlockAckTimeout | Integer | 0-65535 | Specifies the number of TUs without a frame exchange between peers after which the Block Ack is considered to be torn down. |

##### 10.3.14.2.3 When generated

This primitive is generated by the MLME as a result of an MLME-ADDBA.request to indicate the results of that request.

##### 10.3.14.2.4 Effect of receipt

The SME is notified of the results of the Block Ack initiation (or modification).

#### 10.3.14.3 MLME-ADDBA.indication

##### 10.3.14.3.1 Function

This primitive reports the initiation (or modification) of Block Ack by a peer MAC entity.

##### 10.3.14.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDBA.indication	(

PeerQSTAAddress,

DialogToken,

TID,

BlockAckPolicy

BufferSize,

BlockAckTimeout

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerQSTAAddress | MAC Address | N/A | Specifies the address of the peer MAC entity that requested the Block Ack initiation (or modification). |
| DialogToken | Integer | 0-255 | Specifies a number unique to the Block Ack action primitives and frames used in defining or deleting a Block Ack. |
| TID | Integer | 0-15 | Specifies the TID of the data. |
| BlockAckPolicy | Enumeration | Immediate, Delayed | Specifies the Block Ack Policy |
| BufferSize | Integer | 0-127 | Specifies the number of MPDUs that can be held in peerQSTAAddress buffer (in case it is the originator) or that can be in the block for the specified TID (in case it is the recipient). |
| BlockAckTimeout | Integer | 0-65535 | Specifies the number of TUs without a frame exchange between peers after which the Block Ack is considered to be torn down. |

##### 10.3.14.3.3 When generated

This primitive is generated by the MLME as a result of receipt of a Block Ack initiation (or modification) by the specified peer MAC entity in the form of an ADDBA request frame.

##### 10.3.14.3.4 Effect of receipt

The SME is notified of the initiation (or modification) of the Block Ack by the specified peer MAC entity.

#### 10.3.14.4 MLME-ADDBA.response

##### 10.3.14.4.1 Function

The primitive responds to the initiation (or modification) by a specified peer MAC entity.

##### 10.3.14.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-ADDBA.response	(

PeerQSTAAddress,

DialogToken,

TID,

ResultCode,

BlockAckPolicy,

BufferSize,

BlockAckTimeout

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerQSTAAddress | MAC Address | N/A | Specifies the address of the peer MAC entity that attempted the Block Ack initiation (or modification). This value must match the PeerQSTAAddress parameter specified in MLME-ADDBA.indication. |
| DialogToken | Integer | 0-255 | Specifies a number unique to the Block Ack action primitives and frames used in defining or deleting a Block Ack. This value must match the Dialog Token parameter specified in MLME-ADDBA.indication. |
| TID | Integer | 0-15 | Specifies the TID of the data. This value must match the TID specified in MLME-ADDBA.indication. |
| ResultCode | Enumeration | SUCCESS, REFUSED, INVALID PARAMETERS, TIMEOUT | Indicates the result of the corresponding MLME-ADDBA.indication. |
| BlockAckPolicy | Enumeration | Immediate, Delayed | Specifies the Block Ack Policy. Undefined when ResultCode is REFUSED. |
| BufferSize | Integer | 0-127 | Specifies the number of MPDUs that can be in Block for the specified TID. Undefined when ResultCode is REFUSED. |
| BlockAckTimeout | Integer | 0-65535 | Specifies the number of TUs without a frame exchange between peers after which the Block Ack is considered to be torn down. |

##### 10.3.14.4.3 When generated

This primitive is generated by the MLME as a result of an MLME-ADDBA.indication to initiate Block Ack by the specified peer MAC entity.

##### 10.3.14.4.4 Effect of receipt

The primitive causes the MAC entity to send an ADDBA response frame to the specified peer MAC entity.

## 11.4.4 TS setup

Figure 68.2 shows the sequence of messages occurring at a TS setup. This message sequence in this figure and in the subsequent figures does not show the ACK.

The non-AP QSTA SME decides that a TS needs to be created.  How it does this, and how it selects the TSPEC parameters, is beyond the scope of this specification. The SME generates an MLME-ADDTS.request containing a TSPEC. A TSPEC may also be generated autonomously by the MAC without any initiation by the SME. However, if a TSPEC is generated subsequently by the SME, the TSPEC containing the same TSID generated autonomously by the MAC shall be overridden. If one or more TSPECs are initiated by the SME, the autonomous TSPEC shall be terminated.

The non-AP QSTA MAC transmits the TSPEC in an ADDTS request in the corresponding QoS Action frame or (re)association request frame to the HC and starts a response timer called ADDTS timer of duration dot11ADDTSResponseTimout.

The HC MAC receives this management frame and generates an MLME-ADDTS.indication primitive to its SME containing the TSPEC.

The SME in the HC decides whether to admit the TSPEC as specified, or refuse the TSPEC, or not admit but suggest an alternative TSPEC and generates an MLME-ADDTS.response primitive containing the TSPEC and a status value.  The contents of the TSPEC and status field contain values specified in 10.3.11.4.2.

The HC MAC transmits an ADDTS response in the corresponding QoS Action frame or (re) association response containing this TSPEC and status.

The non-AP QSTA MAC receives this management frame and cancels its ADDTS timer.  It generates an MLME-ADDTS.confirm to its SME containing the TSPEC and status.

The non-AP QSTA SME decides whether the response meets its needs or not.  How it does this is beyond the scope of this specification.   In the “SUCCESS” Status cases, the TS is in the active state.  Otherwise, the whole process can be repeated using the same TSID and Direction and a modified TSPEC until the non-AP QSTA SME decides that the granted TXOP is adequate or inadequate and cannot be improved.

Figure 68.2 – TS Setup

The parameters that are set for a TS can be modified in a similar manner, when such a request is generated by the SME through ADDTS.request. When a request for the modification of the TS parameters is accepted by the HC, it shall reset both the short inactivity interval and the inactivity interval timers.

If the AP/HC grants a TXOP for an ADDTS request with Ack Policy set to “Block Acknowledgement” and Direction set to either “Downlink” or “Bidirectional” then it shall initiate a Block Ack negotiation by sending an ADDBA request to the STA which sent the ADDTS request. If a non-AP QSTA is granted a TXOP for an ADDTS request with Ack Policy set to “Block Acknowledgement” and Direction set to either “Uplink” or “Bidirectional” then it shall initiate a Block Ack negotiation by sending an ADDBA request to the AP which granted the TXOP.

## 11.5 Block Ack operation

Block Ack may be set up at the MAC or by the initiation of SME. The set up and deletion of Block Ack at the initiation of the SME is described in this subclause. In this clause, the QSTA with QoS type data frames to send is referred to as the originator, and the receiver of that data is referred to as the recipient.

11.5.1 Set up and modification of the Block Ack parameters

The procedures for setting up and modifying the Block Ack parameters are described in 11.5.1.1 and 11.5.1.2 respectively and illustrated in Figure 68.6.

Figure 68.6 – Block Ack set up

#### 11.5.1.1 Procedure at the originator

Upon receipt of MLME-ADDBA.request, a QSTA that intends to transmit QoS type data frames underthe Block Ack mechanism shall set up the Block Ack using the following procedure.

a)	Check if the intended peer QSTA is capable of participating in the Block Ack mechanism by discovering and examining its “Block Ack” capability bit. If the recipient is capable of participating, the originator sends an ADDBA frame indicating the TID and the Buffer Size.

b)	If an ADDBA response frame is received with the matching Dialog Token and the TID, and with a Result Code set to a value of “SUCCESS”, the QSTA has established Block Ack mechanism with the recipient QSTA and the MLME shall issue an MLME-ADDBA.confirm indicating the successful completion of the Block Ack set up.

c)	If an ADDBA response frame is received with the matching Dialog Token and the TID, and with a Result Code set to a value other than “SUCCESS”, the QSTA has not established Block Ack mechanism with the recipient QSTA and the MLME shall issue an MLME-ADDBA.confirm indicating the failure of the Block Ack set up.

d) 	If there is no response from the recipient within dot11ADDBAFailureTimeout, the QSTA has not established Block Ack mechanism with the recipient QSTA and the MLME shall issue an MLME-ADDBA.confirm with ResultCode “TIMEOUT”.

#### 11.5.1.2 Procedure at the recipient

A recipeint shall operate as follows in order to support Block Ack initialization and modification.

a)	Whenever an ADDBA request frame is received from another QSTA, the MLME shall issue an MLME-ADDBA.indication.

b)	Upon receipt of the MLME-ADDBA.response, the QSTA shall respond by an ADDBA response frame with a Result code as defined in 7.4.3.2.

1)	If the Result code is “SUCCESS” the BlockAck is considered to be established with the originator. Contained in the frame are the type of Block Ack and the number of buffers that have been allocated for the support of this Block.

2)	If the Result code is “REFUSED” the Block Ack is not considered to have been established.

### 11.5.2 Tear down of the Block Ack mechanism

The procedure at the two QSTAs is described in 11.5.2.1 and 11.5.2.2, and illustrated in Figure 68.7.

Figure 68.7 – Block Ack Deletion

#### 11.5.2.1 Procedure at the initiator of the Block Ack teardown

Upon receipt of an MLME-DELBA.request, the QSTA MAC/MLME shall tear down the Block Ack using the following procedure:

a)	The QSTA shall transmit a DELBA frame.

b)	Upon the receipt of an acknowledgement to the DELBA frame, the MLME issues a MLME-DELBA.confirm.

#### 11.5.2.2 Procedure at the recipient of the DELBA frame

A QSTA shall issue a MLME-DELBA.indication with the parameter Reason having a value of “REQUESTED” when a DELBA frame is received.

### 11.5.3 Error recovery upon a peer failure

Every station shall maintain an inactivity timer for every negotiated Block Ack set up. The inactivity timer is reset whenever MPDUs corresponding to the TID for which the Block Ack policy is set are received, regardless of the ack policy that is used for the transmission of that frame. The inactivity timer is not reset whenever MPDUs corresponding to other TIDs are received. When a timeout of Block Ack Timeout is detected, the QSTA shall send a DELBA frame to the peer QSTA, and shall issue a MLME-DELBA.indication with the Reason parameter having a value of “TIMEOUT”. The procedure is illustrated in Figure 68.8.

Figure 68.8 – Error Recovery by the Receiver upon a peer failure

For Buffer size, the recipient of data advertises a single scalar number that is the number of maximum-size fragment buffers available for bursting. Every buffered MPDU will consume one of these buffers regardless of whether the frame contains a whole MSDU or a fragment of an MSDU. In other words, ten maximum-size unfragmented MSDUs will consume the same amount of buffer space at the recipient as 10 small fragments.

The delayed Block Ack mechanism is primarily intended to allow existing implementations to use this feature with minimal hardware changes and also to allow inexpensive implementations that would use the processing power on the host.