IEEE P802.11
Wireless LANs

Title

Date:	November 13, 2003

Author:	Amjad Soomro
	Philips
	345 Scarborough Road, Briarcliff Manor, NY 10510
	Phone: (914) 945 6319
	Fax: (914) 945 6580
	e-Mail: amjad.soomro@philips.com

Abstract

This document has proposed normative text to for a QAP to indicate the type of service period it will provide to a QSTA.

Instruct the editor to modify the referred paragraph in subclause 11.2.3 as follows:

A non-AP QSTA may signal its desire to utilize APSD as the power-save mode delivery method for individual traffic streams by setting to 1 the APSD sub-field of the TS info field contained in the TSPEC element.  The non-AP QSTA shall request scheduled service periods by sending a TSPEC element that contains a TS Info Field with its Schedule bit set to 1.  The non-AP QSTA shall request unscheduled service periods by sending a TSPEC element that contains a TS Info Field with its Schedule bit set to 0. The QAP shall respond with a TS Info Field in TSPEC with the Schedule bit set to 0 or 1 indicating if the QAP will use scheduled or unscheduled service periods when servicing that QSTA.  If scheduled service periods  are indicated to be used by the QAP then the QAP shall return Schedule element indicating whether the requested wakeup time can be accommodated by the QAP, and if not a new or modified wakeup schedule shall be indicated by the Service Interval and the Service Start Time.  If unscheduled service periods  are indicated to be used by the QAP then the  QAP shall not return a Schedule element in the response.