IEEE P802.11
Wireless LANs

IEEE 802.11 TGn Comparison Criteria 
(Phy-related 4.6 sections working document)

Date:	January 12, 2003

Author:	Original 03-0813: Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +44 1223 763457
	e-Mail: adrian.p.stephens@intel.com

04-0053 section 4.6 owner: Jeff Gilbert
Atheros Communications
529 Almanor Ave
408-773-5261
email: gilbertj@atheros.com

Abstract

This document defines comparison criteria that must be addressed by any proposal claiming that it is a complete proposal in response to the IEEE 802.11 TGn call for proposals.

# Contributors

(This will grow to reflect those providing explicit email contributions / review comments of this document. please let Adrian know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |
| Adrian P Stephens | Intel | 15 JJ Thompson Avenue, Cambridge CB3 0FD, United Kingdom | +44 1223 763457 |  | adrian.p.stephens@intel.com |
| Pieter-Paul Giesberts | Agere |  |  |  |  |
| Bruno Jechoux | Mitsubishi | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | jechoux@tcl.ite.mee.com |
| Jeff Gilbert | Atheros | 529 Almanor Ave Sunnyvale, CA 94085 | +1 408-773-5261 |  | gilbertj@atheros.com |
| Bjorn Bjerke | Qualcomm | 9 Damonmill Sq., Suite 2A, Concord, MA 01742, USA | +1 781-276-0912 | +1 781-276-0901 | bbjerke@qualcomm.com |
| Herve Bonneville | Mitsubishi Electric | 1 allée de Beaulieu, CS 10806, 35708 Rennes cedex 7, France | +33 (0)2 23 45 58 58 |  | bonneville@tcftcl.ite.mee.com |
| Rahul Malik | Panasonic | Blk 1022 Tai Seng Ave. #06-3530 Tai Seng Industrial Estate, Singapore 534415 | +65 6550-5482 | +65 6550-5459 | rahul@psl.com.sg |
| Frans Hermodsson | TeliaSonera |  | +46 40 10 51 97 | +46 40 30 70 29 | Frans.Hermodsson@teliasonera.com |
| David Bagby | Calypso Consulting |  |  |  |  |
| Sean Coffey | Texas Instruments |  |  |  |  |
| Günter Kleindl | SIEMENS AG Austria |  | +43 51707 35738 |  | guenter.kleindl@siemens.com |

# Revision History (First is history of original 03-0814 then of new phy-centric 04-0053)

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Empty skeleton submitted to satisfy the automatic document system | 20 October 2003 |
| R1 | APS initial contribution | 31 October 2003 |
| R2 | Updated following FRCC telecon on 4 Nov 2003 to add proposed comparison criteria evaluating degree of support for HT usage models | 10 November 2003 |
| R3 | Updates made during 13 November 2003 TGn session while editing the FR document. Added revision history. Updated with changes made at the FRCC telecon on 18 November 2003. | 19 November 2003 |
| R4 | Merged in comments from: - Pieter-Paul Giesberts (Agere) - Bruno Jechoux (Mitsubishi) Reorganised existing CC into sections according to Pieter-Paul’s contribution. + APS comments on the above | 27 November 2003 28 November 2003 |
| R5 | Updated at 2 December 2003 FRCC telecon. Includes changes agreed at the telecon. Includes proposed definition of maturity by Jeff Gilbert. | 2 December 2003 |
| R6 | Merged in multiple comments and changes - Bjorn Bjerke - Herve Bonneville - Rahul Malik - Frans Hermodsson - David Bagby - Sean Coffey - Jeff Gilbert - Guenter Kleindl Added in "priority" column and give initial priority assignment. | 15 December 2003 |
| R7 04-0053 R0 | Updated during the Dec 16, 2003 telecon of the FRCC. Reorganized phy section by pulling out impairments and turned into a new document since it is not an official continuation of 03-814 but will be merged into 03-0814 once completed. | 16 December 2003 6 January 2004 |

# Introduction

## Purpose of document

A proposal submitted for consideration under the 802.11 TGn selection process [1], and declared to be complete is required to meet the functional requirements defined in [2] and to disclose results according to the comparison criteria defined in this document.

## Form of Disclosure

A proposal shall disclose its results using the template or form defined below in section 5..

It is TBD whether a disclosure has to meet all of the comparison criteria.

## Relationship to Functional Requirements

The main purpose of the comparison criteria is to define metrics to enable comparison of TGn roposals.

In addition, the functional requirements [2] may define that specific criteria meet specific values.   This document defines how those measurements are to be made and reported so that compliance to the functional requirements can be evaluated.

As such, the functional requirements [2] are dependent on this document, but not the other way around.

## Relationship to Simulation Scenarios

The IEEE 802.11 TGn FRCC (Functional Requirements and Comparison Criteria Special Committee) has defined usage models from which simulation scenarios have been created [3].

These simulation scenarios are intended to define the input to a simulation in sufficient detail so that the simulation results from different proposals can be meaningfully compared.

This document may define certain criteria given the conditions defined in a certain simulation scenario.  As such certain parts of this  document are dependent on the simulation scenarios contained in [3], but not the other way around.

## Requirements of the Comparison Criteria Document

(This section may be removed at a later date.  It is really only relevant while we are writing this document)

The criteria defined here:

- Shall be defined unambiguously

- Can be obtained from a reasonable simulation environment, or obtained by examination of the proposed submission

- Are compliant to the 802.11 HT PAR [5] and 5C [6]

Ideally, most criteria should be single values.

In some cases (e.g. transport delay), a metric might need to be presented as a graph or table.  The definition of the metric shall include the exact form in which it is to be presented.

## Summary Status of  11-03-813r5

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet discussed by FRCC | 39 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC |  |
| Number of Comparison Criteria approved by TGn |  |
| Total number of Comparison Criteria | 44 |

## Summary Status of  11-03-813r6

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet discussed by FRCC | 75 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC | 5 |
| Number of Comparison Criteria approved by TGn |  |
| Total number of Comparison Criteria | 80 |

## Summary Status of  11-03-813r7

This document has not been approved by IEEE 802.11 TGn.

| Number of Comparison Criteria not yet approved by FRCC | 74 |
| --- | --- |
| Number of Comparison Criteria approved by FRCC | 6 |
| Number of Comparison Criteria approved by TGn | 0 |
| Total number of Comparison Criteria | 80 |

# Definitions

| Term | Definition |
| --- | --- |
| Goodput | Goodput is defined by totaling the number of bits in MSDUs indicated at the MAC DATA SAP, and dividing by the simulation duration (s). |

# Additional Disclosures

(This section contains requirements for additional information that shall be disclosed with a proposal)

| Number | Name | Definition | Status of this AD |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. |  | PPG (Agere) proposal Agreed 6/0. |  |  |

# Comparison Criteria

| Number | Name | Definition | Simulation Scenario | Status of this CC | Notes | Pri |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ## General |  |  |  |  |  |  |  |
| CC1 | Regulatory compliance | Proposal shall state the regulatory domains it is intended to be compliant with. The proposal shall state its intended regulatory compliance with at least the following domains: USA, Japan, Europe, China. |  | Suggestion made at TGn session. Vote 14/4. with TeliaSonera modification “EU” removed unanimous | Definition of EU is needed. Compliance according to regulatory body within each domain, FCC, CEPT, ARIB etc. | H |  |
| CC2 | Regulatory non-compliance | Proposal shall identify any known problems with existing regulations The proposal shall state any known problems with regulatory compliance with at least the following domains: USA, Japan, Europe, China. |  | Suggestion made at TGn session, split out by Joe Levy from previous. Vote 7/0 unanimous |  | H |  |
| ## Marketability |  |  |  |  |  |  |  |
| W10 CC3 | List of mandatory usage models covered at HT rate | List the mandatory usage models for which 100 Mbps throughput can be achieved. |  | Based on WFA proposal. unanimously |  | M |  |
| W11 CC4 | Cost of Overall Solution that addresses all market segments for both AP and Station | Estimated cost, relative to an 802.11a or g device, of a device capable of meeting the mandatory features of proposal. |  | Based on WFA proposal unanimous |  | M |  |
| CC5 | RF/IF complexity | Give an indication of the complexity of the RF/IF part of the solution, relative to current 802.11a PHYs. |  | PPG (Agere) proposal | Analogue vs RF/IF (i.e. what if IF is digital)? Difficult to quantify in a straightforward manner. Proposal: delete criterion as it stands here. The answers to the specific items under “RF Characteristics” will give sufficient indication of the RF/IF complexity What is a suitable metric for RF/IF complexity? | M |  |
| CC6 | Baseband processing complexity | Give an indication of the baseband processing complexity, relative to current 802.11a PHYs (gate counts, MIPS, etc.). |  | PPG (Agere) proposal |  | M |  |
| CC7 | MAC processing complexity | Give an indication of the MAC processing complexity, relative to current 802.11-1999 and 802.11e implementations |  | RM proposal |  | M |  |
| CC8 | Maturity of solution and technology | Give an indication of the maturity of the solution and its technology. For example, reference to similar technology in successful products. Reference articles in the literature. Alternative proposal by Jeff Gilbert: Give an indication of the maturity of the solution and its technology by listing relevant publications, presentations and implementations within and outside of the IEEE. The burden will be on the proposer to list sufficient relevant publications and implementations to convince the body of the maturity of the key technology of the proposal. Alternative by Pieter-Paul: Give an indication of the maturity of the solution and its technology by listing relevant publications, presentations and production level implementations within and outside of the IEEE. |  | PPG (Agere) proposal No consensus 9/7 PPG to produce better language. Augmented by Jeff Gilbert (Atheros) PPG rewording | Maturity needs precise definition | M |  |
| CC9 | Power consumption estimate | Give an estimate of the power consumption in TX, RX (decoding packet), IDLE (listening but no packet). Specify model and assumptions. To specify for a mode of operation that achieves 100Mbps goodput for the point-point use case. Tx consumption measured at total tx power = 20dBm. Alternate: At 100Mbps aggregated goodput for the use cases in one of the defined mandatory multi-user simulation scenario 4 or 6. Estimate average power consumption per application. Otherwise use a point-to-point use case. Estimate average power consumption per application. Tx consumption measured at total tx power = 20dBm. |  | PPG (Agere) proposal 7/0 with TeliaSonera modification “To specify for a mode of operation that achieves” and “the point-to-point” removed |  | M |  |
| CC10 | Power consumption | Total power consumed (mW) by a realistic implementation (define implementation technologies). TBD - Note, For consistency, this document will need to define which parts of an implementation are included in the power consumption measurements. Show results separately for: - Signal Acquisition - Receiving - Transmitting (at Tx power = 20dBm) Show results for highest achievable PHY rate, 54Mbps (or closest rate) and 6 Mbps (or closest rate). | If relevant, choose one channel model from [4]. | APS proposal |  | M |  |
| ## Interoperability and Coexistence |  |  |  |  |  |  |  |
| CC11 | Backward compatibility and interoperability with 802.11a and h | Identify the means of achieving backward compatibility and interoperability with 802.11a and h. Also indicate for which of the modes and data rates that this is applicable. |  | PPG (Agere) proposal with TeliaSonera modification | Should reference technical specification and perhaps summarise? Does “backwards compatability” have the same definition as in the FR – If so, it should be stated here Is there any difference between backward compatability and interoperability? | M |  |
| CC12 | Backward compatibility and interoperability with 802.11g | Identify the means of achieving backward compatibility and interoperability with 802.11g. Also indicate for which of the modes and data rates that this is applicable. |  | PPG (Agere) proposal | Should reference technical specification and perhaps summarise? | M |  |
| CC13 | Impact on options in previous 802.11 standards | List the impact on the options of other 802.11 standards, such as .11a/g/b or .11e. alternative: List the impact on the following optional features of other 802.11 standards: a) Extended rate sets (802.11a/b/g) b) Transmit power control (802.11a/b/g) c) Block ACK (802.11e) d) Automatic power save delivery (802.11e) e) Direct link protocol (802.11e) “Impact” can have the following values: a) none, b) eliminates the optional feature, c) affects the functionality and/or the performance of the optional feature (specify how) |  | PPG (Agere) proposal | Impact needs definition. Such as needs definition. Suggestion: identify the 4-5 most important/significant optional features most likely to be found in implementations, and indicate level of impact on these (see suggested features on the left) | L |  |
| CC14 | Legacy Share | Two measures are made. Firstly the throughput (T1) of a fully backlogged legacy STA transmitting to its AP. Two measures are made. Firstly the goodput (T1) of a fully backlogged legacy STA (STA1) transmitting to its legacy AP (AP1). Secondly the goodput (T2) of the same legacy STA when a fully-backlogged co-channel co-incident HT STA (STA2) and its HT AP (AP2) are introduced. The legacy share is defined as (T2 / T1). | TBD geometry. TBD channel model. 1500B MSDU size. The geometry is defined based on a 10 m x 20 m rectangle. AP1 and STA1 are located at the upper and lower left hand corners, respectively (10 m apart), while AP2 and STA2 are located at the upper and lower right hand corners, respectively. Channel model: B – NLOS Both links are operating at their maximum rates for the given distance. median MSDU size or mixed size, according to Cisco measured data scenario 4 Usage Models. | APS proposal Telesonaria modification | The proposed geometry is intended to reflect a situation where two co-channel networks operate in close proximity to each other, such as in an apartment building | L |  |
| W9 CC15 | Sharing of medium with legacy devices | Throughput of legacy device in HT network. Compare to throughput of same legacy device in similar conditions in an all-legacy network. |  | Based on WFA proposal |  | L |  |
| CC16 | Legacy Impact | Two measures are made. Firstly the goodput (T1) of a fully backlogged HT STA (STA1) transmitting to its HT AP (AP1). Secondly the goodput (T2) of the same HT STA when a fully-backlogged co-channel co-incident legacy STA (STA2) and its legacy AP (AP2) are introduced. The legacy impact is defined as ((T1-T2) / T1)). | 1500B MSDU size The geometry is defined based on a 10 m X 20 m rectangle. AP1 and STA1 are located at the upper and lower left hand corners, respectively (10 m apart), while AP2 and STA2 are located at the upper and lower right hand corners, respectively. Channel model: B – NLOS Both links are operating at their maximum rates for the given distance. |  | Criterion originally proposed by Mike Moreton The proposed geometry is intended to reflect a situation where two co-channel networks operate in close proximity to each other, such as in an apartment building | L |  |
| W12 CC17 | SAP compatibility | Required changes to the MAC/SAP interface: – List of major functional areas affected – Approx. multiplier of code complexity associated with MAC/SAP baseline interface (1X, 2X, etc.) |  | Based on WFA proposal |  | L |  |
| ## Coverage of Usage Models |  |  |  |  |  |  |  |
| CC18 | HT Usage Models Supported (non QoS) | This metric relates to the ability of the proposal to support the non-QoS application service level of each usage model, as defined by its simulation scenario. (Total TCP Goodput) for each mandatory simulation scenario Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | All mandatory simulation scenarios Note, this is measured with QoS flows turned on. | APS TBD proposal resulting from FRCC telecon on 4 Nov 2003. 4/0 Reworded dec 16th | Would the QoS flows be turned off as in 03/835r1 during the evaluation of this CC? Would prefer that they be turned on. | H |  |
| CC19 | HT Usage Models Supported (QoS) | This metric relates to the ability of the proposal to support the QoS application service level of each usage model, as defined by its simulation scenario. For each flow (i.e. application at a station) with a delay and PLR specification, the proposal shall report the packet loss rate (defined below) and compare to the maximum specified packet loss rate. The proposal shall also report the number of these flows that are less than or equal to the PLR specified in [2] . For the purpose of this criterion, packet loss rate (PLR) is defined by the number of MSDUs that are lost in transit, or whose MAC SAP to MAC SAP transit time exceeds the maximum delay for the application specified in [2] divided by the total number of MSDUs transmitted during the simulation for that application at that STA. | All mandatory simulation scenarios | APS proposal resulting from FRCC telecon on 4 Nov 2003. 0/0 TBD | As in the “non-QoS” CC, would prefer that this measurement be done with the TCP flows turned on – Allows one simulation for both the CCs | H |  |
| ## MAC Related |  |  |  |  |  |  |  |
| ### Performance Measurements at the MAC SAP |  |  |  |  |  |  |  |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | The metric is defined by the sum of goodput across all STA in a BSS. Note, a flow that transits through an AP (i.e. is relayed within the BSS) is not counted as goodput at the AP. A flow that terminates at the AP is counted as goodput. | All mandatory simulation scenarios | APS proposal Started discussion 18 Nov 2003 Reworded on dec 16th. | Use the same language as in the definition of “goodput” in Section 2. Delete notes, as they are redundant. | H |  |
| CC21 | Throughput / Area. | Presented as a curve of Throughput (bps) vs area (m2). Throughput is defined by totaling the number of bits in MSDUs successfully transmitted and dividing by the simulation duration (s) | Present results for STAs in mandatory multi user simulation scenarios (one may be an AP), one is fully backlogged with median MSDU size or mixed size, according to Cisco measured data scenario 4 in [3] addressed to the other. Vary the distance between two of the STA. Report results for all the channel models described in [4]. TBD scenario from [3]. | TeliaSonera proposal | Use a system test simulation scenario in [3]. For example scenario 4 and 6 | M |  |
| CC22 | HT Area | The area value where the Throughput / Area curve passes through 100Mbps. | Report results for all the channel models described in [4]. | TeliaSonera proposal |  | M |  |
| CC23 | 10Mbps Area | The area value where the Throughput / Area curve passes through 10Mbps. | Report results for all the channel models described in [4]. | TeliaSonera proposal |  | M |  |
| CC24 | MAC Efficiency | MAC efficiency is defined as the the aggregate goodput at the MAC-SAP divided by the average physical layer data rate. Proposal for alternative wording: MAC Efficiency is defined as the ratio of the aggregate time occupied by all successful (and unsuccessful ?) MPDU transmissions on the channel (excluding Preamble and PLCP header) to the total simulated time. Note: This definition is valid for the case when there is always at least one STA with available MSDUs at the MAC-SAP.(throughout the duration of the simulation) | All mandatory simulation scenarios | BJC, HBE proposal |  | L |  |
| CC25 | Scalability | Provide analysis of MAC efficiency with PHY data rates increased by a) 50% and b) 100% relative to the maximum PHY rate of the proposal. Give efficiency numbers as percentages of the PHY data rate. |  |  | The intention is to provide the Task Group with an idea of the long term efficiency potential of the MAC. How well does it perform with significantly increased data rates? | L |  |
| CC26 | MSDU Delay | This metric gives the MSDU delay distribution for each flow type, both QoS and non-QoS (i.e. application aggregated on all the STA) Curves shall be represented as 1-cdf(Delay) | All mandatory simulation scenarios | BJC, HBE proposal | Added words to emphasize that ALL flow types are considered | M |  |
| CC27 | Throughput / Range | Presented as a curve of Throughput (bps) vs range (m). Throughput is defined by totaling the number of bits in MSDUs successfully transmitted and dividing by the simulation duration (s). Alternative: Goodput is defined by totaling the number of bits in MSDUs successfully transmitted and dividing by the simulation duration (s). Present three curves: a) Goodput (bps) vs range (m) with 95% coverage b) Goodput (bps) vs range (m) with 50% coverage c) Goodput (bps) vs range (m) with 5% coverage Note: Goodput with X% coverage refers to the goodput achieved with X% of channel realizations | Present results for two isolated STA (one may be an AP), one is fully backlogged with 1500B MSDUs addressed to the other. Vary the distance between the two STA. Report results for all the channel models described in [4]. TBD scenario from [3]. Scenario (Bjorn Bjerke):Proposal for link test scenario (to be added to [3]): Two isolated STAs (one may be an AP), one is fully backlogged with 1500 B MSDUs addressed to the other. Channel models: B – NLOS and E – NLOS Channelization: 20 MHz | APS proposal | Need a link test simulation scenario to go in [3]. Motivation: By reporting 95%, 50% and 5% coverage in the form of three goodput vs range curves, the next 4 criteria can be addressed simply by reading values off the curves. | M |  |
| CC28 | Throughput / Range in 20MHz | Presented as a curve of Throughput (bps) vs range (m) when a 20MHz PHY mode of operation is in use. Throughput is defined by totaling the number of bits in MSDUs successfully transmitted and dividing by the simulation duration (s). | Present results for two isolated STA (one may be an AP), one is fully backlogged with 1500B MSDUs addressed to the other. Vary the distance between the two STA. Report results for all the channel models described in [4]. TBD scenario from [3]. | RM proposal | This follows from a FR; It can be defined in a manner similar to the ‘Throughput/Range’ FR, above This criterion can be merged with the modified version of the previous criterion. | M |  |
| CC29 | HT Range | The range value where the Throughput / Range curve passes through 100Mbps. Alternative The range values where the goodput / Range curves with 95%, 50% and 5% coverage pass through 100Mbps. | Report results for all the channel models described in [4]. | APS proposal |  | M |  |
| CC30 | HT Range in 20MHz | The range value where the Throughput / Range curve (20MHz mode of operation), passes through 100Mbps. | Report results for all the channel models described in [4]. | RM proposal |  | M |  |
| CC31 | 10Mbps range | The range value where the Throughput / Range curve passes through 10Mbps. Alternative: The range values where the goodput / Range curves with 95%, 50% and 5% coverage pass through 10Mbps. | Report results for all the channel models described in [4]. | WFA proposal (moved from FR) |  | M |  |
| CC32 | 10Mbps range in 20MHz channel | The range value where the Throughput / Range curve (20MHz mode of operation) passes through 10Mbps. | Report results for all the channel models described in [4]. | RM proposal |  | M |  |
| CC33 | Uniform BSS operation with HT rate supported | Continuous coverage of HT range and 10Mbps range requirements should be at least 95% (i.e., maximum 1 out of 20 stations within range specified may have to accept lower goodput, also providing for mobile stations and movement in the environment) | TBD | WFA proposal (moved from FR) | Needs implementable definition. This criterion is addressed by the previous two criteria | M |  |
| W1 CC34 | Extended range, residential environment | Range at which point-point throughput of 10 Mbps is supported with channel model B |  | Based on WFA proposal |  | M |  |
| W2 CC35 | Extended range, enterprise environment | Range at which point-point throughput of 10 Mbps is supported with channel model D |  | Based on WFA proposal |  | M |  |
| W3 CC36 | Extended range, hotspot environment | Range at which point-point throughput of 10 Mbps is supported with channel model E |  | Based on WFA proposal |  | M |  |
| W4 CC37 | Throughput at long range, residential environment | Point-point throughput achievable at a range of 100 m with channel model B |  | Based on WFA proposal |  | M |  |
| W5 CC38 | Throughput at long range, enterprise environment | Point-point throughput achievable at a range of 100 m with channel model C |  | Based on WFA proposal |  | M |  |
| W6 CC39 | Throughput at long range, hotspot environment | Point-point throughput achievable at a range of 100 m with channel model D |  | Based on WFA proposal |  | M |  |
| W7 CC40 | Coverage in high throughput mode | Maximum range at which coverage of 100 Mbps point-point throughput is at least 95% |  | Based on WFA proposal | Some discussion of suitable definition of coverage has taken place; no final language available yet. | M |  |
| W8 CC41 | Coverage in low throughput mode | Maximum range at which coverage of 10 Mbps point-point throughput is at least 95% |  | Based on WFA proposal | See note to W8. | M |  |
| ### Throughput and Overhead |  |  |  |  |  |  |  |
| CC42 | Preambles (Adopted in phy – need to review) | What are the possible preambles? |  | PPG (Agere) proposal | Why is this relevant? This is not relevant as a CC. The information can be found in the technical proposal | L |  |
| CC43 | Maximum data throughput | Maximum data throughput at all combinations of: a. Packet sizes of 100B, 1000, and 10000B b. All proposed preamble lengths. |  | PPG (Agere) proposal | Incomplete, other conditions need to be specified. This CC is already more or less covered by the “Coverage of Usage Models” criterion | L |  |
| ### MAC Changes |  |  |  |  |  |  |  |
| CC44 | Required changes to 802.11 MAC | A list of required changes to the 802.11 MAC. |  | PPG (Agere) proposal | "Change" needs precise definition. How is this not satisfied by their technical specification? This is a summary of the MAC portion of the technical proposal. Leave it up to the proposer to highlight main features. | L |  |
| CC45 | Post 2003 assumptions | List any MAC extensions (complete or inprogress) that the proposal assumes which are beyond the published 2003 standard. (I.e. does the proposal assume or require features from TGe, TGi, TGk etc?) |  | DB proposal |  | M |  |
| CC46 | MAC option usage | Does the proposal require the use of any (2003 or later extension) optional MAC features? (ex: PCF). If so, list them. |  | DB proposal |  | M |  |
| CC47 | 2003 MAC compatability | Describe any proposed changes required to the 2003 MAC operation to implement the proposal. For each change, describe the impacts of this change on 2003 MAC based stations and the thruput performance gains as compared to 2003 MAC stations. |  | DB proposal |  | M |  |
| CC48 | MAC parameters | Describe the proposal’s assumed settings of various MAC “control knobs”? (example: fragmentation threshold, RTS CTS usage, etc). Any new knob settings proposed? |  | DB proposal |  | M |  |
| CC49 | MAC SAP Thruput | Give the expected Thruput for the proposal. Primary comparison criteria is of Class 3 Data frames, but if the proposal creates significant thruput gain or loss for other frame types, please call those out. |  | DB proposal |  | L |  |
| CC50 | Encryption impacts | Give the proposal’s thruput for both unsecured and secured traffic. Specify the security mechanism assumed/required for each supported security case (2003 WEP? WPA Tkip? TGi AES?) |  | DB proposal |  | L |  |
| ## PHY Related |  |  |  |  |  |  |  |
| In this section, the performance of the physical layer will be shown given realistic implementation and radio impairments under a subset of channel conditions. The results will be given in terms of PER for a given data rate and thus should be independent of MAC aspects of the proposals. Both phy-only and complete proposals should be required to complete this section. ### PHY Rates |  |  |  |  |  |  |  |
| CC51 | Data rates | A list of PHY layer data rates, and for each data rate, specify the used modulation techniques, number of Tx antennas, coding rate and bandwidth. Specify which of the rates are mandatory and which are optional. |  | PPG (Agere) proposal Accepted unanimously. |  | H |  |
| CC42 | Preambles (Adopted in phy – need to review - PENDING) | Options: 1) What are the proposed preambles? 5 2) Describe and evaluate the cross-correlaation and auto-correlation performance of the STS (short training sequences) used by the proposal. 0 3) State PAPR of preambles used for channel estimation? 0 4) All of the above? 20 5) None? 7 Result: What are the proposed preambles? Describe and evaluate the cross-correlation and auto-correlation properties of all preambles. If the system uses multiple concurrent transmit streams, describe and evaluate the cross-correlation properties of the preambles in the different streams. Describe the auto-correlation properties within each stream. State the PAPR of all preambles. DEADLOCK - For TGn: - These analyses should be conducted on the transmit waveforms independent of any channel mode. - Cross-correlation specified as above (no channel) But autocorrelation analysis should be performed for extreme channel NLOS E and LOS B representing extreme types of channels to avoid poor synchronization in some scenarios. Metric required is the mean and std of peak to sidelobe ratio of the autocorrelation function. Performance of preambles - Assessed independently of channel (tx) - 11 - Assessed in the context of particular channels (to be specified – one or more channel models) - 11 |  | PPG (Agere) proposal | Why is this relevant? This is not relevant as a CC. The information can be found in the technical proposal | CC42 |  |
| ### Channelization |  |  |  |  |  |  |  |
| CC52 | Spectral Mask | Show the transmit spectral mask that all transmissions in the proposal meet. List for each channelization. This must be under the same PA backoff per rate used for performance simulations. |  | PPG (Agere) proposal TBD : Jeff Gilbert |  | H |  |
| CC52.5 (Move to before CC52) | Channelization | Specify the channelization – i.e. the adjacent channel spacing. Notes: None: 1 Just mask: 1 Just channelization: 0 Both: ~22 |  | PPG (Agere) proposal approved unanimously |  | H |  |
| CC53 | Co-channel interference rejection (DEFERRED) | Give the number for the system capacity reduction factor e according to Appendix 1 in 802.11-03/802r7. |  | PPG (Agere) proposal | Question to PPG (Agere): Appendix 1 in 802.11-03/802r7 states that t for each mode can be determined from PER vs SNR curves and s (Step 1) for each mode determined from t. Is it the intent of Appendix 1 of 802.11-03/802r7 in Step 3 to apply the value of e corresponding to the value of s and t for the most robust mode of the proposal? | M |  |
| CC54 | Adjacent channel interference rejection | The adjacent channel rejection shall be measured by setting the desired signal strength 3dB above the rate-dependent sensitivity and raising the power of the interfereing signal (namely another uncorrelated packet at the same data rate present throughout the entire packet) until 10% PER is caused for a PSDU length of 1000 bytes. This should be specified for all data rates listed in CC51. Options: Leave in (clarify): 11 Remove: 8 Reword to be a rx filter requirements spec given specified ACI spec: 3 Add sensitivity CC requriremet (to be used by ACI), also leave ACI – clarify 0 Leave in – clarify rx implications/impairments: 2 Remove: 9 | 1500B MSDU size Proposed geometry: The geometry is defined based on a 10 m X 20 m rectangle. AP1 and STA1 are located at the upper and lower left hand corners, respectively (10 m apart), while AP2 and STA2 are located at the upper and lower right hand corners, respectively. Channel model: B – NLOS Both links are operating at their maximum rates for the given distance. | PPG (Agere) proposal | "Impact" needs definition, and under what circumstances. | L |  |
| ### Efficiency |  |  |  |  |  |  |  |
| CC55 | HT net System capability (Tentative handoff to MAC) | The number of bps/Hz/cell when demonstrating throughput values in a scenario Net System capability as defined in ITU-R [7] | Report results for all the channel models [4], and mandatory multi user scenarios described in Usage models [3]. | TeliaSonera proposal | System capacity reduction due to CCI and ACI is discussed in appendix 1 in [3] | L |  |
| CC58 | HT Spectral Efficiency | The number of bps/Hz during the PSDU carrying a Data MPDU when demonstrating a goodput value of at least 100Mbps. Specify the phy data rate used during this test. Vote: Merge into to CC51: 8 Delete: 3 Leave: 13 Change to over entire packet of given length: 1 | Using simulation scenario 16 defined in [3]. | APS proposal TBD reworded on 16 dec | Is the bps value used in the spectral efficiency calculation measured at the MAC or at the PHY? | H |  |
| ### PHY Performance |  |  |  |  |  |  |  |
| CC59 | AWGN PER performance | Lengths? a) 100B and 1000B? 11 b) 1000B only? 23 c) Defer for guidance of MAC group? 6 a) Identify performance in an AWGN channel with uncorrelated paths, and for packet length of 1000B. Show the PER curves for each supported data rate. 5 b) The rows and columns should be orthogonal to each other. Take the first Nr x Nt elements of the fourier matrix with dimension max(Nr, Nt) 14 c) Scalar multiplied identity matrix to adjust for a given SNR. 5 d) SISO-only AWGN 8 e) MIMO AWGN with non-diagonal matrix 4 - Yes revote? 8 - No no revote? 25 a) Keep b) Removal Decided: Identify performance in idealized channel with uncorrelated paths, and for a packet length of 1000B. The rows and columns of the channel should be orthogonal to each other as follows: Take the first Nr x Nt elements of the fourier matrix with dimension max(Nr, Nt). Show the PER curves for each supported data rate. |  | PPG (Agere) proposal | Why is AWGN relevant? Is the PER performance to be reported for all modes? I assume that this is just for best-case performance only limited by white noise | L |  |
|  |  | Continuation: Identify performance in idealized channel with uncorrelated paths, and for a packet length of 1000B. The rows or columns of the channel should be orthogonal to each other as follows: Take the first Nr x Nt elements of the fourier matrix with dimension max(Nr, Nt). Show the PER curves for each supported data rate. Nr Nt options - Fixed by us? 0 - Nt=Nr= 1,2,3,4? 2 - Nt=Nr=1,2,3,4 if proposal has any mode generating 2 or more independent streams otherwise Nt=Nr=1? 16 d) Nt=Nr=1,2,3,4 if proposal has any mode generating 2 or more streams otherwise Nt=Nr=1 ? 9 |  |  |  |  |  |
|  |  | Would you like: 1) CC59 – AWGNish performance? Yes – 25.5 No –14 2) CC67/CC68 – performance with channel models in 03/940r1 Yes – 37 No – 0 |  |  |  |  |  |
| CC67 | PER performance in non AWGN channels | PER versus Eb/No and Es/No for each supported data rate (where Es is measured at the output of the transmitter) with 1000B packets simulated down to a PER of 0.01 or further. What’s required: Delay spread |  | PPG (Agere) proposal | Create waterfall curves for all channel models defined in 03/940r1. APS comment: type of averaging needs to be defined carefully. Is the PER performance to be reported for all modes? | H |  |
| CC68 | PER performance in non AWGN channels | PER versus distance for each supported data rate using each of the channel model defined in 03/940r1 using 1000B packets simulated down to a PER of 0.01 or further. A maximum total output power of 20dBm from all transmit antennas must be used (even better is to tie to required backoff due to non-linearity) and an antenna gain of 0dBi should be used. Need AGC? What’s required: Noise floor |  | Loosely taken from PPG (Agere) proposal , incorporated by Jeff Gilbert | Create waterfall curves for all channel models defined in 03/940r1. APS comment: type of averaging needs to be defined carefully. Proposed replacement by Jeff Gilbert. May make some other CCs redundant | H |  |
| ### Spectral Mask Characteristics |  |  |  |  |  |  |  |
| CC65 | Spectral characteristics with non-ideal power amplifier | Show spectral characteristics at specified output power. |  | Taken from PPG (Agere) proposal, incorporated by Jeff Gilbert (Atheros) | Proposed replacement by Jeff Gilbert. May make some other CCs redundant. | M |  |
| ### PHY Changes |  |  |  |  |  |  |  |
| CC80 | Required changes to 802.11 PHY | A list of required changes to the 802.11 PHY. |  | APS proposal based on list of required changes to 802.11 MAC, for completeness. | "Change" needs precise definition. How is this not satisfied by their technical specification? | L |  |

## Additional contribution by Herve Bonnevile

(Comment by Adrian:  I am not sure exactly what is being proposed here).

We have computed the percentage of medium occupancy for flows aggregated per station (see table below for scenario 4). We assumed a 100 Mbit/s MAC. As total source throughput exceeds this limit, we served UDP traffic first and then shared remaining bandwidth among TCP sources on an equal basis:

Throughput (TCP sources) =  Th(TCP) * ((100 – Th(UDP)) / Th(total) - Th(TCP))

(TCP ack are not taken into account).

Results shows how flows are (theoretically) spread among stations pairs, tx and rx part separated. The medium is globally full, but many stations participate for a small percentage only.

Hence we consider it is meaningful to compute the time a station spends in tx and rx phases, and then to deduce an estimated power consumption per station based on its effective activity.

Enterprise - Scenario #4

| Station | %tx | %rx | Total |
| --- | --- | --- | --- |
| 0 | 70,64% | 29,36% | 100,00% |
| 1 | 0,05% | 0,21% | 0,26% |
| 2 | 0,05% | 0,21% | 0,26% |
| 3 | 0,05% | 0,21% | 0,26% |
| 4 | 1,04% | 0,21% | 1,25% |
| 5 | 0,21% | 0,21% | 0,42% |
| 6 | 0,05% | 0,21% | 0,26% |
| 7 | 1,00% | 1,00% | 2,00% |
| 8 | 1,00% | 1,00% | 2,00% |
| 9 | 0,00% | 2,00% | 2,00% |
| 10 | 0,00% | 2,00% | 2,00% |
| 11 | 0,00% | 6,25% | 6,25% |
| 12 | 0,00% | 6,25% | 6,25% |
| 13 | 0,00% | 6,25% | 6,25% |
| 14 | 0,00% | 6,25% | 6,25% |
| 15 | 0,00% | 6,25% | 6,25% |
| 16 | 0,00% | 6,25% | 6,25% |
| 17 | 0,00% | 6,25% | 6,25% |
| 18 | 0,00% | 6,25% | 6,25% |
| 19 | 0,00% | 6,25% | 6,25% |
| 20 | 0,00% | 6,25% | 6,25% |
| 21 | 6,25% | 0,00% | 6,25% |
| 22 | 6,25% | 0,00% | 6,25% |
| 23 | 6,25% | 0,00% | 6,25% |
| 24 | 6,25% | 0,00% | 6,25% |
| 25 | 0,15% | 0,15% | 0,30% |
| 26 | 0,15% | 0,15% | 0,30% |
| 27 | 0,15% | 0,15% | 0,30% |
| 28 | 0,15% | 0,15% | 0,30% |
| 29 | 0,15% | 0,15% | 0,30% |
| 30 | 0,15% | 0,15% | 0,30% |

## Physical layer impairments

These impairments are to be used in section 4.6.

| Number | Name | Definition | Status of this IM |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| IM1 | PA non-linearity | Include? Yes? 8+17 = 25 No? 4 Simulation should be run at oversample rate of at least 4x. Use RAPP power amplifier model as specified in document 00/294. Use P-parameter of 3. Specify backoff from full saturation used in the simulation calculated as PABackoff = –10 log10(Average TX Power/Power at saturation) for each rate. At least 4x ? 6+10=16 Exactly 4x? 0+0=0 1) RAPP? 6+8=14 2) Saleh/Gorbani? 0+3=3 done |  | Taken from PPG (Agere) proposal, incorporated by Jeff Gilbert (Atheros) |  |  |
| IM2 | Carrier frequency offset | Simulations should be run using a random total tx and rx carrier frequency offset ranging from –40ppm to +40ppm. The symbol clock shall have the same relative frequency offset as the RF carrier frequency offset. Yes? 14+31 = 45 No? 0 1) Relative to baseband (kHz)? 0 2) Relative to carrier single (ppm)? 14+5=19 3) Relative to carrier with different ppm per band? 2+5=7 40ppm? 25 50ppm? 0 1 Separate distribution or common? Two separate (~half)? 3 One common? 7+13=20 |  | Loosely adapted from PPG (Agere) proposal and modified by Jeff Gilbert (Atheros) |  |  |
| IM3 | Baseband timing offset accuracy | a) The baseband sample timing must be detected from the signal and cannot be assumed to start at a fixed offset. 2 b) Simulations shall include timing acquisition on a per-packet basis. 26 opt Yes? 3+21 = 24 No? 2+4 = 6 Done – accepted option B |  | Jeff Gilbert (Atheros) proposal |  |  |
| IM4 | Phase noise | The phasenoise profile should be determined by state-of-the-art 5GHz phasenoise performance as seen in publications (these numbers are approximate as read from graphs): In ISSCC ’02, Paper 5.4, Atheros reported: -87dBc/Hz up to 5kHz -92dBc/Hz at 200kHz -112dBc/Hz at 1MHz -130dBc/Hz at 10MHz In ISSCC ’03, Paper 20.2, Athena Semiconductors reported: ~-88-95dBc/Hz up to 5kHz -95dBc/Hz at 200kHz -115dBc/Hz at 1MHz -137dBc/Hz at 10MHz In ISSCC ’03, Paper 20.3 Resonext reported: ~ -90dBc/Hz up to 5kHz -87dBc/Hz at 200kHz -110dBc/Hz at 1MHz -125dBc/Hz at 10MHz In ISSCC ’04, Atheros will report: -95-105dBc/Hz up to 5kHz -105dBc/Hz at 200kHz -115dBc/Hz at 1MHz -130dBc/Hz at 10MHz Other data points welcomed. Then a single profile should be determined – this could be some combination of data from several. The proposer should state whether they assume that the phasenoise is independent for the different antennas (multiple synthesizers) or common (single synthesizer with distributed RF carrier to radios for the antennas). Alternatively we could decide that it should be modelled in one speciic way – common or independent. Yes? 12+27 = 39 No? 1 a) Profile specified at spot-frequencies? OLD10 4 b) Single pole filter? OLD16 new9 c) Single poll single zero? 10 d) Multiple pole filter OLD3 3 e) Integrated in band and out of band noise? OLD0 0 Chosen model d but OPEN exact details |  | Loosely adapted from PPG (Agere) proposal and modified by Jeff Gilbert (Atheros) |  |  |
| IM5 | Receiver antenna diversity switching time | If additional switched diversity is used in the system, the different antennas cannot be sampled without at least this much time between the sampling of the two antennas. Proposal of 1us – other suggestions welcomed. . Yes? 4 No? 10+24 |  |  |  |  |
|  | Antenna spacing | Suggested ½ wavelength between antenna – couple If required for clarity – section 7 in channel model document. NOTE: Shoemaker model uses 1 lambda spacing – we need 0.5 lambda |  |  |  |  |
|  | Antenna gain, efficiency, radiation, pattern | Anything required to make channel model fully defined – as in channel model |  |  |  |  |
|  | Noise figure | Yes? 15+16 = 31 No? 1+1 = 2 NF=5dB? 1 NF=10dB? 29 Input referred total noise figure from antenna to output of the A/D will be 10dB. Done Options: a) Input of A/D? 4 b) Output of A/D? 11 Different for 2.4 and 5? |  |  |  |  |
|  | I/Q imbalance | Before / after correction? Yes? 1 No? 9+23 = 32 |  |  |  |  |
|  | DC Offset | Yes? 0 No? 8+21=29 |  |  |  |  |
|  | Input compression on LNA | Yes? 3+2 = 5 No? 5+16 = 21 If included use RAPP model? |  |  |  |  |
|  | AGC Detection | Yes? 0 No? 11+4=15 |  |  |  |  |

# Template for Comparison Criteria Submissions

The results for a complete submission shall include a the values of comparison criteria defined in section 2 using the format defined in this section.

This is TBD.

# References

[1] IEEE 802 11-03/665, TGn Selection Procedure

[2] IEEE 802 11-03/813, TGn Functional Requirements

[3] IEEE 802 11-03/802, TGn Usage Models

[4] IEEE 802 11-03/161r2, TGn Channel Models

[5] IEEE 802.11-02/798r7, Draft PAR for High Throughput Study Group

[6] IEEE 802.11-02/799r6, Criteria for Standards Development (HTSG 5 Criteria Document)

[7] ITU-R 1390 Method