IEEE P802.11
Wireless LANs

Normative Text for STA disassociation behavior

Date:	January 14th, 2004

Author:	Marian Rudolf, Joe Kwak
	InterDigital
	1000 Sherbrooke Ouest, 10e etage, Montreal (QC), Canada, H3A 3G4
	Phone: 514-904-6258
	Fax: 514-904-6344
	e-Mail: marian.rudolf@interdigital.com; joekwak@mindspring.com

# Abstract

This document is a normative text proposal on mandating non-AP STAs to send a DISASSOCIATION frame when leaving a network.as defined in section 5.4.2.4 in the current 802.11 baseline specification.

# References

[1] 11-04-xxxx-r0-K “STA disassociation behavior”, InterDigital

# Normative Text Proposal:

Change paragraph 5.4.2.4 as shown below:

## 5.4.2.4 Disassociation

The disassociation service is invoked whenever an existing association is to be  terminated. Disassociation is a DSS.

In an ESS, this tells the DS to void existing association information. Attempts to send messages via the DS to a disassociated STA will be unsuccessful.

The disassociation service may be invoked by either party to an association (non-AP STA or AP). Disassociation is a notification, not a request. Disassociation cannot be refused by either party to the association.

APs may need to disassociate STAs to enable the AP to be removed from a network for service or for other reasons.

STAs shall disassociate when they leave a network, power down, or become incapable of continuing network service. However, the MAC protocol does not depend on STAs invoking the disassociation service. (MAC management is designed to accommodate loss of an associated STA.)

Insert the following new row item at the bottom of the list in section A.4.11 of TGk draft as shown by revision marks into Annex A – PICS:

# Annex A – PICS

## A.4.11 Radio Resource Measurement Extensions

| RRMxx | Disassociation notification when leaving network | 5.4.2.4 | CF2:M | Yes, No |
| --- | --- | --- | --- | --- |