IEEE P802.11
Wireless LANs

Extensibility Issues in 11k

Date:	Jan 12, 2004

Author	Emily Qi, Ravi Murty, Adrian Stephens

Intel Corporation
	e-Mail: emily.h.qi@intel.com
	ravi.murty@intel.com	
	Adrian.p.stephens@intel.com

Abstract

This submission provides normative text for potential incorporation into the TGk Draft to provide Vendor Specific Measurement definition that intends to solve extensibility issues in 11k. Vendor Specific Measurement can be used for vendors to communication their proprietary measurements through the standard mechanism.

Instructions to Editor

Change the table 0-1 to the following table:

Table 0-1 – Measurement Type Definitions for Measurement Request Element

| Measurement Name | Measurement Type |
| --- | --- |
| Unused | 0-2 |
| Channel Load Request | 3 |
| Noise Histogram Request | 4 |
| Beacon Request | 5 |
| Frame Request | 6 |
| Hidden Node Request | 7 |
| Medium Sensing Time Histogram Request | 8 |
| STA Statistics Request | 9 |
| Vendor Specific Measurement | 255 |
| Reserved | 10-254 |

Change the table 0-6 to the following table:

Table 0-6 – Measurement Type Definitions for Measurement Report Element

| Measurement Name | Measurement Type |
| --- | --- |
| Unused | 0-2 |
| Channel Load Report | 3 |
| Noise Histogram Report | 4 |
| Beacon Report | 5 |
| Frame Report | 6 |
| Hidden Node Report | 7 |
| Medium Sensing Time Histogram | 8 |
| STA Statistics Report | 9 |
| Vendor Specific Measurement | 255 |
| Reserved | 10-254 |

Add the following text after 7.3.2.19.7

##### 7.3.2.19.8 Vendor Specific Measurement Request

Vendor Specific Measurement enables a STA/AP to communicate proprietary measurement through the standard measurement request information element. The Measurement Request field corresponding to a Vendor Specific Measurement is shown in Figure +1.

|  | Organization Uniform Identifier | Vendor Specific Measurement Request details |
| --- | --- | --- |
| Octets: | 3 | variable |

Figure +1 – Measurement Request field format for Vendor Specific Measurement

Organization Uniform Identifier field is assigned by 802.11 Working Group.

The Vendor Specific Measurement Request details field contains the details of the measurement request, and it should be defined by the specific vendor.

Add the following text after 7.3.2.20.7

##### 7.3.2.20.8 Vendor Specific Measurement Report

A  STA receiving Vendor Specific Measurement Request has an optional to ignore it or to response with a Radio Measurement Report frame containing one or more Measurement (Vendor Specific Measurement) Report elements. The format of the Vendor Specific Measurement Report field is shown in Figure +2.

|  | Organization Uniform Identifier | Vendor Specific Measurement Report details |
| --- | --- | --- |
| Octets: | 3 | variable |

Figure +2 – Measurement Report field format for Vendor Specific Measurement

Organization Uniform Identifier field is assigned by 802.

The Vendor Specific Measurement Report details field contains the details of the measurement report, and it should be defined by the specific vendor.