IEEE P802.11
Wireless LANs

Multicast Replay Detection

Date:	January 14, 2004

Author:

Fred Stivers
Texas Instruments
fstiver@ti.com

Replace Figure 10 in clause 7.3.2.9.3 with:

B0              B1         B2             B3      B4                B5      B6                            B15

| Pre-Auth | No Pairwise | PTKSA Replay Counter | GTKSA Replay Counter | Reserved |
| --- | --- | --- | --- | --- |

Replace list item 3 in clause 7.3.4.9.3

3 Replay Counter

A STA or AP sets the Number of Replay Counters subfield (Bits 2 and 3) of the RSN Capability Information field to the value contained in dot11RSNAConfigNumberofReplayCounters. The least significant bit of dot11RSNAConfigNumberofReplayCounters is put in bit 2. See Clauses 8.3.2.3.4 and 8.3.3.4.3. The meaning of the Number of Replay Counters is defined in Table 1.

with:

3 PTKSA Replay Counter

A STA or AP sets the Number of PTKSA Replay Counters subfield (Bits 2 and 3) of the RSN Capability Information field to the value contained in dot11RSNAConfigNumberofPKTSAReplayCounters. The least significant bit of dot11RSNAConfigNumberofPTKSAReplayCounters is put in bit 2. See Clauses 8.3.2.3.4 and 8.3.3.4.3. The meaning of the Number of Replay Counters is defined in Table 1.

Insert the following after Table 4 in clause 7.3.4.9.3

4 GTKSA Replay Counter

A STA sets the Number of GTKSA Replay Counters subfield (Bits 4 and 5) of the RSN Capability Information field to the value contained in dot11RSNAConfigNumberofGKTSAReplayCounters. The least significant bit of dot11RSNAConfigNumberofGTKSAReplayCounters is put in bit 4. See Clauses 8.3.2.3.4 and 8.3.3.4.3. The meaning of the Number of Replay Counters is defined in Table 1.

Replace list item 4 in clause 7.3.4.9.3

4 Reserved

with:

5 Reserved

Replace Table 4 in clause 7.3.4.9.3 with:

Table 1—Number of PTKSA/GTKSA Replay Counters Usage

| Number of Replay Counters value | Meaning |
| --- | --- |
| 0 | 1 replay counter per PTKSA/GTKSA |
| 1 | 2 replay counters per PTKSA/GTKSA |
| 2 | 4 replay counters per PTKSA/GTKSA |
| 3 | 16 replay counters per PTKSA/GTKSA |

Replace list item 2 in clause 8.3.2.3.4

Each TSC (48 bit counter) shall be selected from a single pool by each transmitter for each temporal key—i.e., each transmitter has its own unique TSC for each directional temporal key established.

with:

Each TSC (48 bit counter) shall be selected from a single pool by each transmitter for each PTKSA and each GTKSA.

Replace the first sentence of list item 5 in clause 8.3.2.3.4

A receiver shall maintain a separate set of TKIP TSC replay counters for each MAC address from which it has received TKIP traffic.

with:

A receiver shall maintain a separate set of TKIP TSC replay counters for each PTKSA and each GTKSA.

Replace list item 2 of clause 8.3.3.4.3

The PN (a 48-bit counter) shall be selected from a single pool by each transmitter for each temporal key (TK).

with:

The PN (a 48-bit counter) shall be selected from a single pool by each transmitter for each PTKSA and each GTKSA.

Replace the first sentence of list item 4 in clause 8.3.3.4.3

A receiver shall maintain a separate set of PN replay counters for each MAC address from which it receives CCMP traffic.

with:

A receiver shall maintain a separate set of PN replay counters for each PTKSA and GTKSA.

Replace in each case:

dot11RSNAConfigNumberofReplayCounters

with:

dot11RSNAConfigNumberofPTKSAReplayCounters

Replace definition of dot11RSNAConfigNumberofReplayCounters with:

dot11RSNAConfigNumberOfPTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the number of PTKSA replay counters per association:

0 –> 1 replay counter,

1 –> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

::= { dot11RSNAConfigEntry 20 }

Add the following MIB variable:

dot11RSNAConfigNumberOfGTKSAReplayCounters OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Specifies the number of GTKSA replay counters per association:

0 –> 1 replay counter,

1 –> 2 replay counters,

2 –> 4 replay counters,

3 –> 16 replay counters"

::= { dot11RSNAConfigEntry 20 }

Add dot11RSNAConfigNumberofGTKSAReplayCounters to dot11RSNAadditions OBJECT-GROUP

Add dot11RSNAConfigNumberofGTKSAReplayCounters to dot11RSNAConfigEntry

Add dot11RSNAConfigNumberofGTKSAReplayCounters to dot11RSNBase OBJECT-GROUP