IEEE P802.11
Wireless LANs

Clarifying TKIP MIC processing format

Date:	February 12, 2004

Author:

Fred Stivers
Texas Instruments
fstiver@ti.com

Add the following text as list item 5 to clause 8.3.3.4.3:

5) The recipient shall maintain a separate replay counter for each IEEE 802.11 MSDU priority, and shall use the PN recovered from a received frame to detect replayed frames, subject to the limitation of the number of supported replay counters indicated in the RSN IE Capabilities Field (see Clause 7.3.2.9). A replayed frame occurs when the PN extracted from a received frame is less that or equal to the current replay counter value for the frame’s MSDU priority. A transmitter shall not use IEEE 802.11 MSDU priorities without ensuring that the receiver supports the required number of replay counters. The separate replay counters per MSDU priority accommodates frames that may be delayed due to MSDU priority.