IEEE P802.11
Wireless LANs

Clause 6 Edits

Date:	20th February 2003

Author:	Mike Moreton
	STMicroelectronics
	e-mail:Mike.Moreton@st.com

Abstract

This document contains comment resolution proposals for 802.11i Sponsor Ballot.

# Comment 231: Accept

Request that the relative ordering of decryption, defragmentation etc be normative, as STAs doing this in different orders would break the system.

In the title of section 6.1.4, remove the word “(Informative)”.

# Comment 320: Reject

Comment was that there was text in 6.1.4 that was simply describing the contents of figure Figure 7 (presumably for those cases where the diagram doesn’t make it into the pdf…).  However, changes in draft 7.2 have added meaning not present in the diagram.

“The comment was correct when made, but changes due to other comments have added additional meaning to the descriptive text.”

# Comment 325 and 326: Accept

The figure in 6.1.4 does not show the correct location of the controlled and uncontrolled ports, and does not show the MAC Relay entity.

Change the diagram from:

to:

# Comment 324: Accept

In 6.1.4, the description of queuing misses out PS.

In clause 6.1.4 modify the following sentence as shown:

The only queuing explicitly introduced by this standard is to perform the defragmentation processes and for Power Save defer.

# Comment 321: Accept

In 6.1.4, the description of the receive processing misses out replay detection.

In clause 6.1.4, modify the following sentence as shown:

During reception, a received Data frames goes through processes of MPDU header + CRC validation, duplicate removal, decryption, defragmentation, integrity checking, and replay detection.