IEEE P802.11
Wireless LANs

Proposed Text for Rate Adaptive Version of CC67

Date:	February 23, 2004

Author:	John Ketchum
	Qualcomm
	9 Damonmill Square, Suite 2A
	Concord, MA 01742
	USA
	Phone: 781-2760915 
	Fax: 
	e-Mail: johnk@qualcomm.com

Abstract

Proposed text for a rate adaptive version of CC67 is given.  Comments and contributions from Colin Lanzl, Stan Skafidas, Jeff Gilbert are included. Revisions added based on comments from John Sadowsky.

Show curves for achieved average physical layer data rates as a function of total SNR at a fixed packet error rate to be determined by the proposer.  The data rate shall be averaged over at least 100 independent realizations of the channel.  Plot the average achieved rate, versus SNR per receive antenna , for 1000B packets.   SNR in this case is the ratio of the received power at a single receive antenna, to the input-referred receiver noise power.  The received signal power is as measured in -10dB signal bandwidth at a single receive antenna, is summed over all transmit antennas and averaged over time and receive antennas   Note that when computing the SNR, the signal and noise bandwidth shall be identical.  When measuring PER, averaging should occur over a minimum of 100 packet errors.   This should be simulated for channel models B, D, and F.   The simulations should all include the Doppler effect as specified in the text of the channel model document.   All models should be run without the fluorescent effect but additionally model D should be run with and without the fluorescent effect.   The shadowing variance should be 0.   Simulations should include the effects of rate feedback and rate determination algorithms in the results.  Detailed disclosure of simulation methodology should be included with the results.