IEEE P802.11
Wireless LANs

Comments on Documents 11-03-814, 11-03-802, 11-02-813

Date:	March 8, 2004

Author:	John Ketchum, Bjorn Bjerke, Irina Medvedev, Sanjiv Nanda, Steve Morley
	Qualcomm
	Phone: 781-276-0915
	Fax: 781-276-0901
	e-Mail: johnk@qualcomm.com

Abstract

| Name | Doc_Reference | Section/Item | Comment | Proposed resolution | E, TN, TY |
| --- | --- | --- | --- | --- | --- |
| Remove CC9 | 814r16 | 4.2/CC9 | Transmit power is not power consumption. Everyone’s answer will be the same: 17 dBm | Remove this CC | TN |
| Remove CC50 | 814r16 | 4.4.2/CC50 | Encryption decisions are independent of MAC design | Remove this CC | TN |
| No physical layer impairments in CC59 | 814r16 | 4.5.4/CC59 | CC59 should be run with no physical layer impairments. Not stated explicitly now. | Add text stating that no physical layer impairments are used in these sims | TN |
| CC59 number of antennas | 814r16 | 4.5.4/CC59 | CC59 too many potential antenna configurations | Use a set representative antenna configurations, maybe 5: 1xN, Nx1, N1xN1, N1xN2, N2xN2 | TN |
| CC67 | 814r16 | 4.5.4/CC67 | Correct spelling of fluorescent | Fluorescent | E |
| CC59 and CC67 bandwidth spec | 814r16 |  | Bandwidth spec in CC59, CC67, &CC67.1 should be the same | Use spec in CC59: “Note that when computing the SNR, the signal and noise bandwidth shall be identical.” | TN |
|  | 814r16 | Section 5/IM1 | Confusing language in definition. “Specify how PA backoff from full saturation used in the simulation calculated as PABackoff = –10 log10(Average TX Power/Power at saturation) is computed. | Simulation should be run at oversampling rate of at least 4x. Use RAPP power amplifier model as specified in document 00/294. Use P-parameter of 3. Specify the PA backoff from full saturation used in the simulation. The backoff is calculated as PABackoff = 10 log10(Average TX Power/Psat). For this test, Psat is specified to be 25dBm. | E/TN |
|  | 814r16 | Section 6 | Content of section 6 is redundant and unneeded | Delecte section 6. If clarifications are needed add them to the CCs | E/TN |
| LOS/NLOS | 802r12 | Tables 1, 2, & 3 | LOS and NLOS are not arbitrary—are a function of range in each channel models | Remove references to LOS/NLOS | TY |
| QoS Priorities | 802r12 | Applications section | QoS management is determined by proposal | Remove last three lines in Applications section referring to QoS priorities | TN |
| Coexistence | 802r12 | Coexistence section |  | Need to remove | E |
| Simulation duration | 802r12 | Common conditions | Simulation duration sufficient for accurate results needs to be determined by proposer, and justified in proposal | remove | E/TN |
| Effective loss | 802r12 | Common conditions | Phy layer impairments are being taken care of elsewhere | remove | TY |
| Shadowing | 802r12 | Common conditions | Having non-zero shadowing requires lengthy simulations to average over shadowing ensemble. Sufficient path loss variability reflected in the different locations | Set to 0 | TN |
| Multiple channel models | 802r12 | Simulation Scenarios | Insufficient specification of the application of multiple channel models in a single scenario. See 11-04-0211-00-000n | Use single channel model per scenario | TY |
| Antenna spacings | 802r12 | Simulation Scenarios | Antenna spacings not specified anywhere. | Specify antenna spacing in common sim conditions to be half wave. | TY |
| Reuse eliminated | 802r12 | Scenario 4 | No use for reuse | Eliminate references to reuse in this scenario. | TY |
| Packet sizes | 802r12 | Scenario 4 | Comments on Cisco data on packet sizes no longer relevent | Remove comments on Cisco packet sizes |  |
|  | 802r12 | Scenario 4 STA 6 | Mean rate should be .256 | Change mean rate |  |
|  | 802r12 | Scenario 4 | Units missing in some rates | Add units to mean rate |  |
|  | 802r12 | Scenario 4 STAs 11-20 | TCP sinks also generate ACKs | Add text that explicitly states that TCP acknowledgment traffic needs to be included. |  |
|  | 802r12 | Scenario 5 | Need to resolve comment about inconsistencies in printing rates |  |  |
|  | 802r12 | Scenario 6 | Too many STAs, simulation is more complex than it needs to be | Reduce number of stations to 25, and increase offered traffic levels proportionately |  |
|  | 802r12 | Scenario 9 | TCP sinks also generate ACKs |  |  |
|  | 802r12 | Appendix 1 | No use for reuse | Remove appendix 1 |  |
|  | 813r9 | FR1 |  | Remove “Add usage model…” |  |
|  | 813r9 | FR2 |  | Add reference to Sim Scen 16 |  |