IEEE P802.11
Wireless LANs

Comments on Document 11-03-814

Date:	March 8, 2004

Author:                                            Patrik Eriksson (Acreo AB), Scott Leyonhjelm (University of Victoria)
	Phone: +46 11 363662
	Fax: +46 11 363601
	e-Mail: patrik.eriksson@acreo.se

Abstract

| Name | Doc_Reference | Section/Item | Comment | Proposed resolution | E, TN, TY |
| --- | --- | --- | --- | --- | --- |
| CC51 for adaptive rates | 814r16 | 4.4.1/CC51 | Not clear how CC51 shall be addressed by adaptive rate proposals. The number of average rates can be very high. | Specify basic rates with adaptive loading turned off. For adaptive rate proposals, refer to detailed description of adaptation mechanism in the proposal. | TN |
| CC58 without MAC | 814r16 | 4.4.3/CC52 | How shall the PHY data rate required to achieve 100 Mbps goodput be known without knowledge of the MAC? | Specify test to be carried out at maximum mandatory PHY data rate. | TN |
| CC67.2 sample timing drift | 814r16 | 4.5.4/CC67.2 | Effect of sample timing drift due to symbol clock offset will depend on length of packets. | Define length of packets in simulation. | TN |
| IM1 clarification | 814r16 | 5/IM1 | Sentence ‘Specify how PA backoff from full saturation used in the simulation calculated as PA Backoff=-10log10(Average Tx Power/Power at saturation) is computed” is confusing. Unclear why absolute power level is important. | Change sentence to “Specify the PA backoff from full saturation, calculated as PABackoff = 10 log10(Average TX Power/Psat), used in the simulation Explain why absolute power level is important. | TN |
| Remove IM2 | 814r16 | 5/IM2 | Sensitivity to frequency offset and sample timing drift shown separately by CC67.2 | Remove IM2 | TN |
| IM4 phase noise PSD number | 814r16 | 5/IM4 | Phase noise PSD at 0 Hz -100 dBc/Hz too aggressive. Integrated phase noise over 8 MHz bandwidth is -44 dBc. This favours non-robust high order modulation proposals. | Change phase noise PSD at 0 Hz to -90 dBc/Hz. | TN |