IEEE P802.11
Wireless LANs

Comments on document 11-03-814 (IEEE 802.11 TGn Comparison Criteria)

Date:		March 15, 2004

Author:                                            Patrik Eriksson (Acreo AB), Scott Leyonhjelm (University of Victoria)
	Phone: +46 11 363662
	Fax: +46 11 363601
	e-Mail: patrik.eriksson@acreo.se

Abstract

| Number | Commenter's Name | Doc_Reference | Section/Item | Comment | Proposed resolution | E, TN, TY | Pri | Status | Consensuso |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Patrik Eriksson | 814r16 | 4.4.1/CC51 | Not clear how CC51 shall be addressed by adaptive rate proposals. The number of average rates can be very high. | For adaptive rate proposals, specify the range of achievable rates or, if possible, state the achievable rates in a closed form expression. | TN |  |  |  |
|  | Patrik Eriksson | 814r16 | 4.5.4/CC67.2 | Effect of sample timing offset due to symbol clock offset will depend on length of packets. | Define length of packets in simulation. | TN |  |  |  |
|  | Patrik Eriksson | 814r16 | 5/IM1 | Sentence ‘Specify how PA backoff from full saturation used in the simulation calculated as PA Backoff=-10log10(Average Tx Power/Power at saturation) is computed” is confusing. | Change sentence to “Specify the PA backoff from full saturation, calculated as PABackoff = 10 log10(Average TX Power/Psat), used in the simulation | TN |  |  |  |
|  | Patrik Eriksson | 814r16 | 5/IM2 | Sensitivity to frequency offset and sample timing offset shown separately by CC67.2 | Remove IM2 | TN |  |  |  |
|  | Patrik Eriksson | 814r16 | 5/IM4 | Phase noise PSD at 0 Hz -100 dBc/Hz too aggressive. Integrated phase noise over 8 MHz bandwidth is -44 dBc. This favours non-robust high order modulation proposals. | Change phase noise PSD at 0 Hz to -90 dBc/Hz. | TN |  |  |  |

.