IEEE P802.11
Wireless LANs

- TGn Functional Requirements, Comparison Criteria and Usage Models document comment resolution

Date:	March 9, 2004

Author:	Adrian P Stephens
	Intel Corporation
	15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom
	Phone: +4 771276 3448 (mobile)
	Fax: 
	e-Mail: adrian.p.stephens@intel.com

- Abstract

This document contains a list of comments gathered from submissions to the TGn FRCC special committee.  The purpose of these comments are to address issues in the FRCC documents that would prevent a successfull confirmation vote in the TGn sessions.

Revision History

| Revision | Date | Author | Changes |
| --- | --- | --- | --- |
| 1 | March 9, 2004 | Adrian Stephens | Initial Merge of contributions prior to FRCC telecon |

| Number | Name | Doc_Reference | Section/Item | Comment | Proposed resolution | E, TN, TY | Pri | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Colin Lanzl | 11-03/802r12 | Applications | The last set of lines before the Application Definitions table ask questions that are central to comparison criteria 18 and 19: these call for measurement of performance of non-QoS and QoS flows. Unfortunately, the questions in this section are the only references in all of the Usage Model document that address the assignment of QoS priorities for UDP (or any other) traffic. There is no other clue in this document that helps anyone trying to simulate their system identify which flows have QoS attributes and the values of those attributes that can be used to accurately simulate proposed systems. This is a major problem with the Usage Model document. | In view of the rather extensive simulations that will be required of proposers, I suggest that a very limited number of actual traffic flows be assigned as QoS flows with reasonable (802.11e-like) priorities. Example, only label HDTV and VOIP flows as QoS, with 802.1D user priorities 5 and 7 respectively, with all other flows assigned as non-QoS flows. | TY (will vote no unless resolved) | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Tables 1, 2, & 3 | LOS and NLOS are not arbitrary—are a function of range in each channel models | Remove references to LOS/NLOS | TY | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Common conditions | Phy layer impairments are being taken care of elsewhere | remove | TY | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Simulation Scenarios | Insufficient specification of the application of multiple channel models in a single scenario. See 11-04-0211-00-000n | Use single channel model per scenario | TY | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Simulation Scenarios | Antenna spacings not specified anywhere. | Specify antenna spacing in common sim conditions to be half wave. | TY | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 4 | No use for reuse | Eliminate references to reuse in this scenario. | TY | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Appendix 1 | No use for reuse | Remove appendix 1 | TY | 3 | Open |
|  | John Ketchum | 11-03/802r12 | Applications section | QoS management is determined by proposal | Remove last three lines in Applications section referring to QoS priorities | TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Common conditions | Simulation duration sufficient for accurate results needs to be determined by proposer, and justified in proposal | remove | E/TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Common conditions | Having non-zero shadowing requires lengthy simulations to average over shadowing ensemble. Sufficient path loss variability reflected in the different locations | Set to 0 | TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 4 STA 6 | Mean rate should be .256 | Change mean rate | TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 4 STAs 11-20 | TCP sinks also generate ACKs | Add text that explicitly states that TCP acknowledgment traffic needs to be included. | TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 5 | Need to resolve comment about inconsistencies in printing rates |  | TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 6 | Too many STAs, simulation is more complex than it needs to be | Reduce number of stations to 25, and increase offered traffic levels proportionately | TN | 2 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 9 | TCP sinks also generate ACKs |  | TN | 2 | Open |
|  | Colin Lanzl | 11-03/802r12 | Mappings between Applications, Environment, Channel Model, Use Case, Usage Model and Simulation Scenario | The channel model document has been approved. | The second and third sentences should be changed to read: "Channel models have been defined in 11-03/940r2, TGn Channel Models, with 6 channel models. Each environment will map to a pair of channel models." | E | 1 | Open |
|  | Colin Lanzl | 11-03/802r12 | Environments | Table 2 indicates (5th row) that mandatory simulation scenario 16 uses channel model C-LOS conditions. In my review of both the Usage Model document and the Comparison Criteria document, I can only find channel models B-NLOS and D-NLOS called out in conjunction with simulation scenario 16. This 5th row is not needed and can either be blanked or removed. Addtionally, a review of the simulation scenarios shows that simulation scenario 11 uses both D-LOS and E-NLOS channel models: an entry for 11 needs to be added to the original 7th row for DLOS. | In the seventh row of table 2, add an entry for simulation scenario 11 (next to 4,9). Remove the 5th row of table 2. | E | 1 | Open |
|  | John Ketchum | 11-03/802r12 | Coexistence section |  | Need to remove | E | 1 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 4 | Comments on Cisco data on packet sizes no longer relevent | Remove comments on Cisco packet sizes | E | 1 | Open |
|  | John Ketchum | 11-03/802r12 | Scenario 4 | Units missing in some rates | Add units to mean rate | E | 1 | Open |
|  | Colin Lanzl | 11-03/813r9 | 3.1 | This section needs to be completed. | Add table entries for R2-R9. | E | 1 | Open |
|  | Colin Lanzl | 11-03/813r9 | 4 | Old reference for channel models. | Change reference [4] to: “IEEE 11-03/940r2, TGn Channel Models”. | E | 1 | Open |
|  | John Ketchum | 11-03/813r9 | FR1 |  | Remove “Add usage model…” | E | 1 | Open |
|  | John Ketchum | 11-03/813r9 | FR2 |  | Add reference to Sim Scen 16 | E | 1 | Open |
|  | John Ketchum | 11-03/814r16 | 4.2/CC9 | Transmit power is not power consumption. Everyone’s answer will be the same: 17 dBm | Remove this CC | TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 | 4.4.2/CC50 | Encryption decisions are independent of MAC design | Remove this CC | TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 | 4.5.4/CC59 | CC59 should be run with no physical layer impairments. Not stated explicitly now. | Add text stating that no physical layer impairments are used in these sims | TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 | 4.5.4/CC59 | CC59 too many potential antenna configurations | Use a set representative antenna configurations, maybe 5: 1xN, Nx1, N1xN1, N1xN2, N2xN2 | TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 |  | Bandwidth spec in CC59, CC67, &CC67.1 should be the same | Use spec in CC59: “Note that when computing the SNR, the signal and noise bandwidth shall be identical.” | TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 | Section 5/IM1 | Confusing language in definition. “Specify how PA backoff from full saturation used in the simulation calculated as PABackoff = –10 log10(Average TX Power/Power at saturation) is computed. | Simulation should be run at oversampling rate of at least 4x. Use RAPP power amplifier model as specified in document 00/294. Use P-parameter of 3. Specify the PA backoff from full saturation used in the simulation. The backoff is calculated as PABackoff = 10 log10(Average TX Power/Psat). For this test, Psat is specified to be 25dBm. | E/TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 | Section 6 | Content of section 6 is redundant and unneeded | Delecte section 6. If clarifications are needed add them to the CCs | E/TN | 2 | Open |
|  | Patrik Eriksson | 11-03/814r16 | 4.4.1/CC51 | Not clear how CC51 shall be addressed by adaptive rate proposals. The number of average rates can be very high. | Specify basic rates with adaptive loading turned off. For adaptive rate proposals, refer to detailed description of adaptation mechanism in the proposal. | TN | 2 | Open |
|  | Patrik Eriksson | 11-03/814r16 | 4.4.3/CC52 | How shall the PHY data rate required to achieve 100 Mbps goodput be known without knowledge of the MAC? | Specify test to be carried out at maximum mandatory PHY data rate. | TN | 2 | Open |
|  | Patrik Eriksson | 11-03/814r16 | 4.5.4/CC67.2 | Effect of sample timing drift due to symbol clock offset will depend on length of packets. | Define length of packets in simulation. | TN | 2 | Open |
|  | Patrik Eriksson | 11-03/814r16 | 5/IM1 | Sentence ‘Specify how PA backoff from full saturation used in the simulation calculated as PA Backoff=-10log10(Average Tx Power/Power at saturation) is computed” is confusing. Unclear why absolute power level is important. | Change sentence to “Specify the PA backoff from full saturation, calculated as PABackoff = 10 log10(Average TX Power/Psat), used in the simulation Explain why absolute power level is important. | TN | 2 | Open |
|  | Patrik Eriksson | 11-03/814r16 | 5/IM2 | Sensitivity to frequency offset and sample timing drift shown separately by CC67.2 | Remove IM2 | TN | 2 | Open |
|  | Patrik Eriksson | 11-03/814r16 | 5/IM4 | Phase noise PSD at 0 Hz -100 dBc/Hz too aggressive. Integrated phase noise over 8 MHz bandwidth is -44 dBc. This favours non-robust high order modulation proposals. | Change phase noise PSD at 0 Hz to -90 dBc/Hz. | TN | 2 | Open |
|  | John Ketchum | 11-03/814r16 | 4.5.4/CC67 | Correct spelling of fluorescent | Fluorescent | E | 1 | Open |
|  | Dmitry Akhmetov | 11-03-0814r14 | Comment in 3.4.1 | Common comment for all MAC SAP Measurements: Most of CC require coalucation of the type: something divide by simulation time. I’d like to note that simul;ation time global (for the whole simulation) and simulation time local ( for particular traffic flow) may be different: In my simulation I give 1 second at the beginning of simulation to the TCP flows to establish TCP connection. So, the other flows start activity a second after the beginning of simulation. Thus, simulation time term should be used very carefully. Or to define the that all traffic flows starts at one time. |  | TN | 2 | Open |
|  | Dmitry Akhmetov | 11-03-0814r14 | CC18 | Question: should backward TCP Ack flow be counted as non-QoS flow. My preference is yes, because fomr the point of MAC view it is just another flow comes from LLC. ( and because it is hard to distinguish backward TCPAck flow in simulation from direct flow, especially in case of DCF CAF) |  | TN | 2 | Open |
|  | Dmitry Akhmetov | 11-03-0814r14 | CC19 | I’d like to note here that we have here a problem. If we devide by total number of MSDUs offered at TX MAC SAP during the simulation than: STA may have 1-30 MSDUS in it buffer when siomulation ends. So, having a 1000 MSUDs transmitted and having 30 MSDUs not transmitted (in queue), for SDTV we will have 0.03 PLR while MAX PLR is 5*e-10^7. So, we might conclude that this flow failed to meet its QoS objectives, but this is not true .!!! So, I guess a more precise definition is required. |  | TN | 2 | Open |
|  | Herve Bonneville | 11-03-802r12 | Common conditions | Lack of instructions to how to abstract the PHY in system simulation | Output of TGn Simulation Methodology Ad Hoc committee shall be include. | TY | 3 | Open |
|  | Kowalski | 11-03-802r12 | Usage Models | There is still a huge amount of data called for here, that I think will bog the voter down in deciding which proposal is actually better. It is of utmost importance to try to pare down these usage models and simulation scenarios or summarize them succinctly so that voters are not buried in a blizzard of data that will, no doubt lead to contradictory results. | 1. Remove 6, 16, 17, 18 usage models, and all references thereto, or make them optional. I would say the large enterprise and residential models cover enough things for a meaningful comparison. | TY | 3 | Open |
|  | Herve Bonneville | 11-03-802r12 | Table 4 Application Definition | Applications 6 (Video conf) and 7 (internet streaming) have a PLR set to 10-2. This corresponds to one packet lost every 20 s (case 6) or 10 s (case 7). This requirement is too loose. | Set PER limit to 10-4. | TN | 2 | Open |
|  | Herve Bonneville | 11-03-802r12 | Common conditions | "TBD" on simulation duration. | We shall agree on a simulation time | TN | 2 | Open |
|  | Herve Bonneville | 11-03-802r12 | Authors list | Typo error on e-maill address | Replaced by following: bonneville@tcl.ite.mee.com | E | 1 | Open |
|  | Herve Bonneville | 11-03-802r12 | Applications, Note on the meaning of "Offered load" a"Protocol | The CC document includes actually a measure of packet loss rate. | The use of the future tense can be suppressed. | E | 1 | Open |
|  | Herve Bonneville | 11-03-802r12 | Table 6 "Usage model definitions" | There is 2 scenarios numbered 18 | The second should be 19 | E | 1 | Open |
|  | Herve Bonneville | 11-03-802r12 | Table 6 "Usage model definitions" | Scenarios 9 and 11 are no more used | Remove scenarios 9 and 11 | E | 1 | Open |
|  | Herve Bonneville | 11-03-813r9 | item 2 | Functional requirement does not specify a simulation context | Add reference to the simulation context #16 | TN | 2 | Open |
|  | Dmitry Akhmetov | 11-03-814r14 | CC24 | Need precise definition of how the average data rate is calculated. What is time-weighted average of PHY data rates? What should be the expected value of these criteria? This value is highly dependent of scenario configuration, because PHY data rates (if they are adaptive) will vary from STA to STA and so from data flow to data flow. Second point is the way to average: Imagine you have 20 data flows. You recorded (summarize) data rates for every data flow and you know number of received packets. How you will find total average PHY rate? By division of sum of all data rates by sum of all received bits?, i.e. PHY_RATE_EVER = (SUM_ALL_RATES)/(SUM_ALL_RCVD_PKTS) Or you will find average PHY rate per flow and than sum this averages and divide this number by number of flows? |  | TN | 2 | Open |
|  | Herve Bonneville | 11-03-814r16 | CC59bis | Lack of a CC with same channel conditions than CC59 but without PHY impairments | Include CC as voted on during February 25th FRCC meeting | TY | 3 | Open |
|  | Jeff Gilbert | 11-03-814r16 | CC67.1 | 1% PER operating point is very low and could favor particular proposals, with particular operating points. PER for 11a/g systems typically much higher from collisions alone. Constrains proposals. | Change to 10% or at least add both. | TY | 3 | Open |
|  | Jeff Gilbert | 11-03-814r16 | CC67.1 | Rate vs. SNR for given data rate biases results against systems with a discrete. | Change to plot Rate*(1-PER) with the rate for each location chosen by the proposal. | TY | 3 | Open |
|  | Jeff Gilbert | 11-03-814r16 | CC67.2 | 1% PER operating point is very low – some PHYs may not choose to optimize that point. Constrains proposals. | Use 10% | TY | 3 | Open |
|  | John Sadowsky | 11-03-814r16 | CC67.1 | I personally would like to support Qualcomm’s CC67.1 – however, but not with the 1% PER constraint. It is not that I am against the PER constraint. From a simulation point of view, it would be almost impossible to verify that the system is actually operating at a PER that low. I can tell you that for the idea rate adaptation TP simulations that I’ve done, the PER is on the order of 3-5%. However, I’m only doing 100 simulations for each of 100 channel realizations. In order to do a PER constraint on the order of 1%, I would have to do at least 1000 simulations per channel realization. Yes, you can live with a noisy PER estimate because the noise gets averaged out over the channel realizations. Nonetheless, it could be difficult to realize in practice. |  | TY | 3 | Open |
|  | Kowalski | 11-03-814r16 | 4.5.1, CC42 | Do we really need a sales pitch for “important properties” of preambles here? And what kind of analysis is specified here. | Remove “Summarize the important properties of each part the proposed preambles,” and “Results should include analysis performed on the transmit waveforms independent of any channel model” | TY | 3 | Open |
|  | Kowalski | 11-03-814r16 | 4.5.4, CC67 | You should probably have averaging over a minimum 10/x packets were x is the PER. What’s stated here is just inviting a kind of a contest I’d rather not see. IOW, you’re asking for statistical insignificance at the low end of the PER. | Reword “Averaging should occur over a minimum of 100 packet errors down to 1% PER.” | TY | 3 | Open |
|  | Kowalski | 11-03-814r16 | 4.5.4, CC67 | You should probably have averaging over a minimum 10/x packets were x is the PER. What’s stated here is just inviting a kind of a contest I’d rather not see. IOW, you’re asking for statistical insignificance at the low end of the PER. | Reword “Averaging should occur over a minimum of 100 packet errors down to 1% PER.” | TY | 3 | Open |
|  | Herve Bonneville | 11-03-814r16 | CC67.x | Assessing PHY performances with simulations using one channel and NLOS conditions only is too restictive | Add a simulation incorporating LOS conditions | TN | 2 | Open |
|  | Herve Bonneville | 11-03-814r16 | CC67.x | Including PHY impairmenents will biaises PHY comparisons as it will focus the criteria on how impairments are simulated. | Have a CC without PHY impairments | TN | 2 | Open |
|  | Herve Bonneville | 11-03-814r16 | CC18 & CC19 | The term "all mandatory scenario" is not relevant, as these CC do not refere to scenarios 9 to 19 | Refers to scenarios 1 to 6. Alternatively remove the label mandatory in scenarios 9 to 19 in Usage Model document | TN | 2 | Open |
|  | Kowalski | 11-03-814r16 | 4.4.2 (4.3?), CC50 | Should also identify what needs to be specified over and above 802.11i | Specify what, if anything needs to be changed from 802.11i to meet any changes due to Security. | TN | 2 | Open |
|  | Kowalski | 11-03-814r16 | 4.4.1, CC20 | Without muddying up things any further, I am wondering if a measure that explicitly separates uplink and downlink flows is needed. | Discuss within group, and see if this needs further subdivision. | TN | 2 | Open |
|  | Herve Bonneville | 11-03-814r16 | Authors list | Typo error on e-maill address | Replaced by following: bonneville@tcl.ite.mee.com | E | 1 | Open |