IEEE P802.11
Wireless LANs

Beacon Table Clarification

Date:	March 15th, 2004

Author:	Tim Olson

Cisco Systems, Inc.

170 West Tasman Drive

San Jose, CA 95134

e-Mail: tolson@cisco.com

Abstract

The Beacon Request supports a mode called “Beacon Table”mode where a client STA returns any saved beacon information from any operation that would save such information such as scanning for new APs.  This document provides some clarifying text for the “Beacon Table” mode.

#### 7.3.2.19.1 Beacon Request

Replace Beacon Table text with the following text

- In Beacon Table mode, the measuring STA shall return a Beacon Report containing the current contents of any stored beacon information for any channel without performing additional measurements.  The receiving STA shall ignore specified channel in the Beacon Request when Beacon Table mode is selected.  The beacon information accumulated may be the result of any operation that caused the STA to save these results.  If the STA has no saved beacon information then the STA may either refuse the request or send an empty Beacon Report.