IEEE P802.11
Wireless LANs

Comments of Airgo on FRCC documents

Date:	March 15, 2004

Author:	Allert van Zelst
	Airgo Networks
	Straatweg 66-S, 3621 BR Breukelen, The Netherlands
	Phone: +31 346 259 663
	Fax: +31 346 259 669
	e-Mail: allertvanzelst@airgonetworks.com

Abstract

This document contains the comments of Airgo Networks on the FRCC documents.

# Comments

| Number | Commenter's Name | Doc_Reference | Section/Item | Comment | Proposed resolution | E, TN, TY | Pri | Status | Consensuso |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Allert van Zelst | IEEE 802.11-02/814r16 | 4.5.1 CC42 | It is not clear what kind of analysis is meant in "Results should include analysis performed on the transmit waveforms independent of any channel model." | Remove sentence | TY |  |  |  |
|  | Allert van Zelst | IEEE 802.11-02/814r16 | 4.5.2 CC52 | It is not completely clear what is meant with "List for each channelization.", at least not in combination with the previous sentence: "Show the transmit spectral mask that ALL transmissions in the proposal meet." | Change in: "List the spectral mask requirements for each channelization of the proposal." | TY |  |  |  |
|  | Allert van Zelst | IEEE 802.11-02/814r16 | 4.5.4 CC59 | Missing "Averaging should occur over a minimum of 100 packet errors down to 1% PER." (see CC67) | Add "Averaging should occur over a minimum of 100 packet errors down to 1% PER." | TY |  |  |  |
|  | Allert van Zelst | IEEE 802.11-02/814r16 | 4.5.4 CC67.2 | How is the "highest average SNR possible" defined? In the other CC's the SNR is defined as the average baseband equivalent SNR at the input of each RX baseband processing, so basically this could be infinity?! | Remove sentence | TY |  |  |  |
|  | Allert van Zelst | IEEE 802.11-02/814r16 | 5 IM1 | Missing requirements on maximum total TX power and EIRP. | Add: "To make a fair comparison between all proposals everybody has to use the same total TX power and EIRP. The total TX power shall be 40 mW. The maximum antenna gain shall not exceed 6 dB in any 1 MHz of bandwidth. (This is according to the FCC regulations in the 5.15-5.25 GHz band.) | TY |  |  |  |
|  | Allert van Zelst | IEEE 802.11-02/814r16 | 5 | Missing IM on Adjacent Channel Interference (ACI) | Add: "For the rates simulated in CC67 show PER versus ACI curves. The SNR must be set to the value where the PER without ACI (see curves obtained from CC67) equals 1%. Averaging should occur over a minimum of 100 packet errors down to 1% PER." | TY |  |  |  |

.