IEEE P802.11
Wireless LANs

Comment Resolution

Date:	March 11th, 2004

Author:	Simon Black (InTalk2k Ltd.)

email: simon.black@intalk2k.com

Abstract

This paper proposes normative text to address the following comments raised in the IEEE802.11 TGk D0.9 review process:

| Comment Number(s) | Clause | Summary of Comment |
| --- | --- | --- |
| 136 | 5 | RRM service definitions |
| 79, 80, 84 | 7.3.2.19.1 | Clarification of beacon measurement active scan mode – proposed solution removed from this paper in R1 for further study |
| 19 | 7.3.2.19.1 | Clarification of beacon measurement passive scan mode – proposed solution removed from this paper in R1 for further study |
| 83 | 7.3.2.19.1 | Clarification of beacon measurement reporting |
| 32, 97, 111, 175, 176 | 7.3.2.19.7, 7.3.2.20.7 | Clarification of measurement duration and randomization time for STA statistics measurement |
| 107 | 7.3.2.20.3 | Clarification of CCA busy fraction |
| 210, 211 | 7.3.2.22 | Missing band definition in Site Report |
| 20, 41, 129 | 11.7.1 | Definition of serving and non-serving channel |
| 144 | 11.7.3 | Clarification of randomization interval |
| 232 | 11.7.4 | Reorganise 11.7.2, 11.7.3 and 11.7.4 (partly done in D0.12) |

The editing instructions and text proposed are relative to IEEE802.11k/D0.12.

There are two type of editing instruction in this document. The first are the editing instructions that apply the IEEE802.11k changes to the baseline text – these are part of the normative text and should be included in the IEEE802.11k draft. Editing instruction of this type appear in the standard bold italic style. The second type of editing instruction are intended to be used by the editor in applying the contents to draft D0.12. These instructions should not appear in the D0.12 draft and appear as bold text within [brackets].

# Proposed Resolutions for Open Comments

### Comment #136
RRM service definitions

Proposed service definition text to address this comment is given in a paper dealing with the harmonization of 802.11k/D0.12 and 802.11h (document 04/292r1).

### Comments #19/79/80/83/84
Clarification of beacon measurement modes

This R1 version of this paper just addresses comment 83 by the addition of a paragraph to 11.7.7.1. The resolution of 19, 79, 80 and 84 are for further study.

#### 11.7.7.1 Beacon Report

[Add the following paragraph to 11.7.7.1 following the existing text:]

When more than one Beacon, or Probe Request from a BSS is received in the measurement duration, the contents of the Beacon Report shall be based on the latest to be received.

### Comments #32/97/111/175/176
Clarification of duration and randomization in STA statistics measurement

The proposed resolution for this set of comments is to redraft text for the STA Statistics request and report to introduce the randomization interval and clarify the use of the duration field.

[Amend clause 7.3.2.21.10 as follows – marked changes are relative to D0.12 text, section numbering is harmonized with 802.11h:]

#### 7.3.2.21.10 STA Statistics Request

The STA Statistics Request Measurement enables a STA to query statistics and counters in a peer STA MIB. The Measurement Request field corresponding to a STA Statistics Request is shown in Figure [Editor TBD].

|  | Randomization Interval | Measurement Duration | Group Identity |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure [Editor TBD]—Measurement Request field format for a STA Statistics Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.7.3.

The Measurement Duration field shall be set equal to the duration of the requested measurement in TUs. The STA Statistics request queries the change in STA counters within the Measurement Duration. A Measurement Duration of 0 shall be used to request instantaneous values of the requested STA counters.

Group Identity indicates the requested statistics group according to Table [Editor TBD].

Table [Editor TBD]—Group Identity for a STA Statistics RequestReport

| Statistics Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| Reserved | 1 – 255 |

[Amend clause 7.3.2.22.10 as follows – marked changes are relative to D0.12 text, section numbering is harmonized with 802.11h:]

#### 7.3.2.22.10 STA Statistics Report

The format of the Measurement Report field of an STA Statistics Report is shown in Figure [Editor TBD].

|  | Measurement Duration | Statistics Group Data |
| --- | --- | --- |
| Octets: | 2 | Variable |

Figure [Editor TBD]—Measurement Report field format for a STA Statistics Report

Measurement Duration shall be set equal to the duration over which the Statistics Data was measured, expressed in TUs. A Measurement duration of 0 indicates instantaneous data.

The STA Statistics Report reports the change in STA counters within the Measurement Duration. A Measurement Duration of 0 shall be used to report instantaneous values of the indicated STA counters.

Statistics Group Data shall contain the requested statistics from MIB related to the interface on which the request was received according to [Editor TBD].

Table [Editor TBD]—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 0 | Dot11CountersEntry for the Interface on which the STA Statistics Request was received with the exception of WEPUndecryptableCount: dot11TransmittedFragmentCount (Counter32), dot11MulticastTransmittedFrameCount (Counter32), dot11FailedCount (Counter32), dot11RetryCount (Counter32), dot11MultipleRetryCount (Counter32), dot11FrameDuplicateCount (Counter32), dot11RTSSuccessCount (Counter32), dot11RTSFailureCount (Counter32), dot11ACKFailureCount (Counter32) dot11ReceivedFragmentCount (Counter32), dot11MulticastReceivedFrameCount (Counter32), dot11FCSErrorCount (Counter32), dot11TransmittedFrameCount (Counter32). |
| 1 – 255 | None |

### Comment #107
Clarification of Channel Load Report

The proposed proposed resolution of this comment is to include the missing Channel Load field in the figure illustrating the Channel Load Report field. The title of this figure is corrected and CCA Busy Fraction renamed Channel Load to better match the usage. A definition of Channel Load is given that includes NAV and appropriate units.

[Amend figure 0-13 in clause 7.3.2.22.4 as follows – marked changes are relative to D0.12, section numbering is harmonized with 802.11h:]

|  | Channel Number | Channel Band | Actual Measurement Start Time | Measurement Duration | Channel Load |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 1 |

Figure 0-1—Measurement Report field format for a Channel Load Report

[Amend the fifth paragraph of clause 7.3.2.22.4 as follows – marked changes are relative to D0.12 text:]

Channel Load shall contain the fraction duration over which the measuring STA determined the channel to be busy during the measurement duration. The Channel Load value is defined as Ceiling(255*[channel busy time (microseconds)]/(1024 * [measurement duration (TU)])). Channel busy time shall be the time during which either the physical carrier sense or NAV indicated channel busy, as defined in 9.2.1.

### Comments #210, 211
Missing band definition in site report

The proposed resolution for these comments is the inclusion of the Channel Band field in the site report and the addition of related text defining the contents.

#### 7.3.2.26 Site Report element

[Insert Channel Band field in the Site Report element figure as follows – marked changes are relative to D0.12, section numbering is harmonized with 802.11h:]

|  | Element ID | Length |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |  |  |  |  |  |  |
|  | BSSID | BSSID Match Status |  | Current Channel |  | Channel Band |  | PHY Type | The element contains zero or more quintuplets |  |
| Octets: | 6 | 2 |  | 1 |  | 1 |  | 1 |  |  |

Figure [Ed TBD]—Site Report element format

[Add the following text prior to the final paragraph]

Channel Band shall contain an enumerated value from Table [Editor TBD] specifying the frequency band in which the Current Channel is valid.

### Comments #20, 41, 129
Definitions of serving and non-serving channel

[Add the following new definitions in alphabetic order in clause 3, renumbering as necessary:]

Non-Serving Channel: A channel that is not being used by a STA for the exchange of MAC Service Data Units (MSDUs).

Serving Channel: A channel that is being used by a STA for the exchange of MAC Service Data Units (MSDUs).

### Comment #232
Reorganisation of 11.7.2, 11.7.3 and 11.7.4

Revised normative text for clause 11.7.3 was adopted in January and is already included in D0.12. This leaves the merging of 11.7.2 and 11.7.4 to resolve this comment.

[Replace 11.7.2 with the following, delete 11.7.4, renumber remaining sections in 11.7]

#### 11.7.2 Station responsibility for measuring non-serving channel

All stations are responsible for (1) providing a power-save notification before switching channels to execute non-serving channel measurements or (2) remaining in active mode and relying on application-specific knowledge, or other knowledge to determine that no incoming frames are expected.

A STA shall determine the time between successive non-serving channel measurements by applying a rule that requires it to return to the serving channel for a particular length of time between non-serving channel measurements. This time may be a fixed length, or it may be determined by the STA using application-specific, or other knowledge.

# Comments with no Action Required

### Comment #144
Clarification of randomization interval

Revised normative text for clause 11.7.3 was adopted in January. This new text addresses the comment in that it replaces the wording ‘A STA may adopt any randomization interval between 0 and Randomization Interval’ by ‘Prior to making each measurement in the requested sequence, the STA shall calculate a random delay distributed uniformly in the range 0 to the randomization interval specified in the measurement request.’