IEEE P802.11
Wireless LANs

Toshiba Comments on Usage Model and Comparison Criteria

Date:	March 16, 2004

Author:	Masahiro TAKAGI  and Darren McNamara
	TOSHIBA
	1 Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, JAPAN
	Phone: +81-44-549-2238
	Fax: +81-44-520-1806
	e-Mail: takagi@csl.rdc.toshiba.co.jp

Abstract

# Comments

| Number | Commenter's Name | Doc_Reference | Section/Item | Comment | Proposed resolution | E, TN, TY | Pri | Status | Consensuso |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Masahiro TAKAGI | 11-03/802r13 | VoIP Application (Table.4) | The offered load has range from 0.02 to 0.15 Mbps, but every usage model uses 0.15 Mbps. This means that VoIP packet is generated every 5.33msec. It is too frequenct, we believe that 20msec is typical (G.711 64kbps voice data rate and 160bytes voice data per packet). | If we consider per packet overhead above Link layer, 96 kbps and 240 bytes seems to be an appropriate one | TY |  |  |  |
|  | Masahiro TAKAGI | 11-03/802r13 | Interactive Gaming Controller (Table.4) | This controller generate packet every 800micro second. This seems too frequent and typical gaming Controller traffic characteristic is not known, so this shold not be included as a representative traffic. | Remove interactive gaming controller. | TY |  |  |  |
|  | Masahiro TAKAGI | 11-03/802r13 | Scenario 19 | The location of STA1 (legacy) and STA2 (HT) is not appropriate to evaluate the backward compatibility of proposed PLCP preambles and headers from the CCA point of view. | Change the location of STA1 (legacy) from (0, 10) to (-10, 0). Location of STA2 (10, 0) shall not be changed | TY |  |  |  |
|  | Masahiro TAKAGI | 11-03/814r17 | CC7 | MAC processing complexity should include 802.11i. | Add 802.11i to the definition. | TN |  |  |  |
|  | Masahiro TAKAGI | 11-03/814r17 | CC11 | Backward compatibility should include 802.11g as well as 802.11-1999 (2003) | Change the name of CC11. Add 802.11g to definition. | TN |  |  |  |
|  | Masahiro TAKAGI | 11-03/814r17 | CC15 | Legacy share should be T3/T2 rather than T3/T1. Legacy impact should be (T1 – T4)/T1 rather than (T2 – T4)/T2 T1 : HT goodput. T2 : Legacy goodput. T3 : Legacy goodput sharing with HT. T4 : HT goodput sharing with legacy. | Change as specified in the comment | E or TY |  |  |  |
|  | Masahiro TAKAGI | 11-03/814r17 | CC24 | It is not clear enough how to calculate the averate data rate. How to count IFS and backoff? | Clarify. | TY |  |  |  |