IEEE P802.11
Wireless LANs

Clause 8.5 edits

Date:	March 16, 2004

Author:	Tim Moore 
	Microsoft 
	1 Microsoft Way, Redmond, WA
	Phone: 425-703-9861
	Fax: 
	e-mail:timmoore@microsoft.com

Abstract

This document contains comment resolution proposals for 802.11i Sponsor Ballot. Contributors to this document are Tim Moore.

48

8.3.2..3.2 Figure 21

Change “Plumb” to “Configure”

8.5.1

Change “plumbed” to “configured”

59

8.5.2

1.  The value 0 (Group) indicates the message is not part of a PTK derivation.

To

1.  The value 0 (Group/STAKey) indicates the message is not part of a PTK derivation.

60

8.5.2.1

The originating STA requests the STAKey by sending an EAPOL-Key frame to the AP, with the Request bit set and with a MAC Address IE in the Key Data field.

To

The originating STA requests the STAKey by sending an EAPOL-Key frame to the AP, with the Key Type set to 0, Request bit set to 1 and with a MAC Address IE in the Key Data field.

61

8.6

8.6.1 Mapping PTK to TKIP keys

Clause 8.5.1.2 defines the EAPOL Temporal Key derived from PTK.

A STA shall use bits 0-127 of Temporal Key as its input to the TKIP Phase 1 and 2 Mixing Functions.

A STA shall use bits 128-191 of Temporal Key as the Michael key for MSDUs from the Authenticator’s STA to the Supplicant’s STA or from the initiator STA to peer STA for STAKeys.

A STA shall use bits 192-255 of Temporal Key as the Michael key for MSDUs from the Supplicant’s STA to the Authenticator’s STA or from the peer STA to initiator STA for STAKeys.

8.6.2 Mapping GTK to TKIP keys

Clause 8.5.1.3 defines the EAPOL Temporal Key derived from GTK.

A STA shall use bits 0-127 of Temporal Key as the input to the TKIP Phase 1 and 2 Mixing Functions.

A STA shall use bits 128-191 of Temporal Key as the Michael key for MSDUs from the Authenticator’s STA to the STA or from the initiator STA to peer STA for STAKeys.

A STA shall use bits 192-255 of Temporal Key as the Michael key for MSDUs from the Supplicant’s STA to the Authenticator’s STA or from the peer STA to initiator STA for STAKeys.

10.3.11.1.2

| Authenticator/Supplicant | Boolean | TRUE, FALSE | Whether the key is configured by the Authenticator or Supplicant, TRUE indicates that the Michael integrity key is for Tx. |
| --- | --- | --- | --- |

| Authenticator/Supplicant Initiator/Peer | Boolean | TRUE, FALSE | Whether the key is configured by the Authenticator or Supplicant, TRUE indicates Authenticator or Initiator. |
| --- | --- | --- | --- |

62

8.5.2.2

I:  Install  bit:  Tx/Rx  for  Group  key  and  Install/Not  install  for  the  pairwise  key.  This  is  the EAPOL-Key Information Install Flag bit.

To

I:  Install bit:  Install/Not install  for  the  pairwise  key.  This is the EAPOL-Key Information Install Flag bit.

63

8.5.3

Message 3. Authenticator  Supplicant:  10

EAPOL-Key(0,1,1,1,P,KeyRSC,ANonce,MIC,RSNIE,GTK[N])  11

Message 4. Supplicant  Authenticator: EAPOL-Key(0,1,0,0,P,0,0,MIC,0,0)

to

Message 3. Authenticator  Supplicant:

EAPOL-Key(1,1,1,1,P,KeyRSC,ANonce,MIC,RSNIE,GTK[N])

Message 4. Supplicant  Authenticator: EAPOL-Key(1,1,0,0,P,0,0,MIC,0,0)

64

8.5.3.3

Delete line

“GTK Length = Version Specific “

65

8.5.4.2

Delete text

The Authenticator  shall use MLME-DEAUTHENTICATE.request primitive  to deauthenticate  the STA  if  message 2 of the Group Key Handshake has not been received by the Authenticator after the timeout period of  dot11RSNAConfigGroupUpdateCount  times  dot11RSNAConfigGroupUpdateTimeOut after  initiating  the  Group  Key  Handshake.  The  Authenticator  may  fail  to  complete  the  Group  Key Handshake because the AP is unable to transmit the message for a period of time due to signal fading,

8.5.4.3

If  the  Authenticator  does  not  receive  a  reply  to  its  messages,  its  shall  attempt dot11RSNAConfigPairwiseUpdateCount  transmits  of  the  message,  plus  a  final  timeout.  The retransmit timeout value shall be 100 milliseconds for the first timeout, half the listen interval for the second timeout, and the listen interval for subsequent timeouts. If there is no listen interval, then 100 milliseconds shall be used for all timeout values. If it still has not received a response after this, then the Authenticator’s STA should use MLME-DEAUTHENTICATE.request primitive to deauthenticate the STA.

69

8.5.2

Key Length. This  field  is  two  (2) octets  in  length,  represented as an unsigned binary number. The value defines the length in octets of the key to configure into IEEE 802.11.

to

Key Length. This field is two (2) octets in length, represented as an unsigned binary number. The value defines the length in octets of the Pairwise Temporal Key to configure into IEEE 802.11.

70

8.5.4

Delete text

If  the GTK  cipher  suite  is TKIP,  the Authenticator  shall  initiate  the  exchange  if  its STA detects  a TKIP MIC integrity failure using the GTK, receives a Michael MIC Failure Report, or on a management event.

71

8.5.4

Delete text

If an AP cannot send the EAPOL-Key frame containing a GTK to a STA, the AP may queue the message. If the AP deletes the message, the AP should send a Deauthentication message and then delete the association state by setting the Disconnect failure event in the Authenticator state machine.

72

8.5.4.1

Encrypted Key Data = 0

To

Encrypted Key Data = 1

75

8.5.3.6

Delete text

Informative Note: The “Initial exchange complete” bit is set in the last message from the Authenticator to the Supplicant to inform the Supplicant that the last key required to initialize the Supplicant has been sent. Once set the “Initial exchange complete” bit should be set in any EAPOL-Key frames from the Authenticator until a 4-Way Handshake is initiated.

88

8.5.2

Key Data Length. This field is two (2) octets in length, taken to represent an unsigned binary number. This represents the length of the Key Data field in octets. If the Encrypted Key Data flag is set, the length is the length of the Key Data after encryption.

To

Key Data Length. This field is two (2) octets in length, taken to represent an unsigned binary number. This represents the length of the Key Data field in octets. If the Encrypted Key Data flag is set, the length is the length of the Key Data after encryption (including any padding).

123

8.5.1

In  an  ESS, AA  is  the wireless  shall  be  the BSSID  used  by  the AP,  and  SPA  the MAC address of  the STA.

to

In an ESS, the AA and AP’s BSSID are equal, and the SPA and the STA’s MAC address are equal.

7

8.5.2

The encrypted GTK is included and the GTK Length indicates the unencrypted length of the GTK in octets. The GTK is encrypted as specified by the Key descriptor version.

to

An encapsulated GTK shall be included and the unencrypted length of the GTK is six less than the length of the GTK KDE in octets.  The entire Key Data field shall be encrypted as specified by the Key descriptor version.

The Key Data field is a variable-length field that is used to include any additional data required for the  key  exchange  which  is  not  included  in  the  fixed  fields  of  the  EAPOL-Key  descriptor.  The additional  data  may  be  zero  or  more  Information  Element(s)  (such  as  the  RSN  IE),  GTK(s), STAKey(s), or PMKID(s), or vendor-specific data. Information Elements sent in the Key Data field include  the  Element  ID  and  Length  fields.  Data  other  than  Information  Elements  shall  be encapsulated using the following format:

to

The Key Data field is a variable-length field that is used to include any additional data required for the  key  exchange  which  is  not  included  in  the  fixed  fields  of  the  EAPOL-Key  descriptor.  The additional  data  may  be  zero  or  more  Information  Element(s)  (such  as  the  RSN  IE), and zero or more KDE(s) (such as GTK(s),  STAKey(s), or PMKID(s)). Information Elements sent in the Key Data field include the Element ID and Length fields.  KDEs shall be encapsulated using the following format:

STAs shall ignore any information elements they do not understand.

To

STAs shall ignore any IEs or KDEs they do not understand.

Data  fields  that  are encrypted but do not contain the GroupKey or STAKey information element shall be accepted.

to

Data fields that are encrypted but do not contain the GroupKey or STAKey KDE shall be accepted.

If the GroupKey or STAKey information element is included in the Key Data field but the Key data field is not encrypted the EAPOL-Key frames shall be ignored.

To

If the GroupKey or STAKey KDE is included in the Key Data field but the Key data field is not encrypted the EAPOL-Key frames shall be ignored.

The format of the GTK IE is as follows:

To

The format of the GTK KDE is as follows:

If  the  value  of  the  Tx  is  1  then  the  IEEE  802.1X  component  shall  configure  the  Temporal Key derived from this IE into its IEEE 802.11 STA for both transmission and reception.

To

If  the  value  of  the  Tx  is  1  then  the  IEEE  802.1X  component  shall  configure  the  Temporal Key derived from this KDE into its IEEE 802.11 STA for both transmission and reception.

If the value of the Tx is 0 means IEEE 802.1X component shall configure the Temporal Key derived from this IE into its IEEE 802.11 STA for reception only.

To

If the value of the Tx is 0 means IEEE 802.1X component shall configure the Temporal Key derived from this KDE into its IEEE 802.11 STA for reception only.

The format of the STAKey and Peer MAC Address IE is as follows

To

The format of the STAKey and Peer MAC Address KDE is as follows

The format of the MAC Address IE is as follows

To

The format of the MAC Address KDE is as follows

85

8.5.2

Table 7

| XX-XX-XX | 1 | GTK Key Data Encapsulation |
| --- | --- | --- |
| XX-XX-XX | 2 | STAKey Key Data Encapsulation |
| XX-XX-XX | 3 | MAC Address Key Data Encapsulation |
| XX-XX-XX | 4 | PMKID Key Data Encapsulation |

Figure 37 – MAC Address Encapsulation Format

To

Figure 37 – MAC Address Key Data Encapsulation Format

Figure 38 – PMKID Encapsulation Format

To

Figure 38 – PMKID Key Data Encapsulation Format