IEEE P802.11
Wireless LANs

TGi SBr1 Clause 3 and 7 Motions

Date:	March 17, 2004

Authors:	Frank Ciotti
 	Apacheta Corp.

Dave Halasz
	Cisco Systems

Abstract

Motions drafted to address Sponsor Ballot re-circulation 1 Comments on Clauses 3and 7 of 802.11i draft 8.0.

Clause 3

Comments 24, 106 – Accept

Note that Comment 106 was rejected in an earlier motion (doc 04/329), however we are now accepting it.

In Clause 3, delete the following first definition of Authenticator:

Authenticator: An IEEE 802.1X Port Access Entity acting in the IEEE 802.1X Authenticator role.

In Clause 3, move the definition of “Authenticator Address (AA)” to after the definition of “Authenticator”.

In Clause 3, replace:

Supplicant: An IEEE 802.1X Port Access Entity acting in the IEEE 802.1X Supplicant role.

With:

Supplicant: See the IEEE 802.1X specification for a definition of this concept.

In Clause 3, replace:

Controlled Port: See IEEE 802.1X for a definition of this concept.

With:

Controlled Port: See IEEE 802.1X specification for a definition of this concept.

Clause 7

Comment 10 - Accept

In Clause 7.3.1.4, replace:

Non-AP STAs in an ESS set the Privacy subfield to 0 within transmitted Association Request and Reassociation Request management frames. APs ignore the Privacy subfield within received Association Request and Reassociation Request management frames.

With:

In an RSNA, non-AP STAs in an ESS set the Privacy subfield to 0 within transmitted Association Request and Reassociation Request management frames. APs ignore the Privacy subfield within received Association Request and Reassociation Request management frames.