IEEE P802.11
Wireless LANs

TGi SBr1 Misc Motions

Date:	March 17, 2004

Authors:	Frank Ciotti
 	Apacheta Corp.

Dave Halasz
	Cisco Systems

Abstract

Motions drafted to address Sponsor Ballot re-circulation 1 Comments on 802.11i draft 8.0.

Note: Comment numbers from doc 04/1004

Comment 141 - Accept

In Clause 5.9.3.2, replace:

Since in an IBSS there are two 4-Way Handshakes between any two STA Supplicants and Authenticators, the Pairwise key used between any two STAs is from the 4-Way Handshake initiated by the STA Authenticator with the higher MAC address.

With:

Since in an IBSS there are two 4-Way Handshakes between any two STA Supplicants and Authenticators, the Pairwise key used between any two STAs is from the 4-Way Handshake initiated by the STA Authenticator with the higher MAC address (see Clause 8.5.1 for MAC comparison).

Comment 141 - Accept

In Clause 7.2.2, replace:

The frame body consists of the MSDU or a fragment thereof, and a security header and trailer (if and only if the Protected Frame subfield in the frame control field is set to 1). The frame body is null (0 octets in length) in data frames of Subtype Null function (no data), CF-Ack (no data), CF-Poll (no data), and CF-Ack+CF-Poll (no data). These frame types shall be checked to verify that the frame body is null, and, if not discard the frame without indication to LLC.

With:

The frame body consists of the MSDU or a fragment thereof, and a security header and trailer (if and only if the Protected Frame subfield in the frame control field is set to 1). The frame body is null (0 octets in length) in data frames of Subtype Null function (no data), CF-Ack (no data), CF-Poll (no data), and CF-Ack+CF-Poll (no data).

Comment 142 - Accept

In Clause 10.3.11.1.4, replace:

Receipt of this primitive causes the MAC to set the appropriate keys and to begin using them as indicated.

With:

Receipt of this primitive causes the MAC to set the appropriate keys and to begin using them for future MA-UNITDATA.request and MA-UNITDATA.indication primitives provided the MLME-SETPROTECTION.request has been issued.

Comment 143 - Accept

In Clause 10.3.11.1.2, replace:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Keylist | A set of KeyDescriptors | N/A | The list of keys to be used by the MAC. |

Each KeyDescriptor consists of the following elements:

With:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Keylist | A set of SetKeyDescriptors | N/A | The list of keys to be used by the MAC. |

Each SetKeyDescriptor consists of the following elements:

In Clause 10.3.12.1.2, replace:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Keylist | A set of KeyDescriptors | N/A | The list of keys to be deleted from the MAC |

Each KeyDescriptor consists of the following elements:

With:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Keylist | A set of DeleteKeyDescriptors | N/A | The list of keys to be deleted from the MAC |

Each DeleteKeyDescriptor consists of the following elements:

Comment 144, 145 - Accept

In Clause 10.3.12.1.2, replace the following table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Address | MAC Address | Any valid individual MAC address | This parameter is valid only when the key type is Pairwise, or when the key type is Group and is from an IBSS STA, or when the key is type STAKey |

With:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Key ID | Integer | N/A | Key ID |
| Key Type | Integer | Group, Pairwise, STAKey | Defines whether this key is a Group or Pairwise key, or STAKey key. |
| Address | MAC Address | Any valid individual MAC address | This parameter is valid only when the key type is Pairwise, or when the key type is Group and is from an IBSS STA, or when the key is type STAKey |

Comment 146 – Accept (addressed in draft 8)

Comment 147 – Accept (addressed in motion in Orlando)

Comment 148 – Reject (dual Association)

Comment 149 - Accept

In Clause 11.4.5, replace list item b:

- The state variable for the AP shall be set to 2.

With:

- The state variable for the AP shall be set to 2 if and only if it was not 1.

Comment 150 - Reject