IEEE P802.11
Wireless LANs

Returning Measurement Reports

Date:	March 17th, 2004

Author:	Simon Black (InTalk2k Ltd.)

emails: simon.black@intalk2k.com

Abstract

This paper contains text to address comment #229 which concerns the return of multiple measurement reports in response to a single measurement request frame. Changes marked are relative to IEEE802.11k/D0.12. The editing instruction is relative to D0.12 and is not part of the proposed text.

[Change the ninth and tenth paragraphs of 11.7.6 as follows:]

The result of each Measurement Request frame shall be returned without undue delay to the requesting STA in Measurement Report elements using one or more Measurement Report frames. The measuring STA may choose to truncate its results, if necessary, to limit its response to a single Measurement Report frame.

The Measurement Report frames shall contain the same Dialog Token field as the corresponding Measurement Request frame and each Measurement Report element shall contain the same Measurement Token field as the corresponding Measurement Request element.

The result of each measurement requested in a Measurement Request element shall be reported in a Measurement Report element of type corresponding to the request. The result of each measurement shall be returned without undue delay to the requesting STA.

When more than one Measurement Request element is present in a Measurement Request frame, the STA may return the corresponding Measurement Report elements in one or more Measurement Report frames. Measurement reports may be returned in more than one Measurement Report frame due to the length of the reports, or to ensure that individual reports are returned in a timely manner following completion.

Each Measurement Report frame shall contain the same Dialog Token field value as the corresponding Measurement Request frame. Each Measurement Report element shall contain the same Measurement Token field value as the corresponding Measurement Request element.