IEEE P802.11
Wireless LANs

Title

Date:	April 20, 2004

Author:	John Ketchum
	Qualcomm
	Phone: 781-276-0915
	Fax: 781-276-0901
	e-Mail: johnk@qualcomm.com

Abstract

The following is a proposed revised version of simulation scenario 6.    The main feature of this is a reduction in the number of STAs from 52 to 34:  5 fewer VoIP STAs; 10 fewer Internet File Transfer STAs, but the remaining STAs have twice the mean rate; 3 fewer High quality streaming audio/video STAs, the remaining STAs have twice the mean rate.  The total offered traffic level of the result is little changed from the original, as are the distance statistics.

## Scenario 6: Hot spot

Use channel model E for this scenario.

| STA Name: AP1 |  | Role: AP |  |  | Location: 0,0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| STA1-10 (replicated) | 2 |  | TCP | 300 |  |  |  | Internet File Transfer |
| STA11-14 (replicated) | 2 |  | UDP | 512 |  | 200ms |  | Mid quality streaming audio/video |
| STA15-17 (replicated) | 8 |  | UDP | 512 |  | 200ms |  | High quality streaming audio/video |
| STA18-19 (replicated) | 5 |  | UDP | 1500 |  | 200ms |  | SDTV broadcast |
| STA20-34 (replicated) | 0.15 Mbps |  | Constant, UDP | 100 |  | 30 |  | VoIP |

| STA Name: STA20 – STA34 |  | Role: VoIP Phone |  |  | Location: See table below |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Data Sources |  |  |  |  |  |  |  |  |
| Destination STA | Mean Rate |  | Rate Distribution | MSDU Size |  | MAX Delay ms |  |  |
| AP1 | 0.15 Mbps |  | Constant, UDP | 100 |  | 30 |  | VoIP |

Note, the other STA are sinks only.

Locations (m)

Table 1 - STA locations for SS6

| STA | X | Y |
| --- | --- | --- |
| 1 | 21 | 23 |
| 2 | 21 | -16 |
| 3 | 13 | 1 |
| 4 | 7 | 6 |
| 5 | 21 | 20 |
| 6 | -1 | -5 |
| 7 | -12 | 22 |
| 8 | -11 | -5 |
| 9 | -15 | 6 |
| 10 | -23 | -2 |
| 11 | 13 | 19 |
| 12 | 13 | -19 |
| 13 | -16 | 16 |
| 14 | -18 | -17 |
| 15 | 16 | 2 |
| 16 | 3 | -10 |
| 17 | -5 | 15 |
| 18 | 4 | -1 |
| 19 | -6 | 0 |
| 20 | 21 | 22 |
| 21 | 19 | -11 |
| 22 | 15 | 10 |
| 23 | 9 | -5 |
| 24 | 4 | 12 |
| 25 | -2 | -22 |
| 26 | -8 | 18 |
| 27 | -14 | -18 |
| 28 | -17 | -8 |
| 29 | -23 | 10 |
| 30 | 21 | 0 |
| 31 | 19 | -22 |
| 32 | 8 | 21 |
| 33 | 7 | -14 |
| 34 | 2 | 3 |