IEEE P802.11
Wireless LANs

New Beacon Reporting Conditions

Date:	May 2004

Author:	Emily Qi, Mustafa Demirhan, Mousumi Hazra, Nandakishore Kushalnagar

Intel Corporation

2111 N.E 25th Avenue

Hillsboro, OR 97124

e-Mail: {emily.h.qi, Mustafa.demirhan, Mousumi.m.Hazra, Nandakishore.Kushalnagar } @intel.com

Abstract

#### The beacon reporting condition is used for specifying when a beacon report is issued. This document proposes two new reporting conditions for periodic measurement, which can be used as the indications of a new AP being detected or an existing AP leaving.

#### Add the following texts in Table 0-3

Table 0-3 – Reporting Condition definitions for Beacon Request element

| Condition Description | Reporting Condition |
| --- | --- |
| Report to be issued in the periodic measurement immediately after the RCPI level of the measured STA enters or leaves a range bound by the serving AP’s RCPI and an offset (with hysteresis) from the serving AP’s RCPI. | 11 |
| Report to be issued in the periodic measurement immediately after the RSSI level of the measured STA enters and leaves a range bound by the serving AP’s RSSI and an offset (with hysteresis) from the serving AP’s RSSI. | 12 |
| Reserved | 13-255 |

Add the following paragraph to 7.3.2.21.6 following the existing text:

Hysteresis provides an unsigned 8 bit integer hysteresis value having the same units as used in the Threshold/Offset field.

Multiple reports may be issued when condition #11 or #12 is met during the measurement interval.