Motion: Instruct Editor to make following changes to 802.11k draft D.014:

7.3.2.21.6 Beacon Request

[Add the following texts following the second paragraph of 7.3.2.21.6]

If Channel Number is 0, it indicates that the receiving STA shall pursue iterative measurements for all channels that are valid for the regulatory domain in the specified channel band.

[Add the following texts following figure X-2 – Measurement Interval field:]

If channel Number is 0 and scan mode is set to Active mode or Passive mode, the receiving STA shall iteratively conduct measurements for all channels in the specified channel band. The time between each consecutive measurement is defined in subclause 11.7.2. The receiving STA shall randomly select a channel number to start with and pursue iterative measurements for all channels during the measurement interval.

Move: Jesse Walker