IEEE P802.11
Wireless LANs

Site Report MLME Primitives

Date:	May 10th, 2004

Author:	Simon Black, Hasse Sinivaara

Nokia

e-Mails: simon.black@intalk2kcom, hasse.sinivaara@nokia.com

Abstract

This paper proposes a set of MLME primitives for the site report request-response protocol. It addresses comment #2 in the TG review of IEEE802.11k/D0.14 (comment numbering from 04/480).

This information in this document is based on that first proposed in document 03/0580 and 04/0400.

### 10.3.15 Site Report Request

Insert the following new subsections following 10.3.14

This set of primitives supports the signaling of Site Report Requests.

#### 10.3.15.1 MLME-SITEREPORTREQ.request

#### 10.3.15.1.1 Function

This primitive requests that a Site Report Request frame be sent to a peer MAC entity.

#### 10.3.15.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SITEREPORTREQ.request	(
Peer MAC Address,
Dialog Token,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Site Report Request frame shall be set. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the site report transaction. |

#### 10.3.15.1.3 When Generated

This primitive is generated by the SME to request that a frame be sent to a peer entity to request an Site Report.

#### 10.3.15.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Site Report Request Action management frame. This frame is then transmitted.

#### 10.3.12.2 MLME-SITEREPORTREQ.confirm

#### 10.3.12.2.1 Function

This primitive reports the result of a request to send a Site Report Request frame.

#### 10.3.12.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SITEREPORTREQ.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, UNSPECIFIED FAILURE | Reports the outcome of a request to send a Site Report Request frame. |

#### 10.3.12.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Site Report Request frame completes.

#### 10.3.12.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.15.3 MLME-SITEREPORTREQ.indication

#### 10.3.15.3.1 Function

This primitive indicates that an Site Report Request frame was received from a peer STA.

#### 10.3.15.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SITEREPORTREQ.indication	(
Peer MAC Address,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which an ESS Information Request frame was received. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token in the Site Report Request frame that was received. |

#### 10.3.15.3.3 When Generated

This primitive is generated by the MLME when a valid Site Report Request frame is received.

#### 10.3.15.3.4 Effect of Receipt

No effect is specified.

### 10.3.16 Site Report Response

Insert the following new subsectionsfollowing 10.3.14

This set of primitives supports the signaling of Site Report Responses.

#### 10.3.16.1 MLME-SITEREPORTRESP.request

#### 10.3.16.1.1 Function

This primitive requests that a Site Report Response frame be sent to a peer MAC entity.

#### 10.3.16.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SITEREPORTRESP.request	(
Peer MAC Address,
Dialog Token,
BSS Information Set
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Site Report Request frame shall be set. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the site report transaction. This parameter is set to 0 for an autonomous report |
| BSS Information Set | Set of BSS Information records each as defined in the Site Report element format | Set of BSS Information records each as defined in the Site Report element format | A set of BSS Information records each containing a description of a BSS as defined in the Site Report element format |

#### 10.3.16.1.3 When Generated

This primitive is generated by the SME to request that a frame be sent to a peer entity that contains information on zero or more BSSs.

#### 10.3.16.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Site Report Response Action management frame. This frame is then transmitted.

#### 10.3.16.2 MLME-SITEREPORTRESP.confirm

#### 10.3.16.2.1 Function

This primitive reports the result of a request to send a Site Report Response frame.

#### 10.3.16.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SITEREPORTRESP.confirm	(
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, UNSPECIFIED FAILURE | Reports the outcome of a request to send a Site Report Request frame. |

#### 10.3.16.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Site Report Response frame completes.

#### 10.3.16.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.16.1 MLME-SITEREPORTRESP.indication

#### 10.3.15.1.1 Function

This primitive requests that a Site Report Response frame has been received from a peer entity. This may be in response to an earlier request, or may be an autonomous report.

#### 10.3.15.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SITEREPORTRESP.indication	(
Peer MAC Address,
Dialog Token,
BSS Information Set
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Site Report Request frame shall be set. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the site report transaction. Set to 0 for an autonomous report. |
| BSS Information Set | Set of BSS Information records each as defined in the Site Report element format | Set of BSS Information records each as defined in the Site Report element format | A set of BSS Information records each containing a description of a BSS as defined in the Site Report element format |

#### 10.3.15.1.3 When Generated

This primitive is generated by the MLME when a valid Site Report Response frame is received.

#### 10.3.15.1.4 Effect of Receipt

On receipt of this primitive, BSS information may be available to the station management entity (SME).