IEEE P802.11
Wireless LANs

Clarification for Beacon Reporting Conditions

Date:	May 2004

Author:	Emily Qi,

Intel Corporation

2111 N.E 25th Avenue

Hillsboro, OR 97124

e-mail: Emily.h.qi@intel.com

Joe Kwak

InterDigital Communications
	482 Degas
 	Bolingbrook, IL 60440
	Phone: 630-739-4327
	e-mail: joekwak@mindspring.com

Abstract

The beacon reporting condition is used for specifying when a beacon report is issued. It is not clear if reporting condition #1 to #10 can be used for periodic measurement or single measurement.  This document provides clear clarification for these reporting conditions

Table 0-3 – Reporting Condition definitions for Beacon Request element

Replace Table 0-3 with the following table:

| Condition Description | Reporting Condition |
| --- | --- |
| Report to be issued after each measurement. | 0 |
| For periodic measurement, report to be issued when the RCPI level of the measured STA crosses above an absolute threshold with hysteresis. For single measurement, report to be issued when the RCPI level of the measured STA is above an absolute threshold with hysteresis. | 1 |
| For periodic measurement, report to be issued when the RCPI level of the measured STA crosses below an absolute threshold with hysteresis. For single measurement, report to be issued when the RCPI level of the measured STA is below an absolute threshold with hysteresis. | 2 |
| For periodic measurement, report to be issued when the RSSI level of the measured STA crosses above an absolute threshold with hysteresis For single measurement, report to be issued when the RSSI level of the measured STA is above an absolute threshold with hysteresis for | 3 |
| For periodic measurement, report to be issued when the RSSI level of the measured STA crosses below an absolute threshold with hysteresis. For single measurement, report to be issued when the RSSI level of the measured STA is below an absolute threshold with hysteresis | 4 |
| For periodic measurement, report to be issued when the RCPI level of the measured AP crosses above a threshold defined by an offset (with hysteresis) from the serving STA’s RCPI. For single measurement, report to be issued when the RCPI level of the measured AP is above a threshold defined by an offset (with hysteresis) from the serving STA’s RCPI | 5 |
| For periodic measurement, report to be issued when the RCPI level of the measured AP crosses below a threshold defined by an offset (with hysteresis) from the serving STA’s RCPI. For single measurement, report to be issued when the RCPI level of the measured AP is below a threshold defined by an offset (with hysteresis) from the serving STA’s RCPI. | 6 |
| For periodic measurement, report to be issued when the RSSI level of the measured AP crosses above a threshold defined by an offset (with hysteresis) from the serving STA’s RSSI. For single measurement, report to be issued when the RSSI level of the measured AP is above a threshold defined by an offset (with hysteresis) from the serving STA’s RSSI. | 7 |
| For periodic measurement, report to be issued when the RSSI level of the measured AP crosses below a threshold defined by an offset (with hysteresis) from the serving STA’s RSSI. For single measurement, report to be issued when the RSSI level of the measured AP is below a threshold defined by an offset (with hysteresis) from the serving STA’s RSSI. | 8 |
| For periodic measurement, periodic reports (one per measurement) to begin when the RCPI level of the measured STA enters and remains in a range bound by the serving AP’s RCPI and an offset (with hysteresis) from the serving AP’s RCPI. For single measurement, report to be issued when the RCPI level of the measured STA is within a range bound by the serving AP’s RCPI and an offset (with hysteresis) from the serving AP’s RCPI. | 9 |
| For periodic measurement, periodic reports (one per measurement) to begin when the RSSI level of the measured STA enters and remains in a range bound by the serving AP’s RSSI and an offset (with hysteresis) from the serving AP’s RSSI. For single measurement, report to be issued when the RSSI level of the measured STA is within a range bound by the serving AP’s RSSI and an offset (with hysteresis) from the serving AP’s RSSI. | 10 |
| Reserved | 11-255 |