IEEE P802.11
Wireless LANs

Tutorial – Using OUI’s to 
Identify Cipher and AKM Suites

Date:	May 11, 2004

Authors:	Dave Halasz
	Cisco Systems

Frank Ciotti
 	Apacheta Corp.

Abstract

Tutorial on how the OUI field is used as part of the Suite Selector to identify ‘Cipher Suites’ and ‘Authentication and Key Management Suites’ for use within 802.11i.

# INFORMATIONAL OVERVIEW:

The IEEE 802.11i (Enhanced Security Mechanisms) has specified the use of Organizationally Unique Identifiers (OUI) to as one of the fields used to select the Cipher Suite as well as the Authentication and Key Management (AKM) Suite for use within the 802.11i protocol.  The Cipher Suite Selector is specified in clause 7.3.2.25.1 and the AKM Suite Selector is specified in clause 7.3.2.25.2.  The text which describes the Suite Selectors is included in Section 2 below.

As part of this definition, the IEEE requires the creation of tutorials that indicate how these Suite Selectors use the IEEE OUI.  The tutorial for 802.11i use of the IEEE registered OUI is included for your reference in this document.

# IEEE 802.11i Suite Selectors

IEEE 802.11i Suite Selectors allow for the negotiation of the Cipher Suite and Authentication and Key Management Protocol (AKMP).  The selected AKMP defines the Authentication Type and Key Management Type.  The selected Cipher Suite defines the data confidentiality protocol.

The IEEE 802.11i Suite Selector has the following format:

| OUI – 3 Octets | Suite Type – 1 Octet |
| --- | --- |

The IEEE has assigned the OUI value 00-0F-AC to IEEE 802.11.  IEEE 802.11i uses this OUI to identify Cipher Suites and AKM Suites specific to IEEE 802.11i as shown in Table 1 and Table 2.

Vendor specific Cipher Suites and AKM Suites may be specified and negotiated by using the OUI assigned to that vendor (see Table 1 and Table 2).

OUI values that are neither the IEEE 802.11 OUI value (00-0F-AC) nor vendor specific are reserved.

Table 1 - Cipher Suite Selectors

| OUI | Suite Type | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Use Group cipher suite |
| 00-0F-AC | 1 | WEP-40 |
| 00-0F-AC | 2 | TKIP |
| 00-0F-AC | 3 | Reserved |
| 00-0F-AC | 4 | CCMP – default in an RSNA |
| 00-0F-AC | 5 | WEP-104 |
| 00-0F-AC | 6-255 | Reserved |
| Vendor OUI | Other | Vendor Specific |
| Other | Any | Reserved |

Table 2 - Authentication and Key Management Suite Selector

| OUI | Suite Type | Meaning |  |
| --- | --- | --- | --- |
|  |  | Authentication Type | Key Management Type |
| 00-0F-AC | 0 | Reserved | Reserved |
| 00-0F-AC | 1 | Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in Clause 8.4.6.2 – RSNA default | RSNA Key Management as defined in Clause 8.5 or using PMKSA caching as defined in Clause 8.4.6.2 – RSNA default |
| 00-0F-AC | 2 | Pre-shared Key | RSNA Key Management as defined in Clause 8.5, using Pre-Shared Key |
| 00-0F-AC | 3-255 | Reserved | Reserved |
| Vendor OUI | Any | Vendor Specific | Vendor Specific |
| Other | Any | Reserved | Reserved |