IEEE P802.11
Wireless LANs

Proposed Text for adding BSS Load

Date:	13 May 2004

Author: 	Joe Kwak
	InterDigital Communications
	482 Degas
 	Bolingbrook, IL 60440
	Phone: 630-739-4327
	e-mail: joekwak@mindspring.com

Abstract

This document contains instructions to the editor to add normative text to incorporate a BSS Load into the TGk draft.  BSS Load is similar to the TGe specification of QBSS Load.  In addition the contribution improves the definition of AP Service Load as described in 11-04/550r0.

.

#### 7.2.3.1 Beacon frame format

Insert the order 20 field in Table 5 as follows:

Table 5 – Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 20 | BSS Load | The BSS Load element is only present within Beacon frames generated by APs that have dot11RadioMeasurementEnabled true. |

### 7.2.3.9 Probe Response frame format

Insert following element into Table 12 and modify the requested information elements field accordingly:

Table 12 – Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | BSS Load | The BSS Load element is only present within Beacon frames generated by APs that have dot11RadioMeasurementEnabled true. |

### 7.3.2 Information Elements

Insert Element ID 42 into Table 3-1 and change the Reserved row accordingly:

Table 20 – Element IDs

| Information Element | Element ID |
| --- | --- |
| BSS Load | 46 |

Insert the following new subsection under Section 7.3.2, adjusting the subsection numbers, figure numbers and table numbers as necessary:

7.3.2.28 BSS Load element

The BSS Load element contains information on the current station population ,traffic level, and service level in the BSS.  The element information field is defined in Figure X-1.

| Octets: 1 | 1 | 2 | 1 | 1 |
| --- | --- | --- | --- | --- |
| Element ID (46) | Length | Station Count (optional field) | Channel Utilization (optional field) | AP Service Load |

Figure X-1 – BSS Load element format

The Length field  shall be set to the number of octets in the following fields.

The Station Count field is interpreted as an unsigned integer that indicates the total number of STAs currently associated with this BSS. The Station Count field shall not be present if dot11QoSOptionImplemented and dot11QBSSLoadImplemented exist for this AP and are both true.

The Channel Utilization field is defined as the percentage of time the QAP sensed the medium busy, as indicated by either the physical or virtual  carrier sense mechanism. This percentage is represented as a moving average of ((channel busy time/(dot11ChannelUtilizationBeaconIntervals * dot11BeaconPeriod * 1024)) *255), where ‘channel busy time’ is defined to be the number of microseconds during which the carrier sense mechanism, as defined in 9.2.1, has indicated a channel busy indication, and dot11ChannelUtilizationBeaconIntervals represents the number of consecutive beacon intervals during which the average should be calculated.  The default value of dot11ChannelUtilizationBeaconIntervals is defined in Annex D. The Channel Utilization field shall not be present if dot11QoSOptionImplemented and dot11QBSSLoadImplemented exist for this AP and are both true.

The AP Service Load shall be a scalar indication of the relative level of service loading at an AP. A low value shall indicate more available service capacity than a higher value.  The value 0 shall indicate that this AP is not currently serving any STA. The value 255 shall indicate no additional AP service capacity is available.  The values between 0 and 255 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets measured from the time the packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. Value 1 represents 50 usec, value 254 represents 5.5 msec. The AP shall measure and average the medium access delay for all transmit packets using DCF access over a thirty second period.  The accuracy for the average medium access delay shall be +/- 200 usec or better when averaged over at least 200 packets.

10.3.2.2.2 Semantics of the service primitive

Insert the following entry into the table consisting BSSDescription in the subclause as shown:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSS Load | As defined in Frame Format | As defined in Frame Format | The values from the BSS Load information element if such an element was present in the probe response or beacon, else null. |

### ANNEX D

Modify TGk draftAnnex D, ASN.1 encoding of the MAC and PHY MIB, specification text as follows:

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex                    Unsigned32,

dot11STAStatisticsReportToken                    OCTET STRING,

dot11STAStatisticsIfIndex                        InterfaceIndex,

dot11STAStatisticsSTAAddress                     MacAddress,

dot11STAStatisticsMeasurementDuration            Unsigned32,

dot11STAStatisticsTransmittedFragmentCount       Integer32,

dot11STAStatisticsMulticastTransmittedFrameCount Integer32,

dot11STAStatisticsFailedCount                    Integer32,

dot11STAStatisticsRetryCount                     Integer32,

dot11STAStatisticsMultipleRetryCount             Integer32,

dot11STAStatisticsFrameDuplicateCount            Integer32,

dot11STAStatisticsRTSSuccessCount                Integer32,

dot11STAStatisticsRTSFailureCount                Integer32,

dot11STAStatisticsACKFailureCount                Integer32,

dot11STAStatisticsReceivedFragmentCount          Integer32,

dot11STAStatisticsMulticastReceivedFrameCount    Integer32,

dot11STAStatisticsFCSCount                       Integer32,

dot11STAStatisticsTransmittedFrameCount          Integer32,

dot11STAAPServiceLoad                            Integer

}

Insert at the end of the dot11STAStatisticsReport  Table:

dot11STAAPServiceLoad OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The AP Service Load shall be a scalar indication of the relative level of service loading at an AP. A low value shall indicate more available service capacity than a higher value.  The value 0 shall indicate that this AP is not currently serving any STA. The value 255 shall indicate no additional AP service capacity is available or that this STA is not an AP. The values between 0 and 255 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets measured from the time the packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. Value 1 represents 50 usec, value 254 represents 5.5msec. The AP shall measure and average the medium access delay for all transmit packets using DCF access over a thirty second period.  The accuracy for the average medium access delay shall be +/- 200 usec or better when averaged over at least 200 packets."

::= { dot11STAStatisticsReportEntry 19 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport	TABLE

-- ********************************************************************