IEEE P802.11
Wireless LANs

Proposed Text for adding BSS Load

Date:	15 September 2004

Author: 	Joe Kwak
	InterDigital Communications
	482 Degas
 	Bolingbrook, IL 60440
	Phone: 630-739-4327
	e-mail: joekwak@mindspring.com

Abstract

This document contains editorial instructions to add normative text to the latest TGk draft for incorporating BSS Load.  BSS Load is similar to the TGe specification of QBSS Load with the addition of an improved AP Service Load as described in 11-04/550r1. BSS Load is included as a new statistics group in Statistics Request/Report.   Comments were received during the Portland meeting noting that the AP Service Load parameter did not address the needs of EDCF traffic.  In this version, a new Access Category Service Load is added for EDCF streams.  This contribution addresses LB comment #421 and others.

.

#### 7.2.3.1 Beacon frame format

Insert the order 20 field in Table 5 as follows:

Table 5 – Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 20 | BSS Load | The BSS Load information element shall only be present within Beacon frames generated by APs that have dot11RadioMeasurementEnabled true. |

### 7.2.3.9 Probe Response frame format

Insert following element into Table 12 and modify the requested information elements field accordingly:

Table 12 – Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | BSS Load | The BSS Load information element shall only be present within Probe Response frames generated by APs that have dot11RadioMeasurementEnabled true. |

### 7.3.2 Information Elements

Insert Element ID 42 into Table 3-1 and change the Reserved row accordingly:

Table 20 – Element IDs

| Information Element | Element ID |
| --- | --- |
| BSS Load | 46 |

Modify the following subsection of existing TGk  draft as indicated below:

##### 7.3.2.21.10 STA Statistics Request

The STA Statistics Request Measurement enables a STA to query statistics and counters in a peer STA MIB. The Measurement Request field corresponding to a STA Statistics Request is shown in Figure +1.

|  | Randomization Interval | Measurement Duration | Group Identity |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure +1 – Measurement Request field format for a STA Statistics Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.7.3.

The Measurement Duration field shall be set equal to the duration of the requested measurement in TUs A Measurement Duration of 0 shall be used to request instantaneous values of the requested group statistics.  A non-zero value of Measurement Duration indicates a request for a change in value (increases or decreases) in the statistics of the specified group measured over the specified Measurement Duration.

Group Identity indicates the requested statistics group according to Table +2.

Table +2 – Group Identity for a STA Statistics Request

| Statistics Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| BSS Load as described in 7.3.2.28 | 1 |
| Reserved | 2 - 255 |

Insert the following new subsection under Section 7.3.2, adjusting the subsection numbers, figure numbers and table numbers as necessary:

7.3.2.28 BSS Load element

The BSS Load element contains information on the current station population , traffic level, and service level in the BSS.  The element information field is defined in Figure X-1.

| Octets: 1 | 1 | 2 | 1 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
| Element ID (46) | Length | Station Count (optional field) | Channel Utilization (optional field) | AP Service Load | Access Category Service Load (0 to 4 optional fields) |

Figure X-1 – BSS Load element format

The Length field shall be set to the number of octets in the following fields.

The Station Count field is interpreted as an unsigned integer that indicates the total number of STAs currently associated with this BSS. The Station Count field shall not be present in beacon or probe response frames if dot11QoSOptionImplemented, dot11QBSSLoadImplemented, and dot11RadioMeasurementEnabled are all true.

The Channel Utilization field is defined as the percentage of time the AP sensed the medium busy, as indicated by either the physical or virtual carrier sense mechanism. This percentage is represented as a moving average of ((channel busy time/(dot11ChannelUtilizationBeaconIntervals * dot11BeaconPeriod * 1024)) *255), where channel busy time is defined to be the number of microseconds during which the carrier sense mechanism, as defined in 9.2.1, has indicated a channel busy indication, and dot11ChannelUtilizationBeaconIntervals represents the number of consecutive beacon intervals during which the average should be calculated.  The default value of dot11ChannelUtilizationBeaconIntervals is defined in Annex D. The Channel Utilization field shall not be present in beacon or probe response frames if dot11QoSOptionImplemented, dot11QBSSLoadImplemented, and dot11RadioMeasurementEnabled are all true.

The AP Service Load shall be a scalar indication of the relative level of service loading at an AP. A low value shall indicate more available service capacity than a higher value.  The value 0 shall indicate that this AP is not currently serving any STA. The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for DCF transmitted packets measured from the time the DCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate no additional AP service capacity is available.  The value 255 shall indicate that the AP Service Load is not available.  The AP shall measure and average the medium access delay for all transmit packets using DCF access mechanism over a continuos thirty second measurement window.  The accuracy for the average medium access delay shall be +/- 200 usec or better when averaged over at least 200 packets.

The Access Category (AC) Service Load elements may be provided in the BSS Load only at QAPs. The AC Service Load shall be a scalar indication of the Average Access Delay (AAD) at a QAP for services of the indicated Access Category. A low value shall indicate shorter access delay than a higher value.  The value 0 shall indicate that this QAP is not currently providing services of the indicated  AC. The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets in the indicated AC measured from the time the EDCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate that services at the indicated AC are currently blocked or suspended.  The value 255 shall indicate that the AC Service Load is not available.  The QAP shall measure and average the medium access delay for all transmit packets of the indicated AC using EDCF access mechanism over a continuos thirty second measurement window.  The accuracy for the average medium access delay shall be +/- 200 usec or better when averaged over at least 200 packets.  The AC Service load is formatted as shown in Figure kx-2, below, as two octet sub element with the first octet containing the AC Indication (ACI) and the second octet containing the measured value of the AAD for the indicated AC as defined earlier in this paragraph.

|  | Access Category Indicator (ACI) |  | Average Access Delay (AAD) |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 |  |  |

Figure kX-2 – Access Category Service Load element format

Table kx2 – ACI encoding

| Access Category (AC) | ACI |
| --- | --- |
| Best Effort | 0 |
| Background | 1 |
| Video | 2 |
| Voice | 3 |
| Reserved | 4-255 |

10.3.2.2.2 Semantics of the service primitive

Insert the following entry into the table consisting BSSDescription in the subclause as shown:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| BSS Load | As defined in Frame Format | As defined in Frame Format | The values from the BSS Load information element if such an element was present in the probe response or beacon, else null. |

# Annex A - PICS

Modify the following subsection of existing TGk  draft Annex A as indicated below:

| RRM16 RRM16.1 RRM16.2 | Management Information Base Dot11peerStatsTable BSS Load | Annex D Annex D | CF1:M CF1:M | Yes, No Yes, No |
| --- | --- | --- | --- | --- |
| RRM17 | Country, Power Constraint and Transmit Power Control (TPC) Report elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.9, 7.3.2.13, 7.2.3.16 | M | Yes, No |
| RRM18 | Power Capability elements in Association and Reassociation frames | 7.2.3.4, 7.2.3.5, 11.6.1 | M | Yes, No |
| RRM19 | BSS Load elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.28, | M | Yes, No |
| RRM23 RRM23.1 RRM23.2 | Adaptation of transmit power TPC Report in Beacon and Probe Response frames Dynamic transmit power adaptation | 11.5.4 11.5.4 | CF1:M CF2:O | Yes, No Yes, No |
| RRM17 | Country, Power Constraint and Transmit Power Control (TPC) Report elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.9, 7.3.2.13, 7.2.3.16 | M | Yes, No |

### ANNEX D

Modify dot11STAStatRqstGroupID in TGk draftAnnex D as follows:

dot11STAStatRqstGroupID OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute corresponds to the group identity if dot11RRMRqstType = 9: STA statistics Request. This attribute shall be ignored otherwise.

0 = STA Counters from dot11CountersTable

1 = BSS Load as defined in 7.3.2.28

The rest are reserved."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 30 }

Modify Dot11STAStatisticsReportEntry in TGk draftAnnex D as follows:

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex                    Unsigned32,

dot11STAStatisticsReportToken                    OCTET STRING,

dot11STAStatisticsIfIndex                        InterfaceIndex,

dot11STAStatisticsSTAAddress                     MacAddress,

dot11STAStatisticsMeasurementDuration            Unsigned32,

dot11STAStatisticsTransmittedFragmentCount       Integer32,

dot11STAStatisticsMulticastTransmittedFrameCount Integer32,

dot11STAStatisticsFailedCount                    Integer32,

dot11STAStatisticsRetryCount                     Integer32,

dot11STAStatisticsMultipleRetryCount             Integer32,

dot11STAStatisticsFrameDuplicateCount            Integer32,

dot11STAStatisticsRTSSuccessCount                Integer32,

dot11STAStatisticsRTSFailureCount                Integer32,

dot11STAStatisticsACKFailureCount                Integer32,

dot11STAStatisticsReceivedFragmentCount          Integer32,

dot11STAStatisticsMulticastReceivedFrameCount    Integer32,

dot11STAStatisticsFCSCount                       Integer32,

dot11STAStatisticsTransmittedFrameCount          Integer32,

dot11STAStatisticsStationCount                   Integer,

dot11STAStatisticsChannelUtilization             Integer,

dot11STAStatisticsAPServiceLoad                  Integer,

dot11STAStatisticsACServiceLoad                  Integer,

}

Insert at the end of the dot11STAStatisticsReport  Table:

dot11STAStatisticsStationCount OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The Station Count field is an unsigned integer that indicates the total number of STAs currently associated with this BSS."

::= { dot11STAStatisticsReportEntry 19 }

dot11STAStatisticsChannelUtilization OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The Channel Utilization field is the percentage of time the AP sensed the medium busy, as indicated by either the physical or virtual  carrier sense mechanism. This percentage is represented as a moving average of ((channel busy time/(dot11ChannelUtilizationBeaconIntervals * dot11BeaconPeriod * 1024)) *255), where ‘channel busy time’ is defined to be the number of microseconds during which the carrier sense mechanism, as defined in 9.2.1, has indicated a channel busy indication, and dot11ChannelUtilizationBeaconIntervals represents the number of consecutive beacon intervals during which the average should be calculated."

::= { dot11STAStatisticsReportEntry 20 }

dot11STAStatisticsAPServiceLoad OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The AP Service Load shall be a scalar indication of the relative level of service loading at an AP. A low value shall indicate more available service capacity than a higher value.  The value 0 shall indicate that this AP is not currently serving any STA. The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets measured from the time the packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate no additional AP service capacity is available.  The value 255 shall indicate that the AP Service Load is not available."

::= { dot11STAStatisticsReportEntry 21 }

dot11STAStatisticsACServiceLoad OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

The Access Category (AC) Service Load elements may be provided in the BSS Load only at QAPs. The AC Service Load shall consist of an Access Caategory Indicator (ACI) and an Average Access Delay (AAD). The ACI indicates the Access Category for this service load element,  The AAD shall be a scalar indication of the Average Access Delay at a QAP for services of the indicated Access Category. A low value shall indicate shorter access delay than a higher value.  The value 0 shall indicate that this QAP is not currently providing services of the indicated  AC. The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets in the indicated AC measured from the time the EDCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate that services at the indicated AC are currently blocked or suspended.  The value 255 shall indicate that the AC Service Load is not available. The AC Service load is formatted as a two octet element with the first octet containing the AC Indication (ACI) and the second octet containing the measured value of the AAD for the indicated AC as defined earlier in this paragraph."

::= { dot11STAStatisticsReportEntry 22 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport	TABLE

-- ********************************************************************