IEEE P802.11
Wireless LANs

Japanese Radio Regulations Update

Date:	July 8th, 2004

Author:	Yasuhiko Inoue
	NTT
	1-1 Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847 Japan
	Phone: ++81-46-859-5097
	Fax: ++81-46-855-1752
	e-Mail: yinoue@ansl.ntt.co.jp

Abstract

This document presents the information on Japanese radio regulations related to the TGj activities. Updated tables for the Annex J of the IEEE 802.11j draft (D1.5) are provided. Requirements for 10MHz channel width systems are also contained in this document.

## J.2 Country information element and Regulatory Classes

### J.2.1 Regulatory class specifications

Table J.2.1—Emissions limits sets

| Emissions limits set | United States | CEPT | Japan |
| --- | --- | --- | --- |
| 1 | 47 CFR 15.407 | ETSI EN 301 389-1 | Nomadic use (MPHPT 49.20, 21) |
| 2 | Reserved | Reserved | Interference-limited areasa (MPHPT 49.21) |
| 3 | Reserved | Reserved | Other interference areasa (MPHPT 49.21) |
| 4-255 | Reserved | Reserved | Reserved |

aNOTE—The deployment in Japan of licensed radios in dense urban areas (Interference Limited Areas) and other areas (Other Interference Areas) are specified to allow more radios to operate in dense urban areas, but permitting what can be less expensive radio designs to be used elsewhere.

Behavior Limits Set is an enumerated list of behaviors that are legal for a regulatory domain, and are specified in Table J.2.2.

Table J.2.2—Behavior limits sets

| Behavior limits set | United States | CEPT | Japan |
| --- | --- | --- | --- |
| 1 nomadic use | 47 CFR 15.407 | ETSI EN 301 389-1 | MPHPT 49.20, 49.21 |
| 2 indoor only use | 47 CFR 15.407(e) | ETSI EN 301 389-1 | MPHPT 49.20 |
| 3 transmit power control | Reserved | ETSI EN 301 389-1 | Reserved |
| 4 dynamic frequency selection | Reserved | ETSI EN 301 389-1 | Reserved |
| 5 IBSS prohibited | Reserved | Reserved | MPHPT 49.21 |
| 6 4 ms carrier sensea | Reserved | Reserved | MPHPT 49.20, 49.21 |
| 7 licensed base station | Reserved | Reserved | MPHPT 49.21 |
| 8 mobile station | Reserved | Reserved | MPHPT 49.20, 49.21 |
| 9-255 | Reserved | Reserved | Reserved |

a NOTE—The Japan 4 ms carrier sense rule says no station can transmit for more than 4 ms without carrier sensing, whether transmitting fragments or frames, unless it is controlled by another station.

The regulatory classes specified for 4.9 GHz and 5 GHz operation in Japan are enumerated in Table J.2.5.

Table J.2.5—Regulatory classes for Japan 4.9 GHz and 5 GHz bands

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (dBm) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 20 | 34, 38, 42, 46 | 22 | 1 | 1, 2, 6 |
| 2 | 5 | 20 | 8, 12, 16 | 24 | 2 | 7, 5, 6 |
| 3 | 5 | 20 | 8, 12, 16 | 24 | 2 | 8, 5, 6 |
| 4 | 5 | 20 | 8, 12, 16 | 24 | 3 | 7, 5, 6 |
| 5 | 5 | 20 | 8, 12, 16 | 24 | 3 | 8, 5, 6 |
| 6 | 5 | 20 | 8, 12, 16 | 22 | 1 | 8, 5, 6 |
| 7 | 4 | 20 | 184, 188, 192, 196 | 24 | 2 | 7, 6, 5 |
| 8 | 4 | 20 | 184, 188, 192, 196 | 24 | 2 | 8, 6, 5 |
| 9 | 4 | 20 | 184, 188, 192, 196 | 24 | 3 | 7, 6, 5 |
| 10 | 4 | 20 | 184, 188, 192, 196 | 24 | 3 | 8, 6, 5 |
| 11 | 4 | 20 | 184, 188, 192, 196 | 22 | 1 | 8, 6, 5 |
| 12 | 5 | 10 | 7, 8, 9, 11 | 24 | 2 | 7, 5, 6 |
| 13 | 5 | 10 | 7, 8, 9, 11 | 24 | 2 | 8, 5, 6 |
| 14 | 5 | 10 | 7, 8, 9, 11 | 24 | 3 | 7, 5, 6 |
| 15 | 5 | 10 | 7, 8, 9, 11 | 24 | 3 | 8, 5, 6 |
| 16 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 2 | 7, 6, 5 |
| 17 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 2 | 8, 6, 5 |
| 18 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 3 | 7, 6, 5 |
| 19 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 3 | 8, 6, 5 |
| 20 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 17 | 1 | 8, 6, 5 |
| 21-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

## J.3 Radio performance specifications

(normative)

### J.3.1 Transmit and receive in-band and out-of-band spurious emissions

Spurious transmissions from compliant devices shall conform to national regulations. For operation in the United States, refer to 47 CFR 15.407. For operation in Europe, refer to ETSI EN 301 389-1. For operation in Japan, refer to Japan MPHPT EO 49.20 and 49.21 Section 1.

NOTE—All out-of-band and in-band spurious emission requirements of MPHPT EO 49.20 and 49.21 Section 1 are included in tables J.3.1.1 and J.3.1.2. Figure J.3.1 is an informative summary of the requirements for operation in the Japan 4.9 GHz and 5.0 GHz bands.

Table J.3.1.1—Japan out-of-band emission limits

| Regulatory domain | Band (GHz) | Out-of-band leakage power | Out-of-band frequencies |
| --- | --- | --- | --- |
| For 20MHz channel space systems |  |  |  |
| Japan E.O. Article 49.21 | 4.9 | < 15µW/MHz | 4 880 ≤ f < 4 900MHz, 5 000 < f ≤ 5 020MHz |
| Japan E.O. Article 49.21 | 5.0 | < 15µW/MHz | 5 000 ≤ f < 5 020MHz, 5 100 < f ≤ 5 120MHz |
| Japan E.O. Article 49.21 | 5.0 | < 0.5mW/MHz | 5 020 ≤ f < 5 030MHz, 5 091 < f ≤ 5 100MHz |
| Japan E.O. Article 49.20 | 5.15-5.25 | ≤ 2.5µW/MHz | 5 130 ≤ f ≤ 5 142MHz |
| Japan E.O Article 49.20 | 5.15-5.25 | ≤ 15µW/MHz | 5 142 < f ≤ 5 150MHz, 5 250 ≤ f < 5 258MHz |
| Japan E.O Article 49.20 | 5.15-5.25 | ≤ 0.2µW/MHz | 5 258 ≤ f ≤ 5 270MHz |

| For 10MHz channel space systems |  |  |  |
| --- | --- | --- | --- |
| Japan E.O. Article 49.21 | 4.9 | < 15µW/MHz | 4 895 ≤ f < 4 905MHz, 4955 < f ≤ 4965MHz |
| Japan E.O Article 49.21 | 5.0 | < 15µW/MHz | 5 015 ≤ f < 5 025MHz, 5 065 < f ≤ 5 075MHz |
| Japan E.O. Article 49.21 | 5.0 | < 0.5mW/MHz | 5 025 ≤ f < 5 030MHz, |

Table J.3.1.2—Japan upper limit of spurious emissions

| Regulatory domain | Band (GHz) | Spurious emissions power | Frequencies |
| --- | --- | --- | --- |
| For 20MHz channel space systems |  |  |  |
| Japan E.O Article 7 | 4.9 | ≤ 2µW/MHz | < 4 870MHz |
| Japan E.O Article 7 | 4.9 | ≤ 2.5µW/MHz | 4 870 ≤ f < 4 880MHz, 5 020 < f ≤ 5 270MHz |
| Japan E.O Article 7 | 4.9 | ≤ 0.2µW/MHz | 5 270 < f ≤ 5 342MHz |
| Japan E.O Article 7 | 4.9 | ≤ 1µW/MHz | > 5 342MHz |
| Japan E.O Article 7 | 5.0 | ≤ 2µW/MHz | < 4 990MHz |
| Japan E.O Article 7 | 5.0 | ≤ 2.5µW/MHz | 4 990 ≤ f < 5 000MHz, 5 120 < f ≤ 5 270MHz |
| Japan E.O Article 7 | 5.0 | ≤ 0.2µW/MHz | 5 270 < f ≤ 5 342MHz |
| Japan E.O Article 7 | 5.0 | ≤ 1µW/MHz | > 5 342MHz |
| Japan E.O Article 7 | 5.15-5.25 | ≤ 2.5µW/MHz | f < 5 130MHz, and 5 342MHz < f |
| Japan E.O Article 7 | 5.15-5.25 | < 0.2µW/MHz | 5 270 < f ≤ 5 342MHz |

| For 10 MHz channel space systems |  |  |  |
| --- | --- | --- | --- |
| Japan E.O Article 7 | 4.9 | ≤ 2µW/MHz | < 4 870MHz |
| Japan E.O Article 7 | 4.9 | ≤ 2.5µW/MHz | 4 870 ≤ f < 4 895MHz, 4965 < f ≤ 5 270MHz |
| Japan E.O Article 7 | 4.9 | ≤ 0.2µW/MHz | 5 270 < f ≤ 5 342MHz |
| Japan E.O Article 7 | 4.9 | ≤ 1µW/MHz | > 5 342MHz |
| Japan E.O Article 7 | 5.0 | ≤ 2µW/MHz | < 4 990MHz |
| Japan E.O Article 7 | 5.0 | ≤ 2.5µW/MHz | 4 990 ≤ f < 5 015MHz, 5 075 < f ≤ 5 270MHz |
| Japan E.O Article 7 | 5.0 | ≤ 0.2µW/MHz | 5 270 < f ≤ 5 342MHz |
| Japan E.O Article 7 | 5.0 | ≤ 1µW/MHz | > 5 342MHz |

E.O.: Ordinance Regulating Radio Equipment (as of March 2004)