IEEE P802.11
Wireless LANs

Radio Measurement Action Protection Normative Text

Date:	July 2004

Author:	Jesse Walker and Emily Qi

Intel Corportation

2111 N.E 25th Avenue

Hillsboro, OR 97124

e-Mail: {Jesse.walker; Emily.h.qi } @intel.com

Abstract

This document proposes normative texts for submission 11-04-0685-00-000k-radio-measurement-action-protection.ppt. Action frame can be Protection-capable frame or Non-protection-capable frames. Radio Measurement Actions are Protection-Capable. Protection-Capable Action frames are protected in a similar way as an ordinary Data MPDU. An RSN (IEEE STD 802.11i-2004) information element Capabilities subfield bit is used to signal whether Protection-capable Action Frames will be protected. Sender’s Pairwise Temporal Key protects unicast Protection-Capable Action Frames, and Sender’s GTK is used to protect broadcast/multicast Protection-Capable Action Frame.

Modifications to IEEE 802.11k draft 0.16 and to IEEE STD 802.11i-2004 are provided.

Acknowledgements

#### To Bernard Aboba, Jon Edney, and Mike Moreton for valuable discussion, input, and review

#### Proposing the following normative text to 802.11k draft 0.16:

Insert the following into clause 2:

IEEE STD 802.11i-2004, Standard for Local and Metropolitan Area Networks: Part 11: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications: Amendment i: Medium Access Control (MAC) Security Enhancements

Insert the following at the end of the subclause 7.3.1.11 of IEEE 802.11k draft 0.16:

All Action Frames are classified as Protection-Capable or Non-Protection-Capable.

Protection-Capable Action Frames shall be capable of being protected by either TKIP as defined in Clause 8.3.2 of IEEE STD 802.11i-2004, or by CCMP as defined in Clause 8.3.3 of IEEE STD 802.11i-2004.

Non-Protection-Capable Action Frames are always transmitted and received without any protection.

By default, all Action Frames are Non-Protection-Capable. An Action Frame is Protection-Capable only if explicitly defined as such.

Add the following texts at the end of subsection 5.7.9:

Radio Measurement Action frames are Protection-Capable Action Frames. If local policy requires protection, all Radio Measurement Action Frames shall be protected, and any unprotected received Radio Measurement Action Frames shall be silently discarded.

Replace the body of Clause 7.1.3.1.9 of IEEE STD 802.11i-2004 with:

The Protected Frame field is one bit in length. The Protected Frame field is set to 1 if the Frame Body field contains information that has been processed by a cryptographic encapsulation algorithm. The Protected Frame field is set to 1 only within frames of type Data, and within frames of type Management, subtype authentication or subtype Action. The Protected Frame field is set to 0 in all other frames. When the Protected Frame field is set to 1, in a frame of type Data and type Management, subtype Action, the Frame Body field is protected utilizing the cryptographic encapsulation algorithm and expanded as defined in Clause 8. Only WEP is allowed as the cryptographic encapsulation algorithm for frames of type Management, subtype Authentication. WEP shall not be used with frames of type Management, subtype Action.

NOTE: WEP lacks the data authenticity and replay detection mechanisms needed to meet the requirements for protected Action Frames.

In Clause 7.3.2.25.3 of IEEE STD 802.11i-2004, replace Figure 68 “RSN Capabilities” with the following:

In Clause 7.3.2.25.3 of IEEE STD 802.11i-2004 add a new RSN Capabilities bit “Protected Action Frames” with the following description:

Bit 6 – Protected  Action  Frame. A STA sets the Protected Action Frame subfield (Bit 6) of the RSN information element Capabilities subfield to 1 to indicate that protection is required for all Protection-Capable Action Frames. A STA sets the bit to 0 to indicate it does not support Action Frame protection, or that protection is disabled. STAs set this bit to 1 in their Beacons and Probe Responses if the local security policy requires protection for Protected Action Frames, and sets it to 0 when protection is disabled or unsupported. (Re)-associating STAs set the bit to 1 in (Re-)Association Requests if source sets it if they support Protected Actions Frames and clear it otherwise.

Add the following to the end of Clause 8.3.2.1 of IEEE STD 802.11i-2004:

This same procedure may be applied to Protection-Capable Action Frames, with MPDU data field replacing MSDU data, and, by convention, the MSDU priority taken as the zero octet. TKIP shall be applied to Protection-Capable Action Frames when the negotiated ciphersuite is TKIP and Protected Management Frames are negotiated using the RSN Capabilities Information Field of the RSN information element; see Clause 8.4.3.

Add the following sentence to the end list item 1 of Clause 8.3.2.1.1 of IEEE STD 802.11i-2004:

Protection-Capable Action Frames may be protected by TKIP, with the MPDU data replacing MSDU data field in this description, and by taking the Priority as the octet with value zero. A MAC implementation shall support TKIP protection of Action Frames if it supports both TKIP and Protection-Capable Action Frames.

Add the following sentence to the end list item 5 of Clause 8.3.2.1.2 of IEEE STD 802.11i-2004:

Note that when the received frame is a TKIP protected Protection-Capable Action Frame, contents of the action frame after protection is removed shall be delivered to the SME rather than passing across the MAC SAP.

Add the following sentence at the end of Clause 8.3.2.2 of IEEE STD 802.11i-2004:

When Protected Action Frames are negotiated (See Clause 8.4.3) to be protected by TKIP, TKIP shall use the same Temporal Key for Protected Action Frames and for Data Frames.

Change item 7 of Clause 8.3.2.3.4 of IEEE STD 802.11i-2004 as follows:

For each PTKSA, GTKSA, and STAKeySA, the receiver shall maintain a separate replay counter for each frame priority, and, if Protected Action Frames have been negotiated, it shall maintain a separate counter for TKIP-protected frames of type Action. The receiver shall use the TSC recovered from a received frame to detect replayed frames, subject to the limitations on the number of replay counters supported, as advertised in the RSN information element Capabilities subfield, as described in Clause 8.4.3. A replayed frame occurs when the TSC extracted from a received frame is less than or equal to the current replay counter value for the frame’s priority or frame type. A transmitter shall not reorder frames with different priorities without ensuring that the receiver supports the required number of replay counters. The transmitter shall not reorder frames within a replay counter, but may reorder frames across replay counters. One possible reason for reordering frames is the IEEE 802.11 MSDU priority.

IEEE 802.11 does not define a method to signal frame priority.

Add the following paragraph to the end of Clause 8.3.3.1 of IEEE STD 802.11i-2004:

Protection-Capable Action Frames may be protected with CCMP by taking the Priority as the octet with value zero. A MAC implementation shall support CCMP for protecting Action Frames if CCMP and Protection-Capable Action Frames are both supported.

Change Clause 8.3.3.3.2 of IEEE STD 802.11i-2004 as follows:

The AAD is constructed from the MPDU Header. The AAD does not include the header Duration field, because the Duration field value can change due to normal IEEE 802.11 operation (e.g. a rate change during retransmission). For similar reasons, several sub-fields in the Frame Control field are masked to zero. AAD construction is performed as follows:

- FC – MPDU Frame Control field, with:

  - Subtype (b4 b5 b6) bits masked to zero in a Data MPDU;

  - Retry bit (b11) masked to zero;

  - PwrMgt bit (bit 12) masked to zero;

  - MoreData bit (bit 13) masked to zero in a Data MPDU; and

  - The Protected Frame bit (bit 14) shall always be set to one.

- A1 – MPDU Address 1.

- A2 – MPDU Address 2.

- A3 – MPDU Address 3.

- SC – MPDU Sequence Control field, with the sequence number field (bits 4-15 of the Sequence Control field) masked to zero. The Fragment Number bits are not modified.

- A4 – MPDU Address, if present in the MPDU.

- QC – The Quality of Service Control, if present, a two-octet field that includes the MSDU priority; this field is reserved for future use.

The format of the AAD is shown in Figure 85. The length of the AAD is 22 octets when there is no A4 field and no QC, and 28 octets long when the MPDU includes A4.

Add the following to the end of Clause 8.3.3.3.5 of IEEE STD 802.11i-2004:

Note that a Protected Action Frame uses the same TK as a Data MPDU.

Add the following to the end of Clause 8.3.3.4.1 of IEEE STD 802.11i-2004:

Note that a Protected Action Frame uses the same TK as a Data MPDU.

Change item 7 of Clause 8.3.3.4.3 of IEEE STD 802.11i-2004 as follows:

For each PTKSA, GTKSA, and STAKeySA, the recipient shall maintain a separate replay counter for each MSDU priority, and, if Protected Action Frames have been negotiated, it shall maintain a separate counter for CCMP-protected frames of Action. The receiver shall use the PN recovered from a received frame to detect replayed frames, subject to the limitations on the number of replay counters supported, as advertised in the RSN information element Capabilities subfield, as described in Clause 8.4.3. A replayed frame occurs when the PN extracted from a received frame is less than or equal to the current replay counter value for the frame’s priority or frame type. A transmitter shall not reorder frames with different priorities without ensuring that the receiver supports the required number of replay counters. The transmitter shall not reorder frames within a replay counter, but may reorder frames across replay counters. One possible reason for reordering frames is the IEEE 802.11 MSDU priority.

Add the following to the end of Clause 8.4.3 of IEEE STD 802.11i-2004:

Use of Protected Action Frames is negotiated using the Beacon/Probe Response and (Re-)Associate mechanism.

If the Beacon/Probe Response source sets the Protected Action Frames subfield of the RSN information element Capabilities Information Field to 0, then Actions Frames shall not be protected. In this case the receiver shall discard any protected Action Frames it receives.

If the Beacon/Probe Response source sets the Protected Action Frames subfield to 1, then the negotiated pariwise and multicast/broadcast ciphers suites shall be applied all Protection-Capable Actions Frames, and any received unprotected Protection-Capable Action Frames shall be discarded. In this case, the key protecting unicast Protection-Capable Actions Frames shall be the TK portion of the negotiated PTK, and for broadcast/multicast Protection Capable Action Frames, the assigned GTK. Implementations shall not allow the Protected Action Frame bit of RSN information element Capabilities subfield to be set to 1 when WEP is an advertised cipher suite.

When a STA receives a Beacon or Probe Response with the Protection Management Frame subfield set to 1, it shall respond by setting the Protection Management subfield to 1 in its RSN information element in any (Re-)Associate Request or 4-Way Handshake Message 2, it sends to establish contact with the Beacon/Probe Response source if it support Protected Management Frames, and shall otherwise set this subfield to 0.

In Annex A, add a new row to the PICS as follows:

| PCX 1.10 | Protection Action Frame | 7.3.1.11, 7.4.2, 7.1.3.1.9, 7.3.2.25.3, 8.3.2.1.1, 8.3.2.1.2, 8.3.2.2, 8.3.2.3.4, 8.3.3.3.2, 8.3.3.3.5, 8.3.3.4.1, 8.3.3.4.3, 8.4.3 | PCX.1:O | Yes No |
| --- | --- | --- | --- | --- |

Add the following to the end of the Dot11StationConfigEntry  appearing in Annex D (802.11i):

dot11RSNAProtectedMgmtFramesEnabled	TruthValue

Add the following to Annex D (802.11i):

dot11RSNAProtectedMgmtFramesEnabled	OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This variable indicates whether this STA protects Action

Frames."

::= { dot11StationConfigEntry 27 }