IEEE P802.11
Wireless LANs

Comment Resolution: Action Frame Class Scope

Date:	July 8th, 2004

Author:	Jon Edney
	InTalk2k
	e-Mail:  jon.edney@intalk2k.com

Abstract

Proposed normative text to resolve comment 53 arising from informal ballot on Draft ?

# Normative Text Proposal

The following normative text is propsed to be added to the current TGk draft:

In clause 5.5, replace  the following list:

a)	Class 1 frames (permitted from within States 1, 2, and 3):

2) Management frames

i)	Probe request/response

ii)	Beacon

iii) Authentication:	Successful authentication enables a STA to exchange Class 2 frames.

Unsuccessful authentication leaves the STA in State 1.

iv) Deauthentication:	Deauthentication notification when in State 2 or State 3 changes the STA’s state to State 1. The STA shall become authenticated again prior to sending Class 2 frames.

v)	Announcement traffic indication message (ATIM)

vi) Action

with:

a)	Class 1 frames (permitted from within States 1, 2, and 3):

2) Management frames

i)	Probe request/response

ii)	Beacon

iii) Authentication:	Successful authentication enables a STA to exchange Class 2 frames.

Unsuccessful authentication leaves the STA in State 1.

iv) Deauthentication:	Deauthentication notification when in State 2 or State 3 changes the STA’s state to State 1. The STA shall become authenticated again prior to sending Class 2 frames.

v)	Announcement traffic indication message (ATIM)

vi) Spectrum Management Action

vii)  Radio Resource Measurement Action other than those containing measurement request and report messages

viii) Radio Resource Measurement Action containing measurement request and report messages sent between two stations in an IBSS

In clause 5.5, replace  the following list:

c)	Class 3 frames (if and only if associated; allowed only from within State 3):

2)	Management frames

— Deauthentication: Deauthentication notification when in State 3 implies disassociation as well, changing the STA’s state from 3 to 1. The station shall become authenticated again prior to another association.

with:

c)	Class 3 frames (if and only if associated; allowed only from within State 3):

2)	Management frames

i) Deauthentication: Deauthentication notification when in State 3 implies disassociation as well, changing the STA’s state from 3 to 1. The station shall become authenticated again prior to another association.

ii) Radio Resource Action containing measurement request and report messages