IEEE P802.11
Wireless LANs

Proposed Text for Neighbor Report Enhancements

Date:	July 9, 2004

Authors:

Sudheer Matta		Dan Harkins					Martin Lefkowitz
Trapeze Networks		Trapeze Networks				
e-Mail: matta@trapezenetworks.com		e-Mail: mailto:dharkins@trpz.com			e-Mail: mailto:lefko@sbcglobal.net

Malik Audeh		Walter Johnson					Simon Black
Tropos		Motorola					Intalk2k
e-Mail:		e-Mail: mailto:walter.johnson@motorola.com	e-Mail: mailto:simon.black@intalk2k.com

John Klein		David Nelson					Mike Dove
Symbol Technologies		Enterasys					Broadcom
e-Mail: jklein@sj.symbol.com		e-Mail: mailto:dnelson@enterasys.com		e-Mail: mailto:mdove@broadcom.com

P. Loc		Dorothy Stanley					Kevin Hayes
Marvell		Agere						Atheros
e-Mail: mailto: ploc@marvell.com		e-Mail: mailto:dstanley@agere.com		e-Mail: kevin@atheros.com

Victoria Poncini		Bernard Aboba					Steve Pope
Microsoft		Microsoft					Texas Instruments
e-Mail: vponcini@microsoft.com		e-Mail: mailto:bernarda@microsoft.com		e-Mail: spp@ti.com

Bob O’Hara		Jim Wendt
Airespace		Hewlett Packard
e-Mail: bob@airespace.com		e-Mail: jim.wendt@hp.com

Abstract

This document contains instructions to the editor to add normative text to incorporate enhancements to the Neighbor  Report information element and related text.

Add the following new definition to clause 3.

#### 3. Definitions

Neighbor AP: A neighbor AP is defined as an infrastructure BSS where the BSA overlaps, or is adjacent to the BSA established by the AP sending the neighbor BSS report.

## Replace clause 7.3.2 with the following. Updated “Site Report” to “Neighbor Report”

#### 7.3.2 Information Elements

Insert Element ID 43-44 into Table 20 and change the Reserved row accordingly:

Table 20 – Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | TBD |
| Neighbor Report | TBD |
| RCPI | TBD |
| Reserved | 51TBD-255 |

Replace the Site Report element clause with the following renamed Neighbor Report element clause in the latest TGk draft. Section numbering is harmonized with 802.11h.

#### 7.3.2.26 Neighbor Report element

The purpose of the Neighbor Report is to enable the STA to optimize aspects of neighbor BSS transition, prior to scanning.  The Neighbor Report element contains information on APs which are roaming candidates for STAs.  A Neighbor Report element shall only contain entries of neighboring APs that are legitimate members of ESS(es) satisfying the query.   ESS(es) satisfying a Neighbor Report query shall be within the administrative domain of the AP handling the query.

Since the information in the Neighbor Report may be stale, it should be considered advisory;  information obtained through a scan or other sources shall be considered definitive, overriding information in the Neighbor Report.  For example, where information contained within a Neighbor Report is contradicted by information in the Beacon/Probe Response, the Beacon/Probe Response information shall be considered definitive; similarly, where information is available within a 4-way handshake, it will be considered definitive.

[Informative]: Determination of the neighboring APs can be accomplished be several means, including:

- Configuring an AP with a list of BSSIDs that are neighbors.

- Utilizing the Beacon Report in order to determine which APs can be heard by STAs in the service area.  To guard against pollution of the neighbor report by an erroneous (but not malevolent) STA, an AP may choose to only believe information corroborated by Beacon Reports from multiple STAs.  In addition, an AP should validate that BSSIDs returned in a Beacon Report represent legitimate members of an ESS within the administrative domain of the AP before including them in a neighbor report.

Determination of which neighboring APs are legitimate members of an ESS within the administrative domain of the AP can be accomplished by several means, including:

- Configuring APs with a list of BSSIDs that are members of an ESS, in order to allow screening out neighbor APs that are not legitimate members of an ESS within the administrative domain of the AP.

- Utilizing a secure IAPP protocol such as IEEE 802.11F, in order to determine whether a Reassociation Request has been sent to a legitimate member of the ESS.

The format of the Neighbor Report element is shown in Figure 0-13.

|  | Element ID | Length | Lower Timestamp Reference |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 4 |  |  |  |  |  |  |
|  | BSSID | BSSID Information | Channel Number | Channel Band | PHY Options | Neighbor TBTT Offset (optional) | Beacon Interval (optional) | The element contains zero or more neighbor list entries |  |
| Octets: | 6 | 2 | 1 | 1 | 1 | 2 | 2 |  |  |

|-----------------------------------------Neighbor List Entry-----------------------------------------------|

Figure 0-13 – Neighbor Report element format

The Element ID field shall be equal to the Neighbor Report value in Table 20.

The Length field is dependent on the number of Neighbor List Entries representing the neighboring APs being reported. Each entry describes an AP and consists of BSSID, BSSID Information, Current Channel, Channel Band, PHY Options, and optionally may also include the Neighbor TBTT Offset and Beacon Interval. The minimum value of the Length field is 0 (i.e., with no neighbor APs in the Neighbor Report element).

The Lower Timestamp Reference field is the lower 4 octets of the serving AP’s Timestamp field (defined in clause 7.3.1.10). It references the time at which the Neighbor TBTT Offset field is valid.

The BSSID is the address of the STA contained in the AP.  The subsequent fields: BSSID Information, Channel Number, Channel Band, PHY Options, and optionally Neighbor TBTT Offset and Beacon Interval are presumed to be for the corresponding BSSID.

The BSSID Information field may be used to help determine neighbor AP roaming candidates.  It shall be two octets in length and shall contain the subfields as shown in Figure 0-14.

|  | B0 B1 | B2 | B3 | B4 B8 | B9 B11 | B12 B15 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reachability | RSN | Key Scope | Capabilities | Reserved | Reserved |
| Bits: | 2 | 1 | 1 | 5 | 3 | 4 |

Figure – 0-14 – BSSID Information field format

- The Reachability bits indicate whether the AP represented by this BSSID is reachable by the STA for the purposes of pre-authentication.  The values are as follows: 

00 -  Reserved
01 - Not Reachable
10 - Unknown
11 - Reachable

The value "Reachable" implies that the AP represented by this BSSID are reachable by the STA for the purposes of pre-authentication.  Reachability may be achieved by a variety of mechanisms, including VLANs, tunnels, etc.  In the case where VLANs are supported, an AP is “Reachable” if it is a member of the same VLAN as the STA.  For example, in the case where APs are only members of the default VLAN, an AP will be "Reachable" if the STA is also assigned to the default VLAN, but will otherwise be "Not Reachable".  Where the AP may be a member of multiple VLANs, but the VLAN membership of either the AP or the STA is not known, the value Unknown should be returned.

- The RSN bit, if set, indicates that the AP represented by this BSSID matches the current AP’s RSN IE capabilities..  If the bit is not set, it indicates that either the capabilities do not match or the information is not available to the AP at this time.

- Bit 3 indicates whether the the AP represented by this BSSID has the same authenticator identity as the AP sending this report.

- The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set equal to the equivalent bits within the Capability Information field being sent by the AP being reported. The format of the Capabilities subfield is as follows:

|  | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Measurement | Block Ack |
| Bits: | 1 | 1 | 1 | 1 | 1 |

Figure – 0-15 – Capabilities Subfield

- Bits 9, 10, and 11 are reserved due to use by existing implementations (e.g. WPA, WMX/EDCA and WMX/HCCA).

- Bits 12 – 15 are reserved and shall be set to 0 on transmission and ignored on reception.

Channel Number indicates the current operating channel of the AP represented by this BSSID.

Channel Band shall contain an enumerated value from Table k1 specifying the frequency band in which the Current Channel is valid.

The PHY Options field represents the Condensed PHY type and TBTT Offset Flag as shown in the Figure 0-16.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | TBTT Offset Flag |
| Bits: | 7 | 1 |

Figure – 0-16 – PHY Options field

- Condensed PHY Type is the lower 7 bits of the PHY Type of the AP represented by this BSSID. The PHY type will be encoded as defined by dot11PHYType.

- TBTT Offset Flag is a one bit field that indicates whether the Neighbor List Entry contains the Neighbor TBTT Offset and Beacon Interval fields. The Neighbor Report may have the TBTT Offset Flag set in some Neighbor Report Entries and not set in others.

The Neighbor TBTT Offset field is used to indicate relative timing information for a neighbor AP. The Neighbor TBTT Offset field is composed of three fields as shown in Figure 0-17: a 1 bit Accuracy Granularity field, a 5 bit TSF TU Accuracy field, and a 10 bit TSF TU Offset field.

|  | B0 B9 | B10 B14 | B15 |
| --- | --- | --- | --- |
|  | TBTT TU Offset | TBTT TU Accuracy | Accuracy Granularity |
| Bits: | 10 | 5 | 1 |

Figure 0-17 - Neighbor TBTT Offset field format

- The 1 bit Granularity field pertains to the 5 bit TBTT TU Accuracy field.  If the bit is set to zero, it indicates fine granularity.   If the bit is set to one, it indicates coarse granularity.

- The 5 bit TBTT TU Accuracy field contains the TBTT TU Accuracy of the TBTT TU Offset field. If the Accuracy Granularity field indicates fine granularity, the TBTT TU Accuracy field represents the number of bins in ¼ TU (256us) steps that indicate the accuracy of the TBTT TU Offset field. If the Accuracy Granularity field indicates coarse granularity, the TBTT TU Accuracy field represents the number of bins in 2 TU (2.048ms) steps that indicate the accuracy of the TBTT TU Offset field starting at 10 TUs.  Below 8 TUs, the TBTT TU Accuracy field can be represented using fine granularity.  A value of all zeros for the 6 bits composed of the Accuracy Granularity and TBTT TU Accuracy fields shall mean that the accuracy is either not known or out of range. A value of all zeros for the 16 bit composing the Neighbor TBTT offset field shall mean that the neighbor TBTT Offset field is not supported.

For example, if the Accuracy Granularity field indicates fine granularity (bit 15 = 0) and the TBTT TU Accuracy field contains 00001, it means that the error of the TBTT TU Offset field is less than ¼ TU.  If the value is 10000, it means that the error of the TBTT TU Offset field is less than 8 TUs. If the Accuracy Granularity field indicates coarse granularity (bit 15 = 1) and the TBTT TU Accuracy field is 00000, it means the error of the TBTT TU Offset field is less than 10 TUs. If the TBTT TU Accuracy Field is 11111, it means that the error of the TBTT TU Offset field is less than 72 TUs.

- The 10 bit TBTT TU Offset field contains the TBTT TU Offset expressed in TUs. The TBTT TU Offset field represents the TUs until a neighbor AP’s TBTT.

The Neighbor TBTT Offset field results in a maximum TBTT TU Offset of approximately 1.048 seconds and a minimum accuracy of 256us. The mechanisms employed by the equipment vendor of the sender of the Neighbor Report to acquire and maintain the Neighbor TBTT Offset field is outside the scope of this specification.  It is required in any algorithm adopted that the TBTT TU Offset field must be accurate to within the TBTT TU Accuracy field.

The Beacon Interval field is the beacon interval of the Neighbor AP being reported.  This field is defined in clause 7.3.1.3 and illustrated in Figure 26.

Informative Note:

The following is an example of how a serving AP calculates the Neighbor TBTT Offset field of a Neighbor AP.

At the Lower Timestamp field time when the Neighbor TBTT Offset field is calculated, lets assume the serving AP has a TSF is equal to 0x0000 0000 AF55 0F10 and the neighbour AP has a TSF of 0x0000 0000 0011 F055 with a beacon interval of 0x64.

The serving AP should calculate the value of the Neighbor AP’s TSF value modulo the beacon interval. For this example this results in the following:

0x64 * 0x400 = 0x19000 microseconds

0x0011F055 mod 0x19000 = 0xC055 micrseconds

0xC055 represents the number of microseconds into the beacon interval.  Subtracting this value from the beacon interval in microseconds will result in the number of microseconds until the neighbor TBTT.

0x19000 – 0xC055 = 0xCFAB

To get this value into TUs, the AP divides by the number of microseconds in a TU.

0xCFAB / 0x400 = 0x33

To determine what the accuracy is for the conversion between microseconds and TUs, the modulo of the microseconds left in the neighbour’s beacon interval should be calculated

0xCFAB mod 0x400 = 0x3AB

Representing this value in terms of TBTT TU Accuracy bins results in the following calculation.

0x3AB / 0x100 (256us bin) = 3 + a remainder of 0xAB = 0x4 fine granularity bins.

The Accuracy Granularity field would be set to 0 for fine granularity in this example. Therefore the Neighbor TBTT Offset field is set to 0x1033.

The following is an example of how a STA would use the Neighbor TBTT Offset field to calculate when the Neighbor TBTT will occur.

Using the same values as above, the STA will receive in a Neighbor Report Response frame which includes a Neighbor TBTT Offset field of 0x1033 and a lower Timestamp field of 0xAF550F10.

In calculating when the neighbor’s TBTT will occur, the STA should turn the TBTT TU offset field into a microsecond count.

0x33 * 0x400 = 0xCC00 microseconds

So, the neighbor TBTT will occur in 0xCC00 microseconds with an accuracy of 1024 microseconds. Every subsequent TBTT will occur every 0x19000 microseconds thereafter.

## Replace clause 7.4.2, 7.4.2.3, and 7.4.2.4 with the following. Updated “Site Report” to “Neighbor Report”and added Neighbor Report Request Types to clause 7.4.2.3.

#### 7.4.2 Radio Measurement Action Details

Five Action frame formats are defined for Radio Measurement purposes. An Action field, in the octet field immediately after the Category field, differentiates the five formats. The Action field values associated with each frame format is defined in Table 20f.

Table 20f—Radio Measurement Action field values

| Action field value | Description |
| --- | --- |
| 0 | Measurement Request |
| 1 | Measurement Report |
| 2 | Reserved |
| 3 | Reserved |
| 4 | Neighbor Report Request |
| 5 | Neighbor Report Response |
| 7-255 | Reserved |

#### 7.4.2.3 Neighbor Report Request frame format

The Neighbor Report Request frame uses the Action frame body format and is transmitted by a STA requesting information in the Neighbor Report about neighboring AP’s.  The format of the Neighbor Report Request frame body is shown in Figure 0-4.

|  | Category | Action | Dialog Token | Neighbor Report Request Types | SSID |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure 0-4 – Neighbor Report Request frame body format

The Category field shall be set equal to the value indicating the Radio Measurement category, as specified in Table 1 in 7.3.11.

The Action field shall be set equal to the value indicating Neighbor Report Request, as specified in Table 5 in 7.4.1.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the measurement request to identify the request/report transaction.

The Neighbor Report Request Types field shall be shall be one octets in length and shall contain the subfields as shown in Figure 0-6.

|  | B0 | B1 B7 |
| --- | --- | --- |
|  | Neighbor TBTT Offset Type | Reserved |
| Bits: | 1 | 7 |

Figure – 0-6 – Neighbor Report Request Types Subfield

- Bit 0 – When this bit is set to 1, it indicates whether the TBTT Offset and the Beacon Interval fields may be included in the Neighbor Report.

The SSID element, if present, shall indicate the ESS for which Neighbor Report information is being requested. If the field is omitted, the Neighbor Report Response shall contain Neighbor Report elements for the ESS that the STA is a member.

#### 7.4.2.4 Neighbor Report Response frame format

The Neighbor Report Response frame uses the Action frame body format and is transmitted by a STA in response to a Neighbor Report Request frame or by a STA autonomously providing Neighbor Report information. The format of the Neighbor Report Response frame body is shown in Figure 0-5.

|  | Category | Action | Dialog Token | Neighbor Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure 0-5 - Site Report Response frame body format

The Category field shall be set equal to the value indicating the Radio Measurement category, as specified in Table 1 in 7.3.11.

The Action field shall be set equal to the value indicating Neighbor Report Response, as specified in Table 5 in 7.4.1.

The Dialog Token field shall be set equal to the value in any corresponding Neighbor Report Request frame. If the Neighbor Report Response frame is not being transmitted in response to a Neighbor Report Request frame then the Dialog token shall be set equal to zero. The Neighbor Report Elements field shall contain the Neighbor Report elements described in 7.3.2.22. If the STA has no information in response to the Neighbor Report Request, the Neighbor Report element will be omitted. The number and length of the Neighbor Report Elements in a Neighbor Report frame is limited by the maximum allowed MMPDU size.

## Replace clause 11.8, 11.8.1, and 11.8.2 with the following. Updated “Site Report” to “Neighbor Report” and updated for comment #43.

#### 11.8 Usage of the Neighbor report

A Neighbor report is sent by an AP and it contains information on known neighbor AP’s. A neighbor report may not be exhaustive either by choice, or due to the fact that there may be neighbor APs not known to the AP. The Site report contents shall be derived from the MIB table dot11RRMNeighborReportTable. The mechanism by which the contents of this table are determined is outside the scope of this amendment, but it may include information from measurement reports received from the STA’s within the BSS, information obtained via the management interface, or the DS.

#### 11.8.1 Neighbor Report Request

A STA requesting a neighbor report shall send a Neighbor Report Request frame to its associated AP. If the STA is interested in information concerning neighbor APs matching a specific SSID, it shall specify the SSID element in the Neighbor Report Request frame.

#### 11.8.1 Neighbor Report Response

An AP accepting a Neighbor Report Request shall respond with a Neighbor Report Response frame containing one or more Neighbor Report elements. If the SSID information element is specified in the corresponding Neighbor Report Request frame, the Neighbor Report elements shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the SSID. Otherwise, the Neighbor Report elements shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the current SSID with which the requesting STA is associated.

A Neighbor Report Response frame may also be sent autonomously by an AP. For example, the AP may autonomously send a Neighbor Report Response frame to a recently associated STA without receiving a Neighbor Report Request from that STA, or the AP may autonomously broadcast a Neighbor Report Response frame to all STAs in the BSS.

## Replace items RRM12 and RRM13 with the following in A.4.13 Radio Resource Measurement extensions. . Updated “Site Report” to “Neighbor Report” and updated for comment #43.

| RRM12 | Action Frame Protocol for Neighbor Reports Neighbor Report Request Neighbor Report Response | 7.4 7.4.2.3 7.4.2.4, 7.3.2.26 | M M M | Yes, No Yes, No Yes, No |
| --- | --- | --- | --- | --- |
| RRM13 | Neighbor Report Procedure | 11.8.1, 11.8.2 | M | Yes, No |

# Annex D – ASN.1 encoding of the MAC and PHY MIB

Replace the current dot11RRMSiteReport with the following text:

-- ********************************************************************

-- * dot11RRMNeighborReport	TABLE

-- ********************************************************************

dot11RRMNeighborReportNextIndex OBJECT-TYPE

SYNTAX Integer(0..255)

MAX-ACCESS	read-only

STATUS current

DESCRIPTION

"Identifies the next available index for managing the Neighbor Report table.  If this attribute is 0, it indicates that the Neighbor Report feature is not configurable via SNMP, or the table is full and new rows cannot be accepted."

::= { dot11RRMConfig 3 }

dot11RRMNeighborReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF dot11RRMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains pertinent information on a collection of BSSID’s that are candidates to which STA’s can roam. The rows are created using createAndWait method and fill in the attributes. When the rowStatus is set to active, the row can be included in Neighbor Report IEs. If there is an error, the rowStatus shall be set to notReady by SME. Since this table contains all Neighbor Report IE entries for all interfaces enabled with the Neighbor Report feature, it is possible to have too many entries for one interface, while still remaining under the MaxTableSize. In that situation, SME shall only include Neighbor Report entries with lower dot11RRMNeighborReportIFIndex up to the maximum possible number of entries for a particular interface identified by ifIndex. SME shall set the rowStatus to notInService for those rows that cannot be included in the Neighbor Report element for that interface."

::= { dot11RRMConfig 4 }

dot11RRMNeighborReportEntry OBJECT-TYPE

SYNTAX dot11RRMiteReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMNeighborReportTable"

INDEX { dot11RRMNeighborReportIndex }

::= { dot11RRMNeighborReportTable 1 }

Dot11RRMNeighborReportEntry ::=

SEQUENCE {

Dot11RRMNeighborReportIndex

dot11RRMNeighborReportIfIndex			ifIndex,

dot11RRMNeighborReportBSSID			MacAddress,

dot11RRMNeighborReportReachability		INTEGER,

dot11RRMNeighborReportRSN			TruthValue,

dot11RRMNeighborReportCapSpectrumMgmt		TruthValue,

dot11RRMNeighborReportCapQoS			TruthValue,

dot11RRMNeighborReportCapAPSD			TruthValue,

dot11RRMNeighborReportCapRRM			TruthValue,

dot11RRMNeighborReportCapBlockAck		TruthValue,

dot11RRMNeighborReportKeyScope			TruthValue,

dot11RRMNeighborReportChannelNumber		INTEGER,

dot11RRMNeighborReportChannelBand		INTEGER,

dot11RRMNeighborReportPhyOptions	        INTEGER,

dot11RRMNeighborReportTimestampReference,     TSFType,

dot11RRMNeighborReportNeighborTBTTOffset	Unsigned32,

dot11RRMNeighborReportBeaconInterval		Unsigned32,

dot11RRMNeighborReportRowStatus		RowStatus

}

dot11RRMNeighborReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Neighbor Report configuration table in dot11RRMNeighborReportTable, greater than 0."

::= { dot11RRMNeighborReportEntry 1 }

dot11RRMNeighborReportIfIndex OBJECT-TYPE

SYNTAX ifIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of Neighbor Report entry belongs to."

::= { dot11RRMNeighborReportEntry 2 }

dot11RRMNeighborReportBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The BSSID to which this row of Neighbor Report applies."

::= { dot11RRMNeighborReportEntry 3 }

Dot11RRMNeighborReportReachability OBJECT-TYPE

SYNTAX		INTEGER { reserved(0), notReachable(1), unknown(2), reachable(3) }

MAX-ACCESS	read-write

STATUS		current

DESCRIPTION	“Indicates the reachability status of the AP represented by the

dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 4}

Dot11RRMNeighborReportCapSpectrumMgmt OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the spectrum management capability of the AP represented by dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 5}

Dot11RRMNeighborReportCapQoS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the QoS capability of the AP represented by dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 6}

Dot11RRMNeighborReportCapAPSD OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the APSD capability of the AP represented by dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 7}

Dot11RRMNeighborReportCapRRM OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the RRM capability of the AP represented by dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 8}

Dot11RRMNeighborReportCapBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates the BlockAck capability of the AP represented by dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 9}

Dot11RRMNeighborReportKeyScope OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“Indicates a shared authenticator identity for the AP represented by dot11NeighborReportBSSID”

::= {dot11RRMNeighborReportEntry 10}

dot11RRMNeighborReportChannelNumber OBJECT-TYPE

SYNTAX Integer (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is the current operating channel of the STA returning the report"

::= { dot11RRMNeighborReportEntry 11 }

dot11RRMNeighborReportChannelBand OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This corresponds to the frequency band for the channel list being reported as defined in Table [editor TBD]."

::= { dot11RRMNeighborReportEntry 12 }

dot11RRMNeighborReportPhyOptions OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The PHY Options of the Neighbor AP Entry."

::= { dot11RRMNeighborReportEntry 13 }

dot11RRMNeighborReportTimestampReference OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The full TSF value of Lower Timestamp Reference field as defined in clause 7.3.2.26."

::= { dot11RRMNeighborReportEntry 14 }

dot11RRMNeighborReportNeighborTBTTOffset OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Neighbor TBTT Offset field is as defined in clause 7.3.2.26."

::= { dot11RRMNeighborReportEntry 15 }

dot11RRMNeighborReportBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Beacon Interval field is as defined in clause 7.3.2.26."

::= { dot11RRMNeighborReportEntry 16 }

dot11RRMNeighborReportRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains the row status of the Neighbor Report, essentially used for indicating whether the row has all valid attributes filled in. Then set to active to be used in Neighbor Report information elements. If any parameter is invalid, the SME shall set this attribute back to notReady. It is the responsibility of the manager to correct the parameters."

::= { dot11RRMNeighborReportEntry 17 }

-- ********************************************************************

-- * End of dot11RRMNeighborReport 	TABLE

-- ********************************************************************