IEEE P802.11
Wireless LANs

Frame Encapsulation

Date:	11th July 2004

Author:	Mike Moreton, Peyush Agarwal
	STMicroelectronics
	Phone: +44 1189298008
	e-Mail: mike.moreton@st.com, peyush.agarwal@st.com

Abstract

This submission provides draft text for a mechanism to protect Measurement Request and Measurement Report frames, using a data frame encapsulation mechanism derived from that previously suggested by Microsoft.

.

Note that the presentation describing this submission is embedded in this document on page 2.

PowerPoint Presentation Describing Mechanism

Add the following new clause after clause 7.1.3 :

7.1.4 MAC Frame Encapsulation

This clause defines a mechanism by which an IEEE802.11 MAC frame may be encapsulated within the payload of an 802.11 frame of type Data.   Such encapsulation is used where frames of type other than Data need the protection or non-local delivery services available for frames of type Data.

Use of this mechanism is indicated by a unique SNAP protocol identifier (to be allocated during Sponsor Ballot), and hence frames using this mechanism are treated identically to all other frames of type Data, including use of protection as defined in clause 8 where configured.  For this reason, the encapsulation described in this clause can only be used where communication is between the SMEs of the involved STAs.  Furthermore, in an RSNA this protocol shall be a controlled port protocol, and hence shall be discarded when the controlled port is not authorised.

A STA shall discard any frame addressed to it that uses this encapsulation mechanism, except where use of this mechanism is described elsewhere in this standard.  Equally, unless specified elsewhere, a STA associated with an AP shall discard any frames using this encapsulation mechanism that do not originate from that AP.

The encapsulation mechanism retains the Frame Control and Frame Body fields of the frame to be encapsulated, and carries them within the Frame Body of a frame of type Data.  Figure X shows the Frame Body of a frame of type Data used to encapsulate a frame in this manner.

|  | WEP/CCMP/TKIP Header (optional) | 802.2 LLC | 802.2 SNAP (Protocol Identifier XX:XX – to be allocated during sponsor ballot) | Frame Control Field of Encapsulated Frame | Frame Body of Encapsulated Frame | WEP/CCMP/TKIP Trailer (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | n | 4 | 5 | 2 | m | p |

Figure X: MAC Frame Encapsulation

All sub-fields of the encapsulated Frame Control field except for Type and Sub-Type shall be set to zero on transmission, and ignored on reception.

Add the following at the end of clause 7.4.2.1 Measurement Request frame format

Measurement Request frames may only be sent using the encapsulation defined in clause 7.1.4.  A STA shall discard any Measurements Request frames not encapsulated as described in clause 7.1.4.

Add the following at the end of clause 7.4.2.2 Measurement Report frame format

Measurement Report frames may only be sent using the encapsulation defined in clause 7.1.4.  A STA shall discard any Measurements report frames not encapsulated as described in clause 7.1.4.

Add the following at the end of clause 7.4.2.3 Site Report Request frame format

Site Report Request frames may only be sent using the encapsulation defined in clause 7.1.4.  A STA shall discard any Site Report Request frames not encapsulated as described in clause 7.1.4.

Add the following at the end of clause 7.4.2.4 Site Report Response frame format

Site Report Response frames may only be sent using the encapsulation defined in clause 7.1.4.  A STA shall discard any Site Report Response frames not encapsulated as described in clause 7.1.4.