IEEE P802.11
Wireless LANs

802.11 TGr Minimum Requirements

Date:	July 15, 2004

Author:	Jim Wendt
	Hewlett-Packard Company
	8000 Foothills Blvd.  M/S 5555
	Roseville, CA  95747-5555
	Phone: +1-916-785-5198
	e-Mail: jim.wendt@hp.com

Abstract

This document specifies the minimum requirements for the Fast-BSS-Transition mechanism to be defined by 802.11 TGr.

Table of Contents

1.1	Contributors	1

1.2	Revision History	1

1.3	Overview	2

1.4	Definitions	2

1.4.1	BSS-Transition	2

1.4.2	Fast-BSS-Transition Environment	2

1.4.3	Fast-BSS-Transition Definition	3

1.4.4	Fast-BSS-Transition-Time	3

1.5	General Requirements	3

1.6	Environment Requirements	3

1.6.1	Topology Scenarios	3

1.6.2	PHY/MAC Scenarios	4

1.6.3	RF Scenarios	4

1.7	Performance Requirements	4

1.7.1	Fast-BSS-Transition Time Goals	4

1.8	Processing Requirements	4

1.8.1	STA Processing	4

1.8.2	Power Save	4

1.9	System Requirements	4

1.9.1	Compatibility	4

1.10	Reference Documents List	4

## Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |

## Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Initial requirements framework for group discussion purposes | 15 July 2004 |

## Overview

This document specifies the minimum requirements and minimum thresholds for the Fast-BSS-Transition mechanism which must be met by any proposed solution.

Note that proposed solutions are not precluded from going beyond these minimum requirements.

## Definitions

### BSS-Transition

A BSS-transition is currently defined as follows:

- (clause 5.4.2.1b) “BSS-transition: This type is defined as a station movement from one BSS in one ESS to another BSS within the same ESS.”

- This BSS-transition definition implies the following:

  - The AP for each BSS must be in the same ESS

  - The AP for each BSS in the ESS must have the same SSID

  - The AP for each BSS in the ESS must be connected to the same DS

  - “Stations within the ESS may communicate and mobile stations may move from one BSS to another BSS (within the same ESS) transparently to LLC”

  - The APs for each BSS must be configured for the same security type

  - The DS is notified when the STA complete the BSS-transitions

<this section needs to quote several additional clauses>

### Fast-BSS-Transition Environment

The Fast-BSS-Transition environment is shown in the figure below:

### Fast-BSS-Transition Definition

A Fast-BSS-Transition is defined as a BSS-Transition that meets the Fast-BSS-Transition-Time goals as stated in this document.

### Fast-BSS-Transition-Time

The Fast-BSS-Transition-Time period is shown in the figure below:

The Fast-BSS-Transition-Time is defined as the period between the last possible point in time (tA) where STA-TE communication can pass through the Old-AP to the first possible point in time (tB) where STA-TE communication can pass through the New-AP.

## General Requirements

GEN.1 – The Fast-BSS-Transition mechanism must meet the conditions stated in the TGr PAR.

## Environment Requirements

### Topology Scenarios

TOPO.1 - Fast-BSS-Transitions are only supported for a STA that supports the Fast-BSS-Transition capability.

TOPO.2 - Fast-BSS-Transitions at a minimum should be supported between two APs which both support the Fast-BSS-Transition capability.

### PHY/MAC Scenarios

PHYMAC.1 - A Fast-BSS-Transition shall be possible between two APs that support any of the following standards: 802.11a, 802.11b, 802.11g

- Consideration would be given to future MAC/PHY implementations

PHYMAC.2 - A Fast-BSS-Transition shall be possible between two APs that are using different channels

PHYMAC.3 - A Fast-BSS-Transition shall be possible between two APs that are using the same channel

PHYMAC.4 – A Fast-BSS-Transition does not have to be supported between different 802.11 bands (a/b/g/n/etc)

### RF Scenarios

RF.1 – Fast-BSS-Transition shall be supported across overlapping cells

RF.2 – Fast-BSS-Transition may be supported across tangential cells

RF.3 – Fast-BSS-Transition does not have to be supported across non-overlapping cells

## Performance Requirements

### Fast-BSS-Transition Time Goals

TIME.1 – The Fast-BSS-Transition-Period shall be a maximum of NN milliseconds

## Processing Requirements

### STA Processing

STA.1 – Fast-BSS-Transition is not required to be supported when a STA has already lost the connection to the Old-AP.

### Power Save

POW.1 - Fast-BSS-Transition is not required to be supported when a STA has gone into power-save mode.

## System Requirements

### Compatibility

COMP.1 – Fast-BSS-Transition capable devices shall co-exist with other legacy 802.11 devices.

## Reference Documents List

<this list needs to be completed>

11-03-813-12-000n-functional-requirements

11-04-503-00-

11-04-516-00-frfh-requirements-discussion

11-04-614-01-

11-04-619-01-frfh-requirementsdocument

11-04-629-01-

11-04-666-01-000r-tgr-requirements-strawman

11-04-677-01-000r-fast-bss-transition-use-cases

11-04-678-00-000r-scope

11-04-785-00-

> 10.0.1.21 -- Last submission in list – Cross Domain Handover