IEEE P802.11
Wireless LANs

802.11 TGr Minimum Requirements

Date:	July 15, 2004

Author:	Jim Wendt
	Hewlett-Packard Company
	8000 Foothills Blvd.  M/S 5555
	Roseville, CA  95747-5555
	Phone: +1-916-785-5198
	e-Mail: jim.wendt@hp.com

Abstract

This document specifies the minimum requirements for the Fast-BSS-Transition mechanism to be defined by 802.11 TGr.

Table of Contents

1.1	Contributors	2

1.2	Revision History	2

1.3	Overview	2

1.4	Definitions	2

1.4.1	BSS-Transition	2

1.4.2	Fast-BSS-Transition Environment	2

1.4.3	Fast-BSS-Transition Definition	3

1.4.4	Fast-BSS-Transition-Time	3

1.5	General Requirements	4

1.6	Environment Requirements	4

1.6.1	Topology Scenarios	4

1.6.2	PHY/MAC Scenarios	5

1.6.3	RF Scenarios	5

1.6.4	Security Scenarios	5

1.7	Performance Requirements	5

1.7.1	Fast-BSS-Transition Time Goals	5

1.8	Processing Requirements	5

1.8.1	Power Save	6

1.9	System Requirements	6

1.9.1	Compatibility	6

1.10	Reference Documents List	6

## Contributors

| Name | Company | Address | Phone | Fax | Email |
| --- | --- | --- | --- | --- | --- |

## Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Initial requirements framework for group discussion purposes | 15 July 2004 |
| R1 | Incorporated changes made during Portland TGr meetings on 7/15/04. Doc is being posted with redline markups. A subsequent cleaned-up version will be posted shortly. | 15 July 2004 |

## Overview

This document specifies the minimum requirements and minimum thresholds for the Fast-BSS-Transition mechanism which must be met by any proposed solution.

Note that proposed solutions are not precluded from going beyond these minimum requirements and minimum thresholds

## Definitions

### BSS-Transition

A BSS-transition is currently defined as follows:

- (clause 5.4.2.1b) “BSS-transition: This type is defined as a station movement from one BSS in one ESS to another BSS within the same ESS.”

- This BSS-transition definition implies the following:

  - The AP for each BSS must be in the same ESS

  - The AP for each BSS in the ESS must have the same SSID

  - The AP for each BSS in the ESS must be connected to the same DS

  - “Stations within the ESS may communicate and mobile stations may move from one BSS to another BSS (within the same ESS) transparently to LLC”

  - The DS is notified when the STA completes the BSS-transition

<this section needs to quote several additional clauses>

### Fast-BSS-Transition Environment

The Fast-BSS-Transition environment is shown in the figure below:

<Replace the following diagram with an even prettier version of Darwin’s “portals” diagram. Maybe include the “controlled port” as well since the FBT.2 definition references it>

### Fast-BSS-Transition Definition

FBT.1 - A Fast-BSS-Transition is defined as a BSS-Transition that meets the Fast-BSS-Transition-Time goals as stated in this document.

### Fast-BSS-Transition-Time

The Fast-BSS-Transition-Time period is shown in the figure below:

FBT.2 - The Fast-BSS-Transition-Time is defined as follows:

- Security not enabled:

  - the period between the last possible point in time (tA) where STA-TE communication can pass through the Old-AP, to the first possible point in time (tB) where STA-TE communication can pass through the New-AP

- Security enabled:

  - the period between the last possible point in time (tA) where STA-TE communication can pass through the Old-AP, to the point in time (tB) where the first MSDU can pass through the controlled port

General Requirements

GEN.1 – The Fast-BSS-Transition mechanism must meet the conditions stated in the TGr PAR.

## Environment Requirements

### Topology Scenarios

TOPO.1 - Fast-BSS-Transitions are only supported for a STA that supports the Fast-BSS-Transition capability.

TOPO.2 - Fast-BSS-Transitions at a minimum must be supported between two APs which both support the Fast-BSS-Transition capability.

### PHY/MAC Scenarios

- PHYMAC.1 - A Fast-BSS-Transition shall be possible between two APs that support any of the following standards: 802.11a, 802.11b, 802.11g

- Consideration would be given to future MAC/PHY implementations

PHYMAC.2 - A Fast-BSS-Transition shall be possible between two APs that are using different channels

PHYMAC.3 - A Fast-BSS-Transition shall be possible between two APs that are using the same channel

PHYMAC.4 – A Fast-BSS-Transition should be possible between different 802.11 bands (a/b/g) as those bands are supported by the STA.

### RF Scenarios

RF.1 – Fast-BSS-Transition shall be supported across overlapping BSSes

RF.2 – Fast-BSS-Transition may be supported across tangential BSSes

### Security Scenarios

<Jessie, Nancy, and others will provide the text for this section>

SEC.1 – Fast-BSS-Transitions shall be possible between APs that have PMKs derived from the same Master Key (including both MSKs and EMSKs).

- ? PMK is unique as STA transitions

- ? lifetime of the Security Association

- ? unique per Authentication Binding

- ? link or session = ?

Performance Requirements

### Fast-BSS-Transition Time Goals

Proposals must submit transition time information as part of the proposal which follows a Fast-BSS-Transition-Time measurement methodology and reporting methodology that is TBD

.

Processing Requirements

### Power Save

POW.1 – The Fast-BSS-Transition-Time goal is not required to be met when a STA has gone into power-save mode.

## System Requirements

### Compatibility

COMP.1 - Fast-BSS-Transition capable APs shall be able to concurrently support associations with both legacy STAs and Fast-BSS-Transition capable STAs.

COMP.2 - Fast-BSS-Transition capable STAs shall be able to support associations with both legacy APs and Fast-BSS-Transition capable APs.

Reference Documents List

<this list needs to be completed>

11-03-813-12-000n-functional-requirements

11-04-086-03 -

11-04-503-00-

11-04-516-00-frfh-requirements-discussion

11-04-614-01-

11-04-619-01-frfh-requirementsdocument

11-04-629-01-

11-04-666-01-000r-tgr-requirements-strawman

11-04-677-01-000r-fast-bss-transition-use-cases

11-04-678-00-000r-scope

11-04-785-00-

> 10.0.1.21 -- Last submission in list – Cross Domain Handover