IEEE P802.11
Wireless LANs

- Scenarios Summary

Date:	July 28, 2004

Author:	Richard Paine

Box 3777, MS 7L-49

`	Seattle, Wa 98008
	Phone: 425-865-4921
	Fax: 425-865-2965
	e-Mail: richard.h.paine@boeing.com

Scenario Summary for WNM SG

Public

Hotspot Scenario – Starbucks, Borders, theaters (no roaming until maybe afterwards)

- Single SSID for hotspot, could see other APs

- AP 11k Service beacon

- AP Neighbor report

- STA Neighbor report

- Bandwidth is sometimes limited to DSL

- STA Management Needed

- AP Management Needed

- Roam Capability

HotZone Scenario – Airports, train stations, malls

- Multiple SSIDs (virtual Aps)

- Better backhaul

- STA Management Needed

  - Beacon (passive scan)

    - SSID

    - BSSID

    - ESS Information

    - RCPI

    - Data Rate

    - BER

    - SNR

    - Neighbor Graph

  - Probe

    - RCPI

    - Data Rate

    - BER

    - SNR

    - Neighbor Report

    - Neighbor Graph

    - Location + direction

- AP Management Needed

- Roam Management Capability

Multiple dwelling units (apartments, townhomes, condos).  Residences very close together (will be multiple SSIDs)

- Many SSIDs

- Optimization of a dense radio environment

- Power level optimization

- Load balancing

- Context Transfer Protocol or IAPP back to the device.

- Handover if community WLAN offered by management

- Channel map

- Neighbor report

- parameters stored in the MIB

- STA Management Needed

- AP Management Needed

- No roam

Enterprise

Rogues

- Roque APs (clients collect data and report it to the network manager. APs are too busy to collect information. Think of them as T-cells. )

- Management Needed

- No Roam

Ubiquitous WLAN Scenario (laptop pre-configured to have a single SSID)

- Single or few SSIDs

- Context Transfer Protocol or IAPP back to the device.

- Handover – requires RCPI

- Channel map

- Neighbor report for pre-authentication and preauthorization

- parameters stored in the MIB (dissassociateimminent)

- STA Management Needed

- AP Management Needed

- Roam Capability

Non-Ubiquitous WLAN Scenario (laptop pre-configured to have a single SSID)

- Single or few SSIDs

- Context Transfer Protocol or IAPP back to the device.

- Stateful protocol to maintain state until the device is back in coverage

- Handover – requires RCPI

- Channel map

- Neighbor report for pre-authentication and preauthorization

- parameters stored in the MIB (dissassociateimminent)

- STA Management Needed

- AP Management Needed

- Roam Capability

Hand-held Ubiquitous WLAN Scenario

- handheld pre-configured to have a single SSID

- Power save mode impacts frequency and scope of management

- Needs AP's neighbor report to create its own.

- Management of battery levels for radio transmission

- This neighbor report changes in real-time as he/she roams.

- signal strength measure by initiating a request to do pre-authentication and pre-authorization with the next cell's AP.

- The neighbor report of the STA in the iPAQ continuously reflects the signal strengths and unique characteristics of the STA and its environment.

- The setup is in the pre-authentication and pre-authorization before the handoff.

- handoff in 40ms

- STA Management Needed

- AP Management Needed

- Roam Capability

Handheld Non-Ubiquitous WLAN Scenario

- Single SSID

- the neighbor report indicates that there is no AP in that direction to handoff to.

- The AP maintains state until the next coverage area occurs using the neighbor report information it has.  Before it reaches the boundary of the next coverage area,

- the RCPI and the neighbor report have indicated that it is moving into the range of one AP.

- The trigger is possibly the old neighbor report indicating an associate is imminent or other parameters stored in the MIB.

- The preauthentication and preauthorization occur with the next AP that is in the neighbor report, using hysteresis (history + trending) and neighbor reporting.

- If a different subnet, then the DHCP request is made by the next AP for a new IP address and passed via Context Transfer Protocol or IAPP back to the device through the old AP.

- STA Management Needed

- AP Management Needed

- Roam Capability

High Speed Mobile

High Speed Mobile Setup (may be multiple SSIDs)

- Single or few SSIDs

- Context Transfer Protocol or IAPP back to the device.

- Handover – requires RCPI

- Channel map

- neighbor report for pre-authentication and preauthorization

- parameters stored in the MIB (dissassociateimminent)

- mechanism for rapid changes in APs

- STA Management Needed

- AP Management Needed

- Very fast roaming and handoff

  - Same network easy

  - Different network difficult

Airplane

Passengers very close together (single SSIDs) and they are a captive audience.

- One SSID

- Optimization of a very dense radio environment

- Power level optimization

- Load balancing

- Context Transfer Protocol or IAPP back to the device.

- Channel map

- STA Management Needed

- AP Management Needed

- Roam across four or five APs

Radar Detection

Defense radars detected from Wi-Fi, monitor channels in the background

- Monitor channels

- Dynamic load balancing

Multiple APs and Multiple STAs

Management of APs and STAs from central management

- Monitor power

- VOIP pre-emption

Location-Enabled Management

Location is the flag for managing security and other things

- Managing based on location