IEEE P802.11
Wireless LANs

System Level and Physical Layer Simulation Methodologies and Results

August 13, 2004

John Ketchum, Sanjiv Nanda, Rod Walton, Steve Howard, Mark Wallace, 
Bjorn Bjerke, Irina Medvedev, Santosh Abraham, Arnaud Meylan, Shravan Surineni

QUALCOMM, Incorporated
9 Damonmill Square, Suite 2A
Concord, MA 01742
Phone: 781-276-0915
Fax:  781-276-0901
e-Mail: johnk@qualcomm.com

Abstract

This document provides an overview of the simulation methodologies adopted for the system level and physical layer simulations supporting Qualcomm’s proposal for high throughput extensions to 802.11. Detailed simulation results that address the 802.11n Functional Requirements and Comparison Criteria are also presented.

The salient features of the proposal are:

- Maximum PHY data rates in 20 MHz:

  - 202 Mbps for stations with two antennas

  - 404 Mbps for stations with four antennas

- Highly reliable, high-performance operation with existing 802.11 convolutional codes used in combination with Eigenvector Steering spatial multiplexing techniques

- Backward compatible modulation, coding and interleaving

- Backward compatible preamble and PLCP. Extended SIGNAL field.

- Adaptation of rates and spatial multiplexing mode through low overhead asynchronous feedback. Works with TXOPs obtained through EDCA, HCF or ACF

- Two spatial multiplexing modes:

  - Eigenvector Steering (ES).

  - Spatial Spreading (SS).

- Calibration procedure for ES

- Up to four spatial streams

- Scalable antenna configurations (minimum 1, maximum 4 or more)

- Steered reference: Eigenvector Steered MIMO training sequence to reduce receiver complexity, e.g. at AP steering to dozens of STAs.

- Reduced OFDM symbol overhead through shortened guard interval and additional data subcarriers

- Flexible frame aggregation with limit on maximum aggregated PSDU size

- Mandatory 802.11e Block Ack and Delayed Block Ack

- Elimination of Immediate ACK for MIMO transmissions. Permits reduction in IFS.

- Adaptive coordination function (ACF): Low latency scheduled operation

- QoS-capable IBSS operation

- Mandatory 802.11h TPC and DFS

Through these features we are able to demonstrate excellent performance in a 20 MHz bandwidth for STAs with 2 transmit and 2 receive antennas. Even higher performance is achieved in networks where a subset of high capability, high throughput STAs and the AP are equipped with 4 antennas. The proposed system achieves high throughput and robust performance at increased range and lower power utilization. These features have been characterized, analyzed and implemented in an operational FPGA prototype.

The complete proposal submitted by QUALCOMM consists of the following four documents:

- 11-04-870 High Throughput System Description and Operating Principles (this document.)

  - Section 1 provides an overview of the proposed PHY and MAC enhancements

  - Section 2 provides a detailed description and proposed text for the MAC and PLCP enhancements.

  - Section 3 provides a detailed description and proposed text for the PHY enhancements.

  - Appendix A provides the mathematical background and operating principles for MIMO applicable to the proposal.

- 11-04-871 High Throughput Proposal Compliance Statement.

  - Section 1 addresses compliance with the functional requirements of 802.11n.

  - Section 2 addresses compliance with the PAR and Five Criteria of 802.11n.

  - Section 3 addresses Comparison Criteria of 802.11n.

- 11-04-872 Link Level and System Performance Results for High Throughput Enhancements.

  - Section 1 describes the system simulation methodology

  - Section 2 provides system performance results for the simulation scenarios defined in the 802.11n usage models document.

  - Section 3 describes the PHY simulation methodology

  - Section 4 provides link level simulation results for packet error rate and throughput.

  - Section 5 defines the link abstraction used to capture the packet error model in system level simulations and also provides model verification results.

  - Section 6 provides performance results for the modified preamble.

- 11-04-873 High Throughput Enhancements Presentation – Features and Performance. Summary presentation of the proposal features and performance results.

  - PHY Features

  - MAC Features

  - Link Performance

  - System Performance

Table of Contents

1.	Introduction	10

1.1	Summary of System Simulation Results	10

1.2	Summary of PHY Simulation Results	11

2.	System Simulation Methodology	12

3.	System Simulation Results	14

3.1	Simulation Conditions That Are Fixed	14

3.2	Simulation Conditions That Are Varied	14

3.3	EDCA Parameters	15

3.4	Additional Scenarios	15

3.5	Summary and Observations from System Simulation Results	16

3.5.1	Aggregate Throughput Results	16

3.5.2	MAC Efficiency	17

3.5.3	Summary of QoS Flows Satisfied	18

3.5.4	Summary of Non-QoS Throughput Achieved	19

3.5.5	Rate versus Range	19

3.5.6	Legacy Sharing	19

3.6	What the Columns in the Following Sections Mean	20

3.7	Scenario 1 (Residential BSS) Results	21

3.7.1	Comparison of ACF, HCF and EDCA for Scenario 1 (2x2, 2.4 GHz)	22

3.7.1.1	Scenario 1: 2x2, 2.4 GHz, SGI-EXP, ACF	22

3.7.1.2	Scenario 1: 2x2, 2.4 GHz, SGI-EXP, HCF	23

3.7.1.3	Scenario 1: 2x2, 2.4 GHz, SGI-EXP, EDCA	24

3.7.2	Comparison of ACF, HCF and EDCA for Scenario 1 (2x2, 5.25 GHz)	25

3.7.2.1	Scenario 1: 2x2, 5.25 GHz, SGI-EXP, ACF	25

3.7.2.2	Scenario 1: 2x2, 5.25 GHz, SGI-EXP, HCF	26

3.7.2.3	Scenario 1: 2x2, 5.25 GHz, SGI-EXP, EDCA	27

3.7.3	Comparison of ACF, HCF and EDCA for Scenario 1 (4x4, 5.25 GHz)	28

3.7.3.1	Scenario 1: 4x4, 5.25 GHz, SGI-EXP, ACF	28

3.7.3.2	Scenario 1: 4x4, 5.25 GHz, SGI-EXP, HCF	29

3.7.3.3	Scenario 1: 4x4, 5.25 GHz, SGI-EXP, EDCA	30

3.7.4	Comparison of 2x2 and 4x4 for Scenario 1 (Standard symbols)	31

3.7.4.1	Scenario 1: 2x2, 5.25 GHz, Standard, ACF	31

3.7.4.2	Scenario 1: 4x4, 5.25 GHz, Standard, ACF	32

3.8	Extended Scenario 1 Results	33

3.8.1	Scenario 1 HT	34

3.8.1.1	Scenario 1 HT: 2x2, 5.25 GHz, SGI-EXP, ACF	34

3.8.1.2	Scenario 1 HT: 4x4, 5.25 GHz, SGI-EXP, ACF	35

3.8.2	Scenario 1 EXT	36

3.8.2.1	Scenario 1 EXT: 2x2, 5.25 GHz, SGI-EXP, ACF	36

3.8.2.2	Scenario 1 EXT: Mixed, 5.25 GHz, SGI-EXP, ACF	37

3.8.2.3	Scenario 1 EXT: 4x4, 5.25 GHz, SGI-EXP, ACF	38

3.9	Scenario 4 (Large Enterprise) Results	39

3.9.1	Scenario 4: 2.4 GHz	40

3.9.1.1	Scenario 4: 2x2, 2.4 GHz, SGI-EXP, ACF	40

3.9.1.2	Scenario 4: 4x4, 2.4 GHz, SGI-EXP, ACF	41

3.9.2	Scenario 4: 5.25 GHz	42

3.9.2.1	Scenario 4: 2x2, 5.25 GHz, SGI-EXP, ACF	42

3.9.2.2	Scenario 4: 4x4, 5.25 GHz, SGI-EXP, ACF	43

3.10	Scenario 6 (Hot Spot) Results	44

3.10.1	Comparison of ACF, HCF and EDCA for Scenario 6	45

3.10.1.1	Scenario 6: 2x2, 5.25 GHz, Standard, ACF	45

3.10.1.2	Scenario 6: 2x2, 5.25 GHz, Standard, HCF	46

3.10.1.3	Scenario 6: 2x2, 5.25 GHz, Standard, EDCA	47

3.10.2	Scenario 6: Comparison of 2x2, Mixed and 4x4	48

3.10.2.1	Scenario 6: 2x2, 5.25 GHz, Standard, ACF	48

3.10.2.2	Scenario 6: Mixed, 5.25 GHz, Standard, ACF	49

3.10.2.3	Scenario 6: 4x4, 5.25 GHz, Standard, ACF	50

3.11	Scenario 6 EXT Results	51

3.11.1	Scenario 6 EXT: Comparison of 2x2, Mixed and 4x4	52

3.11.1.1	Scenario 6 EXT: 2x2, 5.25 GHz, Standard, ACF	52

3.11.1.2	Scenario 6 EXT: Mixed, 5.25 GHz, Standard, ACF	53

3.11.1.3	Scenario 6 EXT: 4x4, 5.25 GHz, Standard, ACF	54

3.12	Scenario 16 (Rate versus Range) Results	55

3.12.1	Channel Model B	55

3.12.2	Channel Model D	56

3.13	Results for Scenarios 17, 18, 19 (Legacy Sharing)	57

4.	Physical Layer Simulation Methodology	58

4.1	Simulation Methodology	58

4.2	Eigenvector Steering Transmission Mode	59

4.3	Spatial Spreading Transmission Mode	60

4.4	Common Simulation Conditions	60

5.	Physical Layer Simulation Results	62

5.1	Eigenvector Steering Transmission Mode	62

5.1.1	CC59 – AWGN PER Performance	62

5.1.2	CC67 – PER Performance in Non AWGN Channels	62

5.1.3	CC67.2 – Offset Compensation	64

5.2	Spatial Spreading Transmission Mode	65

5.2.1	CC59 – AWGN PER Performance	65

5.2.2	CC67 – PER Performance in Non AWGN Channels	65

5.2.3	CC67.2 – Offset Compensation	66

5.2.4	Additional Results Obtained with Spatial Spreading Transmission Mode without Cyclic Delay	67

5.3	Some Results Obtained with Short Guard Interval and 52 Data Subcarriers	68

5.3.1	CC59 – AWGN PER Performance	68

5.3.2	CC67 – PER Performance in Non AWGN Channels	68

6.	Error Model for System Simulations Based on PHY Abstraction	71

6.1	Introduction	71

6.2	PHY Abstraction: Post-Detection SNRs	71

6.3	Error Process Generation	72

6.4	Error Model Validation	73

7.	References	75

List of Tables

Table 3-1  EDCA Parameters	13

Table 3-2 Aggregate Throughput Results for Scenarios 1, 1 HT, 1 EXT, 4, 6 and 6 EXT for 2x2, 4x4 and Mixed	14

Table 3-3 MAC Efficiency Results	15

Table 3-4 Summary of QoS Flows Satisfied	16

Table 3-5 Summary of Non-QoS Throughput Achieved	17

Table 4-1: Data Rates	59

Table 5-1: Organization of plots for ES mode	61

Table 5-2: Impact of carrier frequency and symbol clock offsets in channel E (LOS) at 50 dB	62

Table 5-3: Organization of plots for SS mode	63

Table 5-4: Impact of carrier frequency and symbol clock offsets in channel E (LOS) at 50 dB	65

Table 5-5: Organization of plots for spatial spreading mode without cyclic delay	65

Table 5-6: Organization of plots for SGI-52, ES mode	67

Table 5-7: Organization of plots for SGI-52, SS mode	67

List of Figures

Figure 2-1: Schematic of the NS based Simulation Tool for performance studies of the 802.11n MAC	10

Figure 4-1:  Scheduled Access Period (SCAP)	57

Figure 7-1: Transmitter processing	74

Figure 7-2: Channel and impairment models	75

Figure 7-3: Receiver processing	76

Figure 7-4: AWGN performance for 1×1 Eigenvector Steering mode	77

Figure 7-5: AWGN performance for 2×2 Eigenvector Steering mode	78

Figure 7-6: AWGN performance for 3×3 Eigenvector Steering mode	79

Figure 7-7: AWGN performance for 4×4 Eigenvector Steering mode	80

Figure 7-8: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Eigenvector Steering	81

Figure 7-9: Forward link throughput distributions for channel model B, 2×2, Eigenvector Steering	82

Figure 7-10: Reverse link throughput distributions for channel model B, 2×2, Eigenvector Steering	83

Figure 7-11: Forward link packet error rate distributions for channel model B, 2×2, Eigenvector Steering	84

Figure 7-12: Reverse link packet error rate distributions for channel model B, 2×2, Eigenvector Steering	85

Figure 7-13: Throughput vs SNR and Average PER vs SNR for channel model B, 4×4, Eigenvector Steering	86

Figure 7-14: Forward link throughput distributions for channel model B, 4×4, Eigenvector Steering	87

Figure 7-15: Reverse link throughput distributions for channel model B, 4×4, Eigenvector Steering	88

Figure 7-16: Forward link packet error rate distributions for channel model B, 4×4, Eigenvector Steering	89

Figure 7-17: Reverse link packet error rate distributions for channel model B, 4×4, Eigenvector Steering	90

Figure 7-18: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Eigenvector Steering	91

Figure 7-19: Forward link throughput distributions for channel model B, 4×2, Eigenvector Steering	92

Figure 7-20: Reverse link throughput distributions for channel model B, 4×2, Eigenvector Steering	93

Figure 7-21: Forward link packet error rate distributions for channel model B, 4×2, Eigenvector Steering	94

Figure 7-22: Reverse link packet error rate distributions for channel model B, 4×2, Eigenvector Steering	95

Figure 7-23: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Eigenvector Steering	96

Figure 7-24: Forward link throughput distributions for channel model D, 2×2, Eigenvector Steering	97

Figure 7-25: Reverse link throughput distributions for channel model D, 2×2, Eigenvector Steering	98

Figure 7-26: Forward link packet error rate distributions for channel model D, 2×2, Eigenvector Steering	99

Figure 7-27: Reverse link packet error rate distributions for channel model D, 2×2, Eigenvector Steering	100

Figure 7-28: Throughput vs SNR and Average PER vs SNR for channel model D, 4×4, Eigenvector Steering	101

Figure 7-29: Forward link throughput distributions for channel model D, 4×4, Eigenvector Steering	102

Figure 7-30: Reverse link throughput distributions for channel model D, 4×4, Eigenvector Steering	103

Figure 7-31: Forward link packet error rate distributions for channel modelDB,4×4, Eigenvector Steering	104

Figure 7-32: Reverse link packet error rate distributions for channel model D, 4×4, Eigenvector Steering	105

Figure 7-33: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Eigenvector Steering	106

Figure 7-34: Forward link throughput distributions for channel model D, 4×2, Eigenvector Steering	107

Figure 7-35: Reverse link throughput distributions for channel model B, 4×2, Eigenvector Steering	108

Figure 7-36: Forward link packet error rate distributions for channel model D, 4×2, Eigenvector Steering	109

Figure 7-37: Reverse link packet error rate distributions for channel model D, 4×2, Eigenvector Steering	110

Figure 7-38: Throughput vs SNR and Average PER vs SNR for channel model E, 2×2, Eigenvector Steering	111

Figure 7-39: Forward link throughput distributions for channel model E, 2×2, Eigenvector Steering	112

Figure 7-40: Reverse link throughput distributions for channel model E, 2×2, Eigenvector Steering	113

Figure 7-41: Forward link packet error rate distributions for channel model E, 2×2, Eigenvector Steering	114

Figure 7-42: Reverse link packet error rate distributions for channel model E, 2×2, Eigenvector Steering	115

Figure 7-43: Throughput vs SNR and Average PER vs SNR for channel model E, 4×4, Eigenvector Steering	116

Figure 7-44: Forward link throughput distributions for channel model E, 4×4, Eigenvector Steering	117

Figure 7-45: Reverse link throughput distributions for channel model E, 4×4, Eigenvector Steering	118

Figure 7-46: Forward link packet error rate distributions for channel model E, 4×4, Eigenvector Steering	119

Figure 7-47: Reverse link packet error rate distributions for channel model E, 4×4, Eigenvector Steering	120

Figure 7-48: Throughput vs SNR and Average PER vs SNR for channel model E, 4×2, Eigenvector Steering	121

Figure 7-49: Forward link throughput distributions for channel model E, 4×2, Eigenvector Steering	122

Figure 7-50: Reverse link throughput distributions for channel model E, 4×2, Eigenvector Steering	123

Figure 7-51: Forward link packet error rate distributions for channel model E, 4×2, Eigenvector Steering	124

Figure 7-52: Reverse link packet error rate distributions for channel model E, 4×2, Eigenvector Steering	125

Figure 7-53: Performance in channel model B, 2×2, Eigenvector Steering, fixed rates	126

Figure 7-54: Performance in channel model D, 2×2, Eigenvector Steering, fixed rates	127

Figure 7-55: Performance in channel model E, 2×2, Eigenvector Steering, fixed rates	128

Figure 7-56: Performance in channel model B, 4×4, Eigenvector Steering, fixed rates	129

Figure 7-57: Performance in channel model D, 4×4, Eigenvector Steering, fixed rates	130

Figure 7-58: Performance in channel model E, 4×4, Eigenvector Steering, fixed rates	131

Figure 7-59: Fluorescent effect on and off in channel model D, 4×4, Eigenvector Steering, 4 spatial streams	132

Figure 7-60: Fluorescent effect on and off in channel model D, 2×2, Eigenvector Steering, 2 spatial streams	133

Figure 7-61: Impact of carrier frequency and symbol clock offset for 2×2 and 4×4, Eigenvector Steering, channel model E	134

Figure 7-62: AWGN performance for 1×1 Spatial Spreading mode	135

Figure 7-63: AWGN performance for 2×2 Spatial Spreading mode	136

Figure 7-64: AWGN performance for 3×3 Spatial Spreading mode	137

Figure 7-65: AWGN performance for 4×4 Spatial Spreading mode	138

Figure 7-66: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Spatial Spreading	139

Figure 7-67: Forward link throughput distributions for channel model B, 2×2, Spatial Spreading	140

Figure 7-68: Reverse link throughput distributions for channel model B, 2×2, Spatial Spreading	141

Figure 7-69: Forward link packet error rate distributions for channel model B, 2×2, Spatial Spreading	142

Figure 7-70: Reverse link packet error rate distributions for channel model B, 2×2, Spatial Spreading	143

Figure 7-71: Throughput vs SNR and Average PER vs SNR for channel model B, 4×4, Spatial Spreading	144

Figure 7-72: Forward link throughput distributions for channel model B, 4×4, Spatial Spreading	145

Figure 7-73: Reverse link throughput distributions for channel model B, 4×4, Spatial Spreading	146

Figure 7-74: Forward link packet error rate distributions for channel model B, 4×4, Spatial Spreading	147

Figure 7-75: Reverse link packet error rate distributions for channel model B,4×4, Spatial Spreading	148

Figure 7-76: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Spatial Spreading	149

Figure 7-77: Forward link throughput distributions for channel model B, 4×2, Spatial Spreading	150

Figure 7-78: Reverse link throughput distributions for channel model B, 4×2, Spatial Spreading	151

Figure 7-79: Forward link packet error rate distributions for channel model B, 4×2, Spatial Spreading	152

Figure 7-80: Reverse link packet error rate distributions for channel model B, 4×2, Spatial Spreading	153

Figure 7-81: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Spatial Spreading	155

Figure 7-82: Forward link throughput distributions for channel model D, 2×2, Spatial Spreading	156

Figure 7-83: Reverse link throughput distributions for channel model D, 2×2, Spatial Spreading	157

Figure 7-84: Forward link packet error rate distributions for channel model D, 2×2, Spatial Spreading	158

Figure 7-85: Reverse link packet error rate distributions for channel model D, 2×2, Spatial Spreading	159

Figure 7-86: Throughput vs SNR and Average PER vs SNR for channel model D, 4×4, Spatial Spreading	160

Figure 7-87: Forward link throughput distributions for channel model D, 4×4, Spatial Spreading	161

Figure 7-88: Reverse link throughput distributions for channel model D, 4×4, Spatial Spreading	162

Figure 7-89: Forward link packet error rate distributions for channel model D, 4×4, Spatial Spreading	163

Figure 7-90: Reverse link packet error rate distributions for channel model D,4×4, Spatial Spreading	164

Figure 7-91: Throughput vs SNR and Average PER vs SNR for channel model D, 4×2, Spatial Spreading	165

Figure 7-92: Forward link throughput distributions for channel model D, 4×2, Spatial Spreading	166

Figure 7-93: Reverse link throughput distributions for channel model D, 4×2, Spatial Spreading	167

Figure 7-94: Forward link packet error rate distributions for channel model D, 4×2, Spatial Spreading	168

Figure 7-95: Reverse link packet error rate distributions for channel model D, 4×2, Spatial Spreading	169

Figure 7-96: Throughput vs SNR and Average PER vs SNR for channel model E, 2×2, Spatial Spreading	170

Figure 7-97: Forward link throughput distributions for channel model E, 2×2, Spatial Spreading	171

Figure 7-98: Reverse link throughput distributions for channel model E, 2×2, Spatial Spreading	172

Figure 7-99: Forward link packet error rate distributions for channel model E, 2×2, Spatial Spreading	173

Figure 7-100: Reverse link packet error rate distributions for channel model E, 2×2, Spatial Spreading	174

Figure 7-101: Throughput vs SNR and Average PER vs SNR for channel model E, 4×4, Spatial Spreading	175

Figure 7-102: Forward link throughput distributions for channel model E, 4×4, Spatial Spreading	176

Figure 7-103: Reverse link throughput distributions for channel model E, 4×4, Spatial Spreading	177

Figure 7-104: Forward link packet error rate distributions for channel model E, 4×4, Spatial Spreading	178

Figure 7-105: Reverse link packet error rate distributions for channel model E,4×4, Spatial Spreading	179

Figure 7-106: Throughput vs SNR and Average PER vs SNR for channel model E, 4×2, Spatial Spreading	180

Figure 7-107: Forward link throughput distributions for channel model E, 4×2, Spatial Spreading	181

Figure 7-108: Reverse link throughput distributions for channel model E, 4×2, Spatial Spreading	182

Figure 7-109: Forward link packet error rate distributions for channel model E, 4×2, Spatial Spreading	183

Figure 7-110: Reverse link packet error rate distributions for channel model E,4×2, Spatial Spreading	184

Figure 7-111: Performance in channel model B, 2×2, Spatial Spreading, fixed rates	185

Figure 7-112: Performance in channel model D, 2×2, Spatial Spreading, fixed rates	186

Figure 7-113: Performance in channel model E, 2×2, Spatial Spreading, fixed rates	187

Figure 7-114: Performance in channel model B, 4×4, Spatial Spreading, fixed rates	188

Figure 7-115: Performance in channel model D, 4×4, Spatial Spreading, fixed rates	189

Figure 7-116: Performance in channel model E, 4×4, Spatial Spreading, fixed rates	190

Figure 7-117: Fluorescent effect on and off in channel model D, 4×4, Spatial Spreading, 3 spatial streams	191

Figure 7-118: Fluorescent effect on and off in channel model D, 2×2, Spatial Spreading, 2 spatial streams	192

Figure 7-119: Impact of carrier frequency and symbol clock offset in 2×2 and 4×4, Spatial Spreading, channel model E	193

Figure 7-120: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Spatial Spreading without Cyclic Delay	194

Figure 7-121: Forward link throughput distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay	195

Figure 7-122: Reverse link throughput distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay	196

Figure 7-123: Forward link packet error rate distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay	197

Figure 7-124: Reverse link packet error rate distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay	198

Figure 7-125: Throughput vs SNR and Average PER vs SNR for channel model D, 4×2, Spatial Spreading without Cyclic Delay	199

Figure 7-126: Forward link throughput distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay	200

Figure 7-127: Reverse link throughput distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay	201

Figure 7-128: Forward link packet error rate distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay	202

Figure 7-129: Reverse link packet error rate distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay	203

Figure 7-130: Throughput vs SNR and Average PER vs SNR for channel model E, 4×2, Spatial Spreading without Cyclic Delay	204

Figure 7-131: Forward link throughput distributions for channel model E, 4×2, Spatial Spreading without Cyclic Delay	205

Figure 7-132: Reverse link throughput distributions for channel model E, 4×2, Spatial Spreading without Cyclic Delay	206

Figure 7-133: Forward link packet error rate distributions for channel model E, 4×2, Spatial Spreading without Cyclic Delay	207

Figure 7-134: Reverse link packet error rate distributions for channel model E,4×2, Spatial Spreading without Cyclic Delay	208

Figure 7-135: AWGN performance for 1×1 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers	209

Figure 7-136: AWGN performance for 2×2 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers	210

Figure 7-137: AWGN performance for 3×3 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers	211

Figure 7-138: AWGN performance for 4×4 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers	212

Figure 7-139: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Eigenvector Steering, SGI-52	213

Figure 7-140: Forward link throughput distributions for channel model B, 2×2, Eigenvector Steering, SGI-52	214

Figure 7-141: Reverse link throughput distributions for channel model B, 2×2, Eigenvector Steering, SGI-52	215

Figure 7-142: Forward link packet error rate distributions for channel model B, 2×2, Eigenvector Steering, SGI-52	216

Figure 7-143: Reverse link packet error rate distributions for channel model B, 2×2, Eigenvector Steering, SGI-52	217

Figure 7-144: Throughput vs SNR and Average PER vs SNR for channel model B, 4×4, Eigenvector Steering, SGI-52	218

Figure 7-145: Forward link throughput distributions for channel model B, 4×4, Eigenvector Steering, SGI-52	219

Figure 7-146: Reverse link throughput distributions for channel model B, 4×4, Eigenvector Steering, SGI-52	220

Figure 7-147: Forward link packet error rate distributions for channel model B, 4×4, Eigenvector Steering, SGI-52	221

Figure 7-148: Reverse link packet error rate distributions for channel model B, 4×4, Eigenvector Steering, SGI-52	222

Figure 7-149: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Eigenvector Steering, SGI-52	223

Figure 7-150: Forward link throughput distributions for channel model B, 4×2, Eigenvector Steering, SGI-52	224

Figure 7-151: Reverse link throughput distributions for channel model B, 4×2, Eigenvector Steering, SGI-52	225

Figure 7-152: Forward link packet error rate distributions for channel model B, 4×2, Eigenvector Steering, SGI-52	226

Figure 7-153: Reverse link packet error rate distributions for channel model B, 4×2, Eigenvector Steering, SGI-52	227

Figure 7-154: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Eigenvector Steering, SGI-52	228

Figure 7-155: Forward link throughput distributions for channel model D, 2×2, Eigenvector Steering, SGI-52	229

Figure 7-156: Reverse link throughput distributions for channel model D, 2×2, Eigenvector Steering, SGI-52	230

Figure 7-157: Forward link packet error rate distributions for channel model D, 2×2, Eigenvector Steering, SGI-52	231

Figure 7-158: Reverse link packet error rate distributions for channel model D, 2×2, Eigenvector Steering, SGI-52	232

Figure 7-159: Throughput vs SNR and Average PER vs SNR for channel model D, 4×4, Eigenvector Steering, SGI-52	233

Figure 7-160: Forward link throughput distributions for channel model D, 2×2, Eigenvector Steering, SGI-52	234

Figure 7-161: Reverse link throughput distributions for channel model D, 4×4, Eigenvector Steering, SGI-52	235

Figure 7-162: Forward link packet error rate distributions for channel model D, 4×4, Eigenvector Steering, SGI-52	236

Figure 7-163: Reverse link packet error rate distributions for channel model D, 4×4, Eigenvector Steering, SGI-52	237

Figure 7-164: Throughput vs SNR and Average PER vs SNR for channel model D, 4×2, Eigenvector Steering, SGI-52	238

Figure 7-165: Forward link throughput distributions for channel model D, 4×2, Eigenvector Steering, SGI-52	239

Figure 7-166: Reverse link throughput distributions for channel model D, 4×2, Eigenvector Steering, SGI-52	240

Figure 7-167: Forward link packet error rate distributions for channel model D, 4×2, Eigenvector Steering, SGI-52	241

Figure 7-168: Reverse link packet error rate distributions for channel model D, 4×2, Eigenvector Steering, SGI-52	242

Figure 7-169: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Spatial Spreading, SGI-52	243

Figure 7-170: Forward link throughput distributions for channel model B, 2×2, Spatial Spreading, SGI-52	244

Figure 7-171: Reverse link throughput distributions for channel model B, 2×2, Spatial Spreading, SGI-52	245

Figure 7-172: Forward link packet error rate distributions for channel model B, 2×2, Spatial Spreading, SGI-52	246

Figure 7-173: Reverse link packet error rate distributions for channel model B, 2×2, Spatial Spreading, SGI-52	247

Figure 7-174: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Spatial Spreading, SGI-52	248

Figure 7-175: Forward link throughput distributions for channel model D, 2×2, Spatial Spreading, SGI-52	249

Figure 7-176: Reverse link throughput distributions for channel model D, 2×2, Spatial Spreading, SGI-52	250

Figure 7-177: Forward link packet error rate distributions for channel model D, 2×2, Spatial Spreading, SGI-52	251

Figure 7-178: Reverse link packet error rate distributions for channel model D, 2×2, Spatial Spreading, SGI-52	252

Figure 7-179: Throughput vs SNR and Average PER vs SNR for channel model E, 2×2, Spatial Spreading, SGI-52	253

Figure 7-180: Forward link throughput distributions for channel model E, 2×2, Spatial Spreading, SGI-52	254

Figure 7-181: Reverse link throughput distributions for channel model E, 2×2, Spatial Spreading, SGI-52	255

Figure 7-182: Forward link packet error rate distributions for channel model E, 2×2, Spatial Spreading, SGI-52	256

Figure 7-183: Reverse link packet error rate distributions for channel model E, 2×2, Spatial Spreading, SGI-52	257

Figure 7-184: Performance of Eigenvector Steering, 2×2, channel model B,  fixed rates, Short Guard Interval, 52 subcarriers	258

Figure 7-185: Performance of Eigenvector Steering, 2×2, channel model D,  fixed rates, Short Guard Interval, 52 subcarriers	259

Figure 7-186: Performance of Eigenvector Steering, 4×4, channel model B, fixed rates, Short Guard Interval, 52 subcarriers	260

Figure 7-187: Performance of Eigenvector Steering, 4×4, channel model D, fixed rates, Short Guard Interval, 52 subcarriers	261

Figure 7-188: Performance of Eigenvector Steering, 4×4, channel model E, fixed rates, Short Guard Interval, 52 subcarriers	262

Figure 7-189: Simulated and predicted PERs, channel model B, 4×4, Eigenvector Steering, 1, 2, and 3 active spatial streams	263

Figure 7-190: Simulated and predicted PER distributions for channel model B, 4×4, Eigenvector Steering, 2 active spatial streams	264

Figure 7-191: Simulated and predicted PERs, channel model D, 4×4, Eigenvector Steering, 2 and 3 active spatial streams	265

Figure 7-192: Simulated and predicted PERs, channel model B, 4×4, Spatial Spreading, 1, 2, and 3 active spatial streams	266

Figure 7-193: Simulated and predicted PERs, channel model E, 4×4, Spatial Spreading, 1, 2, and 3 active spatial streams	267

# Introduction

In this document we present an overview of the simulation methodologies used to assess the performance of Qualcomm’s MAC and PHY layer proposals for 802.11n. Results of extensive simulations that satisfy the requirements of the 802.11n Functional Requirements and Comparison Criteria documents are also presented. The document consists of four main sections.  In Section 2, we describe the simulation tool used for system level performance simulations. Simulation results obtained with this tool are presented in Section 3. In Section 4, we give a description of the methodology adopted for the physical layer simulations.  In Section 5, various PHY related simulation results are presented. The system level simulations are configured with PHY dependent parameters in order to simulate physical layer events such as packet errors. These parameters represent an abstraction of the physical layer, and are obtained from a specific set of physical layer simulations described in Section 6.

## Summary of System Simulation Results

System simulation results show that 802.11e access mechanisms (HCCA and EDCA) are insufficient to fully exploit the physical layer data rates that are unleashed by physical layer MIMO technology. Even for Scenario 1 with only 11 STAs, HCCA and EDCA prove to be a bottleneck. Scenarios 4 and 6 have many more STAs and consequently even lower MAC efficiency. For Scenario 1, as physical layer data rates are increased going from 2x2 to 4x4 operation, the MAC efficiency of HCCA (enhanced with Frame Aggregation) decreases from around 0.5 to 0.4, while the MAC Efficiency of EDCA (enhanced with Frame Aggregation) decreases from 0.45 to 0.23. Moreover, the requirements of STA-AP QoS flows (in particular, VoIP) cannot be satisfied with EDCA.

Using the proposed ACF (SCHED frame and SCAP), MAC efficiency of 0.6-0.7 is achieved for Scenario 1. For Scenarios 4 and 6, the MAC efficiency is above 0.58 and 0.65, respectively. In Scenario 1, the low latency scheduling mechanism of ACF permits us to achieve a tighter delay bound on the video and audio streaming flows (less than 50 ms) and greater range for the HDTV flows (up to 25 m).

System simulation results demonstrate that the proposed MAC and PHY enhancements achieve robust throughput and range. In the AP-STA (Scenario 16) 100 Mbps throughput above the MAC is achieved in a bandwidth of 20 MHz at the following distance:

- 29 m for 2x2, 5.25 GHz

- 40 m for 2x2, 2.4 GHz

- 47 m for 4x4, 5.25 GHz

- 75 m for 4x4, 2.4 GHz

This corresponds to a physical layer spectral efficiency of approximately 5.85 bps/Hz or equivalently, a MAC efficiency of 0.85.

While remaining backward compatible with 802.11a and 802.11g, and introducing measured enhancements to features already included in 802.11e and 802.11h, the 802.11n MAC is shown to deliver BSS throughput in excess of 100 Mbps.

## Summary of PHY Simulation Results

The physical layer results clearly demonstrate that highly reliable, high throughput operation can be achieved using MIMO eigenvector steering techniques in conjunction with the resources of the existing 802.11 physical layer, such as OFDM symbol design, convolutional coding, and PLCP header design.  An important component of the enhancements that make such MIMO techniques work in the 802.11 framework is the addition of provisions for adaptive rate selection.

MIMO Spatial Spreading provides somewhat diminished performance compared to the eigenvector steering approach.  There are two primary sources of performance degradation.  First is the limitation of the MMSE receiver processing used in this case, which can be shown to suffer a capacity degradation of about 20% relative to the use of eigenvector steering on the same channel.  The second is the difficulty that the convolutional decoder has in dealing with the high SNR variance across an OFDM symbol that is transmitted using spatial spreading and related techniques.

In spite of these shortcomings, Spatial Spreading is a useful and necessary tool for transmitting broadcast/multicast data and for fallback when a transmitting STA does not have enough information to use eigenvector steering.

Adaptive rate selection is a critical component in achieving sustained high throughput operation in any MIMO system, whether transmission is done using eigenvector steering or related approaches, or it is done using spatial spreading or other similar approaches.   This is made clear by the high sustained rates at low PERS that are shown in section Error: Reference source not found.

Results are also shown for the use expanded OFDM symbols that use 52 data subcarriers instead of the standard 48, and a shortened guard interval of 400 ns.  These results demonstrate clear improvements in throughput, in spite of increased vulnerability to time dispersive channels and timing errors.

# System Simulation Methodology

The system level simulations were performed using NS [1].  NS is a widely used simulation tool for the study of communication network systems.  It is an event driven simulator implemented using object oriented methods.  NS captures the layered structure of a communication network.   The function of each layer is implemented within separate objects providing increased flexibility for introducing new protocols at a given layer while reusing existing modules from other layers.  The availability of several flavors of TCP and an implementation of the 802.11 MAC in the simulation make the tool particularly attractive for the purpose of studying 802.11n.

Figure 2-1: Schematic of the NS based Simulation Tool for performance studies of the 802.11n MAC

In Figure 2-1 we present a block diagram to illustrate the NS objects and flow of data traffic in the simulation of data traffic between two nodes.  Each node has a protocol stack that includes a transport layer, network layer and the MAC layer.  A typical simulation scenario consists of several such nodes.  Each of these nodes is configured to receive and/or send one or more streams of traffic using either TCP or UDP.  A single PHY layer object and a single MIMO propagation module are shared by all the nodes in the simulation.  Existing NS modules are used to simulate the operation of the TCP and UDP protocol.

The version of TCP used in the simulations is TCP SACK with delayed ACK.  This version of TCP is based on TCP Reno and therefore includes fast retransmit, fast recovery. In addition, SACK TCP allows the transmission of delayed acknowledgements, thus allowing multiple TCP segments to be recovered with the receipt of a single ACK packet. SACK TCP is used in the current versions of Windows and Linux.

Due to the high data rate (>100Mb/s) available with 802.11n and delays on the order of hundreds of milliseconds, high application layer throughput can be obtained only if the TCP window is allowed to grow to a value larger than 65536 octets.  In the simulations the TCP scale option [4] is used.  This option allows the TCP windows to grow beyond 65KB.  The scale option is available in both Linux and Windows operating systems.  Throughout the system simulations, all TCP sources are policed using a leaky bucket.  The leaky bucket limits the rate of packets sent out of the TCP source to the desired rate for the TCP flow specified in the scenario description.  The parameters used for the TCP sources are:

- Maximal TCP window size: 500 TCP segments.

- Delay Ack timer:  20 ms.

We developed a module for the proposed 802.11n MAC based on an existing 802.11 MAC implementation for NS that includes the EDCA operation [2].  We added features such as the Block ACK, ARQ operation and the newly proposed 802.11n framing.  We also implemented the proposed scheduled mode of operation within the MAC.  The 802.11n module can be configured to operate as an AP or a station.  The MAC module operates in conjunction with the MIMO Propagation Module (MPM) to determine the instantaneous transmit rates and packet error rates.

In addition to the MAC layer, the operation of the MIMO based physical layer has also been implemented in the simulation tool.   The “MIMO Propagation Module” implements the channel models specified by 802.11 TGn [3].  The MPM is configured using the coordinates of each node and the channel model (e.g., Channel B, Channel D, etc) on each node pair.  Once the MPM is configured, it simulates the time evolution of the channel between each node pair.  The MPM can be queried by the MAC to obtain the instantaneous SNR (and data rate) on each MIMO spatial stream for a given node pair.  The MPM can also be queried for the instantaneous packet error rate for a packet based on three properties of the packet: SNR in each spatial stream, MIMO transmission mode (i.e., Spatial Spreading or Eigenvector Steering), and modulation scheme used for the packet.  The MPM uses a table look-up to determine the packet error probability based on the received SNR, modulation scheme and packet size.  For a detailed description of the error model see Section Error: Reference source not found.

# System Simulation Results

We report results for the simulation scenarios defined in the Usage Models document 11-03-0802-19-000n-usage-models.

## Simulation Conditions That Are Fixed

The following parameters are fixed for all system simulation results.

- Bandwidth: 20 MHz.

- Frame Aggregation

  - No Header Compression

- Fragmentation Threshold: 100 kB

- Delayed Block Ack

  - No Compressed Block Ack

- Adaptive Rate Control

- Adaptive Mode Control between ES and SS

## Simulation Conditions That Are Varied

The following parameters are varied. Results are provided for different combinations of these parameters.

- Bands:

  - 2.4 GHz

  - 5.25 GHz

- MIMO:

  - 2x2: All STAs with 2 antennas

  - 4x4: All STAs with 4 antennas

  - Mixed:

    - Scenario 1: the AP and the HDTV/SDTV displays are assumed to have 4 antennas; all other STAs have 2 antennas.

    - Scenario 6: AP and all STAs, except VoIP terminals have 4 antennas; VoIP terminals have 2 antennas.

- OFDM symbols

  - Standard: 0.8 μs Guard Interval, 48 data subcarriers

  - SGI-EXP: 0.4 μs Shortened Guard Interval, 52 data subcarriers. SGI-EXP symbols are not used for Scenario 6 (Hot Spot) due to the larger STA-STA distances.

- Access Mechanisms

  - ACF (SCHED)

  - HCF (Poll)

  - EDCA with additional AC for Block Ack

## EDCA Parameters

The EDCA Parameters used in the simulations are:

| AC | CW min | CW max | AIFS |  |
| --- | --- | --- | --- | --- |
| 0 | 127 | 1023 | 2 | BlockAck/VoIP |
| 1 | 127 | 1023 | 4 | Video HDTV |
| 2 | 127 | 1023 | 8 | Other QoS |
| 3 | 127 | 1023 | 10 | Best effort |

Table 3-1  EDCA Parameters

## Additional Scenarios

Scenario 1 HT is an extension of Scenario 1:

- Additional FTP flow of up to 130 Mbps at 15.6 m from the AP for 2×2.

Scenario 1 EXT is an extension of Scenario 1:

- Additional FTP flow of up to 130 Mbps at 15.6 m from the AP for 2×2.

- Maximum delay requirement for all video/audio streaming flows is decreased from 100/200 ms to 50 ms.

- Two HDTV flows are moved from 5 m from the AP, to 25 m from the AP.

Scenario 6 EXT is an extension of Scenario 6:

- One FTP flow of 2 Mbps at 31.1 m from the AP is increased up to 80 Mbps for 4x4.

## Summary and Observations from System Simulation Results

### Aggregate Throughput Results

NA: Not available

Table 3-2 Aggregate Throughput Results for Scenarios 1, 1 HT, 1 EXT, 4, 6 and 6 EXT for 2x2, 4x4 and Mixed

Observations:

- ACF provides highest total throughput compared to HCF and EDCA.

- ACF satisfies all QoS flows for all Scenarios when SGI-EXP symbols are used.

  - Only in the case standard symbols are used (giving reduced throughput) at 5.25 GHz (giving reduced range), the PLR requirement of gaming flows is not satisfied.

- No increase in throughput for EDCA with 4x4 compared to 2x2.

- Scenario 1 EXT imposes stringent delay (less than 50 ms for streaming) and range (HDTV flows at 25 m) requirements on QoS flows. When 2x2 is used, one or two QoS flows are not satisfied. In the Mixed case, by equipping the AP and the HDTV and SDTV displays with 4 antennas, all QoS flows except the gaming flow are satisfied with an almost 50% increase in total throughput compared to 2x2.

- In Scenario 4, throughput achieved is over 100 Mbps with 2x2 and almost 200 Mbps with 4x4.

- Scenario 6 EXT Mixed case (mixture of 4-antenna and 2-antenna STAs) gives higher TCP throughput than the 4x4 case. This is because there is more time available for TCP flows due to the reduced training sequence overhead for VoIP STAs with 2 antennas compared to VoIP STAs with 4 antennas. Scenario 6 EXT has 30 VoIP flows.

### MAC Efficiency

Table 3-3 MAC Efficiency Results

Observations:

As defined, MAC Efficiency is meaningful only when the offered load for a scenario exceeds the carried load and there is always backlogged traffic at some flow. In the above table, the MAC Efficiency numbers are shown in red for the cases where the medium is forced idle due to no backlog. These numbers are not meaningful.

- For 2x2, the MAC Efficiency for ACF is between 0.65-0.7.

- For 2x2, the MAC Efficiency for HCF and EDCA is around 0.5.

- For 4x4, the MAC Efficiency for HCF and EDCA reduces to 0.4 and 0.2, respectively. ACF manages to sustain a MAC Efficiency around 0.6, even with 4x4.

### Summary of QoS Flows Satisfied

Table 3-4 Summary of QoS Flows Satisfied

Observations:

- Scenario 1 EXT imposes stringent delay (less than 50 ms for streaming) and range (HDTV flows at 25 m) requirements on QoS flows. When 2x2 is used, one or two QoS flows are not satisfied. In the Mixed case, by equipping the AP and the HDTV and SDTV displays with 4 antennas, all QoS flows except the gaming flow are satisfied with an almost 50% increase in total throughput compared to 2x2.

- More QoS flows are satisfied with HCF than with EDCA. However, ACF is required to address stringent QoS requirements.

- QoS for uplink EDCA VoIP flows is not satisfied.

- All QoS Flows are satisfied for Scenario 4.

### Summary of Non-QoS Throughput Achieved

Table 3-5 Summary of Non-QoS Throughput Achieved

Observations:

- Scenario 6 EXT Mixed case (mixture of 4-antenna and 2-antenna STAs) gives higher TCP throughput than the 4x4 case. This is because there is more time available for TCP flows due to the reduced training sequence overhead for VoIP STAs with 2 antennas compared to VoIP STAs with 4 antennas. Scenario 6 EXT has 30 VoIP flows.

### Rate versus Range

The plots for Channel Model B and Channel Model D are roughly similar. A throughput above the MAC of 100 Mbps is achieved at:

- 29 m for 2x2, 5.25 GHz

- 40 m for 2x2, 2.4 GHz

- 47 m for 4x4, 5.25 GHz

- 75 m for 4x4, 2.4 GHz

### Legacy Sharing

ACF efficiently shares the medium with legacy STAs. With equal-time sharing of the medium, the legacy STA in CC15 achieves approximately half the throughput when the HT STA is present. The HT STA in the presence of the legacy STA achieves less than half the throughput compared to the case when the legacy STA is not present. After the medium is released by the legacy STA, the HT STA needs to ramp up its rate by reacquiring MIMO channel estimate.

When equal-time sharing with a legacy STA, the throughput achieved by the HT STA exceeds 60 Mbps for 2x2 and 100 Mbps for 4x4.

## What the Columns in the Following Sections Mean

Detailed results are in the following sections. For each flow we show:

- Application

- Distance: from the Transmitter STA to Receiver STA

- Offered Load (Mbps)

- Delay Tolerance (seconds). For non-QoS (TCP) there is no delay bound specified and it is shown as ‘-1’ in the Tables.

- Achieved Throughput (Mbps) for the flow. Metric 2 as defined for CC 20.

- Mean Delay (ms) for packets from the flow.

- Packet Loss Rate (PLR) achieved. When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

## Scenario 1 (Residential BSS) Results

The following results for Scenario 1 are provided.

- Scenario 1:

- 2×2, 2.4 GHz, SGI-EXP, ACF

- 2×2, 2.4 GHz, SGI-EXP, HCF

- 2×2, 2.4 GHz, SGI-EXP, EDCA

- Scenario 1:

- 2×2, 5.25 GHz, SGI-EXP,  ACF

- 2×2, 5.25 GHz, SGI-EXP, HCF

- 2×2, 5.25 GHz, SGI-EXP, EDCA

- Scenario 1:

- 4×4, 5.25 GHz, SGI-EXP,  ACF

- 4×4, 5.25 GHz, SGI-EXP, HCF

- 4×4, 5.25 GHz, SGI-EXP, EDCA

- Scenario 1:

- 2×2, 5.25 GHz, Standard,  ACF

- 4×4, 5.25 GHz, Standard, ACF

### Comparison of ACF, HCF and EDCA for Scenario 1 (2x2, 2.4 GHz)

#### Scenario 1: 2x2, 2.4 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 2x2, 2.4 GHz, SGI-EXP, HCF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 2x2, 2.4 GHz, SGI-EXP, EDCA

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

### Comparison of ACF, HCF and EDCA for Scenario 1 (2x2, 5.25 GHz)

#### Scenario 1: 2x2, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 2x2, 5.25 GHz, SGI-EXP, HCF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 2x2, 5.25 GHz, SGI-EXP, EDCA

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

### Comparison of ACF, HCF and EDCA for Scenario 1 (4x4, 5.25 GHz)

#### Scenario 1: 4x4, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 4x4, 5.25 GHz, SGI-EXP, HCF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 4x4, 5.25 GHz, SGI-EXP, EDCA

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

### Comparison of 2x2 and 4x4 for Scenario 1 (Standard symbols)

#### Scenario 1: 2x2, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1: 4x4, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

## Extended Scenario 1 Results

- Scenario 1 HT:

- 2×2, 5.25 GHz, SGI-EXP, ACF

- 4×4, 5.25 GHz, SGI-EXP, ACF

- Scenario 1 EXT:

- 2×2, 5.25 GHz, SGI-EXP, ACF

- Mixed, 5.25 GHz, SGI-EXP, ACF

- 4×4, 5.25 GHz, SGI-EXP, ACF

- Mixed: The AP and the HDTV/SDTV displays are assumed to have 4 antennas, all other STAs have 2 antennas.

- Scenario 1 HT demonstrates that higher throughput can be achieved than that specified for Scenario 1 in the usage models.

- Scenario 1 EXT shows an extreme case where the delay and range requirements are made more stringent.

### Scenario 1 HT

#### Scenario 1 HT: 2x2, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1 HT: 4x4, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

### Scenario 1 EXT

#### Scenario 1 EXT: 2x2, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1 EXT: Mixed, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 1 EXT: 4x4, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

## Scenario 4 (Large Enterprise) Results

List of results for Scenario 4:

- Scenario 4:

- 2×2, 2.4 GHz, SGI-EXP, ACF

- 4×4, 2.4 GHz, SGI-EXP, ACF

- Scenario 4:

- 2×2, 5.25 GHz, SGI-EXP, ACF

- 4×4, 5.25 GHz, SGI-EXP, ACF

### Scenario 4: 2.4 GHz

#### Scenario 4: 2x2, 2.4 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 4: 4x4, 2.4 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

### Scenario 4: 5.25 GHz

#### Scenario 4: 2x2, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 4: 4x4, 5.25 GHz, SGI-EXP, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

## Scenario 6 (Hot Spot) Results

List of results for Scenario 6:

- Scenario 6:

- 2×2, 5.25 GHz, Standard,  ACF

- 2×2, 5.25 GHz, Standard, HCF

- 2×2, 5.25 GHz, Standard, EDCA

- Scenario 6:

- 2×2, 5.25 GHz, Standard,  ACF

- Mixed, 5.25 GHz, Standard,  ACF

- 4×4, 5.25 GHz, Standard, ACF

### Comparison of ACF, HCF and EDCA for Scenario 6

#### Scenario 6: 2x2, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 6: 2x2, 5.25 GHz, Standard, HCF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 6: 2x2, 5.25 GHz, Standard, EDCA

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

### Scenario 6: Comparison of 2x2, Mixed and 4x4

#### Scenario 6: 2x2, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 6: Mixed, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 6: 4x4, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

## Scenario 6 EXT Results

List of results for Scenario 6:

- Scenario 6 EXT:

- 2x2, 5.25 GHz, Standard,  ACF

- Mixed, 5.25 GHz, Standard,  ACF

- 4x4, 5.25 GHz, Standard,  ACF

### Scenario 6 EXT: Comparison of 2x2, Mixed and 4x4

#### Scenario 6 EXT: 2x2, 5.25 GHz, Standard, ACF

#### Scenario 6 EXT: Mixed, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

#### Scenario 6 EXT: 4x4, 5.25 GHz, Standard, ACF

When the achieved PLR exceeds the permitted loss rate for the application it is shown in red.

## Scenario 16 (Rate versus Range) Results

These results are for 20 MHz bandwidth, using SGI-EXP OFDM symbols, i.e., shortened guard interval, 52 sub-carriers.

### Channel Model B

### Channel Model D

## Results for Scenarios 17, 18, 19 (Legacy Sharing)

List of results for Scenario 17, 18, 19:

# Physical Layer Simulation Methodology

The proposed 802.11n PHY layer employs MIMO-OFDM with up to four antennas at both the transmitter and receiver. This enables a maximum of four spatial streams to be transmitted simultaneously in the same bandwidth on a single link. Two approaches to spatial multiplexing are proposed, referred to as Eigenvector Steering (ES) and Spatial Spreading (SS), respectively. The proposed PHY layer is described in detail in [4]. This section contains a description of the methodology used to simulate the proposed PHY layer.  In Section 5, we present results of extensive simulations that characterize the performance of both modes of operation.

## Simulation Methodology

In order to obtain performance metrics such as average PHY throughput and packet error rate (PER), the exchange of multiple PPDUs on a single link between two entities (e.g., an AP and a STA) is simulated. The basic time unit during which this exchange takes place is referred to as the Scheduled Access Period (SCAP) [4]. As shown in , the duration of the SCAP is 2.048 ms, and the SCAP is divided into a forward link (FL) segment and a reverse link (RL) segment. In the simulations, a single 1000-byte PPDU is transmitted in each of the FL and RL segments, in what is referred to as “FL data” and “RL data”, respectively.

To allow realistic simulation of channel estimation and rate adaptation in a time-varying channel, a sequence of two or three SCAPs is simulated, depending on whether Spatial Spreading or Eigenvector Steering is employed. Performance metrics such as average throughput and average packet error rate are obtained by simulating a large number of such sequences, each with its own independent channel realization. However, in order to obtain distributions of throughput and packet error rate, a large number of sequences may be simulated with each single channel realization such that a throughput value and a packet error rate may be associated with each realization. Each channel realization provides multiple snapshots of the channel, where the Doppler process evolves from snapshot to snapshot, capturing the time-variance of the channel. Assuming that the channel is stationary throughout the duration of each PPDU, three snapshots are needed per SCAP. In our simulations these snapshots are taken at t0=0 ms, t1=0.5 ms and t2=1.5 ms. The channel realizations are generated in accordance with the TGn channel model document [3] and are implemented as time-varying tapped delay lines. In the following discussion, we assume an AP-centric scenario where FL transmission refers to transmissions from the AP to a STA, and RL transmission refers to transmissions from the STA to the AP. The simulation approach is described below for Eigenvector Steering and Spatial Spreading, respectively.

Figure 4-2:  Scheduled Access Period (SCAP)

## Eigenvector Steering Transmission Mode

In simulations of the Eigenvector Steering transmission mode, channel estimation and rate adaptation take place over three SCAPs (6.144 ms). In this mode, the transmitters at each end of the link have sufficient information about the channel to compute optimum steering vectors.

- First SCAP:

  - STA receives Common MIMO Training Sequence from AP and computes channel estimate at t0.

  - STA sends steered Dedicated MIMO Training Sequence on RL at t2.

  - AP receives steered training sequence and computes estimates of its receive and transmit steering vectors.

- Second SCAP:

  - STA receives Common MIMO Training Sequence from AP and computes channel estimate at (2.048 ms + t0).

  - AP sends steered Dedicated MIMO Training Sequence on FL at (2.048 ms + t1). Based on the received steered training sequence, the STA makes FL rate selection and computes estimates of its receive vectors.

  - STA sends steered Dedicated MIMO Training Sequence on RL at (2.048 ms + t2 ) with FL rate selection. Based on the received steered training sequence, the AP makes RL rate selection and computes estimates of its receive and transmit steering vectors.

- Third SCAP:

  - STA receives Common MIMO Training Sequence from AP and computes channel estimate at (4.096 ms + t0).

  - AP sends steered Dedicated MIMO Training Sequence followed by data PPDU + RL rate selection on FL at (4.096 ms + t1). FL data rates selected in second SCAP. The STA receives the PPDU with receive steering vectors computed from the steered training sequence.

  - STA sends steered Dedicated MIMO Training Sequence followed by PPDU on RL at (4.096 ms + t2). RL data rates selected in second SCAP. The AP receives the PPDU with receive steering vectors computed from the steered training sequence.

## Spatial Spreading Transmission Mode

In simulations of the Spatial Spreading transmission mode, channel estimation and rate adaptation take place over two SCAPs (4.096 ms). In this mode the transmitters do not have sufficient channel information to compute optimum steering vectors.

- First SCAP:

  - STA receives Common MIMO Training Sequence from AP and computes channel estimate at t0.  Based on the received training sequence, the STA makes FL rate selection.

  - STA sends unsteered Dedicated MIMO Training Sequence on RL at t2 with FL rate selection. Based on the received training sequence, the AP makes RL rate selection.

- Second SCAP:

  - STA receives Common MIMO Training Sequence from AP and computes receive vectors based on channel estimate at (2.048 ms + t0).

  - AP sends data PPDU + RL rate selection on FL at (2.048 ms + t1) based on rate choice made in first SCAP. The STA’s receive vectors are based on the previously received training sequence.

  - STA sends unsteered Dedicated MIMO Training Sequence followed by data PPDU on RL at (2.048 ms + t2 ) with data rates selected in first SCAP. The AP’s receive vectors are based on the received training sequence.

## Common Simulation Conditions

The PHY simulator consists of the functional blocks and impairment models depicted in Figure 7-3, Figure 7-4, and Figure 7-5.  Figure 7-3 shows the transmitter processing, Figure 7-4 diagrams the channel and impairment models, and Figure 7-5 illustrates the receiver processing.  The impairment models comply with the requirements set forth by the 802.11n Comparison Criteria document [5].

The following common simulation conditions were adopted for simulations with both Eigenvector Steering and Spatial Spreading:

- Antenna configurations simulated: NAP × NSTA = 1×1, 2×2, 3×3, 4×4, and 4×2

- 802.11a, g OFDM structure (48 data subcarriers, 4 pilot tones, 12 guard tones, symbol duration: 4 μs including guard interval)

- Extended OFDM symbol with shortened guard interval (SGI) of 0.4 μs and 52 data subcarriers

- Data rate per spatial stream ranging from 6 Mbps to 101.11 Mbps

- 802.11n channel models B, D and E, as well as Fourier (AWGN) channel

- 802.11n impairment models:

  - Time-domain channel simulator with 5× oversampling rate (Ts=10 ns)

  - Rapp nonlinear power amplifier model (IM1):

    - Total Tx power = 17 dBm;  Psat = 25 dBm

    - Backoff: 11 dB per PA with NTx=2;  12.77 dB per PA with NTx=3;  14 dB per PA with NTx=4

  - Carrier frequency offset : -13.675 PPM and +/-40 PPM (IM2)

  - Sampling clock frequency offset: -13.675 PPM and +/-40 PPM (IM2)

  - Phase noise at both transmitter and receiver (IM4)

- Targeted packet error rate (PER) performance: mean PER <= 1%

- PPDU size: 1000 bytes

The simulations are all baseband simulations, and for the Eigenvector Steering transmission mode it is assumed that calibration has been performed, ensuring that the FL and RL channels are reciprocal, i.e., the FL and RL channel matrices have the following relationship HFL=HRLT.

Acquisition is performed on a per-packet basis, i.e., the MIMO Training Sequences are all  preceded by a preamble consisting of 10 short and 2 long training symbols similar to the 802.11a/g preamble. Carrier frequency and symbol clock offsets are estimated and compensated for in both the MIMO Training Sequence and data portions of the received packets. Hence, in the simulations, acquisition is performed three times per SCAP, i.e., at t0, t1, and t2.

The rate adaptation algorithm assigns data rates on a per-stream basis, selecting from the set of data rates listed in Table 4-6. In the simulations discussed here rate #10a is unavailable and 256-QAM is used whenever supported by the channel conditions. The rate adaptation algorithm makes rate decisions based on the SNRs per subcarrier per stream computed from either the Common MIMO Training Sequence or the Dedicated MIMO Training Sequence.

| Rate # | Bits per symbol | Code rate | Modulation format | Data rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 48 subcarriers | 52 subcarriers + SGI |
| 0 | 0.5 | 1/2 | BPSK | 6 | 7.22 |
| 1 | 0.75 | 3/4 | BPSK | 9 | 10.83 |
| 2 | 1.0 | 1/2 | QPSK | 12 | 14.44 |
| 3 | 1.5 | 3/4 | QPSK | 18 | 21.67 |
| 4 | 2.0 | 1/2 | 16 QAM | 24 | 28.89 |
| 5 | 2.5 | 5/8 | 16 QAM | 30 | 36.11 |
| 6 | 3.0 | 3/4 | 16 QAM | 36 | 43.33 |
| 7 | 3.5 | 7/12 | 64 QAM | 42 | 50.56 |
| 8 | 4.0 | 2/3 | 64 QAM | 48 | 57.78 |
| 9 | 4.5 | 3/4 | 64 QAM | 54 | 65.00 |
| 10a | 5.0 | 5/6 | 64 QAM | 60 | 72.22 |
| 10b | 5.0 | 5/8 | 256 QAM | 60 | 72.22 |
| 11 | 6.0 | 3/4 | 256 QAM | 72 | 86.67 |
| 12 | 7.0 | 7/8 | 256 QAM | 84 | 101.11 |

Table 4-6: Data Rates

# Physical Layer Simulation Results

Simulation results generated according to the requirements of Comparison Criteria 59, 67, and 67.2 are presented below. The performance results are given in terms of

    - average PHY throughput as a function of SNR,

    - throughput statistics,

    - average PER as a function of  SNR, and

    - PER statistics.

Results are provided for both forward and reverse links with Eigenvector Steering and Spatial Spreading modes of operation, respectively. SNR is given in terms of total Es/N0 per receiver. Note that Es reflects the captured energy in the received 3.2 μs OFDM symbol, exclusive of the guard interval.

## Eigenvector Steering Transmission Mode

### CC59 – AWGN PER Performance

The PER vs Es/N0 performance in AWGN was simulated with the following system configurations: NAP × NSTA = 1×1, 2×2, 3×3, and 4×4. Referring to Table 4-6, the following set of data rates was used: {0, 3, 6, 9, 12}, corresponding to data rates of  where Mbps. For each point, a minimum of 100 packet errors were collected. Perfect synchronization and channel estimation was assumed.

Figure 7-6, Figure 7-7, Figure 7-8, and Figure 7-9 show AWGN performance in Eigenvector Steering mode for 1×1, 2×2, 3×3, and 4×4 configurations, respectively.

### CC67 – PER Performance in Non AWGN Channels

Average physical layer data throughput:

Plots illustrating the average physical layer data throughput as a function of SNR per receiver achieved with adapative rate selection are provided. Also shown on the same figures are the corresponding average packet error rates.  In addition, throughput distributions and packet error rate distributions are shown for each SNR point, each link, and each configuration.  Results are given for the following system configurations:  NAP × NSTA = 2×2, 4×4, and 4×2.  Table 5-7 provides the figure numbers for each of the plots.

In obtaining the results shown in the figures, 100 channel realizations were simulated for each SNR point, and multiple packets were transmitted per channel realization (stopping criterion: 10 packet errors or 400 independent 3-SCAP sequences simulated per channel realization).

|  | B |  |  | D |  |  | E |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2×2 | 4×4 | 4×2 | 2×2 | 4×4 | 4×2 | 2×2 | 4×4 | 4×2 |
| Average throughput | Figure 7-10 | Figure 7-15 | Figure 7-20 | Figure 7-25 | Figure 7-30 | Figure 7-35 | Figure 7-40 | Figure 7-45 | Figure 7-50 |
| FL throughput distribution | Figure 7-11 | Figure 7-16 | Figure 7-21 | Figure 7-26 | Figure 7-31 | Figure 7-36 | Figure 7-41 | Figure 7-46 | Figure 7-51 |
| RL throughput distribution | Figure 7-12 | Figure 7-17 | Figure 7-22 | Figure 7-27 | Figure 7-32 | Figure 7-37 | Figure 7-42 | Figure 7-47 | Figure 7-52 |
| FL PER distribution | Figure 7-13 | Figure 7-18 | Figure 7-23 | Figure 7-28 | Figure 7-33 | Figure 7-38 | Figure 7-43 | Figure 7-48 | Figure 7-53 |
| RL PER distribution | Figure 7-14 | Figure 7-19 | Figure 7-24 | Figure 7-29 | Figure 7-34 | Figure 7-39 | Figure 7-44 | Figure 7-49 | Figure 7-54 |

Table 5-7: Organization of plots for ES mode

Packet Error Rate vs SNR:

In addition to the main simulation results showing average throughput achieved with adaptive rate selection, we also provide a select set of fixed data rate packet error rate results. We include plots showing the average PER as a function of SNR per receiver with fixed data rates for 2×2 and 4×4 system configurations. 100 packet errors were collected for each point, and each packet uses an independent channel realization. A set of data rates considered representative of the proposal is given below.

Figure 7-55, Figure 7-56, and Figure 7-57 show performance of the following data rates in a 2×2

Configuration for channel models B, D, and E, respectively:

- 1 spatial stream - BPSK, R=1/2 (6 Mbps)

- 2 spatial streams - 256-QAM, R=7/8; QPSK, R=3/4 (102 Mbps)

- 2 spatial streams - 256-QAM, R=7/8; 64-QAM, R=2/3 (132 Mbps)

- 2 spatial streams - 256-QAM, R=7/8; 256-QAM, R=7/8 (168 Mbps)

Figure 7-58, Figure 7-59, and Figure 7-60 show performance of the following data rates in a 4×4 configuration for channel models B, D, and E, respectively:

- 1 spatial stream - BPSK, R=1/2  (6 Mbps)

- 2 spatial streams - 16-QAM, R=3/4; QPSK, R=3/4 (54 Mbps)

- 3 spatial streams - 256-QAM, R=7/8; 256-QAM, R=5/8; QPSK, R=3/4 (162 Mbps)

- 4 spatial streams - 256-QAM, R=7/8; 256-QAM, R=3/4; QPSK, R=3/4; BPSK, R=1/2 (180 Mbps)

- 4 spatial streams - 256-QAM, R=7/8; 256-QAM, R=7/8; 256-QAM, R=3/4; 16-QAM, R=3/4 (276 Mbps)

- 4 spatial streams - 256-QAM, R=7/8; 256-QAM, R=7/8; 256-QAM, R=7/8; 256-QAM, R=7/8; (336 Mbps)

Performance in channel D with and without the fluorescent effect:

Figure 7-61 and Figure 7-62 compare the average PER as a function of SNR with and without the fluorescent effect present in channel D. Figure 7-61 shows results obtained with a 4×4 system operating with four active spatial streams and two different rates. The data rates are high, 336 Mbps (rate #12 on all four streams) and 276 Mbps (rates #12, 12, 11, 6), respectively, resulting in a high PER (close to 1.0 in the former case). Figure 7-62 shows results obtained with a 2×2 system where two spatial streams are active. In this case, the data rate is 144 Mbps (rates #12, 10b). The fluorescent effect has no discernible impact on performance.

### CC67.2 – Offset Compensation

Impact of carrier frequency and symbol clock offset on PER at low SNR, channel E:

Figure 7-63 illustrates the impact of carrier frequency and symbol clock offsets on the PER for a 2×2 system and a 4×4 system in channel E. With no carrier frequency or symbol clock offsets, the lowest average SNR at which a 10% PER is achieved is approximately 2 dB for the 4x4 system and approximately 6 dB for the 2x2 system. Both are achieved with the lowest data rate (rate #0, BPSK, R=1/2 – 6 Mbps). There is very little difference in performance when operating with +/- 40 PPM carrier frequency and symbol clock offsets at the same SNRs.

Impact of carrier frequency and symbol clock offset on PER at SNR=50 dB, channel E (LOS):

Table 5-8 shows the impact of carrier frequency and symbol clock offsets on the PER for a 2×2 system and a 4×4 system, respectively, operating at an SNR of 50 dB in channel E (LOS). The following data rates, which are typical of operation at this SNR, are used:

- 2×2: two active spatial streams with rates #11 and 5 (102 Mbps)

- 4×4: four active spatial streams with rates # 12, 12, 8, 3 (234 Mbps)

The table shows the PER performance achieved with 0 PPM and +/-40 PPM carrier frequency and symbol clock offsets. The differences between the three cases are insignificant.

|  | Data rate | 0 PPM | + 40 PPM | - 40 PPM |
| --- | --- | --- | --- | --- |
| 2×2 | 102 Mbps | 1.80e-2 | 1.00e-2 | 2.10e-2 |
| 4×4 | 234 Mbps | 1.44e-1 | 1.13e-1 | 1.34e-1 |

Table 5-8: Impact of carrier frequency and symbol clock offsets in channel E (LOS) at 50 dB

## Spatial Spreading Transmission Mode

### CC59 – AWGN PER Performance

The PER vs Es/N0 performance in AWGN was simulated with the following system configurations: NAP × NSTA = 1×1, 2×2, 3×3, and 4×4.  Referring to Table 4-6, the following set of data rates was used: {0, 3, 6, 9, and 12}, corresponding to data rates of  where Mbps. For each point, a minimum of 100 packet errors were collected. Perfect synchronization and channel estimation was assumed.

Figure 7-64, Figure 7-65, Figure 7-66, and Figure 7-67 show AWGN performance in Spatial Spreading mode for 1×1, 2×2, 3×3, and 4×4 configurations, respectively.

### CC67 – PER Performance in Non AWGN Channels

Average physical layer data throughput:

Plots illustrating the average physical layer data throughput as a function of SNR per receiver achieved with adapative rate selection are provided. Also shown on the same figures are the corresponding average packet error rates.  In addition, throughput distributions and packet error rate distributions are shown for each SNR point, each link, and each configuration.  Results are given for the following system configurations:  NAP  × NSTA = 2×2, 4×4, and 4×2.  Table 5-9 provides the figure numbers for each of the plots.

In obtaining the results shown in the figures, 100 channel realizations were simulated for each SNR point, and multiple packets were transmitted per channel realization (stopping criterion: 10 packet errors or 400 independent 2-SCAP sequences simulated per channel realization).

|  | B |  |  | D |  |  | E |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2×2 | 4×4 | 4×2 | 2×2 | 4×4 | 4×2 | 2×2 | 4×4 | 4×2 |
| Average throughput | Figure 7-68 | Figure 7-73 | Figure 7-78 | Figure 7-83 | Figure 7-88 | Figure 7-93 | Figure 7-98 | Figure 7-103 | Figure 7-108 |
| FL throughput distribution | Figure 7-69 | Figure 7-74 | Figure 7-79 | Figure 7-84 | Figure 7-89 | Figure 7-94 | Figure 7-99 | Figure 7-104 | Figure 7-109 |
| RL throughput distribution | Figure 7-70 | Figure 7-75 | Figure 7-80 | Figure 7-85 | Figure 7-90 | Figure 7-95 | Figure 7-100 | Figure 7-105 | Figure 7-110 |
| FL PER distribution | Figure 7-71 | Figure 7-76 | Figure 7-81 | Figure 7-86 | Figure 7-91 | Figure 7-96 | Figure 7-101 | Figure 7-106 | Figure 7-111 |
| RL PER distribution | Figure 7-72 | Figure 7-77 | Figure 7-82 | Figure 7-87 | Figure 7-92 | Figure 7-97 | Figure 7-102 | Figure 7-107 | Figure 7-112 |

Table 5-9: Organization of plots for SS mode

Packet Error Rate vs SNR:

We include plots illustrating the average packet error rate as a function of SNR per receiver with fixed data rates. 100 packet errors were collected for each point, and each packet uses an independent channel realization. A set of data rates considered representative of the proposal is given below.

Figure 7-113, Figure 7-114, and Figure 7-115 show performance of the following data rates in a 2×2 configuration for channel models B, D, and E, respectively:

- 1 stream: QPSK, R=3/4 (18 Mbps)

- 2 streams: 16-QAM, R=3/4 on both streams (72 Mbps)

- 2 streams: 64-QAM, R=3/4 on both streams (108 Mbps)

Figure 7-116, Figure 7-117, and Figure 7-118 show performance of the following data rates in a 4×4 configuration for channel models B, D, and E, respectively:

- 1 stream: QPSK, R=3/4 (18 Mbps)

- 2 streams: 16-QAM, R=1/2 on both streams (48 Mbps)

- 3 streams: 16-QAM, R=3/4 on all three streams (108 Mbps)

- 3 streams: 64-QAM, R=2/3 on all three streams (144 Mbps)

- 4 streams: 64-QAM, R=2/3 on all four streams (192 Mbps)

Performance in channel D with and without the fluorescent effect:

Figure 7-119 and Figure 7-120 compare the average PER as a function of SNR with and without the fluorescent effect present in channel D. Figure 7-119 shows results obtained with a 4×4 system operating with three active spatial streams. The data rate is high, 252 Mbps (rate #12 on all three streams) resulting in a high PER. Figure 7-120 shows results obtained with a 2×2 system where two spatial streams are active. In this case, the data rate is 120 Mbps (rate #10b on both streams). The fluorescent effect has no discernible impact on performance.

### CC67.2 – Offset Compensation

Impact of carrier frequency and symbol clock offset on PER at low SNR, channel E:

Figure 7-121 illustrates the impact of carrier frequency and symbol clock offsets on the PER for a 2×2 system and a 4×4 system in channel E. With no carrier frequency or symbol clock offsets, the lowest average SNR at which a 10% PER is achieved is approximately 6 dB for the 4x4 system and approximately 9 dB for the 2x2 system. Both are achieved with the lowest data rate (rate #0, BPSK, R=1/2 – 6 Mbps). There is very little difference in performance when operating with +/- 40 PPM carrier frequency and symbol clock offsets at the same SNRs.

Impact of carrier frequency and symbol clock offset on PER at SNR=50 dB, channel E (LOS):

Table 5-10 shows the impact of carrier frequency and symbol clock offsets on the PER for a 2×2 system and a 4×4 system, respectively, operating at an SNR of 50 dB in channel E (LOS). The following data rates, which are typical of operation at this SNR, are used:

- 2×2: two active spatial streams both with rate # 6 (72 Mbps)

- 4×4: two active spatial streams both with rates # 10b (120 Mbps)

The table shows the PER performance achieved with 0 PPM and +/-40 PPM carrier frequency and symbol clock offsets. The differences between the three cases are insignificant.

|  | Data rate | 0 PPM | + 40 PPM | - 40 PPM |
| --- | --- | --- | --- | --- |
| 2×2 | 72 Mbps | 4.60e-2 | 4.60e-2 | 6.60e-2 |
| 4×4 | 120 Mbps | 1.10e-2 | 1.20e-2 | 1.50e-2 |

Table 5-10: Impact of carrier frequency and symbol clock offsets in channel E (LOS) at 50 dB

### Additional Results Obtained with Spatial Spreading Transmission Mode without Cyclic Delay

Plots illustrating the average physical layer data throughput as a function of SNR per receiver achieved with adapative rate selection are provided for channel models B, D, and E. Also shown on the same figures are the corresponding average packet error rates.  In addition, throughput distributions and packet error rate distributions are shown for each SNR point, each link, and each configuration.  Results are given for the following system configuration:  NAP  × NSTA = 4×2.  Table 5-11 provides the figure numbers for each of the plots.

In obtaining the results shown in the figures, 100 channel realizations were simulated for each SNR point, and multiple packets were transmitted per channel realization (stopping criterion: 10 packet errors or 400 independent 3-SCAP sequences simulated per channel realization).

|  | 4×2 |  |  |
| --- | --- | --- | --- |
|  | B | D | E |
| Average throughput | Figure 7-122 | Figure 7-127 | Figure 7-132 |
| FL throughput distribution | Figure 7-123 | Figure 7-128 | Figure 7-133 |
| RL throughput distribution | Figure 7-124 | Figure 7-129 | Figure 7-134 |
| FL PER distribution | Figure 7-125 | Figure 7-130 | Figure 7-135 |
| RL PER distribution | Figure 7-126 | Figure 7-131 | Figure 7-136 |

Table 5-11: Organization of plots for spatial spreading mode without cyclic delay

## Some Results Obtained with Short Guard Interval and 52 Data Subcarriers

### CC59 – AWGN PER Performance

The PER vs Es/N0  performance in AWGN was simulated with the following system configurations: NAP  × NSTA = 1×1, 2×2, 3×3, and 4×4. Referring to Table 4-6, the following set of data rates was used: {0, 3, 6, 9, 12}. For each point, a minimum of 100 packet errors were collected. Perfect synchronization and channel estimation was assumed.

Figure 7-137, Figure 7-138, Figure 7-139, and Figure 7-140 show AWGN performance in Eigenvector Steering mode with short guard interval and 52 data subcarriers for 1×1, 2×2, 3×3, and 4×4 configurations, respectively.

### CC67 – PER Performance in Non AWGN Channels

Average physical layer data throughput:

Plots illustrating the average physical layer data throughput as a function of SNR per receiver achieved with adapative rate selection are provided for both ES mode and SS mode. Also shown on the same figures are the corresponding average packet error rates.  In addition, throughput distributions and packet error rate distributions are shown for each SNR point, each link, and each configuration.  Results are given for the following system configurations:  NAP  × NSTA = 2×2, 4×4, and 4×2.  provides the figure numbers for each of the plots obtained with ES mode, while Table 5-13 provides the figure numbers for the plots obtained with SS mode.

In obtaining the results shown in the figures, 100 channel realizations were simulated for each SNR point, and multiple packets were transmitted per channel realization (stopping criterion: 10 packet errors or 400 independent 3-SCAP sequences simulated per channel realization).

|  | B |  |  | D |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2×2 | 4×4 | 4×2 | 2×2 | 4×4 | 4×2 |
| Average throughput | Figure 7-141 | Figure 7-146 | Figure 7-151 | Figure 7-156 | Figure 7-161 | Figure 7-166 |
| FL throughput distribution | Figure 7-142 | Figure 7-147 | Figure 7-152 | Figure 7-157 | Figure 7-162 | Figure 7-167 |
| RL throughput distribution | Figure 7-143 | Figure 7-148 | Figure 7-153 | Figure 7-158 | Figure 7-163 | Figure 7-168 |
| FL PER distribution | Figure 7-144 | Figure 7-149 | Figure 7-154 | Figure 7-159 | Figure 7-164 | Figure 7-169 |
| RL PER distribution | Figure 7-145 | Figure 7-150 | Figure 7-155 | Figure 7-160 | Figure 7-165 | Figure 7-170 |

Table 5-12: Organization of plots for SGI-52, ES mode

|  | 2×2 |  |  |
| --- | --- | --- | --- |
|  | B | D | E |
| Average throughput | Figure 7-171 | Figure 7-176 | Figure 7-181 |
| FL throughput distribution | Figure 7-172 | Figure 7-177 | Figure 7-182 |
| RL throughput distribution | Figure 7-173 | Figure 7-178 | Figure 7-183 |
| FL PER distribution |  | Figure 7-179 | Figure 7-184 |
| RL PER distribution | Figure 7-175 | Figure 7-180 | Figure 7-185 |

Table 5-13: Organization of plots for SGI-52, SS mode

Packet Error Rate vs SNR (ES mode only):

We include plots illustrating the average packet error rate as a function of SNR per receiver with fixed data rates. 100 packet errors were collected for each point, and each packet uses an independent channel realization. A set of data rates considered representative of the proposal is given below.

Figure 7-186 and Figure 7-187 show performance of the following data rates in a 2×2 configuration for channel models B and D, respectively:

- 1 spatial stream - BPSK, R=1/2 (7.22 Mbps)

- 2 spatial streams - 256-QAM, R=7/8; QPSK, R=3/4 (122.78 Mbps)

- 2 spatial streams - 256-QAM, R=7/8; 64-QAM, R=2/3 (158.89 Mbps)

Figure 7-188, Figure 7-189, and Figure 7-190 show performance of the following data rates in a 4×4 configuration for channel models B, D, and E, respectively:

- 1 spatial stream - BPSK, R=1/2  (7.22 Mbps)

- 2 spatial streams - 16-QAM, R=3/4; QPSK, R=3/4 (65 Mbps)

- 3 spatial streams - 256-QAM, R=7/8; 256-QAM, R=5/8; QPSK, R=3/4 (195 Mbps)

- 4 spatial streams - 256-QAM, R=7/8; 256-QAM, R=3/4; QPSK, R=3/4; BPSK, R=1/2 (216.67 Mbps)

- 4 spatial streams - 256-QAM, R=7/8; 256-QAM, R=7/8; 256-QAM, R=3/4; 16-QAM, R=3/4 (332.22 Mbps)

# Error Model for System Simulations Based on PHY Abstraction

## Introduction

In the NS based system simulator, a MIMO propagation module (MPM) has been implemented which simulates the effects of the underlying PHY for each wireless link. The MPM is not a full PHY link simulator, but rather an abstraction of the PHY. This abstract model is sufficient for generating error processes in the system simulations that accurately reflect the interaction between the MIMO OFDM PHY and the wireless channel as modeled by the TGn channel models [3]. In fact, the MPM implements the channel models between each pair of nodes in the system simulation and is queried by the system simulator to obtain the instantaneous post-detection SNRs associated with a specific link’s channel state at the time of a packet arrival. “Post-detection SNRs” refers to estimates of the receive SNRs that include the effects of transmitter and receiver spatial processing. These SNRs represent an abstract model of the PHY, and provide means for computing packet error probabilities, upon which the error processes are based. The post-detection SNRs are also used for rate control purposes, i.e., for selecting the PHY data rates that can be supported by the channel at a given time. In the following sections, a detailed description is given of how post-detection SNRs are calculated, as well as how the error processes are generated from them.

## PHY Abstraction: Post-Detection SNRs

The operation of the proposed 802.11n PHY layer involves estimating the channel (as well as receive/transmit steering vectors in the case of the Eigenvector Steering mode of operation) using the MIMO Training Sequence and the Steered MIMO Training Sequence. Upon receiving either one of the training sequences, the STA computes its receive vectors (steered or unsteered) and determines the number of spatially segregated streams and associated per-stream data rates that can be reliably supported by the channel. Specifically, these decisions are based on estimates of the SNR in each OFDM subcarrier of each spatial stream computed at the output of the receive MIMO spatial processing. Since the TGn channel models are slowly fading channels relative to typical packet durations, the simulator assumes that the channel is stationary over each individual packet. Each OFDM subcarrier can therefore be treated as if it were an AWGN subchannel with a fixed SNR over the course of a packet.

The received signal vector in subcarrier k is given by

where  is the MIMO channel matrix,  is AWGN, and T[.] is a transformation on the transmitted signal vector  representing transmit spatial processing (if any). The output of the receive spatial processing may be represented as

where R[.] is a transformation on the received signal vector representing receive spatial processing, and  is a noise vector consisting of both AWGN and residual self-interference due to imperfect isolation of the spatial streams.

The post-detection SNR in subcarrier k of stream i is

where F[.] is determined by the spatial processing performed at the transmitter and receiver. Based on the post-detection SNRs, a rate adaptation algorithm decides the number of spatial streams to be used, Ns, and the per-stream data rates that can be supported under the current channel conditions.

As an example, consider a minimum mean squared error (MMSE) receiver for the case where , where I is the identity matrix. In this case, the receive spatial processing is given by

where M(k) is the MMSE spatial filter given by

and

denotes the conjugate transpose of . Defining the matrix , with diagonal elements wii, it can be shown that the post-detection SNR of the i-th element of  is

γ(k,i)=wii1−wii.

## Error Process Generation

Having obtained the post-detection SNRs as well as the number of active spatial streams Ns and the per-stream data rate selections, the packet error probability can be calculated using the following steps. First we define the effective SNR per stream,

SNReff(i)=10X(i)

where

X(i)=1NΣN−1k=0logγ(k,i)−avar(logγ(k,i))

X(i) is simply the geometric mean of the subcarrier SNRs, adjusted by the variance. The constant a is used to fit the approximation to simulation results obtained with the actual PHY link simulator .

The effective SNR for stream i is now used to look up the coded bit error probability for the data rate used in stream i from a table listing the bit error rate vs SNR in AWGN. Next, the approximate error event probability Pe(i) is calculated as

Pe(i)=Pb(i)dmin(i)

where dmin(i) is the minimum distance of the code used in stream i. The probability of error in stream i is approximated as

Pi=1−Pe(i)NiDi

where Ni  is the number of coded bits in stream i, Di=KRi , K is the constraint length of the code, and Ri is the code rate in stream i. Finally, the overall packet error probability with  Nsspatial streams is given by

Ps=1−(1−P0)(1−P1)...(1−PNs−1)

The packet error probability estimate is the basis for the error process in the system simulation. Each time a packet arrives at a receiver, the packet error probability is calculated based on the post-detection SNRs as described above. A weighted coin toss where the weight is simply the packet error probability is then performed in order to decide whether the packet is received in error.

## Error Model Validation

In order to validate the packet error model, full PHY link simulations were conducted in which simulated packet error rates were compared to predicted packet error rates, i.e., packet error rates obtained with the error process model. In order to comply with the MAC related system simulation scenarios, these link simulations were run using perfect synchronization and perfect channel estimation. The phase noise and PA nonlinearity models were both active, however. 10000 channel realizations were generated, and a single 3(2)-SCAP sequence was simulated per channel realization.

Figure 7-191 compares simulated and predicted average packet error rates obtained with a 4×4 system operating in Eigenvector Steering mode in channel B. Results are shown for one, two and three active spatial streams with the following data rates (referring to Table 4-1):

- One stream: rate #3 (18 Mbps)

- Two streams: rates #6 and #3 (54 Mbps)

- Three streams: rates #12, #9, #4 (162 Mbps)

Figure 7-192 shows the distributions of the packet error rates per channel realization for a single SNR point (8dB) for the case of two active streams above. The distributions reflect 100 channel realizations and 1000 3-SCAP sequences per realization. Figure 7-193 shows simulated and predicted average packet error rates obtained with two and three active spatial streams in channel D.

Figure 7-194 and Figure 7-195 compare simulated and predicted average packet error rates obtained with a 4×4 system operating in Spatial Spreading mode in channels B and E, respectively. Results are shown for one, two, and three active spatial streams with the following data rates (referring to Table 4-1):

- One stream: rate #3 (18 Mbps)

- Two streams: rate #4 on both streams (48 Mbps)

- Three streams: rates #6 on all three streams (108 Mbps)

The above results were obtained with fitting constant values of a=0 for the ES case and a=0.6 for the SS case. The results show satisfactory agreement between the simulated and predicted packet error rates for a wide range of cases.

# References

[1] K. Fall, K. Varadhan , The NS Manual,  http://www.isi.edu/nsnam/ns/doc/

[2] P. Garg et al. “Achieving Higher Throughput and QoS in 802.11 Wireless LANS,” Proc IPCC’03.  (NS code: http://mosquitonet.stanford.edu/software/802.11e/ns.tar.gz)

[3] “IEEE 802.11 TGn Channel Models,”  IEEE 802.11-03-940r4

[4] “System Description and Operating Principles for High Throughput Enhancements to 802.11,” IEEE 802.11-04-870r0

[5] “IEEE 802.11 TGn Comparison Criteria,” IEEE 802.11-03-814r30

Figure 7-3: Transmitter processing

Figure 7-4: Channel and impairment models

Figure 7-5: Receiver processing

Figure 7-6: AWGN performance for 1×1 Eigenvector Steering mode

Figure 7-7: AWGN performance for 2×2 Eigenvector Steering mode

Figure 7-8: AWGN performance for 3×3 Eigenvector Steering mode

Figure 7-9: AWGN performance for 4×4 Eigenvector Steering mode

Figure 7-10: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Eigenvector Steering

Figure 7-11: Forward link throughput distributions for channel model B, 2×2, Eigenvector Steering

Figure 7-12: Reverse link throughput distributions for channel model B, 2×2, Eigenvector Steering

Figure 7-13: Forward link packet error rate distributions for channel model B, 2×2, Eigenvector Steering

Figure 7-14: Reverse link packet error rate distributions for channel model B, 2×2, Eigenvector Steering

Figure 7-15: Throughput vs SNR and Average PER vs SNR for channel model B, 4×4, Eigenvector Steering

Figure 7-16: Forward link throughput distributions for channel model B, 4×4, Eigenvector Steering

Figure 7-17: Reverse link throughput distributions for channel model B, 4×4, Eigenvector Steering

Figure 7-18: Forward link packet error rate distributions for channel model B, 4×4, Eigenvector Steering

Figure 7-19: Reverse link packet error rate distributions for channel model B, 4×4, Eigenvector Steering

Figure 7-20: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Eigenvector Steering

Figure 7-21: Forward link throughput distributions for channel model B, 4×2, Eigenvector Steering

Figure 7-22: Reverse link throughput distributions for channel model B, 4×2, Eigenvector Steering

Figure 7-23: Forward link packet error rate distributions for channel model B, 4×2, Eigenvector Steering

Figure 7-24: Reverse link packet error rate distributions for channel model B, 4×2, Eigenvector Steering

Figure 7-25: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Eigenvector Steering

Figure 7-26: Forward link throughput distributions for channel model D, 2×2, Eigenvector Steering

Figure 7-27: Reverse link throughput distributions for channel model D, 2×2, Eigenvector Steering

Figure 7-28: Forward link packet error rate distributions for channel model D, 2×2, Eigenvector Steering

Figure 7-29: Reverse link packet error rate distributions for channel model D, 2×2, Eigenvector Steering

Figure 7-30: Throughput vs SNR and Average PER vs SNR for channel model D, 4×4, Eigenvector Steering

Figure 7-31: Forward link throughput distributions for channel model D, 4×4, Eigenvector Steering

Figure 7-32: Reverse link throughput distributions for channel model D, 4×4, Eigenvector Steering

Figure 7-33: Forward link packet error rate distributions for channel modelDB,4×4, Eigenvector Steering

Figure 7-34: Reverse link packet error rate distributions for channel model D, 4×4, Eigenvector Steering

Figure 7-35: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Eigenvector Steering

Figure 7-36: Forward link throughput distributions for channel model D, 4×2, Eigenvector Steering

Figure 7-37: Reverse link throughput distributions for channel model B, 4×2, Eigenvector Steering

Figure 7-38: Forward link packet error rate distributions for channel model D, 4×2, Eigenvector Steering

Figure 7-39: Reverse link packet error rate distributions for channel model D, 4×2, Eigenvector Steering

Figure 7-40: Throughput vs SNR and Average PER vs SNR for channel model E, 2×2, Eigenvector Steering

Figure 7-41: Forward link throughput distributions for channel model E, 2×2, Eigenvector Steering

Figure 7-42: Reverse link throughput distributions for channel model E, 2×2, Eigenvector Steering

Figure 7-43: Forward link packet error rate distributions for channel model E, 2×2, Eigenvector Steering

Figure 7-44: Reverse link packet error rate distributions for channel model E, 2×2, Eigenvector Steering

Figure 7-45: Throughput vs SNR and Average PER vs SNR for channel model E, 4×4, Eigenvector Steering

Figure 7-46: Forward link throughput distributions for channel model E, 4×4, Eigenvector Steering

Figure 7-47: Reverse link throughput distributions for channel model E, 4×4, Eigenvector Steering

Figure 7-48: Forward link packet error rate distributions for channel model E, 4×4, Eigenvector Steering

Figure 7-49: Reverse link packet error rate distributions for channel model E, 4×4, Eigenvector Steering

Figure 7-50: Throughput vs SNR and Average PER vs SNR for channel model E, 4×2, Eigenvector Steering

Figure 7-51: Forward link throughput distributions for channel model E, 4×2, Eigenvector Steering

Figure 7-52: Reverse link throughput distributions for channel model E, 4×2, Eigenvector Steering

Figure 7-53: Forward link packet error rate distributions for channel model E, 4×2, Eigenvector Steering

Figure 7-54: Reverse link packet error rate distributions for channel model E, 4×2, Eigenvector Steering

Figure 7-55: Performance in channel model B, 2×2, Eigenvector Steering, fixed rates

Figure 7-56: Performance in channel model D, 2×2, Eigenvector Steering, fixed rates

Figure 7-57: Performance in channel model E, 2×2, Eigenvector Steering, fixed rates

Figure 7-58: Performance in channel model B, 4×4, Eigenvector Steering, fixed rates

Figure 7-59: Performance in channel model D, 4×4, Eigenvector Steering, fixed rates

Figure 7-60: Performance in channel model E, 4×4, Eigenvector Steering, fixed rates

Figure 7-61: Fluorescent effect on and off in channel model D, 4×4, Eigenvector Steering, 4 spatial streams

Figure 7-62: Fluorescent effect on and off in channel model D, 2×2, Eigenvector Steering, 2 spatial streams

Figure 7-63: Impact of carrier frequency and symbol clock offset for 2×2 and 4×4, Eigenvector Steering, channel model E

Figure 7-64: AWGN performance for 1×1 Spatial Spreading mode

Figure 7-65: AWGN performance for 2×2 Spatial Spreading mode

Figure 7-66: AWGN performance for 3×3 Spatial Spreading mode

Figure 7-67: AWGN performance for 4×4 Spatial Spreading mode

Figure 7-68: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Spatial Spreading

Figure 7-69: Forward link throughput distributions for channel model B, 2×2, Spatial Spreading

Figure 7-70: Reverse link throughput distributions for channel model B, 2×2, Spatial Spreading

Figure 7-71: Forward link packet error rate distributions for channel model B, 2×2, Spatial Spreading

Figure 7-72: Reverse link packet error rate distributions for channel model B, 2×2, Spatial Spreading

Figure 7-73: Throughput vs SNR and Average PER vs SNR for channel model B, 4×4, Spatial Spreading

Figure 7-74: Forward link throughput distributions for channel model B, 4×4, Spatial Spreading

Figure 7-75: Reverse link throughput distributions for channel model B, 4×4, Spatial Spreading

Figure 7-76: Forward link packet error rate distributions for channel model B, 4×4, Spatial Spreading

Figure 7-77: Reverse link packet error rate distributions for channel model B,4×4, Spatial Spreading

Figure 7-78: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Spatial Spreading

Figure 7-79: Forward link throughput distributions for channel model B, 4×2, Spatial Spreading

Figure 7-80: Reverse link throughput distributions for channel model B, 4×2, Spatial Spreading

Figure 7-81: Forward link packet error rate distributions for channel model B, 4×2, Spatial Spreading

Figure 7-82: Reverse link packet error rate distributions for channel model B, 4×2, Spatial Spreading

Figure 7-83: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Spatial Spreading

Figure 7-84: Forward link throughput distributions for channel model D, 2×2, Spatial Spreading

Figure 7-85: Reverse link throughput distributions for channel model D, 2×2, Spatial Spreading

Figure 7-86: Forward link packet error rate distributions for channel model D, 2×2, Spatial Spreading

Figure 7-87: Reverse link packet error rate distributions for channel model D, 2×2, Spatial Spreading

Figure 7-88: Throughput vs SNR and Average PER vs SNR for channel model D, 4×4, Spatial Spreading

Figure 7-89: Forward link throughput distributions for channel model D, 4×4, Spatial Spreading

Figure 7-90: Reverse link throughput distributions for channel model D, 4×4, Spatial Spreading

Figure 7-91: Forward link packet error rate distributions for channel model D, 4×4, Spatial Spreading

Figure 7-92: Reverse link packet error rate distributions for channel model D,4×4, Spatial Spreading

Figure 7-93: Throughput vs SNR and Average PER vs SNR for channel model D, 4×2, Spatial Spreading

Figure 7-94: Forward link throughput distributions for channel model D, 4×2, Spatial Spreading

Figure 7-95: Reverse link throughput distributions for channel model D, 4×2, Spatial Spreading

Figure 7-96: Forward link packet error rate distributions for channel model D, 4×2, Spatial Spreading

Figure 7-97: Reverse link packet error rate distributions for channel model D, 4×2, Spatial Spreading

Figure 7-98: Throughput vs SNR and Average PER vs SNR for channel model E, 2×2, Spatial Spreading

Figure 7-99: Forward link throughput distributions for channel model E, 2×2, Spatial Spreading

Figure 7-100: Reverse link throughput distributions for channel model E, 2×2, Spatial Spreading

Figure 7-101: Forward link packet error rate distributions for channel model E, 2×2, Spatial Spreading

Figure 7-102: Reverse link packet error rate distributions for channel model E, 2×2, Spatial Spreading

Figure 7-103: Throughput vs SNR and Average PER vs SNR for channel model E, 4×4, Spatial Spreading

Figure 7-104: Forward link throughput distributions for channel model E, 4×4, Spatial Spreading

Figure 7-105: Reverse link throughput distributions for channel model E, 4×4, Spatial Spreading

Figure 7-106: Forward link packet error rate distributions for channel model E, 4×4, Spatial Spreading

Figure 7-107: Reverse link packet error rate distributions for channel model E,4×4, Spatial Spreading

Figure 7-108: Throughput vs SNR and Average PER vs SNR for channel model E, 4×2, Spatial Spreading

Figure 7-109: Forward link throughput distributions for channel model E, 4×2, Spatial Spreading

Figure 7-110: Reverse link throughput distributions for channel model E, 4×2, Spatial Spreading

Figure 7-111: Forward link packet error rate distributions for channel model E, 4×2, Spatial Spreading

Figure 7-112: Reverse link packet error rate distributions for channel model E,4×2, Spatial Spreading

Figure 7-113: Performance in channel model B, 2×2, Spatial Spreading, fixed rates

Figure 7-114: Performance in channel model D, 2×2, Spatial Spreading, fixed rates

Figure 7-115: Performance in channel model E, 2×2, Spatial Spreading, fixed rates

Figure 7-116: Performance in channel model B, 4×4, Spatial Spreading, fixed rates

Figure 7-117: Performance in channel model D, 4×4, Spatial Spreading, fixed rates

Figure 7-118: Performance in channel model E, 4×4, Spatial Spreading, fixed rates

Figure 7-119: Fluorescent effect on and off in channel model D, 4×4, Spatial Spreading, 3 spatial streams

Figure 7-120: Fluorescent effect on and off in channel model D, 2×2, Spatial Spreading, 2 spatial streams

Figure 7-121: Impact of carrier frequency and symbol clock offset in 2×2 and 4×4, Spatial Spreading, channel model E

Figure 7-122: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-123: Forward link throughput distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-124: Reverse link throughput distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-125: Forward link packet error rate distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-126: Reverse link packet error rate distributions for channel model B, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-127: Throughput vs SNR and Average PER vs SNR for channel model D, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-128: Forward link throughput distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-129: Reverse link throughput distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-130: Forward link packet error rate distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-131: Reverse link packet error rate distributions for channel model D, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-132: Throughput vs SNR and Average PER vs SNR for channel model E, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-133: Forward link throughput distributions for channel model E, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-134: Reverse link throughput distributions for channel model E, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-135: Forward link packet error rate distributions for channel model E, 4×2, Spatial Spreading without Cyclic Delay

Figure 7-136: Reverse link packet error rate distributions for channel model E,4×2, Spatial Spreading without Cyclic Delay

Figure 7-137: AWGN performance for 1×1 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers

Figure 7-138: AWGN performance for 2×2 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers

Figure 7-139: AWGN performance for 3×3 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers

Figure 7-140: AWGN performance for 4×4 Eigenvector Steering mode, Short Guard Interval, 52 subcarriers

Figure 7-141: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Eigenvector Steering, SGI-52

Figure 7-142: Forward link throughput distributions for channel model B, 2×2, Eigenvector Steering, SGI-52

Figure 7-143: Reverse link throughput distributions for channel model B, 2×2, Eigenvector Steering, SGI-52

Figure 7-144: Forward link packet error rate distributions for channel model B, 2×2, Eigenvector Steering, SGI-52

Figure 7-145: Reverse link packet error rate distributions for channel model B, 2×2, Eigenvector Steering, SGI-52

Figure 7-146: Throughput vs SNR and Average PER vs SNR for channel model B, 4×4, Eigenvector Steering, SGI-52

Figure 7-147: Forward link throughput distributions for channel model B, 4×4, Eigenvector Steering, SGI-52

Figure 7-148: Reverse link throughput distributions for channel model B, 4×4, Eigenvector Steering, SGI-52

Figure 7-149: Forward link packet error rate distributions for channel model B, 4×4, Eigenvector Steering, SGI-52

Figure 7-150: Reverse link packet error rate distributions for channel model B, 4×4, Eigenvector Steering, SGI-52

Figure 7-151: Throughput vs SNR and Average PER vs SNR for channel model B, 4×2, Eigenvector Steering, SGI-52

Figure 7-152: Forward link throughput distributions for channel model B, 4×2, Eigenvector Steering, SGI-52

Figure 7-153: Reverse link throughput distributions for channel model B, 4×2, Eigenvector Steering, SGI-52

Figure 7-154: Forward link packet error rate distributions for channel model B, 4×2, Eigenvector Steering, SGI-52

Figure 7-155: Reverse link packet error rate distributions for channel model B, 4×2, Eigenvector Steering, SGI-52

Figure 7-156: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Eigenvector Steering, SGI-52

Figure 7-157: Forward link throughput distributions for channel model D, 2×2, Eigenvector Steering, SGI-52

Figure 7-158: Reverse link throughput distributions for channel model D, 2×2, Eigenvector Steering, SGI-52

Figure 7-159: Forward link packet error rate distributions for channel model D, 2×2, Eigenvector Steering, SGI-52

Figure 7-160: Reverse link packet error rate distributions for channel model D, 2×2, Eigenvector Steering, SGI-52

Figure 7-161: Throughput vs SNR and Average PER vs SNR for channel model D, 4×4, Eigenvector Steering, SGI-52

Figure 7-162: Forward link throughput distributions for channel model D, 2×2, Eigenvector Steering, SGI-52

Figure 7-163: Reverse link throughput distributions for channel model D, 4×4, Eigenvector Steering, SGI-52

Figure 7-164: Forward link packet error rate distributions for channel model D, 4×4, Eigenvector Steering, SGI-52

Figure 7-165: Reverse link packet error rate distributions for channel model D, 4×4, Eigenvector Steering, SGI-52

Figure 7-166: Throughput vs SNR and Average PER vs SNR for channel model D, 4×2, Eigenvector Steering, SGI-52

Figure 7-167: Forward link throughput distributions for channel model D, 4×2, Eigenvector Steering, SGI-52

Figure 7-168: Reverse link throughput distributions for channel model D, 4×2, Eigenvector Steering, SGI-52

Figure 7-169: Forward link packet error rate distributions for channel model D, 4×2, Eigenvector Steering, SGI-52

Figure 7-170: Reverse link packet error rate distributions for channel model D, 4×2, Eigenvector Steering, SGI-52

Figure 7-171: Throughput vs SNR and Average PER vs SNR for channel model B, 2×2, Spatial Spreading, SGI-52

Figure 7-172: Forward link throughput distributions for channel model B, 2×2, Spatial Spreading, SGI-52

Figure 7-173: Reverse link throughput distributions for channel model B, 2×2, Spatial Spreading, SGI-52

Figure 7-174: Forward link packet error rate distributions for channel model B, 2×2, Spatial Spreading, SGI-52

Figure 7-175: Reverse link packet error rate distributions for channel model B, 2×2, Spatial Spreading, SGI-52

Figure 7-176: Throughput vs SNR and Average PER vs SNR for channel model D, 2×2, Spatial Spreading, SGI-52

Figure 7-177: Forward link throughput distributions for channel model D, 2×2, Spatial Spreading, SGI-52

Figure 7-178: Reverse link throughput distributions for channel model D, 2×2, Spatial Spreading, SGI-52

Figure 7-179: Forward link packet error rate distributions for channel model D, 2×2, Spatial Spreading, SGI-52

Figure 7-180: Reverse link packet error rate distributions for channel model D, 2×2, Spatial Spreading, SGI-52

Figure 7-181: Throughput vs SNR and Average PER vs SNR for channel model E, 2×2, Spatial Spreading, SGI-52

Figure 7-182: Forward link throughput distributions for channel model E, 2×2, Spatial Spreading, SGI-52

Figure 7-183: Reverse link throughput distributions for channel model E, 2×2, Spatial Spreading, SGI-52

Figure 7-184: Forward link packet error rate distributions for channel model E, 2×2, Spatial Spreading, SGI-52

Figure 7-185: Reverse link packet error rate distributions for channel model E, 2×2, Spatial Spreading, SGI-52

Figure 7-186: Performance of Eigenvector Steering, 2×2, channel model B,  fixed rates, Short Guard Interval, 52 subcarriers

Figure 7-187: Performance of Eigenvector Steering, 2×2, channel model D,  fixed rates, Short Guard Interval, 52 subcarriers

Figure 7-188: Performance of Eigenvector Steering, 4×4, channel model B, fixed rates, Short Guard Interval, 52 subcarriers

Figure 7-189: Performance of Eigenvector Steering, 4×4, channel model D, fixed rates, Short Guard Interval, 52 subcarriers

Figure 7-190: Performance of Eigenvector Steering, 4×4, channel model E, fixed rates, Short Guard Interval, 52 subcarriers

Figure 7-191: Simulated and predicted PERs, channel model B, 4×4, Eigenvector Steering, 1, 2, and 3 active spatial streams

Figure 7-192: Simulated and predicted PER distributions for channel model B, 4×4, Eigenvector Steering, 2 active spatial streams

Figure 7-193: Simulated and predicted PERs, channel model D, 4×4, Eigenvector Steering, 2 and 3 active spatial streams

Figure 7-194: Simulated and predicted PERs, channel model B, 4×4, Spatial Spreading, 1, 2, and 3 active spatial streams

Figure 7-195: Simulated and predicted PERs, channel model E, 4×4, Spatial Spreading, 1, 2, and 3 active spatial streams