IEEE P802.11
Wireless LANs

Class-based Contention Periods (CCP) for the 802.11n MAC

Date:	August 11, 2004

Authors:	Abel Dasylva, Z.Yao, D.Y. Montuno, W. Chen, M. Ouellette, J. Aweya, K. Felske
	Nortel Networks
	3500 Carling ave Nepean ON K2H8E9
	Phone: 613 7635034
	Fax: 613 768-1140
	e-Mail: abel@nortelnetworks.com

Abstract:

A Class-based Contention Period (CCP) scheme is proposed for a simple and effective Qos allocation over wireless LANs. It is a natural extension of EDCA with multiple contention periods, where access to the medium is restricted to a subset of the Access Categories (ACs). Within a contention period, access to the medium is obtained through contention among the frames from allowed classes, as with EDCA contention periods. However, the scheduling of the successive contention periods is done by the Hybrid Coordinator (HC). Contention periods are advertised to stations, through specific control frames transmitted by the HC. CCP enables a better protection of real-time traffic from lower priority frames for up-stream and down-stream traffic.

Content

1.	References	3

2.	Definitions	3

3.	Abbreviations and acronyms	3

4.	Introduction	6

4.1.	Purpose	6

4.2.	General description of MAC enhancements	6

5.	Frame formats	6

5.1.	Control frames	6

5.1.1.	CP-start MPDU	6

5.1.2.	CP-End MPDU	7

5.1.3.	CP-End+CP-Start MPDU	7

5.2.	Management frames	7

6.	MAC sublayer functional description	7

6.1.	EDCA	8

6.1.1.	Qos negotiation	8

6.1.2.	TXOPs	8

6.1.3.	Back-off counter	8

6.1.4.	Congestion window	8

6.2.	CP scheduling (informative)	9

7.	MAC sublayer management	10

8.	Simulation methodology	10

Tables

Error! No table of figures entries found.

Figures

Figure 1: CP-start MPDU	6

Figure 4: WRR CP schedule	9

Figure 5: Hierarchical CP schedule	9

# References

- IEEE Std 802.11®-1999 (Reaff 2003)
Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- IEEE P802.11e/D8.0, February 2004
(Draft Amendment to IEEE Std 802.11, 1999 Edition (Reaff 2003))
Medium Access Control (MAC) Enhancements for Quality of Service (QoS)

- Project Authorization Request  for IEEE 802.11n.
11-02-798r7-HT-SG-Draft-PAR.doc

- IEEE Std 802.11®-1999 (Reaff 2003)
Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- IEEE Std 802.11®-1999
High-speed Physical Layer in 5 GHz Band

# Definitions

- CCP: a refinement of contention period in different types for traffic isolation and bandwidth control

- CP Length: Length of a contention period

- CP Type: Indicates the subset of access categories that are eligible to contend in this period

# Abbreviations and acronyms

| Term | Description |
| --- | --- |
| AC | Access Category |
| ACK | Acknowledgement |
| AGC | Automatic Gain Control |
| AIFS | Arbitration IFS |
| AP | Access Point |
| AP | Access Point |
| BSS | Basic Service Set |
| CAC | Call Admission Control |
| CAP | Controlled Access Phase |
| CCP | Class-based Contention Periods |
| CP | Contention Period |
| CPL | Contention Period Length |
| CPT | Contention Period Type |
| CTS | Clear To Send |
| CTS | Clear To Send |
| DCF | Distributed Coordination Function |
| DS | Distribution System |
| EDCA | Enhanced Distributed Channel Access |
| EDCF | Enhanced DCF |
| FCS | Frame Check Sequence |
| FDM | Frequency Division Multiplexing |
| FSM | Finite State Machine |
| HC | Hybrid Coordinator |
| HCCA | HCF Controlled Channel Access |
| HCF | Hybrid Coordination Function |
| HT | High Throughput |
| IE | Information Element |
| IFS | Inter Frame Spacing |
| LLC | Logical Link Control |
| MAC | Medium Access Controller |
| MPDU | MAC protocol data unit |
| MPDU | MAC PDU |
| MSDU | MAC service data unit |
| MSDU | MAC SDU |
| MU | Mobile Unit |
| NAV | Network Allocation Vector (a MAC-level carrier-sense) |
| OFDM | Orthogonal Frequency Division Multiplexing |
| OFDM | Orthogonal FDM |
| PDU | Protocol Data Unit |
| PHY | Physical Layer |
| PLCP | PHY layer convergence protocol |
| PPDU | PHY protocol data unit |
| PSDU | PHY service data unit |
| Pure Mode | A mode of operation of a BSS in which there are no legacy devices |
| QAP | QoS access point |
| QoS | Quality of Service |
| QSTA | QoS Station |
| RA | Receiver Address |
| RTS | Request to send |
| RX | Receiver |
| SAP | Service Access Point |
| SDU | Service Data Unit |
| SIFS | Short inter-frame spacing |
| SoP | Start of Packet (i.e. PPDU Packet) |
| STA | Station |
| TA | Transmitter Address |
| TC | Traffic Category |
| TGe | 802.11 Task Group e - Quality of Service |
| TGn | 802.11 Task Group n - Enhancements for Higher Throughput |
| TS | Traffic Stream |
| TSID | Traffic Stream Identifier |
| TSPEC | Traffic specification |
| TX | Transmitter |
| TXOP | Transmission opportunity |
| TXOP | Transmission Opportunity |
| UP | User Priority |
| WM | Wireless Medium |

# Introduction

## Purpose

- This document defines proposed enhancements for higher throughput within the scope of the Project Authorization Request (PAR) as formulated for IEEE 802.11 Task Group n.

## General description of MAC enhancements

- The Class-based Contention Periods scheme provide a simple and effective way to provide Qos in wireless LANs. It is obtained by simple extensions of EDCA with contention periods of different types. Within a contention period, channel access is based on contention as with EDCA for a restricted number of ACs. Contention periods are scheduled and advertised by a central coordination function, according to a scheduling policy set by the network operator. The CCP can also interoperate with contention free channel access by allocating periods where no AC is allowed to contend, and traffic is polled by the central coordinator according to HCCA.

- Control frames for the allocation of contention periods

- Interoperation with HCCA

- Interoperation with legacy devices

# Frame formats

When the BSS does not support HCCA, a CP always starts with a CP-start control frame and it ends with  the next CP-start or CP-End control frame. With HCCA, A CP-start frame transmitted in a CFP must be ignored by STAs. An STA that receives traffic from of a given AC must wait for the next CP allowing the traffic from the AC to contend.

## Control frames

### CP-start MPDU

| Octets: 2 | 2 | 6 | 6 | 1 | 4 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | TA | CP mask | FCS |
| MAC header |  |  |  |  |  |

Figure 1: CP-start MPDU

The different fields are described in the following table.

| Field | Size (bytes) | Purpose |
| --- | --- | --- |
| Frame Control | 2 |  |
| Duration | 2 | It is set to the duration of the next CP |
| RA | 6 |  |
| TA | 6 |  |
| CP mask | 1 | Bit mask where AC n corresponds to bit n (if the AC mask is denoted by b7…b0). Four bits are reserved (b4 to b7) for the definition of additional TCs |
| FCS | 4 |  |

### CP-End MPDU

TBD

### CP-End+CP-Start MPDU

TBD

## Management frames

TBD

# MAC sublayer functional description

With CCP,  the start of a CP is explicitly indicated either by CP-start, CFP-End or CFP-End+Ack frames. To ensure proper traffic isolation, the CCP imposes restrictions on the initiation of frame exchange sequences, and on the lengths of TXOPs.

At QSTA that has just emerged from the power save mode, a channel access function of a given AC does not initiate a frame exchange sequence until  the next CP-start frame that initiates a CP supporting transmission of traffic of the AC. Otherwise the QSTA is aware of the last CP-start generated by the HC. At such a QSTA, a channel access function uses EDCA rules to determine when to initiate a frame exchange sequence, with the current CP, subject to the constraint that any initiated exchange should complete within the CP.

## EDCA

With CCP, the normal operation of EDCA is only modified by the allocation of multiple types of CPs. Therefore, when HCCA is also used, multiple CPs may separate two consecutive CFPs. Within a CP, traffic from the allowed ACs contend essentially according to the rules of EDCA, except that any obtained TXOP (by an STA or the HC) or  frame exchange initiated within a CP must end within the same CP. This is fundamental to ensure proper traffic isolation and delay guarantees for real-time traffic.

### Qos negotiation

The provisioning of the different CPs may be done offline based on the projected traffic load for the different ACs. However, in practice this may not be optimal if the traffic demand is not well known or changes frequently e.g. due to the mobility of the STAs. The preferred approach is to dynamically adjust the length of the CPs according to traffic parameters negotiated between the QSTAs and the HC, during a Qos negotiation phase.

### TXOPs

#### TXOPS obtained by QSTAs

An STA that obtains a TXOP within a CP is limited to transmitting frames of one of the ACs supported by the CP (i.e. the ACs that have their bit set in the last CP-start frames). As with EDCA, a QSTA is limited to transmitted frames of the same AC within a TXOP.

#### TXOPs obtained by the HC

No restriction is imposed on the traffic that may be transmitted in the TXOP, provided that the TXOP length respects the CP boundary.

### Back-off counter

Each channel access function maintains a back-off counter according to [1] (Section 9.2.5.2) and the Qos amendments described in [2]. With CCP, a channel access function of a given AC must not decrement its back-off counter in CPs where the AC bit is not set in the preceding CP-start frame. This is ensured by the setting of the duration field in the preceding CP-start frame (see Sections 5.1.1-Error: Reference source not found).

### Congestion window

Within each CP, the back-off procedure described in Section 9.9.1.5 of the 802.11e draft amendment [2] applies within each CP. The only difference with CCP is that internal collisions  are less frequent as they can only occur  among frames from different ACs that can be transmitted in the same CP.

## CP scheduling (informative)

The CCP framework enables the implementation of various Qos and bandwidth allocation policies. In the simplest case a simple Weighted Round Robin discipline (WRR) can be considered, with four CP types, where each CP is dedicated to an AC. The lengths of the different CP types are selected to meet the following constraints.

- Bandwidth requirements of the different flows.

- Delay requirements of the flows.

With the WRR scheduling, it is possible to proceed as follows.

- The length of the scheduling round (the time it takes to serve all the CP types) does not exceed the maximum delay for the most delay sensitive application to be supported.

- The length of a CP type is roughly proportional to the aggregate offered load for the CP.

- The CP length should be greater than the required time to accommodate a complete frame exchange sequence including all IFS and ACKs, for a data frame of the maximum size.

|  | Scheduling round |  |  |  | Next round |  |
| --- | --- | --- | --- | --- | --- | --- |
| CP | 1 | 2 | 3 | 4 | 5 | 6 |
| CP mask | 00001000 | 00000100 | 00000010 | 00000001 | 00001000 | 00000100 |
| Allowed ACs | VO | VI | BK | BE | VO | VI |

Figure 2: WRR CP schedule

In a variation of this allocation scheme, it is possible to proceed hierarchically as follows.

- First allocate the lengths for the real-time CPs (i.e. CPs allowing traffic from AC_VO and AC_VI) in a scheduling round, and allocate the remain time for non-real time traffic, subject to the minimum CP length constraint described above.

- Within the time allocated to real time traffic within a scheduling round, allocate the CP lengths for AC_VO and AC_VI proportionally to their offered load, subject to the minimum CP length constraint.

- Within the time allocated to non real-time traffic within a scheduling round, allocate the CP lengths for AC_BE and AC_BK proportionally to their offered load, subject to the minimum CP length constraint.

In yet another scheduling discipline, it is possible to implement precedence mechanisms by allowing traffic of a given AC in multiple CP types. This is especially desirable for AC_BE and AC_BK where a relative differentiation is preferable. An example of this approach is described in the following figure.

|  | Scheduling round |  |  |  | Next round |  |
| --- | --- | --- | --- | --- | --- | --- |
| CP | 1 | 2 | 3 | 4 | 5 | 6 |
| CP mask | 00001000 | 00000100 | 00000010 | 00000011 | 00001000 | 00000100 |
| Allowed ACs | VO | VI | BK | BK, BE | VO | VI |

Figure 3: Hierarchical CP schedule

For real time classes, to guarantee the Qos received by flows the CCP scheme should be applied with an admission control algorithm that limits the traffic offered to each CP. The admission control algorithm should be implemented by the HC during a Qos negotiation phase, based on the traffic specification of flows as they request admission into a real time CP type.

# MAC sublayer management

TBD

# Simulation results

TBD