IEEE P802.11
Wireless LANs

Class-based Contention Periods (CCP) for the 802.11n MAC

Date:	August 25, 2004

Authors:	Abel Dasylva, Z.Yao, D.Y. Montuno, W. Chen, M. Ouellette, J. Aweya, K. Felske
	Nortel Networks
	3500 Carling ave Nepean ON K2H8E9
	Phone: 613 7635034
	Fax: 613 768-1140
	e-Mail: abel@nortelnetworks.com

Abstract:

A Class-based Contention Period (CCP) scheme is proposed for a simple and effective QoS allocation over wireless LANs. It is a natural extension of EDCA with multiple contention periods, where access to the medium is restricted to a subset of the Access Categories (ACs). Within an Explicit Contention Period (ECP), access to the medium is obtained through contention among the frames from allowed classes, as with EDCA contention periods. However, the scheduling of the successive contention periods is done by the Hybrid Coordinator (HC). ECPs are advertised to stations, through specific control frames transmitted by the HC. CCP enables a better protection of real-time traffic from lower priority frames, for up-stream and down-stream traffic.

Content

1.	References	3

2.	Definitions	4

3.	Abbreviations and acronyms	4

4.	Introduction	7

4.1.	Purpose	7

4.2.	General description of MAC enhancements	7

5.	Frame formats	7

5.1.	Control frames	7

5.1.1.	ECP-Start	7

5.1.2.	ECP-End	8

5.1.3.	ECP-End+ECP-Start	8

5.1.4.	ECP-Access Request	9

5.1.5.	ECP-Access-Request-ACK	9

5.1.6.	ECP-Access Response MPDU	10

5.2.	Data frames	11

5.3.	Management frames	11

5.3.1.	ECP capability element	11

5.3.2.	ECP parameters element	12

6.	MAC sublayer functional description	12

6.1.	Allocation of ECPs	12

6.2.	Channel access within ECPs	14

6.2.1.	Single data frames	14

6.2.2.	Aggregate data frames	14

6.3.	Channel access within LCPs	14

6.4.	Interaction with the power-save function	14

6.5.	ECP scheduling (informative)	15

6.6.	QoS negotiation (informative)	16

7.	MAC sublayer management	16

7.1.	ECP capability	16

7.2.	QoS negotiation	16

7.3.	IBSS operation	17

7.4.	Coexistence with legacy devices	17

8.	Results for Mandatory Comparison Criteria and FR	17

8.1.	Summary of Configurations	17

8.1.1.	MAC 1x1x20	17

8.2.	Results relating to MAC simulations	18

8.3.	MAC Comparison Criteria not related to simulation results	19

8.4.	CC46 – Baseline options	19

8.5.	CC47	19

9.	Appendix	20

9.1.	Usage models	20

9.1.1.	Usage model 4	20

9.1.2.	Usage model 6	21

9.1.3.	Usage model 1	23

Tables

Table 1: Fields for the ECP-Start control frame	8

Table 2: Fields for the ECP-End control frame	8

Table 3: Fields for the ECP-Start+ECP-End control frame	9

Table 4: Fields in the ECP-Access-Request control frame	9

Table 5: Fields in the ECP-Access-Request-ACK control frame	10

Table 6: Fields in the ECP capability information element	12

Table 7: Traffic classification	18

Table 8: QoS flows for UM4	20

Table 9: Non-QoS flows for UM4	21

Table 10: QoS flows for UM6	23

Table 11: Non-QoS flows for UM6	23

Table 12: QoS flows for UM1	24

Table 13: Non-QoS flows for UM1	24

Figures

Figure 1: ECP-Start control frame	7

Figure 2: ECP-End control frame	8

Figure 3: ECP-Start+ECP-End control frame	8

Figure 4: ECP-Access-Request control frame	9

Figure 5: ECP-Access request ACK control frame	10

Figure 6: ECP-Access response control frame	10

Figure 7: ECP capability information element	11

Figure 8: ECP parameter information element	12

Figure 9: ECP allocation: two consecutive ECPs separated by a PIFS	13

Figure 10: ECP allocation: two consecutive ECPs separated by an ECP-Start+ECP-End frame	13

Figure 11: ECP allocation: ECP followed by an LCP	13

Figure 12: WRR ECP schedule	15

Figure 13: Hierarchical ECP schedule	16

# References

- IEEE Std 802.11®-1999 (Reaff 2003)
Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- IEEE P802.11e/D8.0, February 2004
(Draft Amendment to IEEE Std 802.11, 1999 Edition (Reaff 2003))
Medium Access Control (MAC) Enhancements for Quality of Service (QoS)

- Project Authorization Request  for IEEE 802.11n.
11-02-798r7-HT-SG-Draft-PAR.doc

- IEEE Std 802.11®-1999 (Reaff 2003)
Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

- IEEE Std 802.11®-1999
High-speed Physical Layer in 5 GHz Band

# Definitions

- CCP: a refinement of contention period in different types for traffic isolation and bandwidth control

- CP Length: Length of a contention period

- CP Type: Indicates the subset of access categories that are eligible to contend in this period

- Explicit CP: A contention period explicitly advertised by the hybrid coordinator, which allows a restricted number of ACs to contend.

- Legacy CP: A contention period that is not explicitly advertised by the hybrid coordinator, and where legacy STAs (with or without QoS functionality may contend according to DCF).

- Maximum ECP-length: Maximum length of any explicitly allocated CPs.

# Abbreviations and acronyms

| Term | Description |
| --- | --- |
| AC | Access Category |
| ACK | Acknowledgement |
| AGC | Automatic Gain Control |
| AIFS | Arbitration IFS |
| AP | Access Point |
| AP | Access Point |
| BSS | Basic Service Set |
| CAC | Call Admission Control |
| CAP | Controlled Access Phase |
| CCP | Class-based Contention Periods |
| CP | Contention Period |
| CPL | Contention Period Length |
| CPT | Contention Period Type |
| CTS | Clear To Send |
| CTS | Clear To Send |
| DCF | Distributed Coordination Function |
| DS | Distribution System |
| EDCA | Enhanced Distributed Channel Access |
| EDCF | Enhanced DCF |
| FCS | Frame Check Sequence |
| FDM | Frequency Division Multiplexing |
| FSM | Finite State Machine |
| HC | Hybrid Coordinator |
| HCCA | HCF Controlled Channel Access |
| HCF | Hybrid Coordination Function |
| HT | High Throughput |
| IE | Information Element |
| IFS | Inter Frame Spacing |
| LLC | Logical Link Control |
| MAC | Medium Access Controller |
| MPDU | MAC protocol data unit |
| MPDU | MAC PDU |
| MSDU | MAC service data unit |
| MSDU | MAC SDU |
| MU | Mobile Unit |
| NAV | Network Allocation Vector (a MAC-level carrier-sense) |
| OFDM | Orthogonal Frequency Division Multiplexing |
| OFDM | Orthogonal FDM |
| PDU | Protocol Data Unit |
| PHY | Physical Layer |
| PLCP | PHY layer convergence protocol |
| PPDU | PHY protocol data unit |
| PSDU | PHY service data unit |
| Pure Mode | A mode of operation of a BSS in which there are no legacy devices |
| HT AP | QoS access point |
| QoS | Quality of Service |
| HT STA | QoS Station |
| RA | Receiver Address |
| RTS | Request to send |
| RX | Receiver |
| SAP | Service Access Point |
| SDU | Service Data Unit |
| SIFS | Short inter-frame spacing |
| SoP | Start of Packet (i.e. PPDU Packet) |
| STA | Station |
| TA | Transmitter Address |
| TC | Traffic Category |
| TGe | 802.11 Task Group e - Quality of Service |
| TGn | 802.11 Task Group n - Enhancements for Higher Throughput |
| TS | Traffic Stream |
| TSID | Traffic Stream Identifier |
| TSPEC | Traffic specification |
| TX | Transmitter |
| TXOP | Transmission opportunity |
| TXOP | Transmission Opportunity |
| UP | User Priority |
| WM | Wireless Medium |
| ECP | Explicit Contention Period |
| LCP | Legacy Contention Period |

# Introduction

## Purpose

- This document defines proposed enhancements for higher throughput within the scope of the Project Authorization Request (PAR) as formulated for IEEE 802.11 Task Group n.

## General description of MAC enhancements

- The Class-based Contention Periods scheme provide a simple and effective way to provide QoS in wireless LANs. It is obtained by simple extensions of EDCA with explicitly allocated contention periods of different types. Within an Explicit Contention Period (ECP), channel access is based on contention as with EDCA for a restricted number of ACs. Like CFPs, ECPs are allocated and scheduled by the hybrid coordinator. They start with ECP-Start frames and end with ECP-End or ECP-End+ECP-Start frames.

# Frame formats

Three new control frames are defined, ECP-Start, ECP-End and ECP-Start+ECP-End frames to respectively announce the beginning, the end and simultaneous start of two consecutive ECPs.

## Control frames

### ECP-Start

| Octets: 2 | 2 | 6 | 6 |  | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | BSSID |  | ECP type | FCS |

Figure 1: ECP-Start control frame

The different fields are described in the following table.

| Field | Size (bytes) | Purpose |
| --- | --- | --- |
| Frame Control | 2 | See [1][2] |
| Duration | 2 | It is set to the duration of the next CP |
| RA | 6 | Broadcast group address |
| BSSID | 6 | See [1][2] |
| ECP type | 1 | Bit mask where AC n corresponds to bit n (if the AC mask is denoted by b7…b0). Four bits are reserved (b4 to b7) for the definition of additional TCs |
| FCS | 4 | See [1][2] |

Table 1: Fields for the ECP-Start control frame

### ECP-End

This frame announces the end of the current ECP, and the beginning of an LCP.

| Octets: 2 | 2 | 6 | 6 |  | 4 |
| --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | BSSID |  | FCS |

Figure 2: ECP-End control frame

The different fields are described in the following table.

| Field | Size (bytes) | Purpose |
| --- | --- | --- |
| Frame Control | 2 | See [1][2] |
| Duration | 2 | Duration of the frame |
| RA | 6 | Broadcast group address |
| BSSID | 6 | See [1][2] |
| FCS | 4 | See [1][2] |

Table 2: Fields for the ECP-End control frame

### ECP-End+ECP-Start

This frames combines the functions of the ECP-Start and ECP-End frames. It enables a reduction of the number of ECP control frames when multiple consecutive ECPs are allocated.

| Octets: 2 | 2 | 6 | 6 |  | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | BSSID |  | ECP type | FCS |

Figure 3: ECP-Start+ECP-End control frame

The different fields are described in the following table.

| Field | Size (bytes) | Purpose |
| --- | --- | --- |
| Frame Control | 2 | See [1][2] |
| Duration | 2 | Duration of the next ECP |
| RA | 6 | Broadcast group address |
| BSSID | 6 | See [1][2] |
| ECP type | 1 | Indicates the type of the next ECP |
| FCS | 4 | See [1][2] |

Table 3: Fields for the ECP-Start+ECP-End control frame

### ECP-Access Request

This frame is sent by a HT STA to the HT AP to request access to one or more ECP types, for which the access mode is through QoS negotiation.

| Octets: 2 | 2 | 6 |  | 6 | 1 |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | BBSID |  | TA | ECP type 1 | TSPEC 1 |  | ECP type n | TSPEC n |

4

FCS

Figure 4: ECP-Access-Request control frame

| Field | Size (bytes) | Purpose |
| --- | --- | --- |
| Frame Control | 2 | See [1][2] |
| Duration | 2 | Frame duration |
| TA | 6 | Address of the requesting HT STA |
| BSSID | 6 | See [1][2] |
| ECP type n | 1 | Type of ECP to which access is requested |
| TSPEC n | See [1][2] | TSPEC of the traffic offered for ECP type n |
| FCS | 4 | See [1][2] |

Table 4: Fields in the ECP-Access-Request control frame

### ECP-Access-Request-ACK

This frame is sent by the HT AP to an HT STA , to acknowledge the receipt of an ECP-Access Request frame and to inform the HT STA of the number that has been assigned to an access request.

| Octets: 2 | 2 | 6 | 6 |  | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | BBSID |  | Request number | FCS |

Figure 5: ECP-Access request ACK control frame

| Field | Size (bytes) | Purpose |
| --- | --- | --- |
| Frame Control | 2 | See [1][2] |
| Duration | 2 | Frame duration |
| RA | 6 | Address of the HT STA that has issued the ECP- access request |
| BSSID | 6 | See [1][2] |
| Request number | 1 | Number that has been assigned to the request |
| FCS | 4 | See [1][2] |

Table 5: Fields in the ECP-Access-Request-ACK control frame

### ECP-Access Response MPDU

This frame is sent by the HT AP, in response to an ECP-Access Request frame.

| Bits: 16 | 16 | 48 | 48 |  |
| --- | --- | --- | --- | --- |
| Frame Control | Duration | RA | BSSID |  |

| 48 | 48 | 1 |  | 48 | 1 |
| --- | --- | --- | --- | --- | --- |
| Request 1 | ECP type 1 | Resp 1 |  | ECP type n_1 | Resp n_1 |

| 48 | 48 | 1 |  | 48 | 1 |
| --- | --- | --- | --- | --- | --- |
| Request m | ECP type 1 | Resp 1 |  | ECP type n_m | Resp n_m |

32

FCS

Figure 6: ECP-Access response control frame

| Field | Size (bits) | Purpose |
| --- | --- | --- |
| Frame Control | 16 | See [1][2] |
| Duration | 16 | Frame duration |
| TA | 48 | Address of the requesting HT STA |
| BSSID | 48 | See [1][2] |
| ECP type n | 8 | Type of ECP to which access is requested |
| Resp n | 1 | Bit indicating whether access has been granted (bit set to one) for the requested ECP type. |
| FCS | 32 | See [1][2] |

## Data frames

None

## Management frames

### ECP capability element

The ability to support ECPs is indicated by an ECP capability information element included in the following management frames.

- Association requests by STAs: to indicate that the HT STA can intrepret ECP-Start, ECP-End, and ECP-Start+ECP-End frames.

- Association response by APs: to advertise the ECP feature to HT STAs.

The ECP capability information element has the following format.

| Bits: 8 | 8 | 1 | 7 | 8 |
| --- | --- | --- | --- | --- |
| Element id | Length | ECP capability | ECP length | Num. ECP types |

Figure 7: ECP capability information element

| Field | Size (bits) | Purpose |
| --- | --- | --- |
| Element id | 2 | See [1][2] |
| Length | 2 | See [1][2] |
| ECP capability | 1 | Bit indicating whether the STA supports ECPs |
| ECP length | 7 | The maximum length of any ECP in milliseconds. For management frames sent by non AP stations, this field is ignored by the AP. |
| Number of ECP types |  | Bit mask indicating the access mode for each ECP type. Bit n corresponds to ECP n. When the bit is set, access to the ECP type requires preliminary QoS negotiation with the AP. |

Table 6: Fields in the ECP capability information element

### ECP parameters element

Information element indicating the parameters of an ECP type.

| Bits: 8 | 8 | 8 | 1 | 8 |
| --- | --- | --- | --- | --- |
| Element id | Length | ECP type | Mode | AC mask |

Figure 8: ECP parameter information element

| Field | Size (bits) | Purpose |
| --- | --- | --- |
| Element id | 16 | See [1][2] |
| Length | 16 | See [1][2] |
| ECP type | 8 | Index of the ECP type |
| Access mode | 1 | Bit indicating whether default access mode (value 0) or access through QoS negotiation |
| AC mask | 8 | Bit mask indicating the ACs that are allowed. AC n is allowed if and only bit n is set to one |

# MAC sublayer functional description

With CCP,  the start of an ECP is explicitly indicated either by an ECP-start or a ECP-End+ECP-Start frame. Its end is indicated by an ECP-End or an ECP-End+ECP-Start frame. With the CCP framework, to ensure proper traffic isolation, some restrictions are imposed on the initiation of frame exchange sequences, and on the lengths of TXOPs.

## Allocation of ECPs

ECPs are allocated by the HC according to a QoS allocation policy set by the operator. To start a new ECP, the HC sends an ECP-start or an ECP-Start+ECP-End frame, with the duration field of the frame set to the length of the allocated ECP. The set of ACs that are allowed channel access are set in the ECP mask field of the frame announcing the ECP. An ECP has a maximum length not exceeding a value given by MAX_ECP_LENGTH with a recommended value of 5ms. An ECP-start frame may be transmitted as follows:

- At the end of a CFP: A PIFS interval after the completion of a CFP indicated by a CFP-End frame.

- At the end of an ECP: a PIFS interval after the completion of an ECP indicated by an ECP-End frame.

- In LCPs: A PIFS interval after the completion of any frame transmission.

| CFP |  |  | Overall CP |  |  |  |  |  |  |  | Next CFP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PIFS | ECP 1 |  |  | PIFS | ECP 2 |  |  |  |  |
| Beacon |  | CFP-End |  | ECP-Start |  | ECP-End |  | ECP-Start |  | ECP-End | Beacon |  |
| CFP repetition interval |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 9: ECP allocation: two consecutive ECPs separated by a PIFS

| CFP |  |  | Overall CP |  |  |  |  |  |  | Next CFP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PIFS | ECP 1 |  |  | ECP 2 |  |  |  |  |
| Beacon |  | CFP-End |  | ECP-Start |  | ECP-End+ ECP-Start |  |  | ECP-End | Beacon |  |
| CFP repetion interval |  |  |  |  |  |  |  |  |  |  |  |

Figure 10: ECP allocation: two consecutive ECPs separated by an ECP-Start+ECP-End frame

| CFP |  |  | Overall CP |  |  |  |  |  |  |  | Next CFP |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PIFS | ECP |  |  | LCP |  |  |  |  |  |
| Beacon |  | CFP-End |  | ECP-Start |  | ECP-End |  |  |  |  | Beacon |  |
| CFP repetition interval |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 11: ECP allocation: ECP followed by an LCP

## Channel access within ECPs

### Single data frames

Within an ECP, channel access functions from the allowed ACs contend essentially according to EDCA rules with the following minor modifications.

- A channel access function initiates a frame exchange sequence only if there is enough time to complete the exchange within the current ECP, including IFS and acknowledgments.

- Any TXOP obtained by a channel access function must complete with the ECP, and can only be used to transmit frames of a same AC allowed within the ECP.

- The HCCA function cannot obtain access the channel within an ECP.

### Aggregate data frames

Frame aggregation is desirable to better amortize the Preamble+PLCP PHY overhead, and to enhance the throughput efficiency at the MAC SAP. The benefit of this technique increases with the number of MSDUs that are aggregated in PPDUs. However to ensure proper traffic isolation, the within an ECP, an aggregate frame is restricted to carry MPDUs belonging to channel access functions that are allowed by the ECP type, i.e.

- In the case of an ECP type with no required QoS negotiation: frames from the ACs that are allowed by the ECP

- Otherwise: frames produced by channel access functions of appropriate ACs that have been granted access to the ECP type by the HC through QoS negotiation.

## Channel access within LCPs

The beginning of an LCP is indicated by any of the following control frames: CFP-End, ECP-End. During an LCP channel access is according to EDCA [2]. Then all HT STAs including legacy HT STAs (that do not have the capability to detect ECPs) may contend. As before  the HCCA function may obtain polled TXOPs within LCPs. LCPs are not explicitly limited in duration, but they end with the beginning of the next CFP or ECP.

## Interaction with the power-save function

Consider a HT STA capable of interpreting ECP-Start, ECP-End and ECP-Start+ECP-End frame. When such HT STA emerges from power-save mode it has no record of any ongoing ECP or LCP, it should reset an ECP-length timer (ECP_LENGTH_TIME) to a specified value exceeding the maximum CFP length, and the maximum ECP length. Then the HT STA should freeze (no update of backoff-counters, of congestion windows, no initiation of frame exchange sequence) all its channel access functions until one of the following events occurs.

- One of the following frame is received (before timer expiration): ECP-Start, ECP-End, ECP-Start+ECP-End, Beacon frame, CFP-End, or CFP-End+CFP-Start frame:Then the HT STA obtains a record of the  next ECPs, LCPs and CFPs and its sets the states of its channel access functions accordingly.

- ECP-length timer expiration: the HT STA has emerged from power save mode in an LCP and it sets its channel access functions to the channel access rules of an LCP, i.e. EDCA rules.

## ECP scheduling (informative)

The CCP framework enables the implementation of various QoS and bandwidth allocation policies. In the simplest case a simple Weighted Round Robin discipline (WRR) can be considered, with four ECP types, where each ECP is dedicated to an AC. The lengths of the different ECP types are selected to meet the following constraints.

- Bandwidth requirements of the different flows.

- Delay requirements of the flows.

With the WRR scheduling, it is possible to proceed as follows.

- The length of the scheduling round (the time it takes to serve all the ECP types) does not exceed the maximum delay for the most delay sensitive application to be supported.

- The length of a ECP type is roughly proportional to the aggregate offered load for the ECP.

- The ECP length should be greater than the required time to accommodate a complete frame exchange sequence including all IFS and ACKs, for a data frame of the maximum size.

|  | Scheduling round |  |  |  | Next round |  |
| --- | --- | --- | --- | --- | --- | --- |
| ECP | 1 | 2 | 3 | 4 | 5 | 6 |
| ECP mask | 00001000 | 00000100 | 00000010 | 00000001 | 00001000 | 00000100 |
| Allowed ACs | VO | VI | BK | BE | VO | VI |

Figure 12: WRR ECP schedule

In a variation of this allocation scheme, it is possible to proceed hierarchically as follows.

- First allocate the lengths for the real-time ECPs (i.e. ECPs allowing traffic from AC_VO and AC_VI) in a scheduling round, and allocate the remain time for non-real time traffic, subject to the minimum ECP length constraint described above.

- Within the time allocated to real time traffic within a scheduling round, allocate the ECP lengths for AC_VO and AC_VI proportionally to their offered load, subject to the minimum ECP length constraint.

- Within the time allocated to non real-time traffic within a scheduling round, allocate the ECP lengths for AC_BE and AC_BK proportionally to their offered load, subject to the minimum ECP length constraint.

In yet another scheduling discipline, it is possible to implement precedence mechanisms by allowing traffic of a given AC in multiple ECP types. This is especially desirable for AC_BE and AC_BK where a relative differentiation is preferable. An example of this approach is described in the following figure.

|  | Scheduling round |  |  |  | Next round |  |
| --- | --- | --- | --- | --- | --- | --- |
| ECP | 1 | 2 | 3 | 4 | 5 | 6 |
| ECP mask | 00001000 | 00000100 | 00000010 | 00000011 | 00001000 | 00000100 |
| Allowed ACs | VO | VI | BK | BK, BE | VO | VI |

Figure 13: Hierarchical ECP schedule

For real time classes, to guarantee the QoS received by flows the CCP scheme should be applied with an admission control algorithm that limits the traffic offered to each ECP. The admission control algorithm should be implemented by the HC during a QoS negotiation phase, based on the traffic specification of flows as they request admission into a real time ECP type.

## QoS negotiation (informative)

The provisioning of the different CPs may be done offline based on the projected traffic load for the different ACs. However, in practice this may not be optimal if the traffic demand is not well known or changes frequently e.g. due to the mobility of the HT STAs. The preferred approach is to dynamically adjust the length of the ECPs according to traffic parameters negotiated between the HT STAs and the HC, through QoS negotiation.

# MAC sublayer management

## ECP capability

An HT AP indicates the ECP capability by including an ECP capability information element in management frames (e.g. association response frames), with the ECP length field set to the maximum ECP length. For each ECP type the HT AP also indicates the mode, and the allowed ACs in the AC mask field in the corresponding ECP information elements.

An HT STA notifies the HT AP about its ability to interpret ECP frames (i.e. ECP-Start, ECP-End and ECP-Start+ECP-End frames) by including an ECP capability information element in association response frames.

## QoS negotiation

Each ECP type has its access mode that defines how a HT STA gains access to the channel during ECPs of this type. The default mode allows access without preliminary QoS negotiation with the AP. In this mode, within an ECP that allows access to a given AC, all channel access functions from the AC may contend. The other ECP access mode requires  a preliminary QoS negotiation phase that proceeds in three steps.

- The HT STA sends a control frame to the HT AP, to request access to the ECP type. This frame contains information about the traffic to be offered to the ECP type, such as the load, maximum delay.

- Based on the traffic information, the HT AP decides to admit or reject the HT STA to the requested ECP type. Although, this involves an admission control algorithm. The specification of this algorithm is beyond the scope of this document.

- The HT AP sends a control frame to inform the HT STA of the admission decision.

Access to an ECP type is done through an ECP-Access-Request frame. In response to ECP-Access request, the HT AP sends ECP-Access-Response frames. ECP-Access (request or response) frames may be sent in any ECP or LCP.

## IBSS operation

ECPs cannot be allocated in IBSS mode since they require a HC.

## Coexistence with legacy devices

ECPs are implemented in a manner compatible with EDCA. HT STAs that do not have the ECP capability access the channel during CFPs or LCPs, and are ensured channel access  through proper scheduling by the HC. However these legacy HT STAs do not access the channel during ECPs, due to the setting of the duration field in ECP-Start, ECP-End or ECP-End+ECP-Start frames.

# Results for Mandatory Comparison Criteria and FR

## Summary of Configurations

### MAC 1x1x20

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 1 |
| Number of Rx Antennae | 1 |
| Nominal Channel Width | 20 MHz |
| PHY Options enabled | None |
| TCP Model | Reno |
| Channel Access Method | EDCA and CCP Note: the channel access method used is shown in the CC results table using colour coding. |

| AC | Application | Application number |
| --- | --- | --- |
| AC_BE | Content download | A13 |
|  | Internet File transfer | A14 |
| AC_BK | VoD control channel | A2 |
|  | Local File transfer, printing | A15 |
|  | Remote user interface | A24 |
| AC_VI | DV Audio/video | A1 |
|  | SDTV | A3 |
|  | HDTV (Video/Audio) | A4 |
|  | DVD | A5 |
|  | Video Conf | A6 |
|  | Internet Streaming video/audio | A7 |
|  | Internet Streaming audio | A8 |
|  | MP3 Audio | A12 |
|  | Interactive Gaming | A16 |
|  | Interactive Gaming | A17 |
|  | Interactive Gaming | A18 |
|  | Netmeeting application/desktop sharing | A19 |
|  | Video phone | A23 |
| AC_VO | VoIP | A9 |

Table 7: Traffic classification

## Results relating to MAC simulations

Preliminary results have been obtained assuming the 802.11a PHY at 54Mbps. In this case the network is clearly in an overload for all three scenarios, even when the traffic demands are slightly reduced (for many applications, the usage model document specifies a minimum and maximum characteristic offered load, in this cases we systematically consider the lower value). Nonetheless, the allocation of ECPs of appropriate lengths succeeds in protecting the real-time classes from the non-real time traffic. In a coming set of results the demand is to be further scaled down to match the effective bandwidth available with a 54Mbps 802.11a PHY. Additional information of the simulation scenarios is provided in the appendix.

| CC# | Name | Result | CCP |
| --- | --- | --- | --- |
| CC3 | List of goodput results for usage models 1, 4 and 6. | SS1 (Mbps) |  |
|  |  | SS4 |  |
|  |  | SS6 |  |
| CC15(?) | Sharing of medium with legacy devices | T1 (Mbps) |  |
|  |  | T2 (Mbps) |  |
|  |  | T3 (Mbps) |  |
|  |  | T4 (Mbps) |  |
| CC18 | HT Usage Models Supported (non QoS) | SS1 (Mbps/ratio) |  |
|  |  | SS4 |  |
|  |  | SS6 |  |
| CC19 | HT Usage Models Supported (QoS) | SS1 |  |
|  |  | SS4 |  |
|  |  | SS6 |  |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | SS1 |  |
|  |  | SS4 |  |
|  |  | SS6 |  |
| CC24 | MAC Efficiency | SS1 |  |
|  |  | SS4 |  |
|  |  | SS6 |  |

## MAC Comparison Criteria not related to simulation results

| CC# | Name | Disclosure |
| --- | --- | --- |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g |  |
| CC46 | MAC Compatibility and parameters. |  |
| CC47 | MAC extensions |  |

## CC46 – Baseline options

"Provide a list of optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used.  Include references to the sections in the technical proposal document where the complete details are given."

IEEE 802.11e

| Item | Feature | Brief Description | Sections in [?] |
| --- | --- | --- | --- |

## CC47

"Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given."

| Feature | Brief Description | Section in [?] |
| --- | --- | --- |
| Contention Period refinement | Allows for further definition of contention period |  |

# Appendix

## Usage models

Three network models have been developed, one for each usage model scenario.

### Usage model 4

Usage model 4 corresponds to the large enterprise case and includes 31 STAs including the AP.

| AC | CP length (ms) |
| --- | --- |
| AC_VO |  |
| AC_VI |  |
| AC_BK |  |
| AC_BE |  |

| Flow | Source | Destination | Application | Offered load (Mbps) | Thr (Mbps) | PLR(%) | Delay (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AP | STA 7 | Video conf | 0.128 |  |  |  |
| 2 | AP | STA 8 | Video conf | 0.128 |  |  |  |
| 3 | AP | STA 9 | MP3 | 0.064 |  |  |  |
| 4 | AP | STA 10 | MP3 | 0.064 |  |  |  |
| 5 | AP | STA 25 | VoIP | 0.096 |  |  |  |
| 6 | AP | STA 26 | VoIP | 0.096 |  |  |  |
| 7 | AP | STA 27 | VoIP | 0.096 |  |  |  |
| 8 | AP | STA 28 | VoIP | 0.096 |  |  |  |
| 9 | AP | STA 29 | VoIP | 0.096 |  |  |  |
| 10 | AP | STA 30 | VoIP | 0.096 |  |  |  |
| 11 | STA 7 | AP | Video conf | 0.128 |  |  |  |
| 12 | STA 8 | AP | Video conf | 0.128 |  |  |  |
| 13 | STA 25 | AP | VoIP | 0.096 |  |  |  |
| 14 | STA 26 | AP | VoIP | 0.096 |  |  |  |
| 15 | STA 27 | AP | VoIP | 0.096 |  |  |  |
| 16 | STA 28 | AP | VoIP | 0.096 |  |  |  |
| 17 | STA 29 | AP | VoIP | 0.096 |  |  |  |
| 18 | STA 30 | AP | VoIP | 0.096 |  |  |  |

Table 8: QoS flows for UM4

| Flow | Source | Destination | Application | Offered load (Mbps) | Thr (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 19 | AP | STA 1 | Internet file | 1 |  |
| 20 | AP | STA 2 | Internet file | 1 |  |
| 21 | AP | STA 3 | Internet file | 1 |  |
| 22 | AP | STA 4 | Internet file | 1 |  |
| 23 | AP | STA 5 | Internet file | 1 |  |
| 24 | AP | STA 6 | Internet file | 1 |  |
| 25 | AP | STA 11 | Local File transfer | 30 |  |
| 26 | AP | STA 12 | Local File transfer | 30 |  |
| 27 | AP | STA 13 | Local File transfer | 30 |  |
| 28 | AP | STA 14 | Local File transfer | 30 |  |
| 29 | AP | STA 15 | Local File transfer | 30 |  |
| 30 | AP | STA 16 | Local File transfer | 30 |  |
| 31 | AP | STA 17 | Local File transfer | 30 |  |
| 32 | AP | STA 18 | Local File transfer | 30 |  |
| 33 | AP | STA 19 | Local File transfer | 30 |  |
| 34 | AP | STA 20 | Local File transfer | 30 |  |
| 35 | STA 1 | AP | Internet file transfer | 1 |  |
| 36 | STA 2 | AP | Internet file transfer | 1 |  |
| 37 | STA 3 | AP | Internet file transfer | 1 |  |
| 38 | STA 4 | AP | Internet file transfer | 1 |  |
| 39 | STA 5 | AP | Internet file transfer | 1 |  |
| 40 | STA 6 | AP | Internet file transfer | 1 |  |
| 41 | STA 21 | AP | Local file transfer | 30 |  |
| 42 | STA 22 | AP | Local file transfer | 30 |  |
| 43 | STA 23 | AP | Local file transfer | 30 |  |
| 44 | STA 24 | AP | Local file transfer | 30 |  |

Table 9: Non-QoS flows for UM4

### Usage model 6

Usage model 6 considers the case of a hot spot with 35 STAs including the AP. The VoIP traffic imposes the most stringent delay requirement of 30ms. Therefore with WRR the length of the scheduling round should not exceed 30ms. The ECP lengths are set as follows.

| AC | CP length (ms) |
| --- | --- |
| AC_VO |  |
| AC_VI |  |
| AC_BK |  |
| AC_BE |  |

| Flow | Source | Destination | Application | Offered load (Mbps) | Thr (Mbps) | PLR (%) | Delay (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AP | STA 11 | Streaming audio/video | 0.1 |  |  |  |
| 2 | AP | STA 12 | Streaming audio/video | 0.1 |  |  |  |
| 3 | AP | STA 13 | Streaming audio/video | 0.1 |  |  |  |
| 4 | AP | STA 14 | Streaming audio/video | 0.1 |  |  |  |
| 5 | AP | STA 15 | Streaming audio/video | 0.1 |  |  |  |
| 6 | AP | STA 16 | Streaming audio/video | 0.1 |  |  |  |
| 7 | AP | STA 17 | Streaming audio/video | 0.1 |  |  |  |
| 8 | AP | STA 18 | SDTV | 4 |  |  |  |
| 9 | AP | STA 19 | SDTV | 4 |  |  |  |
| 10 | AP | STA 20 | VoIP | 0.096 |  |  |  |
| 11 | AP | STA 21 | VoIP | 0.096 |  |  |  |
| 12 | AP | STA 22 | VoIP | 0.096 |  |  |  |
| 13 | AP | STA 23 | VoIP | 0.096 |  |  |  |
| 14 | AP | STA 24 | VoIP | 0.096 |  |  |  |
| 15 | AP | STA 25 | VoIP | 0.096 |  |  |  |
| 16 | AP | STA 26 | VoIP | 0.096 |  |  |  |
| 17 | AP | STA 27 | VoIP | 0.096 |  |  |  |
| 18 | AP | STA 28 | VoIP | 0.096 |  |  |  |
| 19 | AP | STA 29 | VoIP | 0.096 |  |  |  |
| 20 | AP | STA 30 | VoIP | 0.096 |  |  |  |
| 21 | AP | STA 31 | VoIP | 0.096 |  |  |  |
| 22 | AP | STA 32 | VoIP | 0.096 |  |  |  |
| 23 | AP | STA 33 | VoIP | 0.096 |  |  |  |
| 24 | AP | STA 34 | VoIP | 0.096 |  |  |  |
| 25 | STA 20 | AP | VoIP | 0.096 |  |  |  |
| 26 | STA 21 | AP | VoIP | 0.096 |  |  |  |
| 27 | STA 22 | AP | VoIP | 0.096 |  |  |  |
| 28 | STA 23 | AP | VoIP | 0.096 |  |  |  |
| 29 | STA 24 | AP | VoIP | 0.096 |  |  |  |
| 30 | STA 25 | AP | VoIP | 0.096 |  |  |  |
| 31 | STA 26 | AP | VoIP | 0.096 |  |  |  |
| 32 | STA 27 | AP | VoIP | 0.096 |  |  |  |
| 33 | STA 28 | AP | VoIP | 0.096 |  |  |  |
| 34 | STA 29 | AP | VoIP | 0.096 |  |  |  |
| 35 | STA 30 | AP | VoIP | 0.096 |  |  |  |
| 36 | STA 31 | AP | VoIP | 0.096 |  |  |  |
| 37 | STA 32 | AP | VoIP | 0.096 |  |  |  |
| 38 | STA 33 | AP | VoIP | 0.096 |  |  |  |
| 39 | STA 34 | AP | VoIP | 0.096 |  |  |  |

Table 10: QoS flows for UM6

| Flow | Source | Destination | Application | Offered load (Mbps) | Troughput (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 40 | AP | STA 1 | Internet file transfer | 1 |  |
| 41 | AP | STA 2 | Internet file transfer | 1 |  |
| 42 | AP | STA 3 | Internet file transfer | 1 |  |
| 43 | AP | STA 4 | Internet file transfer | 1 |  |
| 44 | AP | STA 5 | Internet file transfer | 1 |  |
| 45 | AP | STA 6 | Internet file transfer | 1 |  |
| 46 | AP | STA 7 | Internet file transfer | 1 |  |
| 47 | AP | STA 8 | Internet file transfer | 1 |  |
| 48 | AP | STA 9 | Internet file transfer | 1 |  |
| 49 | AP | STA 10 | Internet file transfer | 1 |  |

Table 11: Non-QoS flows for UM6

### Usage model 1

The first usage model concerns the residential case where multimedia traffic is dominant. The lengths of the different ECPs are set as follows.

| AC | CP length (ms) |
| --- | --- |
| AC_VO |  |
| AC_VI |  |
| AC_BK |  |
| AC_BE |  |

| Flow | Source | Destination | Application | Offered load (Mbps) | Troughput (Mbps) | PLR | Delay (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | AP | STA 1 | HDTV |  |  |  |  |
| 2 | AP | STA 3 | HDTV |  |  |  |  |
| 3 | AP | STA 4 | SDTV |  |  |  |  |
| 4 | AP | STA 7 | VoIP |  |  |  |  |
| 5 | AP | STA 8 | VoIP |  |  |  |  |
| 6 | AP | STA 9 | VoIP |  |  |  |  |
| 7 | AP | STA 10 | Internet streaming audio |  |  |  |  |
| 8 |  | STA 11 | MP3 audio |  |  |  |  |
| 9 | STA 1 | AP | VoD control |  |  |  |  |
| 10 | STA 3 | AP | VoD control |  |  |  |  |
| 11 | STA 5 | STA 6 | Video phone |  |  |  |  |
| 12 | STA 6 | STA 5 | Video phone |  |  |  |  |
| 13 | STA 7 | AP | VoIP |  |  |  |  |
| 14 | STA 8 | AP | VoIP |  |  |  |  |
| 15 | STA 9 | AP | VoIP |  |  |  |  |
| 16 | STA 10 | AP | Console to Internet |  |  |  |  |
| 17 | STA 11 | STA 10 | Controller to console |  |  |  |  |
| Aggregate |  |  |  |  |  |  |  |

Table 12: QoS flows for UM1

| Flow | Source | Destination | Application | Offered load (Mbps) | Troughput (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 18 | AP | STA 4 | Internet file transfer |  |  |
| 19 | STA 4 | STA 10 | Local file transfer |  |  |
| Aggregate |  |  |  |  |  |

Table 13: Non-QoS flows for UM1

Although 4 ACs are currently defined, up to 8 TCs may be specified in the mask.