IEEE P802.11
Wireless LANs

TGn Sync Proposal PHY Results

Date:	August 13, 2004

Author:	Syed Aon Mujtaba
	Agere Systems

555 Union Boulevard

Allentown, Pennsylvania 18109, U.S.A.
	Phone: +1 610 712 6616

Cell: +1 908 342 5251
	e-Mail: mujtaba@agere.com

Abstract

This document contains simulation results for the TGn Sync proposal.  These simulations establish compliance with CC59 and CC67.  Additional simulations are provided to highlight the potential performance gains of various features of the TGn Sync proposal.

Table of Contents

1	Introduction	6

1.1	References	6

1.2	Scope	6

2	CC59 Smulations	6

2.1	Results for 20 MHz Channels	7

2.2	Results for 40 MHz Channels	9

3	CC67 Simulations	11

3.1	PHY-1 Simulations	12

3.1.1	Description of Simulator	12

3.1.2	Simulation Results	14

3.2	PHY-2 Simulations	27

3.2.1	Impairments	27

3.2.2	Simulation set up	27

3.2.3	Simulation Results	27

3.3	PHY-3 Simulations	30

3.3.1	Channels	30

3.3.2	Impairments	30

3.3.3	Data Rates & Antenna Configuration	30

3.3.4	Simulation Setup	30

3.3.5	Simulation Results for CC67	31

3.3.6	Simulation Results for CC67.2	32

3.4	PHY-4 Simulations	34

3.4.1	Supported data rates	34

3.4.2	Simulation Results	34

3.4.3	CC67.2 Simulation	38

4	PHY Throughput Simulations	41

4.1	Ideal Throughput Simulations	41

4.1.1	Basic MIMO Simulations	41

4.1.2	Beam Forming Simuations	47

4.1.3	Advanced Coding Simulations	53

4.2	Throughput Simulations for Beamforming with Impairments	59

Table of Figures

Figure 2-1:  20 MHz, 1 spatial stream	7

Figure 2-2:  20 MHz, 2 spatial streams	7

Figure 2-3:  20 MHz, 4 spatial streams	8

Figure 2-4:  20 MHz, 4 spatial streams	8

Figure 2-5:  40 MHz, 1 spatial stream	9

Figure 2-6:  40 MHz, 2 spatial streams	9

Figure 2-7:  40 MHz, 3 spatial streams	10

Figure 2-8:  40 MHz, 4 spatial streams	10

Figure 3-1 : Channel-B (nLOS) 2x2x20MHz, basic MIMO mode	15

Figure 3-2 : Channel-B (nLOS) 2x2x20MHz, advanced BF MIMO  mode	15

Figure 3-3 :  Channel-B (nLOS) 2x3x20MHz, basic MIMO mode	16

Figure 3-4 : Channel-B (nLOS) 2x3x20MHz, advanced BF MIMO mode	16

Figure 3-5 : Channel-B (nLOS) 3x2x20MHz, advanced BF MIMO mode	17

Figure 3-6 : Channel-B (nLOS) 4x4x20MHz, basic MIMO	18

Figure 3-7 : Channel-B (nLOS) 4x4x20MHz, advanced BF MIMO	18

Figure 3-8 : Channel-D (nLOS) 2x2x20MHz, basic MIMO	19

Figure 3-9 : Channel-D (nLOS) 2x2x20MHz, advanced BF MIMO	19

Figure 3-10 : Channel-D (nLOS) 2x3x20MHz, basic MIMO	20

Figure 3-11 : Channel-D (nLOS) 2x3x20MHz, advanced BF MIMO	20

Figure 3-12 : Channel-D (nLOS) 3x2x20MHz, advanced BF MIMO	21

Figure 3-13: Channel-D (nLOS) 4x4x20MHz, basic MIMO	22

Figure 3-14: Channel-D (nLOS) 4x4x20MHz, advanced BF MIMO	22

Figure 3-15 : Channel-E (nLOS) 2x2x20MHz, basic MIMO	23

Figure 3-16 : Channel-E (nLOS) 2x2x20MHz, advanced BF MIMO	23

Figure 3-17 : Channel-E (nLOS) 2x3x20MHz, basic MIMO	24

Figure 3-18 : Channel-E (nLOS) 2x3x20MHz, advanced BF MIMO	24

Figure 3-19 : Channel-E (nLOS) 3x2x20MHz, advanced BF MIMO	25

Figure 3-20 : Channel-E (nLOS) 4x4x20MHz, basic MIMO	26

Figure 3-21 : Channel-E (nLOS) 4x4x20MHz, advanced BF MIMO	26

Figure 3-22:  CC67 simulation of TGn B_NLOS, 40 MHz	28

Figure 3-23: CC67 Simulation for TGn D_NLOS	28

Figure 3-24: CC67 Simulation for TGn E_NLOS	29

Figure 3-25: CC67 Channel B-NLOS, 20MHz, 1000 Byte (with Half GI)	31

Figure 3-26: CC67 Channel D-NLOS, 20MHz, 1000 Byte	32

Figure 3-27: CC67 Channel E-NLOS, 20MHz, 1000 Byte	32

Figure 3-28: CC67.2 Channel E-NLOS, 20MHz, 1000 Byte	33

Figure 3-29:  Channel Model B, NLOS, 20MHz (with Half GI)	35

Figure 3-30:  Channel Model D, NLOS, 20MHz	36

Figure 3-31:  Channel Model E, NLOS, 20MHz	36

Figure 3-32:  Channel Model B, NLOS, 40MHz (with Half GI)	37

Figure 3-33:  Channel Model D, NLOS, 40MHz	37

Figure 3-34:  Channel Model E, NLOS, 40MHz	38

Figure 4-1:  Basic MIMO Throughput Comparison Model B NLOS	42

Figure 4-2:  Basic MIMO Throughput Comparison Model D NLOS	43

Figure 4-3:  Basic MIMO Throughput Comparison Model E NLOS	43

Figure 4-4:  MCS Selection Probabilities for 2x2-20 MHz, Model D NLOS	44

Figure 4-5:  MCS Selection Probabilities for 2x3-20 MHz, Model D NLOS	45

Figure 4-6:  MCS Selection Probabilities for 4x4-20 MHz, Model D NLOS	45

Figure 4-7:  MCS Selection Probabilities for 2x2-40 MHz, Model D NLOS	46

Figure 4-8:  Advanced BF vs. Basic Open Loop Comparison, Model B	48

Figure 4-9:  Advanced BF vs. Basic Open Loop Comparison, Model D	48

Figure 4-10:  Advanced BF vs. Basic Open Loop Comparison, Model E	49

Figure 4-11:  Comparison of Basic and Advanced BF, 2x2 - Model B	49

Figure 4-12:  Comparison of Basic and Advanced BF, 4x2 - Model B	50

Figure 4-13:  Comparison of Basic and Advanced BF, 2x2 - Model D	50

Figure 4-14:  Comparison of Basic and Advanced BF, 4x2 - Model D	51

Figure 4-15:  Comparison of Basic and Advanced BF, 2x2 - Model E	51

Figure 4-16:  Comparison of Basic and Advanced BF, 4x2 - Model E	52

Figure 4-17:  Advanced coding comparison, 2x2 Model B NLOS, no constraint.	54

Figure 4-18:  Advanced coding comparison, 2x2 Model B NLOS, PER < 2%.	55

Figure 4-19:  Advanced coding comparison, 2x2 Model D NLOS, no constraint.	55

Figure 4-20:  Advanced coding comparison, 2x2 Model D NLOS, PER < 2%.	56

Figure 4-21:  Advanced coding comparison, 2x2 Model E NLOS, no constraint.	56

Figure 4-22:  Advanced coding comparison, 2x2 Model E NLOS, PER < 2%.	57

Figure 4-23:  PER for Model B advanced coding simulations.	57

Figure 4-24:  PER for Model D advanced coding simulations.	58

Figure 4-25:  PER for Model E advanced coding simulations.	58

Figure 4-26 : Channel-B (nLOS) Throughput, 2x2 : 2x3 : 3x2	59

Figure 4-27 : Channel-B (nLOS)  Throughput, 4x4	60

Figure 4-28 : Channel-D (nLOS) Throughput, 2x2 : 2x3 : 3x2	60

Figure 4-29 : Channel-D (nLOS) Throughput, 4x4	61

Figure 4-30 : Channel-E (nLOS) Throughput, , 2x2 : 2x3 : 3x2	61

Figure 4-31 : Channel-E (nLOS) Throughput, 4x4	62

# Introduction

## References

[1]	Syed (Aon) Mujataba, IEEE802.11-04-0889-r0, “TGn Sync Proposal Technical Specification,” August 13, 2004.

[2]	Syed (Aon) Mujataba, IEEE802.11-04-0890-r0, “TGn Sync Proposal FRCC Compliance,” August 13, 2004.

[3]	Adrian Stephens, IEEE802.11-03-0814-r31, “802.11 TGn Comparison Criteria,” July 12 2004.

## Scope

This document provides PHY simulation results in support of the TGn Sync proposal for 802.11n as presented in the technical specification [1].  Simulations presented here establish compliance with CC59 (Section 2) and CC67 (Section 3) as defined in [3].

Section 4 contains PHY throughput simulations (not required by the CCs) that shed additional light into the efficacy of the TGn Sync proposal, and establish the potential gains of the advanced coding and beam forming options of the proposal.

# CC59 Smulations

CC59 requires ideal AWGN channel simulations with no impairments.  In this section we provide PER curves for the full Basic MCS set organized in charts according to the number of spatial streams.  In each chart, the number of transmit antennas and the number of receive antennas are the same as the number of spatial streams.

For each SNR, the simulations were run until either least 500 packet errors were observed, or a total of 50,000 packets had been simulated.  In call cases of PER  1%, 500 packet errors were observed, hence exceeding the 100 packet error requirement.

The MCS (modulation coding scheme) definitions and indexing, as defined in [1], for the Basic MIMO set are found in Table 1.  The same definitions are used for both 20 and 40 MHz channels.  There is one exception.  MCS 32 (not listed in the table) is a BPSK rate 1/2 duplicate format transmission mode that provides a 6 Mbps rate for 40 MHz channels.  (The data rate for MCS 0 in 40 MHz is 13.5 Mbps.)

Table 1:  MCS Definition

| MCS Indices for 1/2/3/4 Spatial Streams | Modulation | FEC Code Rate |
| --- | --- | --- |
| 0 / 8 / 16 / 24 | BPSK | 1/2 |
| 1 / 9 / 17 / 25 | QPSK | 1/2 |
| 2 / 10 / 18 / 26 | QPSK | 3/4 |
| 3 / 11 / 19 / 27 | 16 QAM | 1/2 |
| 4 / 12 / 20 / 28 | 16 QAM | 3/4 |
| 5 / 13 / 21 / 29 | 64 QAM | 2/3 |
| 6 / 14 / 22 / 30 | 64 QAM | 3/4 |
| 7 / 15 / 23 / 31 | 64 QAM | 7/8 |

## Results for 20 MHz Channels

Figure 2-1:  20 MHz, 1 spatial stream

Figure 2-2:  20 MHz, 2 spatial streams

Figure 2-3:  20 MHz, 4 spatial streams

Figure 2-4:  20 MHz, 4 spatial streams

## Results for 40 MHz Channels

Figure 2-5:  40 MHz, 1 spatial stream

Figure 2-6:  40 MHz, 2 spatial streams

Figure 2-7:  40 MHz, 3 spatial streams

Figure 2-8:  40 MHz, 4 spatial streams

# CC67 Simulations

This section provides CC67 Simulations from four different simulators.  Each simulation was created and managed by different engineering teams, utilizing different acquisition and channel estimation algorithms.

In compliance with CC67 we provide Set 1 CC67 simulation.  We do not provide the optional throughput simulation as specified in Set 2.

CC67 simulations are intended to establish the practicality of proposed transmission modes in the presence of impairments and acquisition errors.   Clearly various aspects of receiver designs (such as filtering, acquisition algorithms, channel estimation algorithm, etc.) will vary across device manufacturers, so some variation in results is to be expected.  Hence, showing results from multiple and disjoint modem engineering efforts only serves to strengthens the conclusions.

Our various simulation efforts have different capabilities, and not all simulators are capabile of fully satisfying all of the CC67 requirements.  The table below establishes which CC67 requirements are satisfied by simulator.

Table 2:  CC67 Compliance

| CC67 Requirement | PHY-1 | PHY-2 | PHY-3 | PHY-4 | PHY-5 |
| --- | --- | --- | --- | --- | --- |
| PER for 5 MCSs in 20 MHz | x |  | x | x |  |
| PER for minimum rate Basic MCS in 20 MHz | x |  | x | x |  |
| PER for maximum rate Basic MCS in 20 MHz |  |  | x | x |  |
| CC67.2 frequency offset simulations in 20 MHz |  |  | x | x |  |
| PER for maximum rate Basic MCS with fluorescent effect in Model D in 20 MHz |  |  |  | x |  |
| PER for 5 MCSs in 40 MHz |  | x |  | x |  |
| PER for minimum rate Basic MCS in 40 MHz |  | x |  | x |  |
| PER for maximum rate Basic MCS in 40 MHz |  | x |  | x |  |
| CC67.2 frequency offset simulations in 40 MHz |  | x |  | x |  |
| PER for maximum rate Basic MCS with fluorescent effect in Model D in 40 MHz |  |  |  | x |  |
| PER for advanced Beamforming Modes* | x |  |  |  |  |

* These simulations are not required for CC67 compliance.

## PHY-1 Simulations

This subsection provide CC67 simulation results for both Basic MIMO and Advanced Beamforming MIMO.

After here, when we write m x n, “m” is the number of Tx antennas (and it is identical to the number of spatial stream in basic MIMO case), and “n” is the number of Rx antennas.

### Description of Simulator

Our simulator is fully CC compliant.  Impairments include

- IM1 (PA non-linearity) is included. P=3 and OBO=8dB.  Sampling clock is 200MHz for this module.

- IM2 (Carrier frequency offset) is included. Offset value is –13.675ppm, and sampling clock offset is also added. Actual timing acquisition is implemented.

- IM4 (Phase Noise) is included.

- IM6 (Antenna Configuration) is set to what IM6 of CC required, i.e., antenna configuration is linear array and distance between adjacent two antennas is a half .

Regarding channel model, we used the latest TGn Matlab code as it is.

- channel mode- B, D, and E.

- non-LOS models are used.

- Channel is varying during packet, and between the channel sounding packet and target packet [in Advanced beamforming MIMO case]

- Please note that our results don’t include fluorescent effect at the highest rate in channel-D

Other simulator descriptions are as follows;

- Our simulator includes

  - Analog filter model and other FIR filter at each sampling clock converting stage.

  - Packet detection without prior knowledge.

  - Frequency offset compensation and its tracking.

  - Time-of-arrival estimation, and sampling clock offset compensation and its tracking.

  - Actual channel estimation at receiver side (MMSE, without any successive cancellation).

  - Almost ideal AGC (AGC is implemented, but signal path has too wide dynamic range). So, we don’t re-AGC at HT-STF.

- Our simulator doesn’t include

  - 64QAM r=7/8 (both for basic MIMO and Advanced beamforming MIMO).

  - short GI (both for basic MIMO Advanced beamforming MIMO).

  - any advanced coding.

  - Walsh +CDD.

- Other parameters

  - Floating calculation

  - T0=600[nsec] (From OFDM symbol edge to FFT window starting point)

  - Trace-back length of Viterbi decoder is 128.

  - No smoothing filter for channel estimation.

  - PPDU length is 1000 bytes, as CC specified.

  - SNR is calculated as ensemble averaged SNR.

  - When the number of packet error reaches to 100, then quit from this loop.

  - 10,000 seeds of channel realization are used.

- In advanced beamforming MIMO simulation, a subset of Link Adaptation effect is included. In our advanced beamforming MIMO, data rate is dynamically adapted to the eigen-value of spatial channel.  So, we add a restriction to have same data rate with basic MIMO and just compare the required SNR at same data rate.

  - Used rate set table is below.

Table 3-3 : Used MCS Set

| ID# | Stream Count | Stream ID vs Modulation&Coding |  |  |  | Full GI |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | stream 1 | stream 2 | stream 3 | stream 4 | Rate in 20MHz |
| 0 | 1 | BPSK, 1/2 | N/A | N/A | N/A | 6 |
| 1 | 1 | QPSK, 1/2 | N/A | N/A | N/A | 12 |
| 2 | 1 | QPSK, 3/4 | N/A | N/A | N/A | 18 |
| 3 | 1 | 16QAM, 1/2 | N/A | N/A | N/A | 24 |
| 4 | 1 | 16QAM, 3/4 | N/A | N/A | N/A | 36 |
| 5 | 1 | 64QAM, 2/3 | N/A | N/A | N/A | 48 |
| 6 | 1 | 64QAM, 3/4 | N/A | N/A | N/A | 54 |
| 8 | 2 | BPSK, 1/2 | BPSK, 1/2 | N/A | N/A | 12 |
| 9 | 2 | QPSK, 1/2 | QPSK, 1/2 | N/A | N/A | 24 |
| 10 | 2 | QPSK, 3/4 | QPSK, 3/4 | N/A | N/A | 36 |
| 11 | 2 | 16QAM, 1/2 | 16QAM, 1/2 | N/A | N/A | 48 |
| 12 | 2 | 16QAM, 3/4 | 16QAM, 3/4 | N/A | N/A | 72 |
| 13 | 2 | 64QAM, 2/3 | 64QAM, 2/3 | N/A | N/A | 96 |
| 14 | 2 | 64QAM, 3/4 | 64QAM, 3/4 | N/A | N/A | 108 |
| 16 | 3 | BPSK, 1/2 | BPSK, 1/2 | BPSK, 1/2 | N/A | 18 |
| 17 | 3 | QPSK, 1/2 | QPSK, 1/2 | QPSK, 1/2 | N/A | 36 |
| 18 | 3 | QPSK, 3/4 | QPSK, 3/4 | QPSK, 3/4 | N/A | 54 |
| 19 | 3 | 16QAM, 1/2 | 16QAM, 1/2 | 16QAM, 1/2 | N/A | 72 |
| 20 | 3 | 16QAM, 3/4 | 16QAM, 3/4 | 16QAM, 3/4 | N/A | 108 |
| 21 | 3 | 64QAM, 2/3 | 64QAM, 2/3 | 64QAM, 2/3 | N/A | 144 |
| 22 | 3 | 64QAM, 3/4 | 64QAM, 3/4 | 64QAM, 3/4 | N/A | 162 |
| 24 | 4 | BPSK, 1/2 | BPSK, 1/2 | BPSK, 1/2 | BPSK, 1/2 | 24 |
| 25 | 4 | QPSK, 1/2 | QPSK, 1/2 | QPSK, 1/2 | QPSK, 1/2 | 48 |
| 26 | 4 | QPSK, 3/4 | QPSK, 3/4 | QPSK, 3/4 | QPSK, 3/4 | 72 |
| 27 | 4 | 16QAM, 1/2 | 16QAM, 1/2 | 16QAM, 1/2 | 16QAM, 1/2 | 96 |
| 28 | 4 | 16QAM, 3/4 | 16QAM, 3/4 | 16QAM, 3/4 | 16QAM, 3/4 | 144 |
| 29 | 4 | 64QAM, 2/3 | 64QAM, 2/3 | 64QAM, 2/3 | 64QAM, 2/3 | 192 |
| 30 | 4 | 64QAM, 3/4 | 64QAM, 3/4 | 64QAM, 3/4 | 64QAM, 3/4 | 216 |
| 33 | 1 | 256QAM, 3/4 | N/A | N/A | N/A | 72 |
| 34 | 2 | QPSK, 3/4 | BPSK, 1/2 | N/A | N/A | 24 |
| 35 | 2 | QPSK, 3/4 | BPSK, 3/4 | N/A | N/A | 27 |
| 36 | 2 | 16QAM, 1/2 | QPSK, 1/2 | N/A | N/A | 36 |
| 37 | 2 | 16QAM, 3/4 | QPSK, 1/2 | N/A | N/A | 48 |
| 38 | 2 | 16QAM, 3/4 | QPSK, 3/4 | N/A | N/A | 54 |
| 39 | 2 | 64QAM, 2/3 | BPSK, 1/2 | N/A | N/A | 54 |
| 40 | 2 | 64QAM, 3/4 | QPSK, 3/4 | N/A | N/A | 72 |
| 41 | 2 | 64QAM, 2/3 | 16QAM, 1/2 | N/A | N/A | 72 |
| 42 | 2 | 256QAM, 3/4 | 16QAM, 1/2 | N/A | N/A | 96 |
| 43 | 2 | 256QAM, 3/4 | 16QAM, 3/4 | N/A | N/A | 108 |
| 44 | 2 | 256QAM, 3/4 | 256QAM, 3/4 | N/A | N/A | 144 |
| 45 | 3 | 16QAM, 3/4 | 16QAM, 1/2 | QPSK, 1/2 | N/A | 72 |
| 46 | 3 | 64QAM, 2/3 | QPSK, 3/4 | BPSK, 1/2 | N/A | 72 |
| 47 | 3 | 64QAM, 3/4 | 16QAM, 1/2 | QPSK 3/4 | N/A | 96 |
| 48 | 3 | 64QAM, 3/4 | 16QAM, 3/4 | QPSK, 3/4 | N/A | 108 |
| 49 | 3 | 64QAM, 3/4 | 64QAM, 2/3 | BPSK, 1/2 | N/A | 108 |
| 50 | 3 | 64QAM, 3/4 | 64QAM, 3/4 | 16QAM, 3/4 | N/A | 144 |
| 51 | 3 | 256QAM, 3/4 | 64QAM, 2/3 | 16QAM, 1/2 | N/A | 144 |
| 52 | 3 | 256QAM, 3/4 | 64QAM, 3/4 | QPSK 3/4 | N/A | 144 |
| 53 | 3 | 256QAM, 3/4 | 64QAM, 3/4 | 16QAM, 3/4 | N/A | 162 |
| 54 | 3 | 256QAM, 3/4 | 256QAM, 3/4 | QPSK 3/4 | N/A | 162 |
| 55 | 3 | 256QAM, 3/4 | 256QAM, 3/4 | 64QAM, 2/3 | N/A | 192 |
| 56 | 4 | 64QAM, 2/3 | 16QAM, 3/4 | BPSK, 1/2 | BPSK, 1/2 | 96 |
| 57 | 4 | 64QAM, 3/4 | 64QAM, 3/4 | 16QAM, 1/2 | QPSK, 1/2 | 144 |
| 58 | 4 | 256QAM, 3/4 | 64QAM, 3/4 | 64QAM, 2/3 | QPSK, 3/4 | 192 |
| 59 | 4 | 256QAM, 3/4 | 64QAM, 3/4 | 64QAM, 3/4 | QPSK, 1/2 | 192 |
| 60 | 4 | 256QAM, 3/4 | 256QAM, 3/4 | 16QAM, 3/4 | QPSK, 1/2 | 192 |
| 61 | 4 | 256QAM, 3/4 | 256QAM, 3/4 | 64QAM, 2/3 | 16QAM, 1/2 | 216 |
| 62 | 4 | 256QAM, 3/4 | 256QAM, 3/4 | 64QAM, 3/4 | QPSK, 3/4 | 216 |

### Simulation Results

This subsection presents simulation results for both Basic MIMO MCSs and advanced beamforming.

For Basic MIMO the MCS from Table 3-3 is the one that correspondes to the rate indicated in the legend with the number of spatial streams being equal to the number of transmit antennas.  For example, the 24 Mbps curve in Figure 3-9 corresponds to MCS 9 (2 spatial streams, QPSK, code rate = 1/2), and not MCS 3 (1 spatial stream, 16 QAM, code rate = 1/2).

The PER curves for advanced beamforming utilize MCS selection among all MCSs having the same data rate, such that the number of spatial streams does not exceed the number of transmit antennas.

Figure 3-9 : Channel-B (nLOS) 2x2x20MHz, basic MIMO mode

Figure 3-10 : Channel-B (nLOS) 2x2x20MHz, advanced BF MIMO  mode

Figure 3-11 :  Channel-B (nLOS) 2x3x20MHz, basic MIMO mode

Figure 3-12 : Channel-B (nLOS) 2x3x20MHz, advanced BF MIMO mode

Figure 3-13 : Channel-B (nLOS) 3x2x20MHz, advanced BF MIMO mode

Figure 3-14 : Channel-B (nLOS) 4x4x20MHz, basic MIMO

Figure 3-15 : Channel-B (nLOS) 4x4x20MHz, advanced BF MIMO

Figure 3-16 : Channel-D (nLOS) 2x2x20MHz, basic MIMO

Figure 3-17 : Channel-D (nLOS) 2x2x20MHz, advanced BF MIMO

Figure 3-18 : Channel-D (nLOS) 2x3x20MHz, basic MIMO

Figure 3-19 : Channel-D (nLOS) 2x3x20MHz, advanced BF MIMO

Figure 3-20 : Channel-D (nLOS) 3x2x20MHz, advanced BF MIMO

Figure 3-21: Channel-D (nLOS) 4x4x20MHz, basic MIMO

Figure 3-22: Channel-D (nLOS) 4x4x20MHz, advanced BF MIMO

Figure 3-23 : Channel-E (nLOS) 2x2x20MHz, basic MIMO

Figure 3-24 : Channel-E (nLOS) 2x2x20MHz, advanced BF MIMO

Figure 3-25 : Channel-E (nLOS) 2x3x20MHz, basic MIMO

Figure 3-26 : Channel-E (nLOS) 2x3x20MHz, advanced BF MIMO

Figure 3-27 : Channel-E (nLOS) 3x2x20MHz, advanced BF MIMO

Figure 3-28 : Channel-E (nLOS) 4x4x20MHz, basic MIMO

Figure 3-29 : Channel-E (nLOS) 4x4x20MHz, advanced BF MIMO

## PHY-2 Simulations

As required by CC67, 5 data rates in 40MHz are simulated. The data rates are shown in Table 4.

Table 4 Simulated Data Rate

| MCS (#Tx × # Rx) | Modulation | Coding Rate | Date Rate full GI / half GI |
| --- | --- | --- | --- |
| 11 (2×2) | 16 QAM | 1/2 | 108Mbs/120Mbs |
| 14 (2×2) | 64 QAM | 3/4 | 243Mbs/270Mbs |
| 14 (2×3) | 64 QAM | 3/4 | 243Mbs/270Mbs |
| 31 (4×6) | 64 QAM | 7/8 | 567Mbs/630Mbs |
| 32 (2×2) | BPSK | 1/2 | 6Mbs |

### Impairments

According to CC67, the following impairments are simulated.

- IM1 (PA nonlinearity): Rapp model with p = 3, saturation power of 25dBm and total transmit power of 16dBm. The signal is oversampled by 5x.

- IM2 (Frequency offset): -13.675 ppm with carrier frequency of 5.25GHz for CC67 simulations and 40ppm for CC67.2 simulations. Timing and frequency synchronization are implemented.

- IM4 (Phase noise): phase noise model specified in CC.

- IM6 (Antenna configuration): uniform linear array, with antennas spacing of half wavelength.

### Simulation set up

The simulation is set up according to Table 5.

Table 5 Simulation set up

| Receiver type | Linear MMSE, soft decision |
| --- | --- |
| PSDU length | 1000 Byte |
| Number of channel realizations | 2000 |
| Channel models | TGN B, D, E |
| Half GI | Only for channel B |
| Channel estimation | Per tone, no smoothing |

### Simulation Results

#### CC67

Figure 3-11, Figure 3-31 and Figure 3-32 show the simulaiton results of CC67 for TGn channel B_NLOS, D_NLOS and E_NLOS of 40MHz mode, respectively.

Figure 3-30:  CC67 simulation of TGn B_NLOS, 40 MHz

Figure 3-31: CC67 Simulation for TGn D_NLOS

Figure 3-32: CC67 Simulation for TGn E_NLOS

#### CC67.2 Results for 40 MHz

The simulation results of CC67.2 of TGn channel E, non-LOS are shown in Table 6.

Table 6 PER of TGn E_NLOS with SNR = 50dB

| Frequency offset | MCS 11 2×2 | MCS 14 2×2 | MCS 14 2×3 | MCS 31 4×6 | MCS 32 2×2 |
| --- | --- | --- | --- | --- | --- |
| 0ppm | 0.001 | 0.0075 |  | 0.0095 |  |
| +40ppm |  | 0.0095 |  |  |  |
| -40ppm |  |  |  |  |  |

The simulation results of CC67.2 of TGn channel E, LOS are shown in Table 7.

Table 7 PER of TGn E_LOS with SNR = 50dB

| Frequency offset | MCS 11 2×2 | MCS 14 2×2 | MCS 14 2×3 | MCS 31 4×6 | MCS 32 2×2 |
| --- | --- | --- | --- | --- | --- |
| 0ppm |  | 0.0105 | 0.0045 |  |  |
| +40ppm |  | 0.0135 |  |  |  |
| -40ppm |  |  |  |  |  |

## PHY-3 Simulations

This sub-section provides simulation results for both CC67 and CC67.2. For CC67, the performance is evaluated by PER vs SNR curve.

CC67 in the comparison crteria is to show the PER performance in non-AWGN channels.  Our simulation results are fully CC compliant.

### Channels

The performance is evaluated for NLOS version of channel models B, D, and E. The channel models include Doppler effect as specified in the channel model document, and the shadowing variance is set to zero.

### Impairments

.The following impairments are included as specified in the comparison criteria document.

- IM1 (PA non-linearity): Rapp PA model with p=3 and OBO=8dB is used with oversampling rate of 4x.

- IM2 (Carrier frequency offset): Carrier frequency offset value of -13.675ppm (with respect to 5.25GHz) is used, and the symbol clock offset has the same relative offset as the carrier frequency offset. Timing acquisition is performed on a per-packet basis.

- IM4 (Phase Noise): Phase noise is generated by the pole-zero model as specified in CC document.

- IM6 (Antenna Configuration): antenna configuration is a uniform linear array, and the distance between adjacent two antennas is a half wavelength

### Data Rates & Antenna Configuration

The following 5 supported data rates are simulated in 20MHz.

Table 8 Data Rates and Antenna Configuration

| MCS index | Modulation | Coding rate | Data Rate in 20MHz |  | Nss | Ntx | Nrx |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Full GI | Half GI |  |  |  |
| 0 | BPSK | ½ | 6 Mbps |  | 1 | 2 | 2 |
| 14 | 64-QAM | ¾ | 108 Mbps | 120 Mbps | 2 | 2 | 2 |
| 14 | 64-QAM | ¾ | 108 Mbps | 120 Mbps | 2 | 2 | 3 |
| 27 | 16-QAM | ½ | 96 Mbps | 106.67Mbps | 4 | 4 | 4 |
| 31 | 64-QAM | 7/8 | 252 Mbps | 280 Mbps | 4 | 4 | 6 |

InTable 8, Ntx is the number of transmit antennas, Nrx is the number of receive antennas, and Nss is the number of spatial streams.

### Simulation Setup

- PSDU of 1000 Byte

- Minimum of 100 MPDU errors down to 1% PER

- Half GI is applied only for channel B except 6Mbps (MCS 0)

- CDD (Cyclic Delay Diversity) is applied for 6Mbps (MCS 0).

- SNR is defined as in the section 2 of CC document

- No priori channel or synchronization assumed

- Simple per-tone channel estimation algorithm used. No smoothing or denoising

- Timing and frequency acquisition implemented

- Linear MMSE receiver used

### Simulation Results for CC67

Figure 3-33, Figure 3-34, and Figure 3-35 show CC67 simulation results for channel model B, D, and E, respectively. Half GI is applied only for channel model B except for 6Mbps, and  the results for channel model D include the highest data rate with fluorecent effect incorporated.

Figure 3-33: CC67 Channel B-NLOS, 20MHz, 1000 Byte (with Half GI)

#### Channel D-NLOS

Figure 3-34: CC67 Channel D-NLOS, 20MHz, 1000 Byte

#### Channel E-NLOS

Figure 3-35: CC67 Channel E-NLOS, 20MHz, 1000 Byte

### Simulation Results for CC67.2

CC67.2 is to show the impact of carrier frequency offset and symbol clock offset on the performance.

#### Channel E-NLOS

The same 5 data rates as in CC67 are simulated. Table 9compares PER for no offset, -40ppm, and 40ppm at SNR for which PER is around 10% with channel model E-NLOS. For reference, Figure 3-36 shows PER vs SNR for no offset, carrier offset of -40 ppm, and carrier offset of 40 ppm.

Table 9 PER Comparison for Channel E-NLOS

| Data Rate | MCS (Ntx x Nrx) | SNR | 0 ppm | -40 ppm | 40 ppm |
| --- | --- | --- | --- | --- | --- |
| 6 Mbps | 0 (2x2) | 8dB | 0.046 | 0.057 | 0.063 |
| 108 Mbps | 14 (2x2) | 34dB | 0.135 | 0.167 | 0.157 |
| 108 Mbps | 14 (2x3) | 28dB | 0.053 | 0.092 | 0.097 |
| 96 Mbps | 27 (4x4) | 26dB | 0.155 | 0.183 | 0.19 |
| 252 Mbps | 31 (4x6) | 34dB | 0.053 | 0.083 | 0.081 |

Figure 3-36: CC67.2 Channel E-NLOS, 20MHz, 1000 Byte

#### Channel E-LOS

The LOS version of channel model E is simulated with SNR of 50dB for the same 5 supported data rates used in CC67.

Table 10 CC67.2: PER Comparison for Channel E-LOS at SNR=50dB

| Data Rate | MCS (Ntx x Nrx) | SNR | 0 ppm | -40 ppm | 40 ppm |
| --- | --- | --- | --- | --- | --- |
| 6 Mbps | 0 (2x2) | 50dB | <1e-4 | <1e-4 | <1e-4 |
| 108 Mbps | 14 (2x2) | 50dB | 2.6e-3 | 3.8e-3 | 3.7e-3 |
| 108 Mbps | 14 (2x3) | 50dB | 4.1e-4 | 6.2e-4 | 6.2e-4 |
| 96 Mbps | 27 (4x4) | 50dB | <1e-4 | <1e-4 | <1e-4 |
| 252 Mbps | 31 (4x6) | 50dB | <1.7e-4 | <1.7e-4 | <1.7e-4 |

## PHY-4 Simulations

This document contains PHY simulation results of CC67 and CC67.2 for the "TGn Sync" proposal to IEEE 802.11 TGn in compliance to the TGn call for proposals.

### Supported data rates

#### 5 supported data rates in 20MHz mode

According to CC67 and CC67.2, the following 5 data rates are selected. These include the maximum and the minimum data rate. The data rates enclosed by ( ) indicate the data rates with Half GI. Half GI is applied only for Channel Model B. The fluorescent effect is added for the maximum rate, 252Mbps in 20MHz mode, and 567Mbps in 40MHz mode.

In the 6Mbps 40MHz mode, the duplicated format is applied. The same data is transmitted both in the upper channel and the lower channel.

1.	6Mbps (6.67Mbps) :	1x2x20, BPSK, R=1/2 coding

2.	108Mbps (120Mbps) :	2x2x20, 64-QAM, R=3/4 coding

3.	108Mbps (120Mbps) :	2x3x20, 64-QAM, R=3/4 coding

4.	96Mbps (106.67Mbps) :	4x4x20, 16-QAM, R=1/2 coding

5.	252Mbps (280Mbps) :	4x6x20, 64-QAM, R=7/8 coding

#### 5 supported data rates in 40MHz mode

1.	6Mbps (6.67Mbps) :	1x2x40, BPSK, R=1/2 coding, Duplicated Format

2.	108Mbps (120Mbps) :	2x2x40, 16-QAM, R=1/2 coding

3.	243Mbps (270Mbps) :	2x3x40, 64-QAM, R=3/4 coding

4.	243Mbps (270Mbps) :	2x2x40, 64-QAM, R=3/4 coding

5.	567Mbps (630Mbps) :	4x6x40, 64-QAM, R=7/8 coding

### Simulation Results

According to CC67, the above 10 data rates must be simulated for NLOS of Channel Model B, D and E.

Table 11 Simulation Conditions for CC67

| Sampling Rate | 80MHz in 20MHz mode 160MHz in 40MHz mode |
| --- | --- |
| Receiver Type | MMSE |
| PSDU Length | 1,000Bytes |
| Channel Model | B(NLOS), D(NLOS), E(NLOS) |
| Half GI | Applied only for Model B |
| Channel Estimation | Per tone estimation (no smoothing) |
| Timing Acquisition | Matched filtering by L-STF |
| Offset Compensation | Using L-LTF and pilot tones |
| Impairments in CC | IM1,2,4,5,6 (Output Back Off=8dB in IM1) |
| Digital filtering | 32 taps root roll-off filter for both Tx and Rx |

#### Results of CC67 in 20MHz mode

The following figures show the results of CC67. Figure 3-37, Figure 3-38 and Figure 3-39 shows the results for Channel Model B, D and E, respectively. Half GI is applied only for Channel Model B and the results for Channel Model D additionally include a curve for the highest data rate with the fluorescent effect applied.

Figure 3-37:  Channel Model B, NLOS, 20MHz (with Half GI)

Figure 3-38:  Channel Model D, NLOS, 20MHz

Figure 3-39:  Channel Model E, NLOS, 20MHz

#### Results of CC67 in 40MHz mode

The following figures show the results of CC67. Figure 3-40, Figure 3-41 and Figure 3-42 shows the results of Channel Model B, D and E, respectively. As for the 20MHz mode, the half GI is applied only for Channel Model B and the results for Channel Model D additionally include a curve for the highest data rate with the fluorescent effect applied.

Figure 3-40:  Channel Model B, NLOS, 40MHz (with Half GI)

Figure 3-41:  Channel Model D, NLOS, 40MHz

Figure 3-42:  Channel Model E, NLOS, 40MHz

### CC67.2 Simulation

According to CC67.2, the simulation conditions are same as CC67. Therefore, the 10 data rates shown in Section 3.4.1 and 3.4.1.2 are used.

Table 12 Simulation Conditions for CC67.2

| Sampling Rate | 80MHz in 20MHz mode 160MHz in 40MHz mode |
| --- | --- |
| Receiver Type | MMSE |
| PPDU Length | 1,000Bytes |
| Channel Model | E(LOS/NLOS) |
| Half GI | Not applied |
| Channel Estimation | Per tone estimation (no smoothing) |
| Timing Acquisition | Matched filtering by L-STF |
| Offset Compensation | Using L-LTF and pilot tones |
| Impairments in CC | IM1,4,5,6 (Output Back Off=8dB in IM1) |
| Digital filtering | 32 taps root roll-off filter for both Tx and Rx |

#### Results of CC67.2 in 20MHz mode

Table 13 shows the results for CC67.2 in Channel Model E/NLOS. At first, the simulations with no offset (0ppm) are done to find SNR value where the PER falls below 10%. These SNR values are shown in the fourth column with the corresponding PERs in the fifth column. Once these SNR values are found, the simulations for offsets of +/-40ppm are done and the PERs for this are shown in the sixth and seventh columns.

Table 14 shows the results for the LOS case. In this case, the SNR=50dB and the PERs for the different offsets are shown. 0ppm means no offset is added.

Table 13 Results of CC67.2 in 20MHz Mode (Channel Model E/NLOS)

| No | Rate (Mbps) | Mode | SNR | 0ppm | +40ppm | -40ppm |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1x2x20, BPSK, R=1/2 | 13 | 7.86E-02 | 1.70E-01 | 1.67E-01 |
| 2 | 108 | 2x2x20, 64QAM, R=3/4 | 35 | 8.38E-02 | 1.42E-01 | 1.29E-01 |
| 3 | 108 | 2x3x20, 64QAM, R=3/4 | 27 | 8.54E-02 | 1.38E-01 | 1.25E-01 |
| 4 | 96 | 4x4x20, 16QAM, R=1/2 | 25 | 5.86E-02 | 1.05E-01 | 9.08E-02 |
| 5 | 252 | 4x6x20, 64QAM, R=7/8 | 33 | 6.70E-02 | 9.90E-02 | 9.34E-02 |

Table 14 Results of CC67.2 in 20MHz Mode (Channel Model E/LOS)

| No | Rate (Mbps) | Mode | SNR | 0ppm | +40ppm | -40ppm |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1x2x20, BPSK, R=1/2 | 50 | 2.36E-02 | 7.41E-02 | 6.62E-02 |
| 2 | 108 | 2x2x20, 64QAM, R=3/4 | 50 | 3.40E-02 | 9.90E-02 | 8.44E-02 |
| 3 | 108 | 2x3x20, 64QAM, R=3/4 | 50 | 1.51E-02 | 6.42E-02 | 5.54E-02 |
| 4 | 96 | 4x4x20, 16QAM, R=1/2 | 50 | 9.60E-03 | 4.53E-02 | 4.17E-02 |
| 5 | 252 | 4x6x20, 64QAM, R=7/8 | 50 | 2.66E-03 | 1.74E-02 | 2.28E-02 |

#### CC67.2 Results in 40MHz mode

The method of obtaining and presenting results is the same as that described in 3.4.3.1. The results are shown in Table 15 for the NLOS channel and in Table 16 for the LOS case.

Table 15 Results of CC67.2 in 40MHz Mode (Channel Model E/NLOS)

| No | Rate (Mbps) | Mode | SNR | 0ppm | +40ppm | -40ppm |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1x2x40, BPSK, R=1/2 | 10 | 4.45E-02 | 5.78E-02 | 7.24E-02 |
| 2 | 108 | 2x2x40, 16QAM, R=1/2 | 21 | 5.70E-02 | 6.52E-02 | 7.06E-02 |
| 3 | 243 | 2x3x40, 64QAM, R=3/4 | 27 | 4.18E-02 | 4.72E-02 | 4.80E-02 |
| 4 | 243 | 2x2x40, 64QAM, R=3/4 | 33 | 9.36E-02 | 9.56E-02 | 1.08E-01 |
| 5 | 567 | 4x6x40, 64QAM, R=7/8 | 32 | 7.96E-02 | 7.94E-02 | 7.88E-02 |

Table 16 Results of CC67.2 in 40MHz Mode (Channel Model E/LOS)

| No | Rate (Mbps) | Mode | SNR | 0ppm | +40ppm | -40ppm |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | 1x2x40, BPSK, R=1/2 | 50 | 9.00E-04 | 4.40E-03 | 6.20E-03 |
| 2 | 108 | 2x2x40, 16QAM, R=1/2 | 50 | 2.25E-03 | 1.07E-02 | 1.08E-02 |
| 3 | 243 | 2x3x40, 64QAM, R=3/4 | 50 | 5.00E-04 | 3.10E-03 | 3.60E-03 |
| 4 | 243 | 2x2x40, 64QAM, R=3/4 | 50 | 3.27E-03 | 9.52E-03 | 1.10E-02 |
| 5 | 567 | 4x6x40, 64QAM, R=7/8 | 50 | 2.00E-04 | 2.00E-04 | 2.00E-04 |

# PHY Throughput Simulations

The simulations presented in this section are not required by the comparison criteria.

In this section we define PHY Throughput to be the total number of bits delivered divided by the transmission time of the data portions of the transmitted PPDUs.  This throughput does not include PLCP header overheads, or MAC overheads such as inter-frame space or contension time.

## Ideal Throughput Simulations

The simulations presented in this subsection are “ideal” simulations because

- there are no impairments, and

- an ideal MCS selection algorithm is employed.

All of the simulations in this subsection use the MMSE receiver.

One purpose of these simulations is to investigate the potential throughput gains that are achievable using some of the optional features of the proposal; specifically, advanced coding and beam forming.  Throughput simulations are particularly well suited for this purpose.  When comparing different coding scheme, the data rates of the MCSs between to two systems to be compaired may not match.

Throughput depends on MCS selection algorithms.  As is well know, SNR measurment alone is not a good predictor of PER.  It is highly likely that practical MCS selection algorithm will need to be tuned to the particular air interface technology.  Thus, our use of an ideal MCS selection algorithm provides a fair comparison between technologies; that is, one of the candidates is not given an unfair advantage via the particular algorithm employed.

The ideal throughput simulation is described as follows:

- Generate a channel realization H

- For each MCS, simulate 200 packet transmissions.

  - Estimate PER(H,MCS) given the number of error observed in the 200 simulations

  - Calculate TP(H,MCS) = (1 - PER(H,MCS)) * rate(MCS)

- Calculate per channel statistics

  - TP(H) = maxMCS TP(H,MCS)

  - PER(H) = maxMCS PER(H,MCS*)  where MCS* = arg maxMCS TP(H,MCS)

The plotted simulation results are then averaged over 100 channel realizations H.

On a practical note, not all MCSs are actually simulated in step 2.  There are obvious tricks to minimize the amount of simulation.  For example, if the data rate of a given MCS* is less than the max TP(H,MCS) for the MCSs already simulated, then there is no point to simulate MCS*.  By starting with the best MCS from the previous H, this eliminates many of the lower rate MCSs.  Likewise, as the algorithm searches higher rate MCSs and TP(H,MCS) falls below, say, half the current best TP(H,MCS), then we can stop simulating.  The program search parameters were tuned so that there was no difference in results between the reduced MCS search algorithm and exhaustive search.

### Basic MIMO Simulations

Figure 4-43, Figure 4-44 and Figure 4-45 show the throughput results of a comparison of four MIMO configuration, respectively, for Model B, Model D and Model E.  These Basic MIMO simulation utilize MCS 0-15 for the 2x2 and 2x3 cases, and MCS 0-31 for the 4x4 case.

These figures show the “sweet spot” of 140 Mbps.  If we assume a MAC efficiency of 70%, 140 Mbps is the required PHY throughput to reach the 100 Mbps top-of-MAC goal.

It is seen that the 2x2-40 MHz TGn Sync solution has a significant SNR advantage over the 20 MHz configuration.

It must be noted that if these curves were translated to throughput vs. range charts there would be a 3 dB plealty applied to the 40 MHz system due to the fact that the total transmit power is the same for both cases.  After this shift 3 dB, it is clear that 2x2-40 MHz is roughly comparable in performance to 4x4-20 MHz.

However, it is still appropriate to compare throughput on the SNR scale for a very important reason.  All else equal, High SNR implies high RF cost and power consumption.  High SNR requires additional PA back-off, additional bits in data converters, additional dynamic range, less I/Q imbalance and highly linear analog circuits.  These factors drive RF and analog cost and power consumption across the transmit and receive RF chains.

It is therefore clear that the 2x2 40 MHz configuration provides an extremely cost effective solution to achieve the 100 Mbps top-of-MAC throughput goal of 802.11n.

Figure 4-43:  Basic MIMO Throughput Comparison Model B NLOS

Figure 4-44:  Basic MIMO Throughput Comparison Model D NLOS

Figure 4-45:  Basic MIMO Throughput Comparison Model E NLOS

In addition to throughput (and PER) statistics, the simulation algorithm can also collect MCS selection statistics to allow us to examine the relative frequency that various MCSs are selected.  These results are shown in the following figures for Model D NLOS.

Compare the 2x2 and 2x3 20 MHz cases of Figure 4-46 and Figure 4-47.  We see in the 2x2 case when the receiver can choose between MCSs of approximately the same rate but different numbers of spatial steams (1 or 2), that the lowner number of spatial streams generally wins.  This is because in the 2x2 case, 2 spatial stream transmissions lacks diversity.  Moving to the 2x3 chart, we clearly see the impact of the additional diversity in that the lower rate 2 spatial stream cases are chosen more often.  The same trend is evident in 4x4 case in Figure 4-48, where we see a predominance of 3 spatial streams, and very little occurance of 4 spatial streams.

Figure 4-49 shows the 2x2-40 MHz case.  These selection probability curves are quite similar to the 2x2-20 MHz case.

Figure 4-46:  MCS Selection Probabilities for 2x2-20 MHz, Model D NLOS

Figure 4-47:  MCS Selection Probabilities for 2x3-20 MHz, Model D NLOS

Figure 4-48:  MCS Selection Probabilities for 4x4-20 MHz, Model D NLOS

Figure 4-49:  MCS Selection Probabilities for 2x2-40 MHz, Model D NLOS

### Beam Forming Simuations

This subsection examines the impact of beam forming on throughput.  We shall show results for both the ABF (Advanced Beam Forming) and much simpler BBF (Basic Beam Forming).  In both cases we shall show that tremendous gains are achievable.

All of the Beam Forming calculations in this section were preformed using the SVD method.

ABF differs from BBF in two primary ways.  ABF utilizes an extended MCS set, and it allows for variable power loading across spatial streams.

The extended MCS set includes MCSs having different code rates and different modulations across the spatial streams.  Additionaly, the extended MCS set includes 256 QAM.  Normally, 256 QAM would be difficult to support due to the high SNR requirement.  However, beam forming gains allow these high data rate modes to operate with substantially lower SNRs, which potentially brings 256 QAM into the realm of possibility.

Figure 4-50, Figure 4-51 and Figure 4-52 show a comparison of 2x2, 3x2 and 4x2 the Advanced BF to 2x2 and 3x2 open loop.  We see that the 3x2 advanced BF is generally several dB better than the 2x3 solution.

TGn Sync believes that the logical application for beam forming transmission is for very high throughput traffic with stringent QoS requirement; specifically streaming video such has HDTV.  The technology makes sense because high costs and power consumption are contained in the media server, which could be a set-top box AP or a multi-media PC.  Power consumption is not such a problem as these devices are pluged into the wall.  One media server device, however, can serve high rate media to several low cost, low power (battery operated) clients.

Given the deployment secenario just described, the sweet spot configuration is that of the 4 transmit antenna media server device serving 2 receive antenna clients.  The results of Figure 4-50, Figure 4-51 and Figure 4-52 indicate a consistent 10 dB gain relative to the 2x2 baseline.  Of course, for the same configuration, one might argue that there are open loop transmit diversity methods that also provide gain.  However, these will not achieve 10 dB diversity gain.

Next, Figure 4-53 through Figure 4-59 compare what is achievable with substantially less complexity than full advanced BF.  We compare advance to two version of basic BF.  Both basic BF are restricted to MCS 0-15, which do not include 256 QAM, and hence, the peak rate is the same as Basic open loop MIMO.  The two variants are with and without variable power loading across spatial streams.

We see in the 2x2 cases that power loading and the extended MCS set do provide some reasonable gains.  For the 4x2 cases, the simplest Basic BF without variable spatial stream power loading provides nearly as much throughput as the full advanced BF.

Figure 4-50:  Advanced BF vs. Basic Open Loop Comparison, Model B

Figure 4-51:  Advanced BF vs. Basic Open Loop Comparison, Model D

Figure 4-52:  Advanced BF vs. Basic Open Loop Comparison, Model E

Figure 4-53:  Comparison of Basic and Advanced BF, 2x2 - Model B

Figure 4-54:  Comparison of Basic and Advanced BF, 4x2 - Model B

Figure 4-55:  Comparison of Basic and Advanced BF, 2x2 - Model D

Figure 4-56:  Comparison of Basic and Advanced BF, 4x2 - Model D

Figure 4-57:  Comparison of Basic and Advanced BF, 2x2 - Model E

Figure 4-58:  Comparison of Basic and Advanced BF, 4x2 - Model E

### Advanced Coding Simulations

In this subsection we examine the impact of the TGn Sync advanced coding options on throughput via comparison for the 2x2 – 20 MHz configuration.

The Reed-Solomon code is a (200,220) code with 8 bit symbols.  This code is applied on top of the convolutional code.  One impact of RS outer coding is that it reduces the overall rate of the concatenated coding scheme.  Thus, all else equal, the peak data rate with RS outer coding is reduced by 10%.

The LDPC results shown here are based on a (3360,3840) mother LDPC code having code rate 7/8.  This code is shortened to produce rate 1/2, 2/3 and 3/4 versions to match the MCS definitions.

Figure 4-59 through Figure 4-65 show throughput comparisons for channel Models B, D and E, NLOS.  For each model two cases are presented: the unconstrained case and the PER < 2% constraint case.  In the constrained case, as the simulation estimates PER(H,MCS), the selection algorithm does not select an MCS that violates the PER constraint.

The PER constraint case is important for the following reason.  There is clearly a trade-off between maximizing throughput and PER.  Several TGn applications may drive the desire to operate with guaranteed low PHY PER rates.  This is the case for high throughput QoS applications such as video distribution and gaming.  These applications demand very low packet loss rates within a delay constraint.  Low packet loss rates can be achieved via re-transmissions; however, the delay constraint limits the allowable amount of re-transmission.  Thus, operating at low PHY PERs may be required in order to achieve stringent QoS requirements.

The gain of any advanced coding technique will be more pronounced when the system is required to operate at low PERs.  Thus, we should expect the throughput gains of advanced coding to be increased when the PER constraint is applied.  Indeed, this is the case.

Figure 4-65, Figure 4-66 and Figure 4-67 show the resulting PERs with and without the PER < 2% constraint.  It is clear that the constraint yields a PER substantially less than 2%.  The reason is that when the unconstrained maximum throughput MCS has a PER that is greater than 2% (which is usually the case), the selection algorithm is forced to select a more conservative MCS.  It is the PER of the more conservative alternative that is reflected in these constrained PER results.  Thus, the PER threshold is a parameter for controlling PER, and not the actual average PER that is achieved.

Table 17 summarizes the gains at various throughputs.  LDPC consistently shows a 3-3.5 dB gain.  Keeping in mind that Reed-Solomon adds very little complexity, we see that Reed-Solomon yields respectable gains below 80 Mbp.  Due to the 10% reduction in peak rate, the Reed-Solomon curves cross the baseline convolutional only curves at about 100 Mbps.

The true power of LDPC is not actually captured in Table 17.  The 2x2 and 2x3 configurations with 20 MHz channels will achieve the 100 Mbps top-of-MAC goal only when operating at their peak data rates.  As seen in the previous subsection, this requires rather high SNRs.  Even the 2x3 configuration generally requires 30 dB SNR in order to achieve its peak rate.  This has very serious implications on higher RF costs.  The LDPC results of this subsection, however, show a substantially reduced SNR threshold required to achieve peak throughput.

Table 17:  Advanced Coding Gains

|  | RS | LDPC | RS* | LDPC* |
| --- | --- | --- | --- | --- |
| 60 Mbps |  |  |  |  |
| Model B | 1.00 | 2.25 | 1.50 | 3.00 |
| Model D | 1.75 | 3.50 | 1.50 | 3.50 |
| Model E | 0.75 | 2.50 | 1.00 | 3.25 |
| 80 Mbps |  |  |  |  |
| Model B | 0.75 | 2.50 | 1.25 | 3.25 |
| Model D | 1.50 | 3.00 | 2.00 | 3.50 |
| Model E | 1.50 | 3.00 | 2.00 | 3.50 |
| 100 Mbps |  |  |  |  |
| Model B | 0.00 | 3.00 | 0.00 | 4.00 |
| Model D | 0.50 | 3.50 | 0.75 | 3.75 |
| Model E | 0.00 | 3.50 | 0.25 | 4.50 |

* PER < 2%.

Figure 4-59:  Advanced coding comparison, 2x2 Model B NLOS, no constraint.

Figure 4-60:  Advanced coding comparison, 2x2 Model B NLOS, PER < 2%.

Figure 4-61:  Advanced coding comparison, 2x2 Model D NLOS, no constraint.

Figure 4-62:  Advanced coding comparison, 2x2 Model D NLOS, PER < 2%.

Figure 4-63:  Advanced coding comparison, 2x2 Model E NLOS, no constraint.

Figure 4-64:  Advanced coding comparison, 2x2 Model E NLOS, PER < 2%.

Figure 4-65:  PER for Model B advanced coding simulations.

Figure 4-66:  PER for Model D advanced coding simulations.

Figure 4-67:  PER for Model E advanced coding simulations.

## Throughput Simulations for Beamforming with Impairments

This subsection provides additional throughput simulations to compare advanced beamforming with Basic MIMO.  These simulation were performed using the PHY-1 simulator of Section 3.1.  Simulating condition and the parameters of simulator is not identical to the simulator description in section 3.1, and different points are as follows.  These are not CC67 compliant simulation.

- IM1, IM2 and IM4 are not included.

- Ideal packet detection and timing offset compensation is assumed.

- Link adaptation algorithm

  - We implemented our original link adaptation algorithm.

  - It is not ideal one. It could be used in actual implementation.

  - Link adaptation uses MCS set the shown in Table 3-3, without restriction, as far as the number of spatial streams is equal to or less than the number of Tx antennas.

- Channel Seed

  - We used 100x30 PPDUs for showing rough comparison, fewer than required by CC67.

Figure 4-68 : Channel-B (nLOS) Throughput, 2x2 : 2x3 : 3x2

Figure 4-69 : Channel-B (nLOS)  Throughput, 4x4

Figure 4-70 : Channel-D (nLOS) Throughput, 2x2 : 2x3 : 3x2

Figure 4-71 : Channel-D (nLOS) Throughput, 4x4

Figure 4-72 : Channel-E (nLOS) Throughput, , 2x2 : 2x3 : 3x2

Figure 4-73 : Channel-E (nLOS) Throughput, 4x4