IEEE P802.11
Wireless LANs

- TGn Sync Proposal MAC Results

Date:	August 13, 2004

Author:	Syed Aon Mujtaba
	Agere Systems

555 Union Boulevard

Allentown, Pennsylvania 18109, U.S.A.
	Phone: +1 610 712 6616

Cell: +1 908 342 5251
	e-Mail: mujtaba@agere.com

Additional Authors:

Adrian P Stephens, Intel Corporation, (adrian.p.stephens@intel.com)

Andrew Myles, Cisco Corporation (amyles@cisco.com)

Aon Mujtaba, Agere Systems Inc, (mujtaba@agere.com)

Brian Johnston, Nortel Networks Corporation, (brjohnso@nortelnetworks.com)

Daisuke Takeda, Toshiba Corporation, (daisuke.takeda@toshiba.co.jp)

Darren McNamara, Toshiba Corporation, (Darren.McNamara@toshiba-trel.com)

Dongjun (DJ) Lee, Samsung Electronics Co. Ltd., (djthekid.lee@samsung.com)

David Bagby, Calypso Consulting, (david.bagby@ieee.org)

Dmitry Akhmetov, Intel Corporation, (Dmitry.akhmetov@intel.com)

Eldad Perahia, Cisco Corporation, (eperahia@cisco.com)

Huanchun Ye, Atheros Communications Inc., (hcye@atheros.com)

Jan Boer, Agere Systems Inc., (janboer@agere.com)

Jari Jokela, Nokia, (jari.jokela@nokia.com)

Jeff Gilbert, Atheros Communications Inc., (gilbertj@atheros.com)

Joe Pitarresi, Intel Corporation, (joe.pitarresi@intel.com)

Jörg Habetha, Royal Philips Electronics, (joerg.habetha@philips.com)

John Sadowsky, Intel Corporation, (john.sadowsky@intel.com)

Jon Rosdahl, Samsung Electronics Co. Ltd., (jon.rosdahl@partner.samsung.com)

Luke Qian, Cisco Corporation, (lchia@cisco.com)

Masahiro Takagi, Toshiba Corporation, (masahiro3.takagi@toshiba.co.jp)

Monisha Ghosh, Royal Philips Electronics, (monisha.ghosh@philips.com)

Nico van Waes, Nokia, (nico.vanwaes@nokia.com)

Osama Aboul-Magd, Nortel Networks Corporation, (osama@nortelnetworks.com)

Paul Feinberg, Sony Corporation, (paul.feinberg@am.sony.com)

Pen Li, Royal Philips Electronics, (pen.li@philips.com)

Pieter-Paul Giesberts, Agere Systems Inc., (pgiesberts@agere.com)

Richard van Leeuwen, Agere Systems Inc., (rleeuwen@agere.com)

Ronald Rietman, Royal Philips Electronics, (ronald.rietman@philips.com)

Seigo Nakao, SANYO Electric Co. Ltd., (snakao@gf.hm.rd.sanyo.co.jp)

Sergey Shtin, Intel Corporation, (Sergey.shtin@intel.com)

Sheung Li, Atheros Communications Inc., (sheung@atheros.com)

Steve Shellhammer, Intel Corporation (steven.j.shellhammer@intel.com)

Takushi Kunihiro, Sony Corporation, (kuni@wcs.sony.co.jp)

Teik-Kheong (TK) Tan, Royal Philips Electronics, (tktan@philips.com)

Tomoko Adachi, Toshiba Corporation, (tomo.adachi@toshiba.co.jp)

Tomoya Yamaura, Sony Corporation, (yamaura@wcs.sony.co.jp)

Tsuguhide Aoki, Toshiba Corporation, (tsuguhide.aoki@toshiba.co.jp)

Won-Joon Choi, Atheros Communications Inc., (wjchoi@atheros.com)

Xiaowen Wang, Agere Systems Inc., (xiaowenw@agere.com)

Yasuhiko Tanabe, Toshiba Corporation, (yasuhiko.tanabe@toshiba.co.jp)

Yasuhiro Tanaka, SANYO Electric Co. Ltd., (y_tanaka@gf.hm.rd.sanyo.co.jp)

Yoshiharu Doi, SANYO Electric Co. Ltd., (doi@gf.hm.rd.sanyo.co.jp)

Yuichi Morioka, Sony Corporation, (morioka@wcs.sony.co.jp)

- Abstract

This document contains results of the TGn Sync Proposal for the MAC portions of the comparison criteria to IEEE 802.11 TGn.

# Introduction

## Purpose of this Document

This document contains MAC simulation results for the "TGn Sync" proposal to IEEE 802.11 TGn in compliance to the TGn call for proposals.

The TGn comparison criteria call for full disclosure of results and conditions.  We present the results at varying levels of detail in the following documents.  The most summarised form is found in the presentation material [5].  This document contains all of the mandatory results – except per-flow results. Documents [3] and [4] contain the detailed per-flow results and parameter settings as well as results for optional simulation scenarios.

## Simulation Methodology

Document [1] describes in full the methodology used to obtain this result.  This section contains a very brief summary of the methodologies.

Results were obtained using two independent MAC simulations that are called MAC1 and MAC2 in this document.

The MAC1 simulation is based on the commercial Opnet ** simulation tool, with a substantially re-written MAC process and PHY pipeline.  The PHY model built into the pipeline is based on PER vs. post-detection capacity.  The simulation uses the LongNAV method of protection (a MAC-layer technique described in [2, section 8.1.7.1]).

The MAC2 simulation is a discrete event-based simulator written in "C".  The PHY model is based on PER vs. SNR curves.  The simulation supports three protection methods:  LongNAV and pairwise spoofing (described in [1]).  Results reported here from that model use the pairwise spoofing protection method.

### Model Credits

The following people created the models and provided the PHY simulation results embodied in the MAC simulations.

| Model | MAC Model | PHY Curves |
| --- | --- | --- |
| MAC1 | Dmitry Akhmetov, Intel Sergey Shtin, Intel Adrian Stephens, Intel | John Sadowsky, Intel |
| MAC2 | Yuichi Morioka, Sony Kenzoh Nishikawa, Sony Kazuyuki Sakoda, Sony | Darren McNamarra, Toshiba |

## References

[1] IEEE 802.11-04/895, "TGnSync Proposal MAC Simulation Methodology"

[2] IEEE 802.11-04/889, "TGnSync Proposal Technical Specification"

[3] IEEE 802.11-04/893, "TGnSync Proposal MAC1 Simulation Results"

[4] IEEE 802.11-04/894, "TGnSync Proposal MAC2 Simulation Results"

[5] IEEE 802.11-04/888, "TGnSync Proposal"

# Results for Mandatory Comparison Criteria and FR

## Summary of Configurations

### MAC1 2x2x20

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 20 MHz |
| PHY Options enabled | None |
| TCP Model | Reno |
| Protection Method | LongNAV |
| Channel Access Method | Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs |
| QoS parameter Optimization | An HCCA schedule was created manually that satisfied the QoS constraints. |

### MAC1 2x2x40

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 40 MHz |
| PHY Options enabled | None |
| TCP Model | Reno |
| Protection Method | LongNAV |
| Channel Access Method | Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs |
| QoS parameter Optimization | An HCCA schedule was created manually that satisfied the QoS constraints. |

### MAC2 2x2x40

| Parameter | Value |
| --- | --- |
| Number of Tx Antennae | 2 |
| Number of Rx Antennae | 2 |
| Nominal Channel Width | 40 MHz |
| PHY Options enabled | None |
| TCP Model | New Reno |
| ProtectionMethod | Pairwise Spoofing Note, comparative results for the LongNAV are shown in section 4.1 below. |
| Channel access method | EDCA |
| QoS Parameter Optimization | None. The default 802.11e parameters were used, mimicking the effect of a "dumb" QAP. |

## Results relating to MAC simulations

Note, the channel access mechanism used to obtain the results is indicated with these colours:

| Optimised HCCA for QoS flows and EDCA for non-QoS flows mimicking the use of TSPECs | Default parameter EDCA |
| --- | --- |

Add reference for per-flow data in CC18 and CC19.

| CC# | Name | Result |  | MAC1 |  | MAC2 2x2x40 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2x2x20 | 2x2x40 |  |
| CC3 | List of goodput results for usage models 1, 4 and 6. | SS1 (Mbps) |  | 55.2 | 76.8 | 47.7 |
|  |  | SS4 |  | 45.1 | 74.1 | 30.9 |
|  |  | SS6 |  | 44.9 | 62.1 | 12.0 |
| CC15 | Sharing of medium with legacy devices | T1 (Mbps) |  | 90.3 | n/a |  |
|  |  | T2 (Mbps) |  | 35.2 | n/a |  |
|  |  | T3 (Mbps) |  | 42.4 | n/a |  |
|  |  | T4 (Mbps) |  | 20.5 | n/a |  |
| CC18 | HT Usage Models Supported (non QoS) | SS1 (Mbps/ratio) |  | 2.76/0.09 Refer to [3] for per-flow results | 24.4/0.8 Refer to [3] for per-flow results | 10/0.32 Refer to [4] for per-flow results |
|  |  | SS4 |  | 36.0/0.07 Refer to [3] for per-flow results | 65.0/0.14 Refer to [3] for per-flow results | 25.9/0.06 Refer to [4] for per-flow results |
|  |  | SS6 |  | 0.1/0.005 Refer to [3] for per-flow results | 17.24/0.86 Refer to [3] for per-flow results | 0.25/0.01 Refer to [4] for per-flow results |
| CC19 | HT Usage Models Supported (QoS) | SS1 |  | 17/17 Refer to [3] for per-flow results | 17/17 Refer to [3] for per-flow results | 13/17 Refer to [4] for per-flow results |
|  |  | SS4 |  | 18/18 Refer to [3] for per-flow results | 18/18 Refer to [3] for per-flow results | 5/18 Refer to [4] for per-flow results |
|  |  | SS6 |  | 39/39 Refer to [3] for per-flow results | 39/39 Refer to [3] for per-flow results | 3/39 Refer to [4] for per-flow results |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | SS1 | M1 | 55.2 | 76.8 | 47.7 |
|  |  |  | M2 | 55.2 | 76.8 | 33.0 |
|  |  |  | M3 | 55.2 | 76.8 | 22.6 |
|  |  | SS4 | M1 | 45.1 | 74.1 | 30.9 |
|  |  |  | M2 | 45.1 | 74.1 | 4.8 |
|  |  |  | M3 | 45.1 | 74.1 | 3.0 |
|  |  | SS6 | M1 | 44.9 | 62.1 | 12.0 |
|  |  |  | M2 | 44.9 | 62.1 | 5.3 |
|  |  |  | M3 | 44.9 | 62.1 | 0.3 |
| CC24 | MAC Efficiency | SS1 |  | 0.53 | 0.53 | 0.23 |
|  |  | SS4 |  | 0.42 | 0.32 | 0.03 |
|  |  | SS6 |  | 0.44 | 0.38 | 0.07 |
| CC27 | Throughput / Range |  |  | n/a | See section 2.4 |  |
| CC28 | Throughput / Range in 20MHz |  |  | See section 2.5 | n/a |  |
| CC58 | HT Spectral Efficiency | bps/Hz |  | From the FR2 curve, goodput is ~100 Mbps at a PSDU PHY rate of 108 Mbps for channel Models B and D. Spectral Efficiency = 108/20 = 5.4 bps/Hz (for channel models B and D) | From CC27, goodput is 216Mbps at a PSDU PHY rate of 243Mbps for Channel Models B and D. Spectral Efficiency = 243/40 = 6.075 bps/Hz (for channel models B and D) |  |

## MAC Comparison Criteria not related to simulation results

| CC# | Name | Disclosure |
| --- | --- | --- |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Backwards compatibility is provided at the PHY level [2, section 12.3.1.2] essentially by retaining the legacy short and long training fields and the legacy signal field. Backward compatibility is managed within the MAC through the definition of an operating mode [2, section 9.1.1] and rules [2, sections 9.1.3 and 9.1.4] for operation in each operating mode. Protection mechanisms are defined using both PHY-level ("spoofing") [2, section 8.1.7.2] and MAC-level techniques [2, section 8.1.7.1]. |
| CC46 | MAC Compatibility and parameters. | See section 2.6 |
| CC47 | MAC extensions | See section 2.7 |

## CC27

Figure 1 shows results for CC27 from the MAC1 simulation, truncated at 100m range for Channel B and Channel D.

The MAC and PHY parameters used are defined in [3, CC27 tab].   Any parameters (e.g. SIFS spacings and slot timings) not specified there take on the 802.11a values.

Figure 1 - CC27 Results

## CC28

Figure 2 shows results for CC28 from the MAC1 simulation, truncated at 100m range for Channel B and Channel D.

The MAC and PHY parameters used are defined in [3, CC28 tab]. Any parameters (e.g. SIFS spacings and slot timings) not specified there take on the 802.11a values.

Figure 2 - CC28 Results

## CC46 – Baseline options

"Provide a list of optional features of 802.11a/b/d/e/g/h/i/j that are required for HT operation, and a summary description of the manner in which they are used.  Include references to the sections in the technical proposal document where the complete details are given."

IEEE 802.11e

| Item | Feature | Brief Description | Sections in [2] |
| --- | --- | --- | --- |
| CF12 | QoS Supported | In addition to the QoS features described in IEEE 802.11e drafts, The TGn Sync MAC proposal introduces TSPEC renegotiation and TSPEC Packet Loss Priority (PLP) operation | 8.4 |
| QB4 | Block Acknowledgement | TGn Sync MAC proposal requires support for Block Acknowledgment in the context of frame aggregation | 8.1 |
| QB5 | Automatic Power Save Delivery | Modified in the context of received aggregate PPDU | 8.1.6 |
| QD3 | Continuation of EDCA TXOP Support | Under LongNAV rules, a TXOP can be truncated by use of CF-END | 8.1.7.1.2 |

## CC47

"Provide a summary description of MAC extensions beyond 802.11a/b/d/e/g/h/i/j that are required for HT operation. Include references to the sections in the technical proposal document where the complete details are given."

| Feature | Brief Description | Section in [2] |
| --- | --- | --- |
| Frame Aggregation | Allows the transmission of multiple MSDU in a single PSDU | 7.1.1, 7.1.2, 7.4, 8.1 |
| Bi-directional data flow | Allows the aggregation of control MPDUs in one direction with Data MPDUs in the reverse direction under control of the TXOP owner | 8.1.10 |
| Multiple Receiver Frame Aggregation | Allows aggregated frame to be destined to different receivers | 7.1.3, 8.1.11 |
| Compressed Header | Compresses redundant information in Data MPDU headers within the same aggregate | 7.2.2 |
| MAC Protection | Three additional procedures for MAC protection, Long NAV, Pair wise spoofing, and Single Ended spoofing | 8.1.7 |
| Coexistence Mechanisms | Definition of a BSS operating mode (pure, mixed, managed mixed, 20-base managed mixed) for a BSS based on its environment and rules for legacy protection and management of channel width. | 9.1 |
| MAC Support for Closed Loop Link adaptation | Allows the adaptation of modulation coding scheme according to channel conditions | 8.1.8 |
| Power Saving | Reduction of the number of enabled receive chains in order to save power with a hold-on timer that keeps a STA in full capability mode for a period of time known to its peers. | 9.3 |

## FR1

"Demonstrate at least one set of conditions under which 100 Mbps at the top of the MAC SAP can be achieved.  Provide all relevant information to document this."

In 2x2x40 mode of operation, the MAC1 simulation demonstrates a goodput of 216Mbps for SS16 using the simulation methodology described in [1] and the MAC and PHY parameters reported in [3,  "SS16" tab].

## FR2

"Proposal supports at least one mode of operation that supports 100Mbps throughput at the top of the MAC SAP in a 20MHz channel. Provide all relevant information to document this."

The mandatory minimum PHY mode of operation is 2x2, no Short guard interval, no 7/8 coding, no 256 QAM.

This corresponds to 108Mbps instantaneous rate at the PHY.

A maximum rate of 100.4. Mbps is reported in [3, "100Mbps" tab]. The curve crosses 100Mbps at about 6m range.

This is obtained under the conditions specified in CC28 with the following modifications:

| Condition | Value |
| --- | --- |
| MSDU size | 1500 |
| Offered Load | Fully saturated |
| Basic MCS | 16-QAM, r=1/2 |

Note, the difference in "basic MCS" is the main effect here, reducing overhead for the LongNAV initial IAC/RAC exchange.

Figure 3 - FR2 result

# Required Additional Disclosures

| Number | Name | Disclosure |
| --- | --- | --- |
| AD2 | TCP Model Parameters | Refer to [3, "Common" tab]. Refer to [1, section 3.1] (MAC2 tcp parameters) |
| AD3 | MAC simulation methodology | This is described in [1], please refer to [1]. |
| AD4 | MAC simulation occupied channel width | For nxnx40 results, a nominal channel width of 40MHz was occupied. For nxnx20 results, a nominal channel width of 20MHz was occupied. |
| AD5 | Justification of low PLR rates achieved | This disclosure relates specifically to the MAC1 2x2x40 simulation. MAC1 simulation targets < 10% PER at the top of the PHY. The fast link adaptation method described in [1, section 2.2.4] achieves the performance shown in [1, section 5]. Typically PER is < 1% except at extreme range. Assuming a PER of 1%, the specified residual PLR of 10-7 can be reached by permitting 4 transmission attempts (3 retries). Assuming uncorrelated errors, these retry attempts will include (with high probability) only a single Data MPDU. At the PHY rates observed in simulation for the Video flows (average 230Mbps), a single Data MPDU / BAR / BA retry exchange takes ~300us. 3 retries takes ~1ms. The average transport delay observed with the Video traffic is <30ms. Attributing this entirely to channel access delay, 3 retries in a separate TXOP would take ~31ms, which is comfortably less than the 200ms delay bound. |

# Characterisation of MAC Options

## Protection Mechanisms

Selected CC results are presented from the MAC2 simulation to show the relative performances of the two protection modes described in [1] relating to the MAC2 simulation.

| CC# | Result |  | MAC2 2x2x40 LongNav | MAC2 2x2x40 Pairwise Spoofing |
| --- | --- | --- | --- | --- |
| CC3 | SS1 (Mbps) |  | 43.7 | 47.7 |
|  | SS4 |  | 29.2 | 30.9 |
|  | SS6 |  | 11.3 | 12.0 |
| CC18 | SS1 (Mbps/ratio) |  | 2.6/0.08 | 10/0.32 |
|  | SS4 |  | 24.1/0.05 | 25.9/0.06 |
|  | SS6 |  | 0.25/0.01 | 0.25/0.01 |
| CC19 | SS1 |  | 12/17 | 13/17 |
|  | SS4 |  | 3/18 | 5/18 |
|  | SS6 |  | 2/39 | 3/39 |
| CC20 | SS1 | M1 | 43.7 | 47.7 |
|  |  | M2 | 33.6 | 33.0 |
|  |  | M3 | 4.45 | 22.6 |
|  | SS4 | M1 | 29.2 | 30.9 |
|  |  | M2 | 4.8 | 4.8 |
|  |  | M3 | 2.5 | 3.0 |
|  | SS6 | M1 | 11.3 | 12.0 |
|  |  | M2 | 4.9 | 5.3 |
|  |  | M3 | 0.2 | 0.3 |
| CC24 | SS1 |  | 0.23 | 0.23 |
|  | SS4 |  | 0.04 | 0.03 |
|  | SS6 |  | 0.08 | 0.07 |

** Other names and brands may be claimed as the property of others.

Note that the PHY model used for this simulation (results T1-T4) was the same as that used for the other MAC simulations.  As such it included impairments IM1, IM5 and IM6.  This is not expected to significantly affect these results.